3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220039

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2263 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_NR5GC_EmergencyCallRelease |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-12-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementation of this function, TTCN expects a PDU Session Release REQ from the UE before calling the functions f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]) and f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); This is incorrect, as unless this coordination message is sent, the IMS PTC will not trigger call release at IMS level, and thus no PDU Session Release REQ will arrive during the alt stament handling. Also waiting for the coordination message from IMS PTC is before the expected PDU Session Release REQ, which will cause problem. As soon as the call is released at IMS level, the UE may transmit a PDU Session Release REQ, which can come before the IMS PTC sends the coordination message to NR PTC that call is released. This will cause testcase failure This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Moved the call to f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); BEFORE the alt statement. This will enable call release at IMS level, and then start the timer for expected PDU Session Release REQ. Also, some UEs may not send the PDU Session Release REQ but may release the call at IMS level, and then send the coordination message to NR PTC within 5s. This also needs to be handled. Basically, reworked the Alt statement handling to handle both cases. Please see below for detailed change description |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	4

1	Corrections required	4

1.1	Correction to function f_NR5GC_EmergencyCallRelease	4

# Corrections required

## Correction to function f_NR5GC_EmergencyCallRelease

| Function name | f_NR5GC_EmergencyCallRelease |
| --- | --- |
| Reason for change | In the current TTCN implementation of this function, TTCN expects a PDU Session Release REQ from the UE before calling the functions f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]) and f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); This is incorrect, as unless this coordination message is sent, the IMS PTC will not trigger call release at IMS level, and thus no PDU Session Release REQ will arrive during the alt stament handling. Also waiting for the coordination message from IMS PTC is before the expected PDU Session Release REQ, which will cause problem. As soon as the call is released at IMS level, the UE may transmit a PDU Session Release REQ, which can come before the IMS PTC sends the coordination message to NR PTC that call is released. This will cause testcase failure This needs to be adressed. |
| Summary of change | Moved the call to f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); BEFORE the alt statement. This will enable call release at IMS level, and then start the timer for expected PDU Session Release REQ. Also, some UEs may not send the PDU Session Release REQ but may release the call at IMS level, and then send the coordination message to NR PTC within 5s. This also needs to be handled. Basically, reworked the Alt statement handling to handle both cases. Please see below for detailed change description |
| TTCN module | NR5GC_IMSProcedures.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_NR5GC_EmergencyCallRelease(NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Emergency_PDN); // @sic R5s210363 sic@ var GSM_MobilityInfo_Type v_PDUSession := valueof(v_GSM_MobilityInfo); // Must be there as previously established it var O1_Type v_PDUSessionId := v_PDUSession.SessionId; timer t_Wait := 5.0; t_Wait.start; alt { // @sic R5-216829 sic@ [] a_NR5GC_PDUSessionReleaseReq (p_NR_CellId, v_PDUSessionId) // @sic R5s211524 sic@ { t_Wait.stop } [] t_Wait.timeout {} } f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); f_NR5GC_PDUSessionRelease (p_NR_CellId, v_PDUSessionId, cs_GMM_GSM_Cause (omit, '00100100'B)); f_IP_RtpRtcpLoopback(IP, v_PDUSession.PdnIndex, tsc_IMS_MediaPort_M1, omit); // @sic R5s210363 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Emergency Call Release"); }

After Change:

function f_NR5GC_EmergencyCallRelease(NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Emergency_PDN); // @sic R5s210363 sic@ var GSM_MobilityInfo_Type v_PDUSession := valueof(v_GSM_MobilityInfo); // Must be there as previously established it var O1_Type v_PDUSessionId := v_PDUSession.SessionId; timer t_Wait := 5.0; f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]);//WA#11_4_4 t_Wait.start; alt { // @sic R5-216829 sic@ [] a_NR5GC_PDUSessionReleaseReq (p_NR_CellId, v_PDUSessionId) // @sic R5s211524 sic@ { t_Wait.stop; f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]);//WA#11_4_4 } [] t_Wait.timeout { f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]);//WA#11_4_4 } [] IMS[tsc_Index_IMS2].receive(cmr_IMS_IPCAN_Trigger) { t_Wait.stop; } //WA#11_4_4 } //WA#11_4_4 MOVED f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); //COMMENTED OUT f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); f_NR5GC_PDUSessionRelease (p_NR_CellId, v_PDUSessionId, cs_GMM_GSM_Cause (omit, '00100100'B)); f_IP_RtpRtcpLoopback(IP, v_PDUSession.PdnIndex, tsc_IMS_MediaPort_M1, omit); // @sic R5s210363 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Emergency Call Release"); }