3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220043

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2267 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 7.1.1.4.2.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-12-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Accoring to 38.214 sec 5.1.3.1 when qam256 is configured the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and in Table 5.1.3.1-2 MCS is defined upto 27 and the rest are marked as reserved. The changes were proposed as part of R5s211564 and R5s211566 and accepted, but not implemented in 21wk49 |  |  |  |  |  |  |  |  |
| Summary of change: |  | In f_TC_7_1_1_4_2_x_DCI0_1_NR_TestBody value 27 is passed as the max mcs for this test case when resourceAllocationType0 is testesd |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test execution will result in a runtime error due to incorrect max mcs value |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.4.2.4.ENDC, 7.1.1.4.2.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required for test case 7.1.1.4.2.4	2

1.1	Correction to f_TC_7_1_1_4_2_4_ENDC_NR	2

1.2	Correction to f_TC_7_1_1_4_2_4_NR5GC	6

# Corrections required for test case 7.1.1.4.2.4

## Correction to f_TC_7_1_1_4_2_4_ENDC_NR

| Function name | f_TC_7_1_1_4_2_4_ENDC_NR |
| --- | --- |
| Reason for change | Accoring to 38.214 sec 5.1.3.1 when qam256 is configured the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and in Table 5.1.3.1-2 MCS is defined upto 27 and the rest are marked as reserved. The changes were proposed as part of R5s211566 and accepted, but not implemented in 21wk49 |
| Summary of change | In f_TC_7_1_1_4_2_x_DCI0_1_NR_TestBody value 27 is passed as the max mcs for this test case when resourceAllocationType0 is testesd |
| TTCN module | MAC_ENDC_NR |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_7_1_1_4_2_4_ENDC_NR() runs on ENDC_NR_PTC { // 7.1.1.4.2.4 UL-SCH transport block size selection / DCI format 0_1 / RA type 0/RA Type 1 / 256QAM / Transform precoding disabled var template (value) NR_TBS_CalculationParameters_Type v_CalculationParameters := cs_NR_TBS_CalculationParameters_711424; //@sic R5s202610 sic@ var template (value) CellGroupConfig v_SCGConfig; var template(value) SpCellConfig v_SpCellConfig ; var template (value) NR_UplinkBWP_List_Type v_NR_UplinkBWP_List ; //Init Cell parameters f_ENDC_NR_Init(); if (f_NR_CellInfo_GetIsFR1(nr_Cell1)) { v_CalculationParameters.MaxNoOfDmrsAddPos := 0; } f_NR_Init_7_1_1_4_2_x(nr_Cell1, formats0_1_And_1_1, qam256, omit); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_Only); //Configure on SS the SCG DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode f_NR_ENDC_ConfigAM_UM(nr_Cell1, 1, 0); // Wait for Trigger from EUTRA indicating start of Test Body f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); f_NR_TestBody_Set(true); //@siclog Steps 1-5 siclog@ f_TC_7_1_1_4_2_x_DCI0_1_NR_TestBody(tsc_NR_DRB2, v_CalculationParameters,resourceAllocationType1, 27);//@sic R5s211566 sic@ if (pc_ra_Type0_PUSCH) { if ( not pc_dynamicSwitchRA_Type0_1_PUSCH ) { //@siclog Steps 5aA1-5aa2 siclog@ f_NR_Set_PUSCH_ResourceAllocation(nr_Cell1, resourceAllocationType0); v_NR_UplinkBWP_List := f_NR_CellInfo_GetUplinkBWP_List(nr_Cell1); v_SpCellConfig := f_NR_GetReconfigurationWithSyncDef_MCG(nr_Cell1); v_SCGConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_SCG , omit, omit, omit, omit, v_SpCellConfig //SpCellConfig ); f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_SCGConfig,-,-,-,reconfigurationWithSync_CheckRACH); // Now send octetstrings to EUTRA f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_UplinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_UplinkBWP_List)); } //@siclog Steps 6-10 siclog@ f_TC_7_1_1_4_2_x_DCI0_1_NR_TestBody(tsc_NR_DRB2, v_CalculationParameters,resourceAllocationType0); } f_NR_TestBody_Set(false); // Send Trigger to EUTRA indicating Finish of Test Body f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); }

After Change:

function f_TC_7_1_1_4_2_4_ENDC_NR() runs on ENDC_NR_PTC { // 7.1.1.4.2.4 UL-SCH transport block size selection / DCI format 0_1 / RA type 0/RA Type 1 / 256QAM / Transform precoding disabled var template (value) NR_TBS_CalculationParameters_Type v_CalculationParameters := cs_NR_TBS_CalculationParameters_711424; //@sic R5s202610 sic@ var template (value) CellGroupConfig v_SCGConfig; var template(value) SpCellConfig v_SpCellConfig ; var template (value) NR_UplinkBWP_List_Type v_NR_UplinkBWP_List ; //Init Cell parameters f_ENDC_NR_Init(); if (f_NR_CellInfo_GetIsFR1(nr_Cell1)) { v_CalculationParameters.MaxNoOfDmrsAddPos := 0; } f_NR_Init_7_1_1_4_2_x(nr_Cell1, formats0_1_And_1_1, qam256, omit); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_Only); //Configure on SS the SCG DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode f_NR_ENDC_ConfigAM_UM(nr_Cell1, 1, 0); // Wait for Trigger from EUTRA indicating start of Test Body f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); f_NR_TestBody_Set(true); //@siclog Steps 1-5 siclog@ f_TC_7_1_1_4_2_x_DCI0_1_NR_TestBody(tsc_NR_DRB2, v_CalculationParameters,resourceAllocationType1, 27);//@sic R5s211566 sic@ if (pc_ra_Type0_PUSCH) { if ( not pc_dynamicSwitchRA_Type0_1_PUSCH ) { //@siclog Steps 5aA1-5aa2 siclog@ f_NR_Set_PUSCH_ResourceAllocation(nr_Cell1, resourceAllocationType0); v_NR_UplinkBWP_List := f_NR_CellInfo_GetUplinkBWP_List(nr_Cell1); v_SpCellConfig := f_NR_GetReconfigurationWithSyncDef_MCG(nr_Cell1); v_SCGConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_SCG , omit, omit, omit, omit, v_SpCellConfig //SpCellConfig ); f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_SCGConfig,-,-,-,reconfigurationWithSync_CheckRACH); // Now send octetstrings to EUTRA f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_UplinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_UplinkBWP_List)); } //@siclog Steps 6-10 siclog@ f_TC_7_1_1_4_2_x_DCI0_1_NR_TestBody(tsc_NR_DRB2, v_CalculationParameters,resourceAllocationType0, 27); } f_NR_TestBody_Set(false); // Send Trigger to EUTRA indicating Finish of Test Body f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); }

## Correction to f_TC_7_1_1_4_2_4_NR5GC

| Function name | f_TC_7_1_1_4_2_4_NR5GC |
| --- | --- |
| Reason for change | Accoring to 38.214 sec 5.1.3.1 when qam256 is configured the UE shall use IMCS and Table 5.1.3.1-2 to determine the modulation order (Qm) and in Table 5.1.3.1-2 MCS is defined upto 27 and the rest are marked as reserved. The changes were proposed as part of R5s211564 and accepted, but not implemented in 21wk49 |
| Summary of change | In f_TC_7_1_1_4_2_x_DCI0_1_NR_TestBody value 27 is passed as the max mcs for this test case when resourceAllocationType0 is testesd |
| TTCN module | MAC_NR5GC |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_7_1_1_4_2_4_NR5GC() runs on NR5GC_PTC { // 7.1.1.4.2.4 UL-SCH transport block size selection / DCI format 0_1 / RA type 0/RA Type 1 / 256QAM / Transform precoding disabled var DRB_Identity v_DRBId; var template (value) NR_TBS_CalculationParameters_Type v_CalculationParameters:= cs_NR_TBS_CalculationParameters_711424; var template (omit) NR_RadioBearerList_Type v_SS_Drb_ConfigList; //@sic R5s202610 sic@ var template (value) CellGroupConfig v_CellGroupConfig; var template(value) SpCellConfig v_SpCellConfig ; var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; var template (value) NR_UplinkBWP_List_Type v_NR_UplinkBWP_List ; //Init Cell parameters f_NR5GC_Init(NR_1); //if ( f_NR_CellInfo_GetIsFR1(nr_Cell1)) { v_CalculationParameters.MaxNoOfDmrsAddPos := 0;}; //@sic R5s200421 sic@ f_NR_Init_7_1_1_4_2_x(nr_Cell1, formats0_1_And_1_1, qam256, omit); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeA_ON); //Retrieve DRB Id of the first PDU session v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, v_DRBId, -, -, -, cs_SDAP_Configuration_None)}; //Configure on SS the DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1, TEST_LOOPModeA_ON, cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, -, //p_IpPduDelayTime -, -, //MAC_CellGroupConfig -, v_SS_Drb_ConfigList ); f_NR_TestBody_Set(true); //@siclog Steps 1-5 siclog@ f_TC_7_1_1_4_2_x_DCI0_1_NR_TestBody(v_DRBId,v_CalculationParameters, resourceAllocationType1,27);//@sic R5s211564 sic@ //@siclog Steps 5aA1-5aA2 siclog@ if (pc_ra_Type0_PUSCH) { if ( not pc_dynamicSwitchRA_Type0_1_PUSCH ) { f_NR_Set_PUSCH_ResourceAllocation(nr_Cell1, resourceAllocationType0); v_NR_UplinkBWP_List := f_NR_CellInfo_GetUplinkBWP_List(nr_Cell1); v_SpCellConfig := f_NR_GetReconfigurationWithSyncDef_MCG(nr_Cell1); v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG , omit, omit, omit, omit, v_SpCellConfig //SpCellConfig ); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext,-,-,-,false); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_UplinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_UplinkBWP_List)); SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)); } //@siclog Steps 6-10 siclog@ f_TC_7_1_1_4_2_x_DCI0_1_NR_TestBody(v_DRBId,v_CalculationParameters,resourceAllocationType0 ); } f_NR_TestBody_Set(false); //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

After Change:

function f_TC_7_1_1_4_2_4_NR5GC() runs on NR5GC_PTC { // 7.1.1.4.2.4 UL-SCH transport block size selection / DCI format 0_1 / RA type 0/RA Type 1 / 256QAM / Transform precoding disabled var DRB_Identity v_DRBId; var template (value) NR_TBS_CalculationParameters_Type v_CalculationParameters:= cs_NR_TBS_CalculationParameters_711424; var template (omit) NR_RadioBearerList_Type v_SS_Drb_ConfigList; //@sic R5s202610 sic@ var template (value) CellGroupConfig v_CellGroupConfig; var template(value) SpCellConfig v_SpCellConfig ; var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; var template (value) NR_UplinkBWP_List_Type v_NR_UplinkBWP_List ; //Init Cell parameters f_NR5GC_Init(NR_1); //if ( f_NR_CellInfo_GetIsFR1(nr_Cell1)) { v_CalculationParameters.MaxNoOfDmrsAddPos := 0;}; //@sic R5s200421 sic@ f_NR_Init_7_1_1_4_2_x(nr_Cell1, formats0_1_And_1_1, qam256, omit); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeA_ON); //Retrieve DRB Id of the first PDU session v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, v_DRBId, -, -, -, cs_SDAP_Configuration_None)}; //Configure on SS the DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1, TEST_LOOPModeA_ON, cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, -, //p_IpPduDelayTime -, -, //MAC_CellGroupConfig -, v_SS_Drb_ConfigList ); f_NR_TestBody_Set(true); //@siclog Steps 1-5 siclog@ f_TC_7_1_1_4_2_x_DCI0_1_NR_TestBody(v_DRBId,v_CalculationParameters, resourceAllocationType1,27);//@sic R5s211564 sic@ //@siclog Steps 5aA1-5aA2 siclog@ if (pc_ra_Type0_PUSCH) { if ( not pc_dynamicSwitchRA_Type0_1_PUSCH ) { f_NR_Set_PUSCH_ResourceAllocation(nr_Cell1, resourceAllocationType0); v_NR_UplinkBWP_List := f_NR_CellInfo_GetUplinkBWP_List(nr_Cell1); v_SpCellConfig := f_NR_GetReconfigurationWithSyncDef_MCG(nr_Cell1); v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG , omit, omit, omit, omit, v_SpCellConfig //SpCellConfig ); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext,-,-,-,false); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_UplinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_UplinkBWP_List)); SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)); } //@siclog Steps 6-10 siclog@ f_TC_7_1_1_4_2_x_DCI0_1_NR_TestBody(v_DRBId,v_CalculationParameters,resourceAllocationType0, 27 ); // } f_NR_TestBody_Set(false); //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };