3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220078

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2289 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR CA test case 8.1.4.1.9.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-12-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add NR CA test case 8.1.4.1.9.1 to the IWD_21wk49 ATS |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 8.1.4.1.9.1 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.1.9.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1 Overview	2

1.1 Verification Test Summary	2

2 Corrections required	3

3 Branches executed	3

4 Execution Log Files	3

4.1 HiSilicon Balong 5000 UE	3

5 References	3

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR CA test case 8.1.4.1.9.1 which is part of the NR5GC test suite in iWD_TTCN3-B2020-09_D21wk49.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Ting Gao

gaoting@starpointcomm.com

## Verification Test Summary

Test Case: 	8.1.4.1.9.1.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D21wk49

System Simulator used:	Starpoint SP9500 Test System

UE used:	HiSilicon Balong 5000

Verification Status:	PASS

# Corrections required

None.

## 2.1	Additional changes from MCC TF160

| Function name | f_TC_8_1_4_1_9_x_NR5GC_TestBody |
| --- | --- |
| Reason for change | The current code is missing the parameter rach-ConfigDedicated in steps 7 and 18. |
| Summary of change | Added rach-ConfigDedicated. |
| TTCN module | RRC_NR_CA_IntraNR_Handover_NR5GC.ttcn |

function f_TC_8_1_4_1_9_x_NR5GC_TestBody(NR_CellId_Type p_PCell,

NR_CellId_Type p_SCell) runs on NR5GC_PTC

{

var template (value) RACH_ConfigGeneric v_RACH_ConfigGeneric;

...

//@siclog "Step 7" siclog@

//The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform inter frequency handover to NR Cell 3/10 and to release SCell NR Cell 3.

v_RACH_ConfigGeneric := f_NR_CellInfo_GetRACH_ConfigGeneric(p_PCell, tsc_NR_BWP_Id);

v_RRCReconfigurationHO := f_NR5GC_RRCReconfiguration_IntraNR_HO(p_SCell,

-,-,-,-,-,-,
                                                                     cs_NR_RACH_ConfigDedicated_Uplink(cs_38508_RACH_ConfigDedicated(v_RACH_ConfigGeneric, 63, f_NR_CellInfo_GetSSB_Index(p_SCell))),

-,-,

{tsc_SCellIndex_1} //SCellToReleaseList

);

SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, v_RRCReconfigurationHO, crs_MacBearerRouting(p_PCell)));     //@sic R5-213516 sic@

...

//@siclog "Step 18" siclog@

//The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform handover to NR Cell 3/10 and to release SCell NR Cell3/10.

SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, v_RRCReconfigurationHO, crs_MacBearerRouting(nr_Cell1)));    //@sic R5-213516 sic@

# Branches executed

## This NR5GC TC was executed in Band combination “CA_n41C” using NEA2/NIA2 algorithms.

# Execution Log Files

## HiSilicon Balong 5000 UE

The HiSi Balong 5000 UE passed this test case on Starpoint SP9500 Test System. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test Case Execution log file: 
\TC_8_1_4_1_9_1_NR5GC_CA_n41C_PASS.spm

- PIXCIT settings used:

\PCT_PICS_PIXIT.sppar

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s220079: Supporting information for agreement of NR CA test case 8.1.4.1.9.1 in FR1 R5s220079 is replaced with new log file including correct IE setting in R5s220220. |
| --- | --- |