3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220088

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4659 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE test case 9.2.1.1.20 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2021-12-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to current TTCN implementation, the SIB2 for Cell I for Step 1 1indicates that access is barred, thus the UE does not camp on this cell at Step 2. After this, at Steps 3 and 4, the SIB2 is changed to defaults, tehreby allowing Access. The UE then performs a successful ATTACH between Steps 8 to 16a. Later at Step 18 it is expected that the UE will not be trying to access Cell I, as the SIB2 content again indicates that the cell is barred. However, some UEs may try to camp on Cell I at this point because they will not be reading the updated SIB2, as the sysinfoValueTag for SIB1 is unchanged. This is in-line with 36.331 cl 5.2.1.3 : Quote SystemInformationBlockType1 (or MasterInformationBlock-NB in NB-IoT) includes a value tag systemInfoValueTag, that indicates if a change has occurred in the SI messages. UEs may use systemInfoValueTag, e.g. upon return from out of coverage, to verify if the previously stored SI messages are still valid Unquote These UEs will simply not read further system information when they detect that the valueTag in SIB1 is identical to the stored system information. Thus, such UEs will still considered the SIB2 broadcast in Steps 3-4 to be valid and camp on Cell I, thus failing the testcase unfairly. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Called function f_EUTRA_CellInfo_IncrementValueTagSI_SIB2(eutra_CellI) to increase the value tag of SI at Step 17. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.1.1.20 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## Change 1

| Function name | fl_TC_9_2_1_1_20_Body() |
| --- | --- |
| Reason for change | According to current TTCN implementation, the SIB2 for Cell I for Step 1 1indicates that access is barred, thus the UE does not camp on this cell at Step 2. After this, at Steps 3 and 4, the SIB2 is changed to defaults, tehreby allowing Access. The UE then performs a successful ATTACH between Steps 8 to 16a. Later at Step 18 it is expected that the UE will not be trying to access Cell I, as the SIB2 content again indicates that the cell is barred. However, some UEs may try to camp on Cell I at this point because they will not be reading the updated SIB2, as the sysinfoValueTag for SIB1 is unchanged. This is in-line with 36.331 cl 5.2.1.3 : Quote SystemInformationBlockType1 (or MasterInformationBlock-NB in NB-IoT) includes a value tag systemInfoValueTag, that indicates if a change has occurred in the SI messages. UEs may use systemInfoValueTag, e.g. upon return from out of coverage, to verify if the previously stored SI messages are still valid Unquote These UEs will simply not read further system information when they detect that the valueTag in SIB1 is identical to the stored system information. Thus, such UEs will still considered the SIB2 broadcast in Steps 3-4 to be valid and camp on Cell I, thus failing the testcase unfairly. This needs to be addressed. |
| Summary of change | Called function f_EUTRA_CellInfo_IncrementValueTagSI_SIB2(eutra_CellI) to increase the value tag of SI at Step 17. |
| TTCN module | NAS_Attach_EOnly.ttcn |
| MCC160 Comment | Accepted |

Before Change

function fl_TC_9_2_1_1_20_Body() runs on EUTRA_PTC

{

…….

//+  The SS set the cell type of cell I to the ''Serving cell'', set the cell type of cell K to the '' Non- Suitable cell'', and set SystemInformationBlockType2 parameters as described below.

//          The UE is switched on.

//@siclog "Step 17" siclog@

f_EUTRA_CellInfo_SetSysInfo_AccessBarring(eutra_CellI, cs_AccessBarringInformation_TemporaryBarring);

f_EUTRA_Cellinfo_SetSysInfo_IntraFreqRes(eutra_CellI, allowed); // @sic R5-104729 sic@

f_EUTRA_ModifySysinfoUE_Off (eutra_CellI);

v_CellPowerList := {

cs_CellPower(eutra_CellK, tsc_NonSuitableCellRS_EPRE),

cs_CellPower(eutra_CellI, tsc_ServingCellRS_EPRE)

};

f_EUTRA_SetCellPowerList(v_CellPowerList);   // @sic R5s110726 additional changes for change 2 sic@

f_EUTRA_SwitchOnUEandStartIP(eutra_CellK, -, true); // @sic R5s120065 sic@ @sic R5s120597r1 sic@

……

}

After change

function fl_TC_9_2_1_1_20_Body() runs on EUTRA_PTC

{

…….

//+  The SS set the cell type of cell I to the ''Serving cell'', set the cell type of cell K to the '' Non- Suitable cell'', and set SystemInformationBlockType2 parameters as described below.

//          The UE is switched on.

//@siclog "Step 17" siclog@

f_EUTRA_CellInfo_SetSysInfo_AccessBarring(eutra_CellI, cs_AccessBarringInformation_TemporaryBarring);

f_EUTRA_Cellinfo_SetSysInfo_IntraFreqRes(eutra_CellI, allowed); // @sic R5-104729 sic@

f_EUTRA_CellInfo_IncrementValueTagSI_SIB2(eutra_CellI);

f_EUTRA_ModifySysinfoUE_Off (eutra_CellI);

v_CellPowerList := {

cs_CellPower(eutra_CellK, tsc_NonSuitableCellRS_EPRE),

cs_CellPower(eutra_CellI, tsc_ServingCellRS_EPRE)

};

f_EUTRA_SetCellPowerList(v_CellPowerList);   // @sic R5s110726 additional changes for change 2 sic@

f_EUTRA_SwitchOnUEandStartIP(eutra_CellK, -, true); // @sic R5s120065 sic@ @sic R5s120597r1 sic@

……

}