3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220089

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2294 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC Logged MDT FR1 test case 8.1.6.1.2.11 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-12-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Cell ID updated as per prose at step 15-22. - UE use SRB2 when it will be having logMeasReport_r16 as per 38.331 cl 5.7.10.3 at step 24. “1> if the logMeasReport is included in the UEInformationResponse: 2> submit the UEInformationResponse message to lower layers for transmission via SRB2; 2> discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers; 1> else: - submit the UEInformationResponse message to lower layers for transmission via SRB1.” - UE can have more than 6 entries as per prose 38.331 cl 5.5a.3.2 at step 24 While T330 is running, the UE shall: 1> perform the logging in accordance with the following: 2> if the reportType is set to periodical in the VarLogMeasConfig: 3> if the UE is in any cell selection state (as specified in TS 38.304 [20]): 4> perform the logging at regular time intervals, as defined by the loggingInterval in the VarLogMeasConfig Thepart highlighted is missing from TS 38.331 v16.3.1. TP2 isbased on the old specification. - For evaluation of each entry for MeasResultServCell 1 and MeasResultServCell 11 there should be or operation at step24. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Cell ID updated to cell 11 - UEInformationResponse reception modified for SRB2. - Removed upper limit for no of logMeasInfoList entried. - Added or operation instead And for checking each entry on MeasResultServCell. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will fail conformant UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.2.11.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.523-1 CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

3	Branches executed	3

4	Execution Log Files	3

4.1	Mediatek MT6983	3

5	References	4

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 8.1.6.1.2.11 which is part of the NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	8.1.6.1.2.11

ATS Version:	iwd-TTCN3-B2020-09_D21wk49

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR,

UE used:	Mediatek MT6983

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_8_1_6_1_2_11_NR5GC_TestBody () |
| --- | --- |
| Reason for change | - Cell ID updated as per prose at step 15-22. - UE use SRB2 when it will be having logMeasReport_r16 as per 38.331 cl 5.7.10.3 at step 24. “1> if the logMeasReport is included in the UEInformationResponse: 2> submit the UEInformationResponse message to lower layers for transmission via SRB2; 2> discard the logged measurement entries included in the logMeasInfoList from VarLogMeasReport upon successful delivery of the UEInformationResponse message confirmed by lower layers; 1> else: - submit the UEInformationResponse message to lower layers for transmission via SRB1.” - UE can have more than 6 entries as per prose 38.331 cl 5.5a.3.2 at step 24 While T330 is running, the UE shall: 1> perform the logging in accordance with the following: 2> if the reportType is set to periodical in the VarLogMeasConfig: 3> if the UE is in any cell selection state (as specified in TS 38.304 [20]): 4> perform the logging at regular time intervals, as defined by the loggingInterval in the VarLogMeasConfig Thepart highlighted is missing from TS 38.331 v16.3.1. TP2 isbased on the old specification. - For evaluation of each entry for MeasResultServCell 1 and MeasResultServCell 11 there should be or operation at step24. |
| Summary of change | - Cell ID updated to cell 11 - UEInformationResponse reception modified for SRB2. - Removed upper limit for no of logMeasInfoList entried. - Added or operation instead And for checking each entry on MeasResultServCell |
| TTCN module | RRC_MDT_IntraNR_Logged_NR5GC.ttcn |
| MCC160 Comment | - Accepted - Accepted - Accepted conditional to prose CR agreement at RAN5#94e - Accepted |

Before Change

function f_TC_8_1_6_1_2_11_NR5GC_TestBody() runs on NR5GC_PTC { var PLMN_Identity v_PlmnID_NRCell1 := f_NR_CellInfo_GetPLMN (nr_Cell1); var PLMN_Identity v_PlmnID_NRCell11 := f_NR_CellInfo_GetPLMN (nr_Cell11); var CellIdentity v_CellIdentity_NRCell1 := f_NR_CellInfo_GetCellIdentity(nr_Cell1); var CellIdentity v_CellIdentity_NRCell11 := f_NR_CellInfo_GetCellIdentity(nr_Cell11); var TrackingAreaCode v_TAC_NRCell1 := f_NR_CellInfo_GetTAC (nr_Cell1); var TrackingAreaCode v_TAC_NRCell11 := f_NR_CellInfo_GetTAC (nr_Cell11); var AbsoluteTimeInfo_r16 v_AbsoluteTimeInfo_r16 := f_MDT_GetAbsolute_TimeStamp(); var template (present) MeasQuantityResults v_MeasQuantityResults; var NR_SRB_COMMON_IND v_ReceivedAsp; var LogMeasInfo_r16 v_LogMeasInfo_r16; var integer v_FieldCnt; var boolean v_EntryFound_NR_Cell_1 := false; var boolean v_EntryFound_NR_Cell_11 := false; var integer i:=0; var float v_WaitForLogging := 10.0; var float v_WaitNoReaction := 15.0; //@siclog "Step 1" siclog@ // SS transmits a LoggedMeasurementConfiguration message including to configure the UE to perform logging of measurement results while in RRC_IDLE. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_LoggedMeasurementConfiguration(v_PlmnID_NRCell1, v_AbsoluteTimeInfo_r16, cs_LoggedMeasurementConfiguration_reportType_Periodical))); //@siclog "Steps 2" siclog@ // The SS transmits an RRCRelease message to move the UE to RRC_IDLE state. f_NR_RRCRelease(nr_Cell1); //@siclog "Step 3" siclog@ // Wait 10s to allow UE to activate logging f_Delay(v_WaitForLogging); //@siclog "Step 4" siclog@ // The SS changes NR Cell 1 and NR Cell 11 levels according to the row "T1" in table 8.1.6.1.2.11.3.2-1/2. f_NR_SetCellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@siclog "Step 5" siclog@ // Wait 15 seconds. Note: It is checked that UE does not perform the logging while not in 'camped normally' state. f_Delay(v_WaitNoReaction); //@siclog "Step 6" siclog@ // The SS changes NR Cell 1 and NR Cell 11 levels according to the row "T2" in table 8.1.6.1.2.11.3.2-1/2. f_NR_SetCellPower (nr_Cell11, -85, -82); //@sic R5-216562 sic@ //@siclog "Step 7-11" siclog@ // Steps 1 to 5 of generic test procedure in TS 38.508-1 subclause 4.9.5.2.2-1 are performed on NR Cell 11 f_NR5GC_MobilityRegistration(nr_Cell11, -,-,-,-, noRrcConnectionRelease); //@siclog "Step 12" siclog@ // Wait 15 seconds. Note: It is checked that UE does not perform the logging while in RRC_CONNECTED state f_Delay(v_WaitNoReaction); //@siclog "Step 13" siclog@ // The SS transmits an RRCRelease message to move the UE to RRC_INACTIVE state. f_NR_RRCRelease(nr_Cell11); //@siclog "Step 14" siclog@ // Wait 10s to allow UE to activate logging f_Delay(v_WaitForLogging); //@siclog "Step 15-22" siclog@ // Steps 1 to 8 of the generic procedure in TS 38.508 Table 4.5.4.2-3 are executed to successfully complete the service request procedure on NR Cell 11. f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1); // siclog "Step 23" siclog@ // The SS transmits a UEInformationRequest message on NR Cell 11. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell11, -, cs_NR_UEInformationRequest_LOG)); // Logged MDT // siclog "Step 24" siclog@ // Check: Does the UE transmit a UEInformationResponse message including logMeasReport with serving cell measurements for NR Cell 1 and NR Cell 2? // Note: The number of entries in logMeasInfoList should not be more than the expected logged measurement result entries within 20 seconds of logging periods. v_MeasQuantityResults := f_NR_Get_MeasQuantityResults(); SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell11, cr_38508_UEInformationResponse(tsc_NR_RRC_TI_Def, -,-, cr_LogMeasReport_r16(v_AbsoluteTimeInfo_r16, cr_TraceReference_r16(v_PlmnID_NRCell1), '001A'O, '05'O, ?, omit,omit,omit )))) -> value v_ReceivedAsp; v_FieldCnt := lengthof(v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasInfoList_r16); if ((v_FieldCnt < 2) or (v_FieldCnt > 6)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 24: Less than 2 or more than 6 LogMeasurementInfo entries") } while ((i < v_FieldCnt) and (not v_EntryFound_NR_Cell_1 and not v_EntryFound_NR_Cell_11)) { v_LogMeasInfo_r16 := v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasInfoList_r16[i]; if (match(v_LogMeasInfo_r16, cr_LogMeasInfo_r16(-, cr_CGI_Info_Logging_r16(v_PlmnID_NRCell1, v_CellIdentity_NRCell1, v_TAC_NRCell1), cr_MeasResultServCell_r16(v_MeasQuantityResults), omit, omit))) { v_EntryFound_NR_Cell_1 := true; } else if (match(v_LogMeasInfo_r16, cr_LogMeasInfo_r16(-, cr_CGI_Info_Logging_r16(v_PlmnID_NRCell11, v_CellIdentity_NRCell11, v_TAC_NRCell11), cr_MeasResultServCell_r16(v_MeasQuantityResults), omit, omit))) { v_EntryFound_NR_Cell_11 := true; } i:=i+1; } if (v_EntryFound_NR_Cell_1 and v_EntryFound_NR_Cell_11) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 24"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 24: condition not fulfilled for the LogMeasurementInfo entries"); } }//end of f_TC_8_1_6_1_2_11_NR5GC_TestBody

After Change

function f_TC_8_1_6_1_2_11_NR5GC_TestBody() runs on NR5GC_PTC { var PLMN_Identity v_PlmnID_NRCell1 := f_NR_CellInfo_GetPLMN (nr_Cell1); var PLMN_Identity v_PlmnID_NRCell11 := f_NR_CellInfo_GetPLMN (nr_Cell11); var CellIdentity v_CellIdentity_NRCell1 := f_NR_CellInfo_GetCellIdentity(nr_Cell1); var CellIdentity v_CellIdentity_NRCell11 := f_NR_CellInfo_GetCellIdentity(nr_Cell11); var TrackingAreaCode v_TAC_NRCell1 := f_NR_CellInfo_GetTAC (nr_Cell1); var TrackingAreaCode v_TAC_NRCell11 := f_NR_CellInfo_GetTAC (nr_Cell11); var AbsoluteTimeInfo_r16 v_AbsoluteTimeInfo_r16 := f_MDT_GetAbsolute_TimeStamp(); var template (present) MeasQuantityResults v_MeasQuantityResults; var NR_SRB_COMMON_IND v_ReceivedAsp; var LogMeasInfo_r16 v_LogMeasInfo_r16; var integer v_FieldCnt; var boolean v_EntryFound_NR_Cell_1 := false; var boolean v_EntryFound_NR_Cell_11 := false; var integer i:=0; var float v_WaitForLogging := 10.0; var float v_WaitNoReaction := 15.0; //@siclog "Step 1" siclog@ // SS transmits a LoggedMeasurementConfiguration message including to configure the UE to perform logging of measurement results while in RRC_IDLE. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_LoggedMeasurementConfiguration(v_PlmnID_NRCell1, v_AbsoluteTimeInfo_r16, cs_LoggedMeasurementConfiguration_reportType_Periodical))); //@siclog "Steps 2" siclog@ // The SS transmits an RRCRelease message to move the UE to RRC_IDLE state. f_NR_RRCRelease(nr_Cell1); //@siclog "Step 3" siclog@ // Wait 10s to allow UE to activate logging f_Delay(v_WaitForLogging); //@siclog "Step 4" siclog@ // The SS changes NR Cell 1 and NR Cell 11 levels according to the row "T1" in table 8.1.6.1.2.11.3.2-1/2. f_NR_SetCellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@siclog "Step 5" siclog@ // Wait 15 seconds. Note: It is checked that UE does not perform the logging while not in 'camped normally' state. f_Delay(v_WaitNoReaction); //@siclog "Step 6" siclog@ // The SS changes NR Cell 1 and NR Cell 11 levels according to the row "T2" in table 8.1.6.1.2.11.3.2-1/2. f_NR_SetCellPower (nr_Cell11, -85, -82); //@sic R5-216562 sic@ //@siclog "Step 7-11" siclog@ // Steps 1 to 5 of generic test procedure in TS 38.508-1 subclause 4.9.5.2.2-1 are performed on NR Cell 11 f_NR5GC_MobilityRegistration(nr_Cell11, -,-,-,-, noRrcConnectionRelease); //@siclog "Step 12" siclog@ // Wait 15 seconds. Note: It is checked that UE does not perform the logging while in RRC_CONNECTED state f_Delay(v_WaitNoReaction); //@siclog "Step 13" siclog@ // The SS transmits an RRCRelease message to move the UE to RRC_INACTIVE state. f_NR_RRCRelease(nr_Cell11); //@siclog "Step 14" siclog@ // Wait 10s to allow UE to activate logging f_Delay(v_WaitForLogging); //@siclog "Step 15-22" siclog@ // Steps 1 to 8 of the generic procedure in TS 38.508 Table 4.5.4.2-3 are executed to successfully complete the service request procedure on NR Cell 11. f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell11); // siclog "Step 23" siclog@ // The SS transmits a UEInformationRequest message on NR Cell 11. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell11, -, cs_NR_UEInformationRequest_LOG)); // Logged MDT // siclog "Step 24" siclog@ // Check: Does the UE transmit a UEInformationResponse message including logMeasReport with serving cell measurements for NR Cell 1 and NR Cell 2? // Note: The number of entries in logMeasInfoList should not be more than the expected logged measurement result entries within 20 seconds of logging periods. v_MeasQuantityResults := f_NR_Get_MeasQuantityResults(); SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell11,tsc_NR_RbId_SRB2, cr_38508_UEInformationResponse(tsc_NR_RRC_TI_Def, -,-, cr_LogMeasReport_r16(v_AbsoluteTimeInfo_r16, cr_TraceReference_r16(v_PlmnID_NRCell1), '001A'O, '05'O, ?, omit,omit,omit )))) -> value v_ReceivedAsp; v_FieldCnt := lengthof(v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasInfoList_r16); // As per Proposed CR 38.523-1, logMeasInfoList can have more than 6 entries. if (v_FieldCnt < 2) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 24: Less than 2 LogMeasurementInfo entries") } while ((i < v_FieldCnt) and (not v_EntryFound_NR_Cell_1 or not v_EntryFound_NR_Cell_11)) { v_LogMeasInfo_r16 := v_ReceivedAsp.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.logMeasReport_r16.logMeasInfoList_r16[i]; if (match(v_LogMeasInfo_r16, cr_LogMeasInfo_r16(-, cr_CGI_Info_Logging_r16(v_PlmnID_NRCell1, v_CellIdentity_NRCell1, v_TAC_NRCell1), cr_MeasResultServCell_r16(v_MeasQuantityResults), omit, omit))) { v_EntryFound_NR_Cell_1 := true; } else if (match(v_LogMeasInfo_r16, cr_LogMeasInfo_r16(-, cr_CGI_Info_Logging_r16(v_PlmnID_NRCell11, v_CellIdentity_NRCell11, v_TAC_NRCell11), cr_MeasResultServCell_r16(v_MeasQuantityResults), omit, omit))) { v_EntryFound_NR_Cell_11 := true; } i:=i+1; } if (v_EntryFound_NR_Cell_1 and v_EntryFound_NR_Cell_11) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 24"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 24: condition not fulfilled for the LogMeasurementInfo entries"); } }//end of f_TC_8_1_6_1_2_11_NR5GC_TestBody

# Branches executed

On Anritsu, the test case was executed on NR band n78 using NR ciphering nea2 and integrity algorithm nia2.

# Execution Log Files

## Mediatek MT6983

The Mediatek MT6983 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anritsu\ TC_8_1_6_1_2_11_LOG.xml

- PICS/PIXIT parameter file:

Anritsu\ TC_8_1_6_1_2_11_PIXIT.txt

# References

| [1] | R5s220090 - Supporting information for addition of NR5GC Logged MDT FR1 test case 8.1.6.1.2.11 |
| --- | --- |