3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220107

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2309 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for function f_MultipleQoSPreamble_Part1() |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the function f_MultipleQoSPreamble_Part1(), following 5GS registration procedure and in the case where pc_noOf_PDUsSameConnection=0, the TTCN will trigger a PDU session establishment and then expect PDU SESSION ESTABLISHMENT request. If the UE has set the Follow-on request in REGISTRATION REQUEST to “no follow-on request pending”, it will start timer T3540 and behave according to 24.501 5.3.1.3: b) shall start the timer T3540 for a UE in 3GPP access if: ………… 2) the UE has set the Follow-on request indicator to "No follow-on request pending" in the REGISTRATION REQUEST message; In case b) and f), - ………………... - if there is no user-plane resources established for PDU sessions, upon receiving a request from the upper layers to perform services other than emergency service fallback only for a UE in 3GPP access or establishing an emergency PDU session, the UE shall wait for the local release of the established N1 NAS signalling connection upon expiry of timer T3540 or wait for timer T3540 being stopped, before initiating NAS signalling; or This indicates that a UE which indicated “no follow-on pending” will not react to the trigger to create a PDU session until after T3540 has expired. Then the UE will locally release the N1 NAS signalling connection and initiate a new connection for the PDU session establishment by sending RRCSetupRequest, which will cause test case failure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If the UE has indicated “no follow-on request pending” and pc_noOf_PDUsSameConnection=0, initiate local connection release and handle a new SERVICE REQUEST procedure before expecting PDU SESSION ESTABLISHMENT REQUEST. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail these test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.4.2.NR5GC, 8.1.5.4.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

2.2	Change 2	6

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk49 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_MultipleQoSPreamble_Part1 () |
| --- | --- |
| Reason for change | In the function f_MultipleQoSPreamble_Part1(), following 5GS registration procedure and in the case where pc_noOf_PDUsSameConnection=0, the TTCN will trigger a PDU session establishment and then expect PDU SESSION ESTABLISHMENT request. If the UE has set the Follow-on request in REGISTRATION REQUEST to “no follow-on request pending”, it will start timer T3540 and behave according to 24.501 5.3.1.3: b) shall start the timer T3540 for a UE in 3GPP access if: ………… 2) the UE has set the Follow-on request indicator to "No follow-on request pending" in the REGISTRATION REQUEST message; In case b) and f), - ………………... - if there is no user-plane resources established for PDU sessions, upon receiving a request from the upper layers to perform services other than emergency service fallback only for a UE in 3GPP access or establishing an emergency PDU session, the UE shall wait for the local release of the established N1 NAS signalling connection upon expiry of timer T3540 or wait for timer T3540 being stopped, before initiating NAS signalling; or This indicates that a UE which indicated “no follow-on pending” will not react to the trigger to create a PDU session until after T3540 has expired. Then the UE will locally release the N1 NAS signalling connection and initiate a new connection for the PDU session establishment by sending RRCSetupRequest, which will cause test case failure. |
| Summary of change | If the UE has indicated “no follow-on request pending” and pc_noOf_PDUsSameConnection=0, initiate local connection release and handle a new SERVICE REQUEST procedure before expectgin PDU SESSION ESTABLISHMENT REQUEST. |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment | Accepted in principle but called existing function f_NR5GC_RRCIdleToConnected_Steps2_4. Only need to check if pc_noOf_PDUsSameConnection=0 as in this case, the UE will never set the FOR bit |

Before Change:

function f_MultipleQoSPreamble_Part1 (NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC return GSM_MobilityInfo_Type { var NR_SRB_COMMON_IND v_ReceivedAsp; var GSM_MobilityInfo_Type v_GSM_MobilityInfo_Type; // turn on NR cell f_NR_SetCellPower (p_NR_CellId, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); // Switch on UE f_UT_SwitchOnUE(UT); f_NR5GC_Registration(p_NR_CellId, TEST_LOOPModeB_ON, Initial_NoSecurity); if (pc_noOf_PDUsSameConnection == 0) { // @sic R5s200751, R5s201358 sic@ f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1); // @sic R5-204399, R5s210084 sic@ } // Receive PDU Establishment SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB1, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo_Type, v_ReceivedAsp.Signalling.Nas[0].Pdu)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed"); } return v_GSM_MobilityInfo_Type; }

After Change:

function f_MultipleQoSPreamble_Part1 (NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC return GSM_MobilityInfo_Type { var NR_SRB_COMMON_IND v_ReceivedAsp; var GSM_MobilityInfo_Type v_GSM_MobilityInfo_Type; var B1_Type v_followOn ; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var B4_Type v_ServiceType := '0000'B; // turn on NR cell f_NR_SetCellPower (p_NR_CellId, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); // Switch on UE f_UT_SwitchOnUE(UT); v_ReceivedMsg := f_NR5GC_Registration(p_NR_CellId, TEST_LOOPModeB_ON, Initial_NoSecurity); v_followOn := v_ReceivedMsg.Pdu.Msg.registration_Request.registrationType.fOR; if (pc_noOf_PDUsSameConnection == 0) { // @sic R5s200751, R5s201358 sic@ if(v_followOn == '0'B) { f_NR_RRCRelease(p_NR_CellId); } f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1); // @sic R5-204399, R5s210084 sic@ if(v_followOn == '0'B) { //RRC connection establishment, security and SERVICE ACCEPT, similar to e.g. steps 2-7 in the function f_NR5GC_RRC_Idle_Extension() // Steps 2 - 4 var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (p_NR_CellId); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, ?); // @sic R5s200489 sic@ if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, -)) { // @sic R5s210189 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } // Steps 5-6 // RRC Security Mode procedure f_NR_RRC_ActivateSecurity (p_NR_CellId, valueof(v_ReceivedMsg.SecurityProtection.NasCount)); // Use NasCount returned from Service Req // FFS - Prose DOES NOT specify PDU Session Status to be reflected back from Req message // Step 7 SRB.send (cas_NR_SRB_NasPdu_REQ(p_NR_CellId, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg(v_ReceivedMsg.Pdu.Msg.service_Request.pduSessionStatus)))); // @sic R5s201387 Baseline Moving sic@ } } // Receive PDU Establishment SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB1, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo_Type, v_ReceivedAsp.Signalling.Nas[0].Pdu)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed"); } return v_GSM_MobilityInfo_Type; }

### MCC160 Implementation

function f_MultipleQoSPreamble_Part1 (NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC return GSM_MobilityInfo_Type

{

var NR_SRB_COMMON_IND v_ReceivedAsp;

var GSM_MobilityInfo_Type v_GSM_MobilityInfo_Type;

// turn on NR cell

f_NR_SetCellPower (p_NR_CellId, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2);

// Switch on UE

f_UT_SwitchOnUE(UT);

f_NR5GC_Registration(p_NR_CellId, TEST_LOOPModeB_ON, Initial_NoSecurity);

if (pc_noOf_PDUsSameConnection == 0) { // @sic R5s200751, R5s201358 sic@

// UE will release signalling connection after T3540

// so release connection and allow it to move back to connected

f_NR_RRCRelease(p_NR_CellId);

f_NR5GC_RRC_Idle_Extension_Steps1_7 (p_NR_CellId);

}

// Receive PDU Establishment

SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB1, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp;

if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo_Type,

v_ReceivedAsp.Signalling.Nas[0].Pdu))

{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed");

}

return v_GSM_MobilityInfo_Type;

}

function f_NR5GC_RRC_Idle_Extension_Steps1_7(NR_CellId_Type p_NR_CellId,

template (present) B4_Type p_ServiceType := '0000'B)  runs on NR5GC_PTC

{

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (p_NR_CellId);

var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams);

var integer v_NoOfExistingSessions := f_NR5GC_MobileInfo_GetNoOfSessions(); // @sic R5-206291 sic@

var PDU_PDN_DNN_Type v_PDN_Type;

var charstring v_DNN := "";

timer t_Wait := 10.0;

// Step 0 @sic R5-204399 sic@

t_Wait.start;

alt {

[] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(p_NR_CellId, cr_38508_RRCSetupRequest(?, ?)))) {

t_Wait.stop;

}

[] t_Wait.timeout {

// Step 1

if (v_NoOfExistingSessions == 0) { // @sic R5-206291 sic@

/* Only trigger if there's no sessions established automatically

to ensure there's (at least) one session established for the test body */

v_PDN_Type := f_GetDefaultAPN_DNNType_FromPICS(); // @sic R5-211372 sic@

select (v_PDN_Type) {

case (Internet_DNN, None) {

v_DNN := pc_APN_ID_Internet;

}

case (IMS_DNN) {

v_DNN := pc_APN_ID_IMS;

}

}

f_UT_RequestPDUSession(UT, 1, v_DNN); // @sic R5s210084, R5-211372 sic@

} else { // @sic R5s210084 sic@

// if an establishment request message hasn't been sent, or triggered, then don't need to carry on

return;

}

}

}

// Steps 2 - 4

v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, ?); // @sic R5s200489 sic@

if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu,  cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (),

tsc_NasKsi_NativeSecurityContext),

v_S_TMSI, p_ServiceType)) {   // @sic R5s210189 sic@

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed");

}

// Steps 5-6

// RRC Security Mode procedure

f_NR_RRC_ActivateSecurity (p_NR_CellId, valueof(v_ReceivedMsg.SecurityProtection.NasCount)); // Use NasCount returned from Service Req

// FFS - Prose DOES NOT specify PDU Session Status to be reflected back from Req message

// Step 7

SRB.send (cas_NR_SRB_NasPdu_REQ(p_NR_CellId, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg(v_ReceivedMsg.Pdu.Msg.service_Request.pduSessionStatus)))); // @sic R5s201387 Baseline Moving sic@

}

function f_NR5GC_RRC_Idle_Extension(NR_CellId_Type p_NR_CellId,

NR_TESTMODE_STATE_Type p_TestMode,

RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease,

template (value) UE_TestLoopModeA_NR_LB_Setup_Type p_UE_TestLoopModeA_NR_LB_Setup := cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling,

UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O,

template (omit) SDAP_Config p_SDAP_Config := omit,

template S_NSSAI_Type p_ExpectedS_NSSAI := *,

template DNN p_ExpectedDNN := *,

template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B),

template (omit) DNN p_DNN := omit,

template (omit) QoS_Rules p_QoS_Rules := omit,

template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O),

template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI, // @sic R5s190919 sic@

template (omit) QoSFlowDescr p_QoSFlowDescr := omit,

template (present) B4_Type p_ServiceType := '0000'B)  runs on NR5GC_PTC

{ // Split into steps1_7 @sic R5s220107 sic@

f_NR5GC_RRC_Idle_Extension_Steps1_7(p_NR_CellId,

p_ServiceType);

// Step 8

f_NR5GC_PDUSessionEstablishment (p_NR_CellId,

pc_noOf_PDUsNewConnection,

tsc_NR_RbId_SRB1, // @sic R5-206291 sic@

p_SDAP_Config, // @sic R5s210134 sic@

p_ExpectedS_NSSAI,

p_ExpectedDNN,

p_SSC_Mode,

p_DNN,

p_QoS_Rules,

p_Session_AMBR,

p_S_NSSAI,

p_QoSFlowDescr);

//Step 9a1

if (p_ReleaseConnection == rrcConnectionRelease) {

// Release RRC to enter RRC_IDLE

f_NR_RRCRelease(p_NR_CellId);

} else {

// Step 8Aa1 - 8Aa2

select (p_TestMode) {

case (TEST_LOOPModeA_ON) {

f_NR5GC_CloseUE_TestLoopModeA (p_NR_CellId, p_UE_TestLoopModeA_NR_LB_Setup);

}

case (TEST_LOOPModeB_ON) {

f_NR5GC_CloseUE_TestLoopModeB (p_NR_CellId, p_IpPduDelayTime);

}

case (PING_Or_TEST_LOOPModeB_ON) {

if (not pc_IP_Ping) {

f_NR5GC_CloseUE_TestLoopModeB (p_NR_CellId, p_IpPduDelayTime);

}

}

}

}

}

## Change 2

| Function name | f_NR5GC_Registration() |
| --- | --- |
| Reason for change | In the function f_MultipleQoSPreamble_Part1(), following 5GS registration procedure and in the case where pc_noOf_PDUsSameConnection=0, the TTCN will trigger a PDU session establishment and then expect PDU SESSION ESTABLISHMENT request. If the UE has set the Follow-on request in REGISTRATION REQUEST to “no follow-on request pending”, it will start timer T3540 and behave according to 24.501 5.3.1.3: b) shall start the timer T3540 for a UE in 3GPP access if: ………… 2) the UE has set the Follow-on request indicator to "No follow-on request pending" in the REGISTRATION REQUEST message; In case b) and f), - ………………... - if there is no user-plane resources established for PDU sessions, upon receiving a request from the upper layers to perform services other than emergency service fallback only for a UE in 3GPP access or establishing an emergency PDU session, the UE shall wait for the local release of the established N1 NAS signalling connection upon expiry of timer T3540 or wait for timer T3540 being stopped, before initiating NAS signalling; or This indicates that a UE which indicated “no follow-on pending” will not react to the trigger to create a PDU session until after T3540 has expired. Then the UE will locally release the N1 NAS signalling connection and initiate a new connection for the PDU session establishment by sending RRCSetupRequest, which will cause test case failure. |
| Summary of change | Adding a return type so that the REGISTRATION REQUEST message is returned from the function, so that the follow-on request field can be checked in the calling function (see change 1) |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment | Not needed. If pc_noOf_PDUsSameConnection=0 then the UE will not send another message on this connection, so the FOR bit doesn’t need to be checked. |

Before Change:

function f_NR5GC_Registration(NR_CellId_Type p_NR_CellId, NR_TESTMODE_STATE_Type p_TestMode := TESTMode_OFF, TypeOfRegistration_Type p_RegType := Initial_Secure, template (omit) PLMN_List p_EPLMNs := omit, template (omit) NG_NetworkFeatureSupport p_NwkFeatureSupport := cs_NG_NetworkFeatureSupport_Def, InterworkWithoutN26 p_IwkN26 := NOT_SUPPORTED) runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; /*skipping code*/ // Steps 5 - 15 f_NR5GC_RRC_Idle_Steps5_15(v_GMM_MobilityInfo, p_NR_CellId, v_ReceivedMsg, v_PLMN, p_TestMode, p_RegType, p_EPLMNs, p_NwkFeatureSupport, p_IwkN26); // @sic R5-202564 sic@ }

After Change:

function f_NR5GC_Registration(NR_CellId_Type p_NR_CellId, NR_TESTMODE_STATE_Type p_TestMode := TESTMode_OFF, TypeOfRegistration_Type p_RegType := Initial_Secure, template (omit) PLMN_List p_EPLMNs := omit, template (omit) NG_NetworkFeatureSupport p_NwkFeatureSupport := cs_NG_NetworkFeatureSupport_Def, InterworkWithoutN26 p_IwkN26 := NOT_SUPPORTED) runs on NR5GC_PTC return NG_NAS_MSG_Indication_Type { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; /*skipping code*/ // Steps 5 - 15 f_NR5GC_RRC_Idle_Steps5_15(v_GMM_MobilityInfo, p_NR_CellId, v_ReceivedMsg, v_PLMN, p_TestMode, p_RegType, p_EPLMNs, p_NwkFeatureSupport, p_IwkN26); // @sic R5-202564 sic@ return v_ReceivedMsg; }