3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220110

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0772 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for IMS 5GS call control test cases 7.10 and 7.25 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As per the prose CR R5-217998, if the video media is received, it should be accepted and not checked. The current implementation in f_IMS_MTCallSetup_NoSdpInINVITE_Step1_6() uses the template cr_SDP_Media_Video with default parameters, which means only the transport type “RTP/AVPF” will be accepted. The UE may indicate a different transport type e.g. “RTP/AVP”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Allow any value for media transport type. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail these test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.10.NR5GC, 7.25.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk49 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_IMS_MTCallSetup_NoSdpInINVITE_Step1_6() |
| --- | --- |
| Reason for change | As per the prose CR R5-217998, if the video media is received, it should be accepted and not checked. The current implementation in f_IMS_MTCallSetup_NoSdpInINVITE_Step1_6() uses the template cr_SDP_Media_Video with default parameters, which means only the transport type “RTP/AVPF” will be accepted. The UE may indicate a different transport type e.g. “RTP/AVP”. |
| Summary of change | Allow any value for media transport type. |
| TTCN module | IMS_NR5GC_CallControlTestcases.ttcn |
| MCC160 Comment | Accepted |

Before Change

function f_IMS_MTCallSetup_NoSdpInINVITE_Step1_6(boolean p_PreconditionsRequired) runs on IMS_PTC return IMS_InviteRequestWithSdp_Type { /* NOTE: the returned InviteRequestWithSdp contains the SDP offer sent by the UE in the 183 Session Progress */ var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var IMS_ResponseWithSdp_Type v_ResponseWithSdp; var template (present) SDP_attribute_list v_PreconditionAttributes := {}; var template (present) SDP_Message v_SdpOfferAudioOnly; var template (present) SDP_Message v_SdpOfferAudioVideo; var template (present) SDP_Message v_SdpOffer; var template (value) MessageHeader v_MessageHeader_Prack; var template (value) SDP_Message v_SdpAnswer; var IMS_SDP_AudioCodecList_Type v_AudioCodecList; var boolean v_CheckForbiddenCodecParameters := true; var InternetProtocol_Type v_Protocol; var SipUrl v_ContactUrl; // Step 2: INVITE v_AudioCodecList := {}; // => no SDP v_InviteRequestWithSdp := f_IMS_MTCallSetup_NR5GC_A_5_X_Step1(p_PreconditionsRequired, v_AudioCodecList); // Step 3 - 4: 100 Trying, 183 Session Progress if (p_PreconditionsRequired) { v_PreconditionAttributes := cr_SDP_PreconditionAttributes(c_none, c_none, c_mandatory, c_optional); /* @sic R5s210462 change 2 -> R5-212770 has been withdrawn at TSGR5#91 => des:qos kept optional for remote sic@ */ } v_AudioCodecList := tsc_IMS_SDP_AudioCodecList_EVS_WBAMR_AMR; v_SdpOfferAudioOnly := f_IMS_BuildSDP_InitialOfferAudioWithPreconditions_RX(v_AudioCodecList, v_PreconditionAttributes); /* @sic R5s210462 change 2 sic@ */ v_SdpOfferAudioVideo := v_SdpOfferAudioOnly; v_SdpOfferAudioVideo.media_list[1] := cr_SDP_Media_Desc(cr_SDP_Media_Video, ?, ?, ?); // @sic R5-217998: SDP offer may contain video sic@ v_SdpOffer := (v_SdpOfferAudioOnly, v_SdpOfferAudioVideo); v_ResponseWithSdp := f_IMS_MTCallSetup_Receive100Trying_Receive183SessionProgress(v_InviteRequestWithSdp.Invite, v_SdpOffer, v_InviteRequestWithSdp.RoutingInfo.Protocol, p_PreconditionsRequired); /* @sic R5s210393: p_PreconditionsRequired sic@ */ f_IMS_SDP_CheckMediaDescr_AudioCodecs(v_ResponseWithSdp.SdpMessage.media_list[0], v_AudioCodecList, v_CheckForbiddenCodecParameters); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 4"); … … … }

After Change

function f_IMS_MTCallSetup_NoSdpInINVITE_Step1_6(boolean p_PreconditionsRequired) runs on IMS_PTC return IMS_InviteRequestWithSdp_Type { /* NOTE: the returned InviteRequestWithSdp contains the SDP offer sent by the UE in the 183 Session Progress */ var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var IMS_ResponseWithSdp_Type v_ResponseWithSdp; var template (present) SDP_attribute_list v_PreconditionAttributes := {}; var template (present) SDP_Message v_SdpOfferAudioOnly; var template (present) SDP_Message v_SdpOfferAudioVideo; var template (present) SDP_Message v_SdpOffer; var template (value) MessageHeader v_MessageHeader_Prack; var template (value) SDP_Message v_SdpAnswer; var IMS_SDP_AudioCodecList_Type v_AudioCodecList; var boolean v_CheckForbiddenCodecParameters := true; var InternetProtocol_Type v_Protocol; var SipUrl v_ContactUrl; // Step 2: INVITE v_AudioCodecList := {}; // => no SDP v_InviteRequestWithSdp := f_IMS_MTCallSetup_NR5GC_A_5_X_Step1(p_PreconditionsRequired, v_AudioCodecList); // Step 3 - 4: 100 Trying, 183 Session Progress if (p_PreconditionsRequired) { v_PreconditionAttributes := cr_SDP_PreconditionAttributes(c_none, c_none, c_mandatory, c_optional); /* @sic R5s210462 change 2 -> R5-212770 has been withdrawn at TSGR5#91 => des:qos kept optional for remote sic@ */ } v_AudioCodecList := tsc_IMS_SDP_AudioCodecList_EVS_WBAMR_AMR; v_SdpOfferAudioOnly := f_IMS_BuildSDP_InitialOfferAudioWithPreconditions_RX(v_AudioCodecList, v_PreconditionAttributes); /* @sic R5s210462 change 2 sic@ */ v_SdpOfferAudioVideo := v_SdpOfferAudioOnly; v_SdpOfferAudioVideo.media_list[1] := cr_SDP_Media_Desc(cr_SDP_Media_Video(-, ?), ?, ?, ?); // @sic R5-217998: SDP offer may contain video sic@ v_SdpOffer := (v_SdpOfferAudioOnly, v_SdpOfferAudioVideo); v_ResponseWithSdp := f_IMS_MTCallSetup_Receive100Trying_Receive183SessionProgress(v_InviteRequestWithSdp.Invite, v_SdpOffer, v_InviteRequestWithSdp.RoutingInfo.Protocol, p_PreconditionsRequired); /* @sic R5s210393: p_PreconditionsRequired sic@ */ f_IMS_SDP_CheckMediaDescr_AudioCodecs(v_ResponseWithSdp.SdpMessage.media_list[0], v_AudioCodecList, v_CheckForbiddenCodecParameters); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 4"); … … … }