3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220125

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2325 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR test case 8.1.1.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | With the tdd_UL_DL_ConfigurationCommon configuration provided for cell 2, DL-UL pattern looks as below (D – Downlink Slot , U – Uplink Slot, F – Flexible Slot ) 0; 1; 2; 3; 4; 5; 6; 7; 8; 9. D; D; D; D; D; D; D; F; U; U; D; D; D; D; D; D; D; F; U; U. At the beginning of the test case , nr cell2 is configured with prach_ConfigurationIndex = (160) . As per table 6.3.3.2-3 in spec 38.211 , Prach is expected to come in the subframe = 9 . In response, RAR will be scheduled at a Subframe which makes sure (K2+ Δ)th subframe is receiving Msg3 at the UL slot. From Rar to Message 3 timing , Please refer refer 38.214 section 6.1.2.1.1 When the UE transmits a PUSCH scheduled by RAR or by the fallbackRAR, the Δ value specific to the PUSCH subcarrier spacing µPUSCH is applied in addition to the K2 value . As per the test case configuration K2 = 6 and Δ = 3 (when running the test in 30Khz SCS); When receiving the Prach in Subframe 9 slot = 1 , RAR is sent on Subframe = 5 Slot = 0 , so that Msg3 can be received in Subframe = 9 Slot = 1 ( from RAR it takes K2 + Δ slots to receive Msg3) ; prach_ConfigurationIndex = (160) is working well in TDD pattern . At step 22A , SS changes the prach-configuration index as “157” for which prach is expected to be received in the subframe = 4 ( every SFN ) If RACH is received in subframe = 4 in an even SFN , RAR Tx can be scheduled on sf = 0 slot = 0 (or) sf = 5 slot = 0 . Considering the interference due to SSB block transmission sf = 0 slot = 0 can not be used for RAR transmission . Sending Rar at sf = 5 slot = 0 will make the Rar window to expire (sl20 is the rar window configured) . |
| Summary of change: |  | Increase the Rar window size ( from sl20 sl40) when configuring prach configuration index as “157” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.4.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required	3

1.1	New function f_NR_CellInfo_SetRaResponseWindow()	3

1.2	Correction to the function fl_TC_8_1_1_4_1_TestBody()	6

# Corrections required

## New function f_NR_CellInfo_SetRaResponseWindow()

| Function name | f_NR_CellInfo_SetRaResponseWindow() |
| --- | --- |
| Reason for change | With the prach configuration index configured in step 22A , Rar window is getting expired before RAR is getting transmitted |
| Summary of change | increase the Rar window size ( from sl20 sl40) when configuring prach configuration index as “157” Implemented the new function which can be used to set the RaResponse Window size |
| TTCN module | ttcn-3/ttcn/develop/Common/NR5GC/NR/NR_CellInfo.ttcn |
| MCC160 Comment | Accepted |

Implementation:

/* * @desc Assign the parameterised Ra_ResponseWindow in the CellArray UL BWP and SIB1(if available) of the cell * @param p_NR_CellId * @param p_RA_ResponseWindow * @status APPROVED (ENDC, NR5GC) */ function f_NR_CellInfo_SetRaResponseWindow(NR_CellId_Type p_NR_CellId, RACH_ConfigGeneric.ra_ResponseWindow p_RA_ResponseWindow)runs on NR_BASE_PTC { var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP (p_NR_CellId, tsc_NR_BWP_Id); //Initial BWP if (v_NR_CellInfo.Sysinfo.NR_SysinfoCombination != MibOnly){ v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.servingCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.rach_ConfigGeneric.ra_ResponseWindow := p_RA_ResponseWindow; } v_NR_UplinkBWP.Common.R15.rach_ConfigCommon.setup.rach_ConfigGeneric.ra_ResponseWindow := p_RA_ResponseWindow; f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); //@sic R5s190455 sic@ f_NR_CellInfo_SetUplinkBWP(p_NR_CellId, v_NR_UplinkBWP, tsc_NR_BWP_Id); }

## Correction to the function fl_TC_8_1_1_4_1_TestBody()

| Function name | fl_TC_8_1_1_4_1_TestBody() |
| --- | --- |
| Reason for change | With the prach configuration index configured in step 22A , Rar window is getting expired before RAR is getting transmitted |
| Summary of change | increase the Rar window size ( from sl20 sl40) when configuring prach configuration index as “157” |
| TTCN module | ttcn-3/ttcn/develop/Common/NR5GC/8_1_1/RRC_Resume_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function fl_TC_8_1_1_4_1_TestBody() runs on NR5GC_PTC { //@sic R5-197053, R5-198912, R5s191051 sic@ var template (omit) GSM_MobilityInfo_Type v_PDUSessionInfo; var integer v_DRBId; var integer v_ModificationPeriodFrames; var float v_ModificationPeriod; var template (value) CellGroupConfig v_CellGroupConfig; var NR_DownlinkBWP_Type v_NR_DownlinkBWP_Cell2; // @sic R5-204436 sic@ var PDSCH_Config v_PDSCH_Config; // @sic R5-204436 sic@ var template (omit) NR_BWP_DownlinkDedicated_PDSCH_Config_Type v_BWP_DownlinkDedicated_PDSCH_Config; // @sic R5-204436 sic@ var integer v_PrachConfigurationIndex; // @sic R5-213465 sic@ var NR_PhysicalParameters_Type v_PhysicalParameters; // @sic R5-213465 sic@ var template (omit) NR_SRB_COMMON_IND v_SrbIndByRef := omit; // not used (needed when other information of the RRC message needs to be access) // @sic R5-213465 sic@ var template (omit) NR_SYSTEM_IND v_SysIndByRef := omit; // to store SYSTEM_IND if it takes over the RRC message // @sic R5-213465 sic@ var template (present) TimingInfo_Type v_TimingInfo; // @sic R5-213465 sic@ timer t_WaitTime := 60.0; <<< SKIPPED CODE >>> //@siclog "Steps 21 - 22" siclog@ //@sic R5-214592 sic@ //Void //@siclog "Step 22A" siclog@ //@sic R5-213465, R5-214592 sic@ //The SS changes the prach-ConfigurationIndex in the system information. if (f_NR_CellInfo_GetIsFR1(nr_Cell2)){ v_PrachConfigurationIndex := 157; //FR1 FDD - As per 38.211 Table 6.3.3.2-2 //FR1 TDD - As per 38.211 Table 6.3.3.2-3 // nSFN mod 1 = 0=> any SFN Subframe 4 v_TimingInfo := cr_TimingInfo(?, (4)); } else { // FR2 v_PrachConfigurationIndex := 156; // @sic R5-217275, R5s211579 sic@ // slots (19,39) in 60kHz - As per 38.211 Table 6.3.3.2-4 // SCS RA = 120kHz, "Number of PRACH slots within a 60 kHz slot" in Table 6.3.3.2-4 is equal to 1, then nRAslot = 1 v_TimingInfo := ( cr_TimingInfo(?, 4, -, cs_SlotTimingInfo_Numerology3(7)), //Slot 39 in 120kHz cr_TimingInfo(?, 9, -, cs_SlotTimingInfo_Numerology3(7)) //Slot 79 in 120kHz ); } //Set Prach configuration index in SIB1 and in Cell Array f_NR_CellInfo_SetPRACH_ConfigurationIndex(nr_Cell2, v_PrachConfigurationIndex); //Reconfigure SS with new PRACH parameters v_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell2); f_NR_SS_CommonCellConfig(nr_Cell2, cads_NR_CellConfigPhysicalLayerUplink(nr_Cell2, -, cs_NR_CellConfigPhysicalLayerUplink_Def(-,-,cs_NR_UplinkBWPs(v_PhysicalParameters.UL_BWPs)))); //Reconfigure SS to report PRACH preambles f_NR_SS_SystemIndCtrlConfig(nr_Cell2, cds_NR_SystemIndCtrl_RachPreamble(enable)); //@siclog "Step 22B" siclog@ //@sic R5-213465, R5s210689, R5-214592 sic@ //The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. f_NR_ModifySysinfo(nr_Cell2, true, RRC_INACTIVE); << SKIPPED CODE >> }

After Change:

function f_ReconfigureDRBs(NR_CellId_Type p_NR_CellId, template (omit) NG_NAS_MSG_RequestList_Type p_NASMsg, template (value) RadioBearerConfig p_RadioBearerConfig, template (value) CellGroupConfig p_CellGroupConfig, template (value) NR_RadioBearerList_Type p_SS_Config, NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB1, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, boolean p_ReceiveForRRCreconfigurationComplete := true, template (omit) NR_RoutingInfoSUL_Type p_NR_RoutingInfoSUL := omit, template (omit) PDSCH_ServingCellConfig.nrofHARQ_ProcessesForPDSCH p_NrofHARQ_ProcessesForPDSCH := omit, template (omit) NR_SecurityParams_Type p_SecurityParams := omit, template (omit) MacBearerRouting_Type p_MacBearerRoutingCA := omit ) runs on NR_BASE_PTC { //@sic R5-197053, R5-198912, R5s191051 sic@ var template (omit) GSM_MobilityInfo_Type v_PDUSessionInfo; var integer v_DRBId; var integer v_ModificationPeriodFrames; var float v_ModificationPeriod; var template (value) CellGroupConfig v_CellGroupConfig; var NR_DownlinkBWP_Type v_NR_DownlinkBWP_Cell2; // @sic R5-204436 sic@ var PDSCH_Config v_PDSCH_Config; // @sic R5-204436 sic@ var template (omit) NR_BWP_DownlinkDedicated_PDSCH_Config_Type v_BWP_DownlinkDedicated_PDSCH_Config; // @sic R5-204436 sic@ var integer v_PrachConfigurationIndex; // @sic R5-213465 sic@ var NR_PhysicalParameters_Type v_PhysicalParameters; // @sic R5-213465 sic@ var template (omit) NR_SRB_COMMON_IND v_SrbIndByRef := omit; // not used (needed when other information of the RRC message needs to be access) // @sic R5-213465 sic@ var template (omit) NR_SYSTEM_IND v_SysIndByRef := omit; // to store SYSTEM_IND if it takes over the RRC message // @sic R5-213465 sic@ var template (present) TimingInfo_Type v_TimingInfo; // @sic R5-213465 sic@ timer t_WaitTime := 60.0; var RACH_ConfigGeneric.ra_ResponseWindow v_RA_ResponseWindow := sl40; << SKIPPED CODE >> //@siclog "Step 22A" siclog@ //@sic R5-213465, R5-214592 sic@ //The SS changes the prach-ConfigurationIndex in the system information. if (f_NR_CellInfo_GetIsFR1(nr_Cell2)){ v_PrachConfigurationIndex := 157; //FR1 FDD - As per 38.211 Table 6.3.3.2-2 //FR1 TDD - As per 38.211 Table 6.3.3.2-3 // nSFN mod 1 = 0=> any SFN Subframe 4 v_TimingInfo := cr_TimingInfo(?, (4)); } else { // FR2 v_PrachConfigurationIndex := 156; // @sic R5-217275, R5s211579 sic@ // slots (19,39) in 60kHz - As per 38.211 Table 6.3.3.2-4 // SCS RA = 120kHz, "Number of PRACH slots within a 60 kHz slot" in Table 6.3.3.2-4 is equal to 1, then nRAslot = 1 v_TimingInfo := ( cr_TimingInfo(?, 4, -, cs_SlotTimingInfo_Numerology3(7)), //Slot 39 in 120kHz cr_TimingInfo(?, 9, -, cs_SlotTimingInfo_Numerology3(7)) //Slot 79 in 120kHz ); } //Set Prach configuration index in SIB1 and in Cell Array f_NR_CellInfo_SetPRACH_ConfigurationIndex(nr_Cell2, v_PrachConfigurationIndex); f_NR_CellInfo_SetRaResponseWindow(nr_Cell2,v_RA_ResponseWindow); //Reconfigure SS with new PRACH parameters v_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell2); f_NR_SS_CommonCellConfig(nr_Cell2, cads_NR_CellConfigPhysicalLayerUplink(nr_Cell2, -, cs_NR_CellConfigPhysicalLayerUplink_Def(-,-,cs_NR_UplinkBWPs(v_PhysicalParameters.UL_BWPs)))); //Reconfigure SS to report PRACH preambles f_NR_SS_SystemIndCtrlConfig(nr_Cell2, cds_NR_SystemIndCtrl_RachPreamble(enable)); //@siclog "Step 22B" siclog@ //@sic R5-213465, R5s210689, R5-214592 sic@ //The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. f_NR_ModifySysinfo(nr_Cell2, true, RRC_INACTIVE); << SKIPPED CODE >> }