3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220197

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2342 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR test case 8.1.4.1.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1)In the current implementation of function fl_TC_8_1_4_1_6_TestBody , at step 2 , after sending the reconfiguration to trigger HO , following local configuration messages are missing to address the below requirements : - Reconfiguration to trigger HO will be resulting in SS retransmissions while UE is doing Handover failure based procedure ; To stop this reconfiguration retransmission rb release and reset is required before changing the CRNTI ( otherwise this RRC Configuration retx will still get continued with updated CRNTI and interfere the reestablishment procedure) - To reconfigure DCCH/DTCH DCI on CSS and reconfigure RACH procedure contention-based with UE Contention Resolution Identity MAC CE and with a new C-RNTI ; this configuration is required so that reestablishment can go with DCI on CSS. RRCReestablishment will not be received by the UE if not sent via CSS. - To Reconfigure UL DCCH DCI on USS and stop SR is required before receiving the RRC Setup Request followed by Reestablishment sent with Integrity failure 2) f_NR_RRCRelease_Local function is used after sending the reestablishment with Integrity failure – but UE could send rrc setup request before finishing up with all the timing based release configurations ; To address this issue ,All these local configurations are made with immediate activation , and moved after the reception of Rrc Setup Request ; this is to to avoid the race condition of UE sending Rrc setup request and SS applying Local configurations |  |  |  |  |  |  |  |  |
| Summary of change: |  | Before sending Reestablishment message - RB release and configuration are sent so that reconfiguration retx can be avoided - Reconfiguration of DCCH/DTCH DCI on CSS and reconfigure RACH procedure contention-based with UE Contention Resolution Identity MAC CE and with a new C-RNTI are done , so that reestablishment can go with DCI on CSS. RRCReestablishment will not be received by the UE if not sent via CSS. - Reestablishment DL Peer Message is sent with immediate activation Before receiving the RRC Setup request, - Reconfiguration of UL DCCH DCI on USS and stop SR are done - f_NR_RRCRelease_Local is removed , all the configurations are made with immediate activation time after receiving the rrc setup request , this would avoid having the race condition of receiving rrc setup request and local configurations are still getting applied |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.1.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required	3

1.1	Correction to the function fl_TC_8_1_4_1_6_TestBody	3

1.2	Correction to the module RRC_IntraNR_Handover_NR5GC	10

# Corrections required

## Correction to the function fl_TC_8_1_4_1_6_TestBody

| Function name | fl_TC_8_1_4_1_6_TestBody |
| --- | --- |
| Reason for change | 1)In the current implementation of function fl_TC_8_1_4_1_6_TestBody , at step 2 , after sending the reconfiguration to trigger HO , following local configuration messages are missing to address the below requirements : - Reconfiguration to trigger HO will be resulting in SS retransmissions while UE is doing Handover failure based procedure ; To stop this reconfiguration retransmission rb release and reset is required before changing the CRNTI ( otherwise this RRC Configuration retx will still get continued with updated CRNTI and interfere the reestablishment procedure) - To reconfigure DCCH/DTCH DCI on CSS and reconfigure RACH procedure contention-based with UE Contention Resolution Identity MAC CE and with a new C-RNTI ; this configuration is required so that reestablishment can go with DCI on CSS. RRCReestablishment will not be received by the UE if not sent via CSS. - To Reconfigure UL DCCH DCI on USS and stop SR is required before receiving the RRC Setup Request followed by Reestablishment sent with Integrity failure 2) f_NR_RRCRelease_Local function is used after sending the reestablishment with Integrity failure – but UE could send rrc setup request before finishing up with all the timing based release configurations ; To address this issue ,All these local configurations are made with immediate activation , and moved after the reception of Rrc Setup Request ; this is to to avoid the race condition of UE sending Rrc setup request and SS applying Local configurations |
| Summary of change | Before sending Reestablishment message - RB release and configuration are sent so that reconfiguration retx can be avoided - Reconfiguration of DCCH/DTCH DCI on CSS and reconfigure RACH procedure contention-based with UE Contention Resolution Identity MAC CE and with a new C-RNTI are done , so that reestablishment can go with DCI on CSS. RRCReestablishment will not be received by the UE if not sent via CSS. - Reestablishment DL Peer Message is sent with immediate activation Before receiving the RRC Setup request, - Reconfiguration of UL DCCH DCI on USS and stop SR are done - f_NR_RRCRelease_Local is removed , all the configurations are made with immediate activation time after receiving the rrc setup request , this would avoid having the race condition of receiving rrc setup request and local configurations are still getting applied |
| TTCN module | develop/Common/NR5GC/8_1_4/ RRC_IntraNR_Handover_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function fl_TC_8_1_4_1_6_TestBody () runs on NR5GC_PTC { var RNTI_Value v_RNTI_Value := f_NR_CellInfo_GetRNTI(nr_Cell1); var PhysCellId v_PhysCellId := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1); var ShortMAC_I v_ShortMAC_I; var template (value) DL_DCCH_Message v_RRCReconfigurationHO; var NextHopChainingCount v_NextHopChainingCount; var float v_T304Min := f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, 1.0); //T304 is ms1000 according to 38.508-1 Table 4.6.3-19. var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1 := { cs_NR_CellPower(nr_Cell2, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) }; //As per Table 8.1.4.1.6.3.2-1 var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2 := { cs_NR_CellPower(nr_Cell2, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE) }; //As per Table 8.1.4.1.6.3.2-1 var SubFrameTiming_Type v_TimingAtT; var IntegerList_Type v_DrbIdList := f_NR_GetActiveDRBs(nr_Cell1); //@sic R5s201221 sic@ timer t_T304Min; // Reconfigure nr_Cell2 to no respond to any RA Preamble // Table 8.1.4.1.6.3.2-4: The UE attempts to perform the handover using MAC Random Access Preamble on NR Cell 2. SS does not respond. f_NR_SS_RachProcedureConfig(nr_Cell2, cs_NR_RachProcedureConfig_NoResponse); //@siclog "Step 1" siclog@ // The SS changes the power level setting according to the row "T1". f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 2" siclog@ //The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform handover to NR Cell 2. v_RRCReconfigurationHO := f_NR5GC_RRCReconfiguration_IntraNR_HO(nr_Cell2); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, v_RRCReconfigurationHO)); t_T304Min.start(v_T304Min); //EXCEPTION: In parallel to the events described in step 9 the steps specified in Table 8.1.4.1.6.3.2-4 should take place. //Nothing to do nr_Cell2 already configured to not respond to any RA Preamble //@siclog "Step 3" siclog@ //The SS changes the power level setting according to the row "T2". f_NR_SetCellPowerList(v_CellPowerList_AtT2); v_ShortMAC_I := f_NR_Calculate_ShortMAC(nr_Cell1, nr_Cell1); //@siclog "Step 4" siclog@ //Check: Does the UE transmit an RRCReestablishmentRequest message on NR Cell 1? t_T304Min.timeout; SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCReestablishmentRequest(cr_NR_ReestabUE_Identity (v_RNTI_Value, v_PhysCellId, v_ShortMAC_I), handoverFailure))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); //UE re-establish SRB1 - UE releases SRB2 and DRB //In SS: re-establish SRB1 and SRB2 - Release DRBs //@sic R5s201221 sic@ f_NR_SS_SRBs_DRBs_Release(nr_Cell1, cs_TimingInfo_Now, v_DrbIdList); f_NR_SS_SRBs_DRBs_Config(nr_Cell1, cs_TimingInfo_Now, omit, true); //@siclog "Step 5" siclog@ //The SS transmits an RRCReestablishment message to resume SRB1 operation and re-activate security on NR Cell 1. //Note: RRCReestablishment includes a wrong nextHopChainingCount value causing an integrity check failure. //@sic R5-200185 sic@ v_NextHopChainingCount := 7; f_NR_SS_AS_ActivateSecurity_Current(nr_Cell1, 3, -, true); //Use a different value than UE to generate an integrity failure (step 6) @sic R5s201221 sic@ v_TimingAtT := f_NR_GetNextSendOccasion_DL(nr_Cell1); SRB.send (cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_NR(v_TimingAtT), cs_38508_RRCReestablishment_Def(tsc_NR_RRC_TI_Def,v_NextHopChainingCount))); f_NR_RRCRelease_Local(nr_Cell1, v_TimingAtT); //@siclog "Step 6" siclog@ //Check: Does the test result of test procedure in TS 38.508-1 [4] subclause 4.9.5 indicate that the UE has performed mobility registration on NR cell 1 started in NR RRC_IDLE? //@sic R5-200185 sic@ f_NR5GC_CheckCampedOnNewCell(nr_Cell1); }

After Change:

function fl_TC_8_1_4_1_6_TestBody () runs on NR5GC_PTC { var RNTI_Value v_RNTI_Value := f_NR_CellInfo_GetRNTI(nr_Cell1); var PhysCellId v_PhysCellId := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1); var ShortMAC_I v_ShortMAC_I; var NG_NAS_MSG_Indication_Type v_NasInd; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (value) DL_DCCH_Message v_RRCReconfigurationHO; var NextHopChainingCount v_NextHopChainingCount; var float v_T304Min := f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, 1.0); //T304 is ms1000 according to 38.508-1 Table 4.6.3-19. var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1 := { cs_NR_CellPower(nr_Cell2, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) }; //As per Table 8.1.4.1.6.3.2-1 var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2 := { cs_NR_CellPower(nr_Cell2, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE) }; //As per Table 8.1.4.1.6.3.2-1 var SubFrameTiming_Type v_TimingAtT; var IntegerList_Type v_DrbIdList := f_NR_GetActiveDRBs(nr_Cell1); //@sic R5s201221 sic@ timer t_T304Min; var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); // Reconfigure nr_Cell2 to no respond to any RA Preamble // Table 8.1.4.1.6.3.2-4: The UE attempts to perform the handover using MAC Random Access Preamble on NR Cell 2. SS does not respond. f_NR_SS_RachProcedureConfig(nr_Cell2, cs_NR_RachProcedureConfig_NoResponse); //@siclog "Step 1" siclog@ // The SS changes the power level setting according to the row "T1". f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 2" siclog@ //The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform handover to NR Cell 2. v_RRCReconfigurationHO := f_NR5GC_RRCReconfiguration_IntraNR_HO(nr_Cell2); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, v_RRCReconfigurationHO)); t_T304Min.start(v_T304Min); //EXCEPTION: In parallel to the events described in step 9 the steps specified in Table 8.1.4.1.6.3.2-4 should take place. //Nothing to do nr_Cell2 already configured to not respond to any RA Preamble //@siclog "Step 3" siclog@ //The SS changes the power level setting according to the row "T2". f_NR_SetCellPowerList(v_CellPowerList_AtT2); v_ShortMAC_I := f_NR_Calculate_ShortMAC(nr_Cell1, nr_Cell1); //UE re-establish SRB1 - UE releases SRB2 and DRB //In SS: re-establish SRB1 and SRB2 - Release DRBs //In SS: It helps with clearing the RB buffer handling reconfiguration retx //@sic R5s201221 sic@ f_NR_SS_SRBs_DRBs_Release(nr_Cell1, cs_TimingInfo_Now, v_DrbIdList, true); f_NR_SS_SRBs_DRBs_Config(nr_Cell1, cs_TimingInfo_Now, omit, true); // Target Cell: Reconfigure DCCH/DTCH DCI on CSS and reconfigure RACH procedure contention-based with UE Contention Resolution Identity MAC CE and with a new C-RNTI. f_NR_CellInfo_SetRNTI(nr_Cell1, tsc_C_RNTI_Value4); f_NR_SS_C_RNTI_Config(nr_Cell1, tsc_C_RNTI_Value4, -, tsc_NoCnfReq, true); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_RachProcedure_DcchDtchToCSS_Config_REQ(nr_Cell1, cs_TimingInfo_Now, tsc_CnfReq, cs_NR_RachProcedureConfig_CBRA_Msg4Based(v_NR_PhysicalParameters, cs_NR_RachProcedureMsg4WithNoRrcMsg), cs_NR_SearchSpaceDlDciAssignment_CSSType1(v_NR_PhysicalParameters), cs_NR_SearchSpaceUlDciAssignmentCSS(v_NR_PhysicalParameters, cs_NR_DciFormat_0_0_Params))); //@siclog "Step 4" siclog@ //Check: Does the UE transmit an RRCReestablishmentRequest message on NR Cell 1? t_T304Min.timeout; SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCReestablishmentRequest(cr_NR_ReestabUE_Identity (v_RNTI_Value, v_PhysCellId, v_ShortMAC_I), handoverFailure))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); //UE re-establish SRB1 - UE releases SRB2 and DRB //In SS: re-establish SRB1 and SRB2 - Release DRBs //removed and moved after the retransmission of reconfiguration at step2 //@sic R5s201221 sic@ //f_NR_SS_SRBs_DRBs_Release(nr_Cell1, cs_TimingInfo_Now, v_DrbIdList); //f_NR_SS_SRBs_DRBs_Config(nr_Cell1, cs_TimingInfo_Now, omit, true); //@siclog "Step 5" siclog@ //The SS transmits an RRCReestablishment message to resume SRB1 operation and re-activate security on NR Cell 1. //Note: RRCReestablishment includes a wrong nextHopChainingCount value causing an integrity check failure. //@sic R5-200185 sic@ v_NextHopChainingCount := 7; f_NR_SS_AS_ActivateSecurity_Current(nr_Cell1, 3, -, true); //Use a different value than UE to generate an integrity failure (step 6) @sic R5s201221 sic@ //v_TimingAtT := f_NR_GetNextSendOccasion_DL(nr_Cell1); SRB.send (cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, cs_38508_RRCReestablishment_Def(tsc_NR_RRC_TI_Def,v_NextHopChainingCount))); //Reconfigure UL DCCH DCI on USS f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_SearchSpaceUlDciAssignment_REQ(nr_Cell1, cs_TimingInfo_Now, f_NR_SearchSpaceUlDciAssignment(v_NR_PhysicalParameters))); // Stop periodic TA f_NR_ULGrantConfiguration_Stop(nr_Cell1,cs_TimingInfo_Now); //@siclog "Step 6" siclog@ //Check: Does the test result of test procedure in TS 38.508-1 [4] subclause 4.9.5 indicate that the UE has performed mobility registration on NR cell 1 started in NR RRC_IDLE? //@sic R5-200185 sic@ f_NR_RRC_SetupRequest_Def(nr_Cell1); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2A"); //In SS: re-establish SRB1 and SRB2 - Release DRBs //UE re-establish SRB1 - UE releases SRB2 and DRB as part of mobility registration f_NR_SS_RRC_ReleaseSecurity(nr_Cell1,cs_TimingInfo_Now); v_DrbIdList := f_NR_GetActiveDRBs(nr_Cell1); f_NR_SS_SRBs_DRBs_Release(nr_Cell1, cs_TimingInfo_Now, v_DrbIdList, true); f_NR_SS_SRBs_DRBs_Config(nr_Cell1, cs_TimingInfo_Now, omit, true); //@sic R5-202621 sic@ //SS transmit an RRCConnectionSetup message on on the NGC Cell 23. f_NR_RRCSetup_Def (nr_Cell1); //@sic R5-202621 sic@ //Check: Does the UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and a REGISTRATION REQUEST message indicating "mobility registration updating" is sent to update the registration of the actual tracking area? v_NasInd := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_NasInd.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2C"); //@sic R5-201135 sic@ //The SS transmits a SecurityModeCommand message. //The UE transmits a SecurityModeComplete message. f_NR_RRC_ActivateSecurity (nr_Cell1, v_NasInd.SecurityProtection.NasCount); //@sic R5-201135 sic@ //Steps 4-5 of TS 38.508-1 Table 4.9.5.2.2-1 of the generic procedure are performed. f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell1, v_GMM_MobilityInfo); //@siclog "Step 6" siclog@ //removed and the steps are handled in split way as above //Check: Does the test result of test procedure in TS 38.508-1 [4] subclause 4.9.5 indicate that the UE has performed mobility registration on NR cell 1 started in NR RRC_IDLE? //@sic R5-200185 sic@ //f_NR5GC_CheckCampedOnNewCell(nr_Cell1); //WA#WI=906590 }

## Correction to the module RRC_IntraNR_Handover_NR5GC

| Module name | RRC_IntraNR_Handover_NR5GC |
| --- | --- |
| Reason for change | Modules are imported to make the compilation OK to support the changes mentioned in section 1.1 |
| Summary of change | Import modules are done to make the compilation OK |
| TTCN module | develop/Common/NR5GC/8_1_4/ RRC_IntraNR_Handover_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

module RRC_IntraNR_Handover_NR5GC { import from CommonDefs all; import from CommonAspDefs all; import from NR_RRC_ASN1_Definitions language "ASN.1:2002" all with {encode "UNALIGNED_PER_OctetAligned"}; import from NR_Component_Base all; import from NR_Component_NR5GC all; import from NR_CommonDefs all; import from NR_ASP_SrbDefs all; import from Common4G5G_Templates all; import from NR_CellCfg_Templates all; import from NR_RRC_Templates all; import from NR_ASP_Templates_SRB all; import from NR_Measurement_Templates all; import from NR5GC_RRC_Templates all; import from NR_Component_NR5GC all; import from NR_CellInfo all; import from NR_ConfigurationSteps all; import from NG_NAS_SecurityFunctions all; import from NR_SecuritySteps all; import from EUTRA_NR_SecurityFunctions all; import from NR_SecurityFunctions all; import from NG_SecurityDefinitionsAndExternalFunctions all; import from NG_NasEmu_CtrlFunctions all; import from NasEmu_Common4G5G all; import from NR_Measurement_Functions all; import from NR5GC_CommonFunctions all; import from NR5GC_NASSteps all; import from NR5GC_Handover all; import from NR5GC_RRCSteps all; import from NR_Timing all; import from NG_NAS_TypeDefs all; import from NG_NAS_Common all; import from NG_NAS_Templates all; import from NAS_CommonTemplates all; import from NG_NASTemplateFunctions all;

After Change:

module RRC_IntraNR_Handover_NR5GC { import from CommonDefs all; import from CommonAspDefs all; import from NR_RRC_ASN1_Definitions language "ASN.1:2002" all with {encode "UNALIGNED_PER_OctetAligned"}; import from NR_Component_Base all; import from NR_Component_NR5GC all; import from NR_CommonDefs all; import from NR_ASP_SrbDefs all; import from Common4G5G_Templates all; import from NR_CellCfg_Templates all; import from NR_RRC_Templates all; import from NR_ASP_Templates_SRB all; import from NR_Measurement_Templates all; import from NR5GC_RRC_Templates all; import from NR_Component_NR5GC all; import from NR_CellInfo all; import from NR_ConfigurationSteps all; import from NG_NAS_SecurityFunctions all; import from NR_SecuritySteps all; import from EUTRA_NR_SecurityFunctions all; import from NR_SecurityFunctions all; import from NG_SecurityDefinitionsAndExternalFunctions all; import from NG_NasEmu_CtrlFunctions all; import from NasEmu_Common4G5G all; import from NR_Measurement_Functions all; import from NR5GC_CommonFunctions all; import from NR5GC_NASSteps all; import from NR5GC_Handover all; import from NR5GC_RRCSteps all; import from NR_Timing all; import from NG_NAS_TypeDefs all; import from NG_NAS_Common all; import from NG_NAS_Templates all; import from NAS_CommonTemplates all; import from NG_NASTemplateFunctions all; import from NR_DCI_Configuration all; //WA#WI=906590 import from NG_NAS_MsgContainers all; //WA#WI=906590