3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220202

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0774 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_IMS_CallWithSessionTimer_CheckInviteAfter422 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test case implemention requires that Call-ID, CSeq, To, and From headers are included in INVITE after message 422 is sent. According to RFC 4028 on Session Timer says in clause 7.3, UE may start a new dialog then all header checks mentioned above will not be required. This document is submitted along with a Prose CR for 34.229-5, where RFC 4028 is detailed and INVITE message contents have been changed accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed Call-ID, CSeq, To, and From header checks on INVITE after 422 message. Note : A Prose CR will be riased at RAN5#94e |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.27.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 34.229-5 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | IMS testcase 22.1(legacy) is also affected |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	3

1.1	Correction to function f_IMS_CallWithSessionTimer_CheckInviteAfter422	3

# Corrections required

## Correction to function f_IMS_CallWithSessionTimer_CheckInviteAfter422

| Function name | f_IMS_CallWithSessionTimer_CheckInviteAfter422 |
| --- | --- |
| Reason for change | Test case implemention requires that Call-ID, CSeq, To, and From headers are included in INVITE after message 422 is sent. According to RFC 4028 on Session Timer says in clause 7.3, UE may start a new dialog then Call-ID, CSeq, To, and From header checks will not be required. |
| Summary of change | Removed Call-ID, CSeq, To, and From header checks on INVITE after 422 message. |
| TTCN module | IMS_NR5GC\NR5GC\IMS_Testsuite_NR5GC.ttcn |
| MCC160 Comment | Accepted according to approval of R5-220211, R5-220231 |

Before Change:

<<SKIPPED CODE>> function f_IMS_CallWithSessionTimer_CheckInviteAfter422(IMS_InviteRequestWithSdp_Type p_InviteRequestWithSdp, MessageHeader p_MessageHeaderPrevInvite, integer p_DeltaSec) runs on IMS_PTC { var MessageHeader v_MessageHeaderReceived := p_InviteRequestWithSdp.Invite.msgHeader; var template (present) MessageHeader v_MessageHeaderExpected := cr_MessageHeader_Dummy; var CSeq v_PrevCSeq := p_MessageHeaderPrevInvite.cSeq; v_MessageHeaderExpected.callId := p_MessageHeaderPrevInvite.callId; v_MessageHeaderExpected.fromField := f_IMS_Response_FromHeaderRX(p_MessageHeaderPrevInvite); v_MessageHeaderExpected.toField := cr_ToDef(f_Addr_Union_GetSipUrl(p_MessageHeaderPrevInvite.toField.addressField)); v_MessageHeaderExpected.cSeq := cr_CseqDef(v_PrevCSeq.seqNumber + 1, v_PrevCSeq.method); v_MessageHeaderExpected.sessionExpires := cr_SessionExpires(p_DeltaSec); v_MessageHeaderExpected.minSE := cr_MinSE(p_DeltaSec); if (not match(v_MessageHeaderReceived, v_MessageHeaderExpected)) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "INVITE not as expected according to previous INVITE and/or delta-sec value"); } } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function f_IMS_CallWithSessionTimer_CheckInviteAfter422(IMS_InviteRequestWithSdp_Type p_InviteRequestWithSdp, MessageHeader p_MessageHeaderPrevInvite, integer p_DeltaSec) runs on IMS_PTC { var MessageHeader v_MessageHeaderReceived := p_InviteRequestWithSdp.Invite.msgHeader; var template (present) MessageHeader v_MessageHeaderExpected := cr_MessageHeader_Dummy; var CSeq v_PrevCSeq := p_MessageHeaderPrevInvite.cSeq; <<REMOVED CODE>> /* v_MessageHeaderExpected.callId := p_MessageHeaderPrevInvite.callId; v_MessageHeaderExpected.fromField := f_IMS_Response_FromHeaderRX(p_MessageHeaderPrevInvite); v_MessageHeaderExpected.toField := cr_ToDef(f_Addr_Union_GetSipUrl(p_MessageHeaderPrevInvite.toField.addressField)); v_MessageHeaderExpected.cSeq := cr_CseqDef(v_PrevCSeq.seqNumber + 1, v_PrevCSeq.method);*/ <<REMOVED CODE>> v_MessageHeaderExpected.sessionExpires := cr_SessionExpires(p_DeltaSec); v_MessageHeaderExpected.minSE := cr_MinSE(p_DeltaSec); if (not match(v_MessageHeaderReceived, v_MessageHeaderExpected)) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "INVITE not as expected according to previous INVITE and/or delta-sec value"); } } <<SKIPPED CODE>>