3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220214

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2354 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function fl_TC_8_2_4_1_1_x_ENDC_NR_TestBody |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to 38.331 cl 6.2.2, RadioLinkMonitoringConfigRadioLinkMonitoringRSpurpose can only be initialized to value BeamFailure for a Scell. In the current TTCN implementation of function fl_TC_8_2_4_1_1_x_ENDC_NR_TestBody this needs to be initialized. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Declared and initialized a new variable v_RadioLinkMonitoringConfig, and then assigned v_RadioLinkMonitoringConfig.setup.failureDetectionResourcesToAddModList[0] := cs_RadioLinkMonitoringRS(0,beamFailure,0); during ServingCellConfig initialization. Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.4.1.1.1.ENDC, 8.2.4.1.1.3.ENDC. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

1	Corrections required	2

1.1	Change 1	2

# Corrections required

## Change 1

| Function name | fl_TC_8_2_4_1_1_x_ENDC_NR_TestBody() |
| --- | --- |
| Reason for change | According to the definition of RadioLinkMonitoringConfig in 38331. For SCell, network only configures the value of purpose to beamFailure. |
| Summary of change | Declared and initialized a new variable v_RadioLinkMonitoringConfig, and then assigned v_RadioLinkMonitoringConfig.setup.failureDetectionResourcesToAddModList[0] := cs_RadioLinkMonitoringRS(0,beamFailure,0); during ServingCellConfig initialization. Please see below. |
| TTCN module | RRC_CarrierAggregation_ENDC_NR.ttcn |
| MCC160 Comment | Postponed conditional to prose CR agreement on 38.523-1 |

Before change:

function fl_TC_8_2_4_1_1_x_ENDC_NR_TestBody(NR_CellId_Type  p_PSCell,

NR_CellId_Type  p_SCell) runs on NR_BASE_PTC

{

var template(value) CellGroupConfig v_SCGConfig;

var template (value) BWP_DownlinkDedicated v_BWP_DownlinkDedicated;

var SearchSpace v_SearchSpaceUSS := f_NR_CellInfo_GetSearchSpaceConfigAtUE(p_SCell, tsc_NR_SearchSpaceId_USS, tsc_NR_BWP_Id);

var template (value) ServingCellConfig v_ServingCellConfig;

var NR_DownlinkBWP_Type v_NR_DownlinkBWP := f_NR_CellInfo_GetDownlinkBWP(p_SCell, tsc_NR_BWP_Id);

var CellGroupId v_CellGroupId := f_NR_CellInfo_Get_CellGroupId(p_PSCell); //@sic R5s201387 sic@

//@siclog "Steps 1 - 2" siclog@

f_NR_AddMod_1SCell_ENDC(p_PSCell, p_SCell);

//@siclog "Step 2A" siclog@

//Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.1-1A indicate that the UE is capable of exchanging IP data on DRB #n associated with the first PDU session on NR Cell 3?

//@sic R5-203666 sic@

f_NR_CheckDataPath_CA(p_PSCell, p_SCell, "2A", true,tsc_SCellIndex_2);

//@siclog "Steps 3 - 4" siclog@

//The SS transmits a RRCConnectionReconfiguration message including NR RRCReconfiguration message to modify the startingBitOfFormat2-3 of the NR SCell

v_BWP_DownlinkDedicated:= cs_38508_BWP_DownlinkDedicated(cs_38508_PDCCH_Config (v_SearchSpaceUSS, cs_PDCCH_TPC_SRS1),

v_NR_DownlinkBWP.Pdsch.ConfigDedicated.R15,

-,

f_NR_CellInfo_GetRadioLinkMonitoringConfig(p_SCell));

v_ServingCellConfig :=  cs_NR_ServingCellConfigDef(v_BWP_DownlinkDedicated,

omit,  //@sic R5s2106184 sic@

tsc_NR_BWP_Id,

omit,                 //UplinkConfig

cs_NR_Pdcch_ServingCellConfigSetup(cs_38508_PDCCH_ServingCellConfig),

cs_NR_Pdsch_ServingCellConfigSetup(cs_38508_PDSCH_ServingCellConfig));

v_SCGConfig := cs_38508_CellGroupConfig(v_CellGroupId,

omit,  //p_RLC_Bearer_Config

omit,  //RLC_BearerToRelease

omit,  //MAC_CellGroupConfig

omit,  //PhysicalCellGroupConfig

omit,  //SpCellConfig

{cs_SCellConfig_Def(omit, v_ServingCellConfig, tsc_SCellIndex_2)});

f_NR_SendRRCReconfigurationContentsToEUTRA(omit, v_SCGConfig);

//Nothing to do in SS as SRS is an uplink configuration

//@siclog "Step 4A" siclog@

//Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.1-1A indicate that the UE is capable of exchanging IP data on DRB #n associated with the first PDU session on NR Cell 3?

//@sic R5-203666 sic@

f_NR_CheckDataPath_CA(p_PSCell, p_SCell, "4A", false,tsc_SCellIndex_2);

//@siclog "Steps 5 - 6" siclog@

f_NR_Release_1SCell_ENDC(p_PSCell, p_SCell, tsc_SCellIndex_2);

}

After change:

function fl_TC_8_2_4_1_1_x_ENDC_NR_TestBody(NR_CellId_Type  p_PSCell,

NR_CellId_Type  p_SCell) runs on NR_BASE_PTC

{

var template(value) CellGroupConfig v_SCGConfig;

var template (value) BWP_DownlinkDedicated v_BWP_DownlinkDedicated;

var SearchSpace v_SearchSpaceUSS := f_NR_CellInfo_GetSearchSpaceConfigAtUE(p_SCell, tsc_NR_SearchSpaceId_USS, tsc_NR_BWP_Id);

var template (value) ServingCellConfig v_ServingCellConfig;

var NR_DownlinkBWP_Type v_NR_DownlinkBWP := f_NR_CellInfo_GetDownlinkBWP(p_SCell, tsc_NR_BWP_Id);

var CellGroupId v_CellGroupId := f_NR_CellInfo_Get_CellGroupId(p_PSCell); //@sic R5s201387 sic@

var template (omit) NR_BWP_DownlinkDedicated_RadioLinkMonitoringConfig_Type v_RadioLinkMonitoringConfig := f_NR_CellInfo_GetRadioLinkMonitoringConfig(p_SCell);

//@siclog "Steps 1 - 2" siclog@

f_NR_AddMod_1SCell_ENDC(p_PSCell, p_SCell);

//@siclog "Step 2A" siclog@

//Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.1-1A indicate that the UE is capable of exchanging IP data on DRB #n associated with the first PDU session on NR Cell 3?

//@sic R5-203666 sic@

f_NR_CheckDataPath_CA(p_PSCell, p_SCell, "2A", true,tsc_SCellIndex_2);

v_RadioLinkMonitoringConfig.setup.failureDetectionResourcesToAddModList[0] := cs_RadioLinkMonitoringRS(0,beamFailure,0);

//@siclog "Steps 3 - 4" siclog@

//The SS transmits a RRCConnectionReconfiguration message including NR RRCReconfiguration message to modify the startingBitOfFormat2-3 of the NR SCell

v_BWP_DownlinkDedicated:= cs_38508_BWP_DownlinkDedicated(cs_38508_PDCCH_Config (v_SearchSpaceUSS, cs_PDCCH_TPC_SRS1),

v_NR_DownlinkBWP.Pdsch.ConfigDedicated.R15,

-,

v_RadioLinkMonitoringConfig);

v_ServingCellConfig :=  cs_NR_ServingCellConfigDef(v_BWP_DownlinkDedicated,

omit,  //@sic R5s2106184 sic@

tsc_NR_BWP_Id,

omit,                 //UplinkConfig

cs_NR_Pdcch_ServingCellConfigSetup(cs_38508_PDCCH_ServingCellConfig),

cs_NR_Pdsch_ServingCellConfigSetup(cs_38508_PDSCH_ServingCellConfig));

v_SCGConfig := cs_38508_CellGroupConfig(v_CellGroupId,

omit,  //p_RLC_Bearer_Config

omit,  //RLC_BearerToRelease

omit,  //MAC_CellGroupConfig

omit,  //PhysicalCellGroupConfig

omit,  //SpCellConfig

{cs_SCellConfig_Def(omit, v_ServingCellConfig, tsc_SCellIndex_2)});

f_NR_SendRRCReconfigurationContentsToEUTRA(omit, v_SCGConfig);

//Nothing to do in SS as SRS is an uplink configuration

//@siclog "Step 4A" siclog@

//Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.1-1A indicate that the UE is capable of exchanging IP data on DRB #n associated with the first PDU session on NR Cell 3?

//@sic R5-203666 sic@

f_NR_CheckDataPath_CA(p_PSCell, p_SCell, "4A", false,tsc_SCellIndex_2);

//@siclog "Steps 5 - 6" siclog@

f_NR_Release_1SCell_ENDC(p_PSCell, p_SCell, tsc_SCellIndex_2);

}