3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220215

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0776 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IMS Emergency call test case 10.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To verify IMS 5GS test case 10.3 using IWD 21wk49 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to IMS 5GS Emgency call test case 10.3 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

3	Branches executed	6

4	Execution Log Files	6

HiSilicon	6

Samsung	6

5	References	6

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of IMS test case 10.3 which is part of the iwd-TTCN3-B2020-09_D21wk49.zip test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case:	10.3.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D21wk49.zip

System Simulator used:	Rohde & Schwarz 5G Protocol Conformance Test platform

UE used:	Hisilicon Balong 5000 UE and Samsung (SM-G996U) UE

Verification Status:	PASS

# Corrections required

  - Correction to function f_TC_10_3_NR5GC_IMS2

| Function name | f_TC_10_3_NR5GC_IMS2 |
| --- | --- |
| Reason for change | Change 1: Coordination message needs to be send to IPCAN to inform that Emergency Call has been established at step 15. Change 2: ContactUrlUE is trying to be assigned even with no dialog for emergency call has been stablished. Change 3: Test configuration when end emergency call should be IPCAN_SpeechAndEmergencyCall, as current scenario when emergency call is placed, there is a normal call “On Hold”. Change 4: Coordination message needs to be send to IPCAN to inform that Emergency Call has been ended at step 17 and PDU session needs to be released. |
| Summary of change | Change 1: Added CoOrdMsg send to inform IPCAN that emergency call has been stablished. Change 2: Moved ContactUrlUE assigment after emergency call has been stablished Change 3: change IPCAN_EmergencyCall to IPCAN_SpeechAndEmergencyCall as test configuration Change 4: Added CoOrdMsg send to inform IPCAN that emergency call has been released. |
| TTCN module | IMS_NR5GC_EmergencyCallTestcases.ttcn |
| MCC160 Comment | 1. Accepted. Used function f_IMS_EmergencyCallSetup_NR5GC_A_6_Step1 instead which includes the required coordination message. See additional changes. 2. Accepted 3. Accepted 4. Accepted. |

Before

<<SKIPPED CODE>>

function f_TC_10_3_NR5GC_IMS2() runs on IMS_PTC

{ // Emergency call with emergency registration / Emergency SIP signalling and media in parallel with another ongoing IM CN subsystem signalling and media / 5GS

var IMS_DATA_REQ v_IMS_DATA_REQ;

var SipUrl v_ContactUrlUE;

<<MOVED CODE>>

v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrl(dialog);

<<MOVED CODE>>

f_IMS_PTC_Init(PDN_2);

f_IMS_IMS_WaitForTrigger(OtherPDN);

// @siclog "Step 6" siclog@

// UE is made to initiate an emergency call

f_IMS_CC_StartCall(IPCAN_EmergencyCall_NormalService);

// @siclog "Step 7-10" siclog@

// Check: Does the UE initiate and complete the IMS emergency registration? (Steps 1-4 of Annex A.3)

f_IMS_EmergencyRegistration();

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 10");

// Emergency call setup: 34.229-5 Annex A.6

// @siclog "Step 11" siclog@

// Check: Does the UE send correctly composed INVITE request for emergency call? (Step 1 of Annex A.6)

v_IMS_DATA_REQ := f_IMS_EmergencyCallSetup_Step1(-); // INVITE for an Emergency call

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 11");

// @siclog "Step 12-14" siclog@

// SS sends 100 Trying, followed by 180 Ringing, followed by 200 OK. (Steps 2-4 of Annex A.6)

// @siclog "Step 15" siclog@

// Check: Does the UE acknowledge? (Step 5 of Annex A.6)

f_IMS_EmergencyCallSetup_Step2ToEnd(v_IMS_DATA_REQ);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 15");

// @siclog "Step 16" siclog@

// SS sends BYE to release the emergency call. (Step 1 of Annex A.8)

// @siclog "Step 17" siclog@

// Check: Does the UE send 200 OK response? (Step 2 of Annex A.8)

f_IMS_CallReleaseMT(v_ContactUrlUE);

f_IMS_CC_EndCall(IPCAN_EmergencyCall_NormalService, IPCAN_EmergencyCall);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 17");

// @siclog "Step 18" siclog@

// UE is made to resume the original voice call

f_UT_CallResume(MMI);

}

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

function f_TC_10_3_NR5GC_IMS2 () runs on IMS_PTC

{ // Emergency call with emergency registration / Emergency SIP signalling and media in parallel with another ongoing IM CN subsystem signalling and media / 5GS

var IMS_DATA_REQ v_IMS_DATA_REQ;

var SipUrl v_ContactUrlUE;

f_IMS_PTC_Init(PDN_2);

f_IMS_IMS_WaitForTrigger(OtherPDN);

// @siclog "Step 6" siclog@

// UE is made to initiate an emergency call

f_IMS_CC_StartCall(IPCAN_EmergencyCall_NormalService);

// @siclog "Step 7-10" siclog@

// Check: Does the UE initiate and complete the IMS emergency registration? (Steps 1-4 of Annex A.3)

f_IMS_EmergencyRegistration();

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 10");

// Emergency call setup: 34.229-5 Annex A.6

// @siclog "Step 11" siclog@

// Check: Does the UE send correctly composed INVITE request for emergency call? (Step 1 of Annex A.6)

v_IMS_DATA_REQ := f_IMS_EmergencyCallSetup_Step1(-); // INVITE for an Emergency call

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 11");

// @siclog "Step 12-14" siclog@

// SS sends 100 Trying, followed by 180 Ringing, followed by 200 OK. (Steps 2-4 of Annex A.6)

// @siclog "Step 15" siclog@

// Check: Does the UE acknowledge? (Step 5 of Annex A.6)

f_IMS_EmergencyCallSetup_Step2ToEnd(v_IMS_DATA_REQ);

// Inform NR: Emergency Call established

f_IMS_IPCAN_SendCoOrdMsg(IPCAN);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 15");

// @siclog "Step 16" siclog@

// SS sends BYE to release the emergency call. (Step 1 of Annex A.8)

// @siclog "Step 17" siclog@

// Check: Does the UE send 200 OK response? (Step 2 of Annex A.8)

v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrl(dialog);

f_IMS_CallReleaseMT(v_ContactUrlUE);

f_IMS_CC_EndCall(IPCAN_EmergencyCall_NormalService, IPCAN_SpeechAndEmergencyCall);

// f_NR5GC_EmergencyCallRelease(nr_Cell1);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 17");

f_IMS_IPCAN_SendCoOrdMsg(IPCAN);

// @siclog "Step 18" siclog@

// UE is made to resume the original voice call

f_UT_CallResume(MMI);

}

<<SKIPPED CODE>>

  - Correction to function f_TC_10_3_NR5GC_IMS1

| Function name | f_TC_10_3_NR5GC_IMS1 |
| --- | --- |
| Reason for change | Call release procedure for normal call is missing after step 22 |
| Summary of change | Added release call procedure after step 22 |
| TTCN module | IMS_NR5GC_EmergencyCallTestcases.ttcn |
| MCC160 Comment | Accepted Prose change: R5-220214. |

Before

<<SKIPPED CODE>>

function f_TC_10_3_NR5GC_IMS1() runs on IMS_PTC

{

…

// @siclog "Step 19-22" siclog@

// Check: Does the UE send re-INVITE and completes resumption of the original voice call? (Steps 1-4 of Annex A.17)

f_IMS_MOCallHoldResume(RESUME, v_InviteRequest, v_CallInfo.MessageHeader183, v_CallInfo.LatestSdpOffer);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 22");

f_IMS_TestBody_Set(false);

f_IMS_CC_Postamble(IPCAN_MO_SpeechCall);

…

}  <<SKIPPED CODE>>

After

<<SKIPPED CODE>>

function f_TC_10_3_NR5GC_IMS1() runs on IMS_PTC

{

…

// @siclog "Step 19-22" siclog@

// Check: Does the UE send re-INVITE and completes resumption of the original voice call? (Steps 1-4 of Annex A.17)

f_IMS_MOCallHoldResume(RESUME, v_InviteRequest, v_CallInfo.MessageHeader183, v_CallInfo.LatestSdpOffer);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 22");

f_IMS_CallReleaseMO(v_InviteRequest);

f_IMS_TestBody_Set(false);

f_IMS_CC_Postamble(IPCAN_MO_SpeechCall);

}

<<SKIPPED CODE>>

Additional change from MCC160

1. Wrapper functions for Emergency Call Setup procedure 34.229-5 A.6 step 1 resp. 2ToEnd

2. Wrapper function used in step 11 and step 15

3. Convert number to dial in f_UT_RequestIMSCall to a URI

1. Wrapper functions for Emergency Call Setup procedure 34.229-5 A.6 step 1 resp. 2ToEnd

/*

* @desc      34.229-5 A.6

* @param     p_Context           (default value: A_2_1_A7)

* @param     p_GeolocationInfoAvailable (default value: false)

* @param     p_OtherCalleeUri    (default value: omit)

* @return    INVITE_Request

* @status    APPROVED (NR5GC)

*/

function f_IMS_EmergencyCallSetup_NR5GC_A_6(INVITE_A_2_1_Context_Type p_Context := A_2_1_A7,

boolean p_GeolocationInfoAvailable := false, // @sic R5-220214 sic@

template SipUrl p_OtherCalleeUri := omit) runs on IMS_PTC return INVITE_Request

{

var IMS_DATA_REQ v_IMS_DATA_REQ;

v_IMS_DATA_REQ := f_IMS_EmergencyCallSetup_NR5GC_A_6_Step1(p_Context, p_GeolocationInfoAvailable, p_OtherCalleeUri);

return f_IMS_EmergencyCallSetup_NR5GC_A_6_Step2ToEnd(v_IMS_DATA_REQ, p_GeolocationInfoAvailable); // @sic R5s210875 sic@

}

/*

* @desc      34.229-5 A.6 Step 1

* @param     p_Context           (default value: A_2_1_A7)

* @param     p_GeolocationInfoAvailable (default value: false)

* @param     p_OtherCalleeUri    (default value: omit)

* @return    IMS_DATA_REQ

* @status    APPROVED (NR5GC)

*/

function f_IMS_EmergencyCallSetup_NR5GC_A_6_Step1(INVITE_A_2_1_Context_Type p_Context := A_2_1_A7,

boolean p_GeolocationInfoAvailable := false, // @sic R5-220214 sic@

template SipUrl p_OtherCalleeUri := omit) runs on IMS_PTC return IMS_DATA_REQ

{ // This is the same as step 1 of 34.229-1 C.22

return f_IMS_EmergencyCallSetup_Step1 (p_Context,

p_GeolocationInfoAvailable,

p_OtherCalleeUri);

}

/*

* @desc      Emergency speech call setup according to 34.229-5 Annex A.6 (apart from Invite)

* @param     p_IMS_DATA_REQ

* @param     p_GeolocationInfoAvailable (default value: false)

* @return    INVITE_Request

* @status    APPROVED (NR5GC)

*/

function f_IMS_EmergencyCallSetup_NR5GC_A_6_Step2ToEnd(IMS_DATA_REQ p_IMS_DATA_REQ,

boolean p_GeolocationInfoAvailable := false) runs on IMS_PTC return INVITE_Request // @sic R5-220214 sic@

{

f_IMS_IPCAN_SendCoOrdMsg(IPCAN);

return f_IMS_EmergencyCallSetup_Step2ToEnd (p_IMS_DATA_REQ, p_GeolocationInfoAvailable);

}

2. Wrapper function used in step 11 and step 15

<<SKIPPED CODE>>

function f_TC_10_3_NR5GC_IMS2() runs on IMS_PTC

{

…

// Emergency call setup: 34.229-5 Annex A.6

// @siclog "Step 11" siclog@

// Check: Does the UE send correctly composed INVITE request for emergency call? (Step 1 of Annex A.6)

v_IMS_DATA_REQ := f_IMS_EmergencyCallSetup_NR5GC_A_6_Step1(); // INVITE for an Emergency call @sic R5s220215 sic@

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 11");

// @siclog "Step 12-14" siclog@

// SS sends 100 Trying, followed by 180 Ringing, followed by 200 OK. (Steps 2-4 of Annex A.6)

// @siclog "Step 15" siclog@

// Check: Does the UE acknowledge? (Step 5 of Annex A.6)

f_IMS_EmergencyCallSetup_NR5GC_A_6_Step2ToEnd(v_IMS_DATA_REQ); // @sic R5s220215 sic@

// Inform NR: Emergency Call established

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 15");

// @siclog "Step 16" siclog@

…

<<SKIPPED CODE>>

3. Convert number to dial in f_UT_RequestIMSCall to a URI

function f_UT_RequestIMSCall(UT_PTC_MTC_PORT p_Port,

charstring p_NumToDial := px_IMS_CalleeUri,

Ut_CnfReq_Type p_CnfRequired := NO_CNF_REQUIRED)

{

var boolean v_DontWaitForCnf := true;

var charstring v_NumToDial := p_NumToDial;

if (regexp (p_NumToDial, "([0-9]+)", 0) == "") { // If string is a number, have to convert this into a URI

v_NumToDial := "tel:" & v_NumToDial;

}

fl_UT_ApplyCommand(p_Port, cas_UT_Req(REQUEST_IMS_VOICE_CALL, p_CnfRequired, {cs_Ut_Parameter_Uri(v_NumToDial)}), v_DontWaitForCnf); // @sic R5-134290, R5-145723, R5s160855 sic@

}

# Branches executed

The test case was executed on NR band n78 using NR ciphering algorithm nea1 and integrity algorithm nia1on Rohde & Schwarz 5G Protocol Conformance Test platform with Hisilicon Balong 5000 UE

The test case was executed on NR band n41 using NR ciphering algorithm nea1 and integrity algorithm nia1 on Rohde & Schwarz 5G Protocol Conformance Test platform with Samsung SM-G996U UE

# Execution Log Files

## HiSilicon

The HiSilicon Balong 5000 UE passed this test case on Rohde & Schwarz 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_10.3_Hisilicon.log

(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file (in .html format) the complete test case execution can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

## Samsung

The Samsung (SM-G996U) UE passed this test case on Rohde & Schwarz 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_10.3_Samsung.log

(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file (in .html format) the complete test case execution can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s220216: Supporting information for Addition of IMS Emergency call test case 10.3 |
| --- | --- |