3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220235

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0777 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IMS 5GS Call Control test case 7.16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-01-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To verify IMS 5GS test case 7.16 using IWD 21wk49 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to IMS 5GS test case 7.16 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.16.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

3	Branches executed	8

4	Execution Log Files	8

HiSilicon	8

5	References	8

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of IMS test case 7.16 which is part of the iwd-TTCN3-B2020-09_D21wk49.zip test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case:	7.16.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D21wk49.zip

System Simulator used:	Rohde & Schwarz 5G Protocol Conformance Test platform

UE used:	Hisilicon Balong 5000 UE

Verification Status:	PASS

# Corrections required

  - Correction to function f_TC_7_16_NR5GC_IMS1

| Function name | f_TC_7_16_NR5GC_IMS1 |
| --- | --- |
| Reason for change | Change 1: Preamble TestConfiguration set is for audio call test cases while this is a video call test case. Change 2: Video codec input parameter to create the SPD offer is set to H264, H265 is the required codec for NR video test cases. |
| Summary of change | Change 1: Changed TestConfiguration to video call. Change 2: Changed video codec input parameter to to build the SDP offer to H265. |
| TTCN module | IMS_NR5GC\IMS_Testsuite_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before

<<SKIPPED CODE>>

function f_TC_7_16_NR5GC_IMS1() runs on IMS_PTC

{ // MTSI MT Video call with preconditions at both originating UE and terminating UE / 5GS

var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp;

var IMS_ResponseWithSdp_Type v_ResponseWithSdp;

var template (value) SDP_Message v_SdpOffer;

var SipUrl v_ContactUrl;

f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED);

f_IMS_TestBody_Set(true);

// @siclog "Step 0A - 0H" siclog@

f_IMS_CC_StartCall(IPCAN_MT_VideoCall);

// @siclog "Step 1" siclog@                 INVITE

v_SdpOffer := f_IMS_BuildSDP_InitialOfferAudioVideo_TX(tsc_IMS_SDP_AudioCodecList_EVS_WBAMR_AMR, tsc_IMS_SDP_VideoCodecList_H264_only);

v_InviteRequestWithSdp := f_IMS_MTCallSetup_Video_SendInvite(SetupVideoCall, v_SdpOffer);

// @siclog "Step 2 - 3" siclog@             100 Trying, 183 Session Progress

v_ResponseWithSdp := f_IMS_MTCallSetup_Video_Receive183(SetupVideoCall, v_InviteRequestWithSdp, EVS_16000, H265_90000);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 3");

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

function f_TC_7_16_NR5GC_IMS1() runs on IMS_PTC

{ // MTSI MT Video call with preconditions at both originating UE and terminating UE / 5GS

var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp;

var IMS_ResponseWithSdp_Type v_ResponseWithSdp;

var template (value) SDP_Message v_SdpOffer;

var SipUrl v_ContactUrl;

f_IMS_CC_Preamble(IPCAN_VideoCall, IMS_REGISTERED);

f_IMS_TestBody_Set(true);

// @siclog "Step 0A - 0H" siclog@

f_IMS_CC_StartCall(IPCAN_MT_VideoCall);

// @siclog "Step 1" siclog@                 INVITE

v_SdpOffer := f_IMS_BuildSDP_InitialOfferAudioVideo_TX(tsc_IMS_SDP_AudioCodecList_EVS_WBAMR_AMR, tsc_IMS_SDP_VideoCodecList_H265_only);

v_InviteRequestWithSdp := f_IMS_MTCallSetup_Video_SendInvite(SetupVideoCall, v_SdpOffer);

// @siclog "Step 2 - 3" siclog@             100 Trying, 183 Session Progress

v_ResponseWithSdp := f_IMS_MTCallSetup_Video_Receive183(SetupVideoCall, v_InviteRequestWithSdp, EVS_16000, H265_90000);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 3");

<<SKIPPED CODE>>

  - Correction to function f_IMS_BuildSDP_MTCallAudioVideo_RX

| Function name | f_IMS_BuildSDP_MTCallAudioVideo_RX |
| --- | --- |
| Reason for change | Function f_IMS_BuildSDP_MTCallAudioVideo_RX is used for both Legacy and NR test cases. Current implementation sets H264 codec in case of the test case run has video media. This needs to be corrected to set H264 in case of a Legacy test case is run and H265 in case a NR test case is run. |
| Summary of change | Set adecuate video codec, H264 or H265, depending on test case needs. |
| TTCN module | Common\IMS\IMS_SDP_MessagesVideo.ttcn |
| MCC160 Comment | Accepted |

Before

<<SKIPPED CODE>>

function f_IMS_BuildSDP_MTCallAudioVideo_RX(VideoCallTypeMT_Type p_VideoCallType,

template (omit) SDP_KindOfAnswer_Type p_KindOfAnswer := omit,

IMS_SDP_AudioCodec_Type p_AudioCodec := AMR_16000,

IMS_SDP_VideoCodec_Type p_VideoCodec := H264_90000,

boolean p_WithPreconditions := true)

runs on IMS_PTC

return template (present) SDP_Message

{ /* @sic R5-172930: function restructured to cope with different flavours ('normal' IMS, WLAN; Setup video call, add video, remove video; INVITE, UPDATE) sic@ */

/* @sic R5-172930: p_FmtAudio, p_FmtVideo removed sic@ */

/* @sic R5-213524: p_AudioCodec, p_VideoCodec, p_WithPreconditions sic@ */

var template (present) SDP_attribute_rtpmap_codec v_AudioCodec;

var template (present) SDP_attribute_list v_MediaAttributes_Audio;

var template (present) SDP_attribute_list v_MediaAttributes_Video;

var template (present) SDP_attribute_list v_PreconditionAttributes_Audio := {}; // @sic R5-170782 sic@

var template (present) SDP_attribute_list v_PreconditionAttributes_Video := {}; // @sic R5-170782 sic@

var template (present) SDP_attribute_list v_PreconditionAttributes_Final := cr_SDP_PreconditionAttributes(c_sendrecv, c_sendrecv, c_mandatory, c_mandatory);  /* preconditions when resources are fully available */

var template (present) SDP_attribute_fmtp_format_specific_params v_FmtpParams_Audio := ?;

var template (present) SDP_attribute_fmtp_format_specific_params v_FmtpParams_Video := ?;

var charstring v_FmtAudio;

var charstring v_FmtVideo := tsc_SDP_FmtVideo;               /* @sic R5-197050: tsc_SDP_FmtVideo instead of fl_IMS_BuildSDP_FmtVideo sic@ */

var boolean v_IsWLAN := f_IMS_PTC_RanTypeIsWLAN();

var boolean v_IsInitialAnswer := match(InitialAnswer, p_KindOfAnswer);

v_AudioCodec := f_SDP_AttributeRtpmapAudioRX(p_AudioCodec);  // @sic R5-168058; R5-168142 sic@

v_FmtAudio := f_SDP_AudioCodec_GetFmt(p_AudioCodec);         /* @sic R5-197050: fix mapping of fmt values sic@ */

if (p_WithPreconditions) {

if (not v_IsWLAN and v_IsInitialAnswer) {  // i.e. 183 Session Progress (C.26 step 4 or 17.2 step 3)

v_PreconditionAttributes_Video := cr_SDP_PreconditionAttributes((c_none, c_sendrecv), c_none, c_mandatory, c_mandatory);

v_PreconditionAttributes_Video := f_SDP_Attributes_Add_RX(v_PreconditionAttributes_Video, cr_SDP_Attribute_conf_qos);         // @sic R5-216443: NR5GC sic@

if (p_VideoCallType == SetupVideoCall) {                 // C.26 step 4: same preconditions for video and audio

v_PreconditionAttributes_Audio := v_PreconditionAttributes_Video;

} else {                                                 // 17.2 step 3: audio is there already

v_PreconditionAttributes_Audio := v_PreconditionAttributes_Final;

}

} else {  // WLAN or 200 OK

v_PreconditionAttributes_Audio := v_PreconditionAttributes_Final;

v_PreconditionAttributes_Video := v_PreconditionAttributes_Final;

}

}

if ((p_AudioCodec == EVS_16000) and v_IsInitialAnswer) {   /* @sic R5-213524 sic@ */

v_FmtpParams_Audio := cr_Fmtp_ParamList(cr_EVS_Fmtp_AudioParamsBrBwMaxRed("13.2", "swb"));

}

select (p_VideoCodec) {                                    /* @sic R5-213524 sic@ */

case (H264_90000) { v_FmtpParams_Video := cr_SDP_FmtpParamsH264_MT; }

case (H265_90000) { v_FmtpParams_Video := cr_SDP_FmtpParamsH265("1", "93"); }

}

v_MediaAttributes_Audio := {

cr_SDP_Attribute_rtpmap(v_FmtAudio, v_AudioCodec),

cr_SDP_Attribute_fmtp(v_FmtAudio, v_FmtpParams_Audio)

};

v_MediaAttributes_Video := {

cr_SDP_Attribute_rtpmap(v_FmtVideo, cr_RTPMAP_H264_90000),

cr_SDP_Attribute_fmtp(v_FmtVideo, v_FmtpParams_Video)  /* @sic R5s141329 - MCC160 implementation sic@

@sic R5-150703: same parameters for C.26 and 17.2 sic@ */

};

if (f_IMS_PTC_RanTypeIsNR5GC()) {    // @sic R5-216215 sic@

v_MediaAttributes_Video := f_SDP_Attributes_Add_RX(v_MediaAttributes_Video, cr_SDP_Attribute_acfg(1, "t=1"));

}

return f_IMS_BuildSDP_AudioVideo_RX(-,

cr_SDP_Media_Audio(v_FmtAudio),

v_MediaAttributes_Audio,

v_PreconditionAttributes_Audio,

cr_SDP_Media_Video(v_FmtVideo),

v_MediaAttributes_Video,

v_PreconditionAttributes_Video);

}

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

function f_IMS_BuildSDP_MTCallAudioVideo_RX(VideoCallTypeMT_Type p_VideoCallType,

template (omit) SDP_KindOfAnswer_Type p_KindOfAnswer := omit,

IMS_SDP_AudioCodec_Type p_AudioCodec := AMR_16000,

IMS_SDP_VideoCodec_Type p_VideoCodec := H264_90000,

boolean p_WithPreconditions := true)

runs on IMS_PTC

return template (present) SDP_Message

{ /* @sic R5-172930: function restructured to cope with different flavours ('normal' IMS, WLAN; Setup video call, add video, remove video; INVITE, UPDATE) sic@ */

/* @sic R5-172930: p_FmtAudio, p_FmtVideo removed sic@ */

/* @sic R5-213524: p_AudioCodec, p_VideoCodec, p_WithPreconditions sic@ */

var template (present) SDP_attribute_rtpmap_codec v_AudioCodec;

var template (present) SDP_attribute_rtpmap_codec v_VideoCodec;

var template (present) SDP_attribute_list v_MediaAttributes_Audio;

var template (present) SDP_attribute_list v_MediaAttributes_Video;

var template (present) SDP_attribute_list v_PreconditionAttributes_Audio := {}; // @sic R5-170782 sic@

var template (present) SDP_attribute_list v_PreconditionAttributes_Video := {}; // @sic R5-170782 sic@

var template (present) SDP_attribute_list v_PreconditionAttributes_Final := cr_SDP_PreconditionAttributes(c_sendrecv, c_sendrecv, c_mandatory, c_mandatory);  /* preconditions when resources are fully available */

var template (present) SDP_attribute_fmtp_format_specific_params v_FmtpParams_Audio := ?;

var template (present) SDP_attribute_fmtp_format_specific_params v_FmtpParams_Video := ?;

var charstring v_FmtAudio;

var charstring v_FmtVideo := tsc_SDP_FmtVideo;               /* @sic R5-197050: tsc_SDP_FmtVideo instead of fl_IMS_BuildSDP_FmtVideo sic@ */

var boolean v_IsWLAN := f_IMS_PTC_RanTypeIsWLAN();

var boolean v_IsInitialAnswer := match(InitialAnswer, p_KindOfAnswer);

v_AudioCodec := f_SDP_AttributeRtpmapAudioRX(p_AudioCodec);  // @sic R5-168058; R5-168142 sic@

v_FmtAudio := f_SDP_AudioCodec_GetFmt(p_AudioCodec);         /* @sic R5-197050: fix mapping of fmt values sic@ */

if (p_WithPreconditions) {

if (not v_IsWLAN and v_IsInitialAnswer) {  // i.e. 183 Session Progress (C.26 step 4 or 17.2 step 3)

v_PreconditionAttributes_Video := cr_SDP_PreconditionAttributes((c_none, c_sendrecv), c_none, c_mandatory, c_mandatory);

v_PreconditionAttributes_Video := f_SDP_Attributes_Add_RX(v_PreconditionAttributes_Video, cr_SDP_Attribute_conf_qos);         // @sic R5-216443: NR5GC sic@

if (p_VideoCallType == SetupVideoCall) {                 // C.26 step 4: same preconditions for video and audio

v_PreconditionAttributes_Audio := v_PreconditionAttributes_Video;

} else {                                                 // 17.2 step 3: audio is there already

v_PreconditionAttributes_Audio := v_PreconditionAttributes_Final;

}

} else {  // WLAN or 200 OK

v_PreconditionAttributes_Audio := v_PreconditionAttributes_Final;

v_PreconditionAttributes_Video := v_PreconditionAttributes_Final;

}

}

if ((p_AudioCodec == EVS_16000) and v_IsInitialAnswer) {   /* @sic R5-213524 sic@ */

v_FmtpParams_Audio := cr_Fmtp_ParamList(cr_EVS_Fmtp_AudioParamsBrBwMaxRed("13.2", "swb"));

}

select (p_VideoCodec) {                                    /* @sic R5-213524 sic@ */

case (H264_90000) {

v_FmtpParams_Video := cr_SDP_FmtpParamsH264_MT;

v_VideoCodec := cr_RTPMAP_H264_90000;

}

case (H265_90000) {

v_FmtpParams_Video := cr_SDP_FmtpParamsH265("1", "93");

v_VideoCodec := cr_RTPMAP_H265_90000;

}

}

v_MediaAttributes_Audio := {

cr_SDP_Attribute_rtpmap(v_FmtAudio, v_AudioCodec),

cr_SDP_Attribute_fmtp(v_FmtAudio, v_FmtpParams_Audio)

};

v_MediaAttributes_Video := {

cr_SDP_Attribute_rtpmap(v_FmtVideo, v_VideoCodec ),

cr_SDP_Attribute_fmtp(v_FmtVideo, v_FmtpParams_Video)  /* @sic R5s141329 - MCC160 implementation sic@

@sic R5-150703: same parameters for C.26 and 17.2 sic@ */

};

if (f_IMS_PTC_RanTypeIsNR5GC()) {    // @sic R5-216215 sic@

v_MediaAttributes_Video := f_SDP_Attributes_Add_RX(v_MediaAttributes_Video, cr_SDP_Attribute_acfg(1, "t=1"));

}

return f_IMS_BuildSDP_AudioVideo_RX(-,

cr_SDP_Media_Audio(v_FmtAudio),

v_MediaAttributes_Audio,

v_PreconditionAttributes_Audio,

cr_SDP_Media_Video(v_FmtVideo),

v_MediaAttributes_Video,

v_PreconditionAttributes_Video);

}

<<SKIPPED CODE>>

  - Correction to function f_NR5GC_IPCAN_MT_VideoCall

| Function name | f_NR5GC_IPCAN_MT_VideoCall |
| --- | --- |
| Reason for change | According to 7.16 test procedure step 0A-0H, Table 4.9.26.2.2-1 of TS 38.508-1 should be performed. Actual implementation sets different parameters than specified in table above mentioned. |
| Summary of change | Set correct Stablishment cause and Service Type input parameters in function f_NR5GC_SpeechCall_Steps2_8. |
| TTCN module | IMS\NR5GC_7\IMS_NR5GC_CallControlTestcases.ttcn |
| MCC160 Comment | Accepted |

Before

<<SKIPPED CODE>>

function f_NR5GC_IPCAN_MT_VideoCall(IMS_IPCAN_CO_ORD_PORT p_Port,

NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC return TriggerResult_Type

{

var TriggerResult_Type v_Result := NORMAL;

var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo;

f_NR_Paging_Def(p_NR_CellId, RRC_IDLE);

v_GSM_MobilityInfo := f_NR5GC_SpeechCall_Steps2_8(p_NR_CellId, mo_VideoCall, '0001'B);

f_IMS_IPCAN_SendCoOrdMsg(p_Port);

v_Result := f_IMS_IPCAN_WaitForTriggerOrAbort(p_Port);  // wait until IMS PTC is ready to configure video DRB

if (v_Result == NORMAL) {                               // => procedure shall not be aborted

f_NR5GC_ModifyPDUSession_AddVideo(p_NR_CellId, valueof(v_GSM_MobilityInfo));

}

f_IMS_IPCAN_SendCoOrdMsg(p_Port);

return v_Result;

}

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

function f_NR5GC_IPCAN_MT_VideoCall(IMS_IPCAN_CO_ORD_PORT p_Port,

NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC return TriggerResult_Type

{

var TriggerResult_Type v_Result := NORMAL;

var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo;

f_NR_Paging_Def(p_NR_CellId, RRC_IDLE);

v_GSM_MobilityInfo := f_NR5GC_SpeechCall_Steps2_8(p_NR_CellId, mt_Access, '0010'B);

f_IMS_IPCAN_SendCoOrdMsg(p_Port);

v_Result := f_IMS_IPCAN_WaitForTriggerOrAbort(p_Port);  // wait until IMS PTC is ready to configure video DRB

if (v_Result == NORMAL) {                               // => procedure shall not be aborted

f_NR5GC_ModifyPDUSession_AddVideo(p_NR_CellId, valueof(v_GSM_MobilityInfo));

}

f_IMS_IPCAN_SendCoOrdMsg(p_Port);

return v_Result;

}

<<SKIPPED CODE>>

# Branches executed

The test case was executed on NR band n78 using NR ciphering algorithm nea1 and integrity algorithm nia1 on Rohde & Schwarz 5G Protocol Conformance Test platform with Hisilicon Balong 5000 UE

# Execution Log Files

## HiSilicon

The HiSilicon Balong 5000 UE passed this test case on Rohde & Schwarz 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_7.16_Hisilicon.log

In the log file (in .html format) the complete test case execution can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s220236: Supporting information for addition of IMS 5GS Call Control test case 7.16 |
| --- | --- |