3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220239

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0779 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IMS 5GS Call Control test case 7.17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To verify IMS 5GS test case 7.17 using IWD 21wk49 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to IMS 5GS test case 7.17 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.17.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Changes 2.2 and 2.3 from R5s220235 are required as well to verify this testcase. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

3	Branches executed	5

4	Execution Log Files	5

HiSilicon	5

5	References	5

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of IMS test case 7.17 which is part of the iwd-TTCN3-B2020-09_D21wk49.zip test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case:	7.17.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D21wk49.zip

System Simulator used:	Rohde & Schwarz 5G Protocol Conformance Test platform

UE used:	Hisilicon Balong 5000 UE

Verification Status:	PASS

# Corrections required

  - Correction to function f_TC_7_17_NR5GC_IMS1

| Function name | f_TC_7_17_NR5GC_IMS1 |
| --- | --- |
| Reason for change | Change 1: Preamble TestConfiguration set is for audio call test cases while this is a video call test case. Change 2: Test procedure is set for MT voice call when this is a video call test case. Change 3: Video codec input parameter to create the SPD offer is set to H264, H265 is the required codec for NR video test cases. Change 4: f_IMS_MTCallSetup_Receive180Ringing function is not setting any 2nd input parameter, taking then the default value and setting preconditions to true. This is wrong as this test case should be run without preconditions. |
| Summary of change | Change 1: Changed TestConfiguration to video call. Change 2: Changed Test procedure to IPCAN_MT_VideoCall Change 3: Changed video codec input parameter to to build the SDP offer to H265. Change 4: Set as 2nd input parameter of f_IMS_MTCallSetup_Receive180Ringing function, tsc_SDP_WithoutPreconditions. |
| TTCN module | IMS_NR5GC\IMS_Testsuite_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before

<<SKIPPED CODE>>

function f_TC_7_17_NR5GC_IMS1() runs on IMS_PTC

{ // MTSI MT Video call without preconditions at both originating UE and terminating UE / 5GS

var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp;

var IMS_ResponseWithSdp_Type v_ResponseWithSdp;

var template (value) SDP_Message v_SdpOffer;

f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED);

f_IMS_TestBody_Set(true);

// @siclog "Step 0A - 0H" siclog@

f_IMS_CC_StartCall(IPCAN_MT_SpeechCall);

// @siclog "Step 1" siclog@                 INVITE

v_SdpOffer := f_IMS_BuildSDP_InitialOfferAudioVideo_TX(tsc_IMS_SDP_AudioCodecList_EVS_WBAMR_AMR, tsc_IMS_SDP_VideoCodecList_H264_only, tsc_SDP_WithoutPreconditions);

v_InviteRequestWithSdp := f_IMS_MTCallSetup_Video_SendInvite(SetupVideoCall, v_SdpOffer);

// @siclog "Step 2 - 3" siclog@             100 Trying, 183 Session Progress

v_ResponseWithSdp := f_IMS_MTCallSetup_Video_Receive183(SetupVideoCall, v_InviteRequestWithSdp, EVS_16000, H265_90000, tsc_SDP_WithoutPreconditions);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 3");

// @siclog "Step 4 - 5" siclog@             PRACK, 200 OK

f_IMS_MTCallSetup_SendPRACK_ReceiveOK(v_InviteRequestWithSdp.Invite, v_ResponseWithSdp.Response.msgHeader);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 5");

// @siclog "Step 5A" siclog@

f_IMS_CC_TriggerDedicatedBearerActivation(IPCAN_MT_VideoCall, tsc_SDP_WithoutPreconditions);

// @siclog "Step 6 - 8" siclog@             180 Ringing, PRACK (conditional), 200 OK (conditional)

f_IMS_MTCallSetup_Receive180Ringing(v_InviteRequestWithSdp);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 10");

// @siclog "Step 9 - 11" siclog@            MMI (Accept Call), 200 OK, ACK

f_IMS_MTCallSetup_AcceptAndAcknowledge(v_InviteRequestWithSdp);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 12");

f_IMS_TestBody_Set(false);

// Call release and postamble:

f_IMS_CC_PostambleWithCallReleaseMT(IPCAN_MO_SpeechCall);

}<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

function f_TC_7_17_NR5GC_IMS1() runs on IMS_PTC

{ // MTSI MT Video call without preconditions at both originating UE and terminating UE / 5GS

var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp;

var IMS_ResponseWithSdp_Type v_ResponseWithSdp;

var template (value) SDP_Message v_SdpOffer;

f_IMS_CC_Preamble(IPCAN_VideoCall, IMS_REGISTERED);

f_IMS_TestBody_Set(true);

// @siclog "Step 0A - 0H" siclog@

f_IMS_CC_StartCall(IPCAN_MT_VideoCall);

// @siclog "Step 1" siclog@                 INVITE

v_SdpOffer := f_IMS_BuildSDP_InitialOfferAudioVideo_TX(tsc_IMS_SDP_AudioCodecList_EVS_WBAMR_AMR, tsc_IMS_SDP_VideoCodecList_H265_only, tsc_SDP_WithoutPreconditions);

v_InviteRequestWithSdp := f_IMS_MTCallSetup_Video_SendInvite(SetupVideoCall, v_SdpOffer);

// @siclog "Step 2 - 3" siclog@             100 Trying, 183 Session Progress

v_ResponseWithSdp := f_IMS_MTCallSetup_Video_Receive183(SetupVideoCall, v_InviteRequestWithSdp, EVS_16000, H265_90000, tsc_SDP_WithoutPreconditions);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 3");

// @siclog "Step 4 - 5" siclog@             PRACK, 200 OK

f_IMS_MTCallSetup_SendPRACK_ReceiveOK(v_InviteRequestWithSdp.Invite, v_ResponseWithSdp.Response.msgHeader);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 5");

// @siclog "Step 5A" siclog@

f_IMS_CC_TriggerDedicatedBearerActivation(IPCAN_MT_VideoCall, tsc_SDP_WithoutPreconditions);

// @siclog "Step 6 - 8" siclog@             180 Ringing, PRACK (conditional), 200 OK (conditional)

f_IMS_MTCallSetup_Receive180Ringing(v_InviteRequestWithSdp, tsc_SDP_WithoutPreconditions);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 10");

// @siclog "Step 9 - 11" siclog@            MMI (Accept Call), 200 OK, ACK

f_IMS_MTCallSetup_AcceptAndAcknowledge(v_InviteRequestWithSdp);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 12");

f_IMS_TestBody_Set(false);

// Call release and postamble:

f_IMS_CC_PostambleWithCallReleaseMT(IPCAN_MT_VideoCall);

}

<<SKIPPED CODE>>

# Branches executed

The test case was executed on NR band n78 using NR ciphering algorithm nea1 and integrity algorithm nia1 on Rohde & Schwarz 5G Protocol Conformance Test platform with Hisilicon Balong 5000 UE

# Execution Log Files

## HiSilicon

The HiSilicon Balong5000 IMS UE passed this test case on Rohde & Schwarz 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_7.17_Hisilicon.log

In the log file (in .html format) the complete test case execution can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s220240: Supporting information for addition of IMS 5GS Call Control test case 7.17 |
| --- | --- |