3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220317

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2391 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of NR5GC Rel-16 RRC test case 8.1.4.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add NR5GC Rel-16 RRC test case 8.1.4.4.1 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied toNR5GC Rel-16 RRC test case 8.1.4.4.1 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.4.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

3	Branches executed	12

4	Execution Log Files	13

4.1	HiSilicon Balong 5000	13

5	References	13

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 8.1.4.4.1 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	8.1.4.4.1

ATS Version:	iwd-TTCN3-B2020-09_D12wk49

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	Samsung SM-G996B

Verification Status:	PASS

# Corrections required

In addition to the below changes, the common changes of R5s220271 are also required and applied.

## 2.1 Change 1

| Function name | fl_TC_8_1_4_4_1_TestBody () |
| --- | --- |
| Reason for change | - At step 2, 10 & 17, The SS transmits an RRCReconfiguration message including ConditionalReconfiguration. Security Keys are not updated as per the specific message contents. Hence there is no need for SS to regenerate the Security Keys. However corresponding parameter of function f_NR5GC_CHO_RRCReconfiguration_SSConfig is set as true by default to indicate Key has been refreshed. - RACH preamble is expected at step #3 ,#11, & step 18. However in order for the TTCN to handle this, RACH preamble reception needs to be enabled before receiving and disabled after receiving the RACH. - At Step 9: The SS transmits an RRCReconfiguration message including MeasConfig and ConditionalReconfiguration to set NR Cell 1 and NR Cell 4 as target candidate cells and configure event A5 as trigger event in NR cell 2. Measurement Object ID shall be added for NR Cell 2 rather than NR Cell 1. p_SourceCellId & p_TargetCellId shall be NR Cell 2 & NR Cell 4 rather than NR Cell 1 & 2. - At Step 16: The SS transmits an RRCReconfiguration message including MeasConfig and ConditionalReconfiguration for intra-frequency event A3 and A5 in NR cell 4. Measurement Object ID shall be added for NR Cell 4 rather than NR Cell 1 |
| Summary of change | - Modified parameter p_RBConfig_KeyChange in function call f_NR5GC_CHO_RRCReconfiguration_SSConfig() to false inorder not to regenerate the security Keys. ( Step 2, 10 & 17) - Function calls are applied to enable and disable reception of RACH around steps 3, 11 & 18 - Step 9: Measurement Object ID is corrected to use NR Cell 2 rather than NR Cell 1. p_SourceCellId & p_TargetCellId is corrected to use NR Cell 2 & NR Cell 4 rather than NR Cell 1 & 2. - Step 16: Measurement Object ID is corrected to use NR Cell 4 rather than NR Cell 1. |
| TTCN module | RRC_Conditional_Handover_NR5GC.ttcn |
| MCC160 Comment | - Not Accepted, according to Table 8.1.4.4.1.3.3-8, this testcase called the RRCReconfiguration-HO configuration with condition RBConfig_KeyChange. So the key need to refresh. - Accepted in principle, enable and check the Rach indication according to the comment from ROHDE & SCHWARZ - Accepted - Accepted |

Before Change

function fl_TC_8_1_4_4_1_TestBody() runs on NR5GC_PTC { var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT3; //var NR_AbsoluteCellPower_Type v_CellPower_Serving_FR1; var NR_AbsoluteCellPower_Type v_CellPower_EntryCondition_FR1; var NR_AbsoluteCellPower_Type v_CellPower_Leaving_FR1; var RSRP_Range v_Rsrp; var RSRP_Range v_RsrpThres1; var RSRP_Range v_RsrpThres2; var integer v_DRBId; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasConfig v_MeasConfig_Step_1; var template (value) MeasConfig v_MeasConfig_Step_9; var template (value) MeasConfig v_MeasConfig_Step_16; var template (value) ConditionalReconfiguration_r16 v_ConditionalReconfiguration_r16_Step_1; var template (value) ConditionalReconfiguration_r16 v_ConditionalReconfiguration_r16_Step_9; var template (value) ConditionalReconfiguration_r16 v_ConditionalReconfiguration_r16_Step_16; var template (value) NR_DRBInfoList_Type v_DRBInfoList := f_NR5GC_MobileInfo_GetDRBInfoList(); var PDUSessionInfoList_Type v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); timer t_Watchdog := 10.0; //Power levels of NR cell 1, NR cell 2 and NR cell 4 FR1 acc. Table 8.1.4.4.1.3.2-1, Table 8.1.4.4.1.3.2-2 FFS /* ¦ Cell 1 ¦ Cell 2 ¦ Cell 4 * ----------------------------------------- * T0 ¦ -82 ¦ -91 ¦ -91 * ----------------------------------------- * T1 ¦ -91 ¦ -79 ¦ -91 * ----------------------------------------- * T2 ¦ -91 ¦ -91 ¦ -79 * ----------------------------------------- * T3 ¦ -79 ¦ -91 ¦ -91 */ // v_CellPower_Serving_FR1 := -82; v_CellPower_Leaving_FR1 := -91; v_CellPower_EntryCondition_FR1 := -79; v_CellPowerList_AtT1 := {cs_NR_CellPower(nr_Cell1, v_CellPower_Leaving_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell2, v_CellPower_EntryCondition_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell4, v_CellPower_Leaving_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // FR2 is FFS v_CellPowerList_AtT2 := {cs_NR_CellPower(nr_Cell1, v_CellPower_Leaving_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell2, v_CellPower_Leaving_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell4, v_CellPower_EntryCondition_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // FR2 is FFS v_CellPowerList_AtT3 := {cs_NR_CellPower(nr_Cell1, v_CellPower_EntryCondition_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell2, v_CellPower_Leaving_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell4, v_CellPower_Leaving_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // FR2 is FFS if(f_NR_CellInfo_GetIsFR1(nr_Cell1)){ v_Rsrp := 2; v_RsrpThres1 := 76; v_RsrpThres2 := 70; } else{ v_Rsrp := 2; //FFS dummy value FR2 v_RsrpThres1 := 76; //FFS dummy value FR2 v_RsrpThres2 := 70; //FFS dummy value FR2 } //@siclog "Steps 1-2" siclog@ //The SS transmits an RRCReconfiguration message including MeasConfig and ConditionalReconfiguration to set NR Cell 2 and NR Cell 4 as target candidate cells and configure event A3 as trigger event in NR cell 1. //The UE transmits an RRCReconfigurationComplete message in NR cell 1. v_ReportConfigList := {cs_NR_ReportConfigToAddMod_NR(tsc_NR_IdReportConfigId1, cs_38508_CondTriggerConfig(v_Rsrp)), cs_NR_ReportConfigToAddMod_NR(tsc_NR_IdReportConfigId2, cs_38508_CondTriggerConfig_EventA5(v_RsrpThres1, v_RsrpThres2))}; v_MeasConfig_Step_1 := f_NR_GenerateMeasurementConfigNR({cs_MeasObjectId1(nr_Cell1)}, v_ReportConfigList, {cs_NR_MeasId_Config_id1_obj1_conf1}, -, -, -, -); v_ConditionalReconfiguration_r16_Step_1 := cs_ConditionalReconfiguration_r16( -, cs_CondReconfigToAddModList_r16_2EntryCR_1EntryMI(1, tsc_NR_MeasId1, f_NR_GenerateCondRRCReconfig_r16_Octetstring(nr_Cell2), 2, tsc_NR_MeasId1, f_NR_GenerateCondRRCReconfig_r16_Octetstring(nr_Cell4))); f_NR5GC_CHO_RRCReconfiguration_SSConfig( nr_Cell1, nr_Cell2, v_DRBInfoList, -,-, cs_38508_RRCReconfiguration_CHO(-,v_MeasConfig_Step_1, v_ConditionalReconfiguration_r16_Step_1), -, -, -); //@siclog "Step 3" siclog@ //Check: Does the UE initiate a random access procedure transmitting Preamble in NR Cell 2 or in NR cell 4 within 10s? t_Watchdog.start; alt { []SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell2, cr_TimingInfo_Any, ?)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 3, PRACH sent by UE "); } []t_Watchdog.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); } }; //@siclog "Step 4" siclog@ //SS adjusts the cell-specific reference signal level according to row "T1" f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 5" siclog@ //Check: Does the UE initiate a random access procedure by transmitting Preamble in NR Cell 2 within 10s? t_Watchdog.start; SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell2, cr_TimingInfo_Any, ?)); t_Watchdog.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); //@siclog "Step 6-7" siclog@ //The SS transmits Random Access Response. //The UE transmits RRCReconfigurationComplete message in NR Cell 2. f_NR5GC_CHO_RRCReconfigurationComplete( nr_Cell1, nr_Cell2, -); //@siclog "Steps 8" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.1-1 indicate that the UE is capable of exchanging IP data on DRB1 and NR Cell 2? v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); f_NR5GC_CheckDataPath(nr_Cell2, f_NR5GC_GetPdnIndex(v_PDUSessionInfoList[0]), dedicatedBearer, v_DRBId, "Step 8"); //@siclog "Steps 9-10" siclog@ //The SS transmits an RRCReconfiguration message including MeasConfig and ConditionalReconfiguration to set NR Cell 1 and NR Cell 4 as target candidate cells and configure event A5 as trigger event in NR cell 2. //The UE transmits an RRCReconfigurationComplete message in NR cell 2. v_MeasConfig_Step_9 := f_NR_GenerateMeasurementConfigNR({cs_MeasObjectId1(nr_Cell1)}, v_ReportConfigList, {cs_NR_MeasId_Config_id1_obj1_conf2}); v_ConditionalReconfiguration_r16_Step_9 := cs_ConditionalReconfiguration_r16( -, cs_CondReconfigToAddModList_r16_2EntryCR_1EntryMI(1, tsc_NR_MeasId2, f_NR_GenerateCondRRCReconfig_r16_Octetstring(nr_Cell1), 2, tsc_NR_MeasId2, f_NR_GenerateCondRRCReconfig_r16_Octetstring(nr_Cell4))); f_NR5GC_CHO_RRCReconfiguration_SSConfig( nr_Cell1, nr_Cell2, v_DRBInfoList, -, -, cs_38508_RRCReconfiguration_CHO(-,v_MeasConfig_Step_9, v_ConditionalReconfiguration_r16_Step_9), -, -, -, false); //Sujith@81441 NoRefresh //@siclog "Step 11" siclog@ //SS adjusts the cell-specific reference signal level according to row "T2". f_NR_SetCellPowerList(v_CellPowerList_AtT2); //@siclog "Step 12" siclog@ //Check: Does the UE initiate a random access procedure by transmitting Preamble in NR Cell 4 within 10s? t_Watchdog.start; SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell4, cr_TimingInfo_Any, ?)); t_Watchdog.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 12"); //@siclog "Steps 13-14" siclog@ //Check:The SS transmits Random Access Response. //The UE transmits RRCReconfigurationComplete message in NR Cell 4. f_NR5GC_CHO_RRCReconfigurationComplete( nr_Cell2, nr_Cell4, -); //@siclog "Step 15" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.1-1 indicate that the UE is capable of exchanging IP data on DRB1 and NR Cell 4? f_NR5GC_CheckDataPath(nr_Cell4, f_NR5GC_GetPdnIndex(v_PDUSessionInfoList[0]), dedicatedBearer, v_DRBId, "Step 15"); //@siclog "Steps 16-17" siclog@ //The SS transmits an RRCReconfiguration message including MeasConfig and ConditionalReconfiguration for intra-frequency event A3 and A5 in NR cell 4. //The UE transmits an RRCReconfigurationComplete message in NR cell 4. v_MeasConfig_Step_16 := f_NR_GenerateMeasurementConfigNR({cs_MeasObjectId1(nr_cell1 )}, v_ReportConfigList, {cs_NR_MeasId_Config_id1_obj1_conf1, cs_NR_MeasId_Config_id2_obj1_conf2}); v_ConditionalReconfiguration_r16_Step_16 := cs_ConditionalReconfiguration_r16( -, cs_CondReconfigToAddModList_r16_2EntryCR_2EntryMI(1, tsc_NR_MeasId1, tsc_NR_MeasId2, f_NR_GenerateCondRRCReconfig_r16_Octetstring(nr_Cell1), 2, tsc_NR_MeasId1, tsc_NR_MeasId2, f_NR_GenerateCondRRCReconfig_r16_Octetstring(nr_Cell2))); f_NR5GC_CHO_RRCReconfiguration_SSConfig( nr_Cell4, nr_Cell1, v_DRBInfoList, -, -, cs_38508_RRCReconfiguration_CHO(-,v_MeasConfig_Step_16, v_ConditionalReconfiguration_r16_Step_16), -, -, -, false); //@siclog "Step 18" siclog@ //SS adjusts the cell-specific reference signal level according to row "T3". f_NR_SetCellPowerList(v_CellPowerList_AtT3); //@siclog "Step 19" siclog@ //Check: Does the UE initiate a random access procedure by transmitting Preamble in NR Cell 1 within 10s? t_Watchdog.start; SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo_Any, ?)); t_Watchdog.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 19"); //@siclog "Steps 20-21" siclog@ //The SS transmits Random Access Response. //The UE transmits RRCReconfigurationComplete message in NR Cell 1. f_NR5GC_CHO_RRCReconfigurationComplete( nr_Cell4, nr_Cell1, -); //@siclog "Step 22" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.1-1 indicate that the UE is capable of exchanging IP data on DRB1 and NR Cell 1? f_NR5GC_CheckDataPath(nr_Cell1, f_NR5GC_GetPdnIndex(v_PDUSessionInfoList[0]), dedicatedBearer, v_DRBId, "Step 22"); //@siclog "Step 23" siclog@ //SS adjusts the cell-specific reference signal level according to row "T1". f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 24" siclog@ //Check: Does the UE transmit RRCReconfigurationComplete message in NR Cell 2 within 10s? t_Watchdog.start; alt { []SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell2, cr_38508_RRCReconfigurationComplete)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 24, RRCReconfigurationComplete sent by UE "); } []t_Watchdog.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 24"); } }; }// End of fl_TC_8_1_4_4_1_TestBody

After Change

function fl_TC_8_1_4_4_1_TestBody() runs on NR5GC_PTC { var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT3; //var NR_AbsoluteCellPower_Type v_CellPower_Serving_FR1; var NR_AbsoluteCellPower_Type v_CellPower_EntryCondition_FR1; var NR_AbsoluteCellPower_Type v_CellPower_Leaving_FR1; var RSRP_Range v_Rsrp; var RSRP_Range v_RsrpThres1; var RSRP_Range v_RsrpThres2; var integer v_DRBId; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasConfig v_MeasConfig_Step_1; var template (value) MeasConfig v_MeasConfig_Step_9; var template (value) MeasConfig v_MeasConfig_Step_16; var template (value) ConditionalReconfiguration_r16 v_ConditionalReconfiguration_r16_Step_1; var template (value) ConditionalReconfiguration_r16 v_ConditionalReconfiguration_r16_Step_9; var template (value) ConditionalReconfiguration_r16 v_ConditionalReconfiguration_r16_Step_16; var template (value) NR_DRBInfoList_Type v_DRBInfoList := f_NR5GC_MobileInfo_GetDRBInfoList(); var PDUSessionInfoList_Type v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); timer t_Watchdog := 10.0; //Power levels of NR cell 1, NR cell 2 and NR cell 4 FR1 acc. Table 8.1.4.4.1.3.2-1, Table 8.1.4.4.1.3.2-2 FFS /* ¦ Cell 1 ¦ Cell 2 ¦ Cell 4 * ----------------------------------------- * T0 ¦ -82 ¦ -91 ¦ -91 * ----------------------------------------- * T1 ¦ -91 ¦ -79 ¦ -91 * ----------------------------------------- * T2 ¦ -91 ¦ -91 ¦ -79 * ----------------------------------------- * T3 ¦ -79 ¦ -91 ¦ -91 */ // v_CellPower_Serving_FR1 := -82; v_CellPower_Leaving_FR1 := -91; v_CellPower_EntryCondition_FR1 := -79; v_CellPowerList_AtT1 := {cs_NR_CellPower(nr_Cell1, v_CellPower_Leaving_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell2, v_CellPower_EntryCondition_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell4, v_CellPower_Leaving_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // FR2 is FFS v_CellPowerList_AtT2 := {cs_NR_CellPower(nr_Cell1, v_CellPower_Leaving_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell2, v_CellPower_Leaving_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell4, v_CellPower_EntryCondition_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // FR2 is FFS v_CellPowerList_AtT3 := {cs_NR_CellPower(nr_Cell1, v_CellPower_EntryCondition_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell2, v_CellPower_Leaving_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell4, v_CellPower_Leaving_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // FR2 is FFS if(f_NR_CellInfo_GetIsFR1(nr_Cell1)){ v_Rsrp := 2; v_RsrpThres1 := 76; v_RsrpThres2 := 70; } else{ v_Rsrp := 2; //FFS dummy value FR2 v_RsrpThres1 := 76; //FFS dummy value FR2 v_RsrpThres2 := 70; //FFS dummy value FR2 } //@siclog "Steps 1-2" siclog@ //The SS transmits an RRCReconfiguration message including MeasConfig and ConditionalReconfiguration to set NR Cell 2 and NR Cell 4 as target candidate cells and configure event A3 as trigger event in NR cell 1. //The UE transmits an RRCReconfigurationComplete message in NR cell 1. v_ReportConfigList := {cs_NR_ReportConfigToAddMod_NR(tsc_NR_IdReportConfigId1, cs_38508_CondTriggerConfig(v_Rsrp)), cs_NR_ReportConfigToAddMod_NR(tsc_NR_IdReportConfigId2, cs_38508_CondTriggerConfig_EventA5(v_RsrpThres1, v_RsrpThres2))}; v_MeasConfig_Step_1 := f_NR_GenerateMeasurementConfigNR({cs_MeasObjectId1(nr_Cell1)}, v_ReportConfigList, {cs_NR_MeasId_Config_id1_obj1_conf1}, -, -, -, -); v_ConditionalReconfiguration_r16_Step_1 := cs_ConditionalReconfiguration_r16( -, cs_CondReconfigToAddModList_r16_2EntryCR_1EntryMI(1, tsc_NR_MeasId1, f_NR_GenerateCondRRCReconfig_r16_Octetstring(nr_Cell2), 2, tsc_NR_MeasId1, f_NR_GenerateCondRRCReconfig_r16_Octetstring(nr_Cell4))); f_NR5GC_CHO_RRCReconfiguration_SSConfig( nr_Cell1, nr_Cell2, v_DRBInfoList, -,-, cs_38508_RRCReconfiguration_CHO(-,v_MeasConfig_Step_1, v_ConditionalReconfiguration_r16_Step_1), -, -, -, false); //NoRefresh //@siclog "Step 3" siclog@ //Check: Does the UE initiate a random access procedure transmitting Preamble in NR Cell 2 or in NR cell 4 within 10s? t_Watchdog.start; alt { []SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell2, cr_TimingInfo_Any, ?)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 3, PRACH sent by UE "); } []t_Watchdog.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); } }; //@siclog "Step 4" siclog@ //SS adjusts the cell-specific reference signal level according to row "T1" f_NR_SetCellPowerList(v_CellPowerList_AtT1); //Reconfigure SS to stop reporting PRACH preambles f_NR_SS_SystemIndCtrlConfig(nr_Cell2, cds_NR_SystemIndCtrl_RachPreamble(enable)); //@siclog "Step 5" siclog@ //Check: Does the UE initiate a random access procedure by transmitting Preamble in NR Cell 2 within 10s? t_Watchdog.start; SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell2, cr_TimingInfo_Any, ?)); t_Watchdog.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); //Reconfigure SS to stop reporting PRACH preambles f_NR_SS_SystemIndCtrlConfig(nr_Cell2, cds_NR_SystemIndCtrl_RachPreamble(disable)); //@siclog "Step 6-7" siclog@ //The SS transmits Random Access Response. //The UE transmits RRCReconfigurationComplete message in NR Cell 2. f_NR5GC_CHO_RRCReconfigurationComplete( nr_Cell1, nr_Cell2, -); //@siclog "Steps 8" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.1-1 indicate that the UE is capable of exchanging IP data on DRB1 and NR Cell 2? v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); f_NR5GC_CheckDataPath(nr_Cell2, f_NR5GC_GetPdnIndex(v_PDUSessionInfoList[0]), dedicatedBearer, v_DRBId, "Step 8"); //@siclog "Steps 9-10" siclog@ //The SS transmits an RRCReconfiguration message including MeasConfig and ConditionalReconfiguration to set NR Cell 1 and NR Cell 4 as target candidate cells and configure event A5 as trigger event in NR cell 2. //The UE transmits an RRCReconfigurationComplete message in NR cell 2. v_MeasConfig_Step_9 := f_NR_GenerateMeasurementConfigNR({cs_MeasObjectId1(nr_Cell2 /*nr_Cell1*/)}, v_ReportConfigList, {cs_NR_MeasId_Config_id1_obj1_conf2}); v_ConditionalReconfiguration_r16_Step_9 := cs_ConditionalReconfiguration_r16( -, cs_CondReconfigToAddModList_r16_2EntryCR_1EntryMI(1, tsc_NR_MeasId2, f_NR_GenerateCondRRCReconfig_r16_Octetstring(nr_Cell1), 2, tsc_NR_MeasId2, f_NR_GenerateCondRRCReconfig_r16_Octetstring(nr_Cell4))); f_NR5GC_CHO_RRCReconfiguration_SSConfig( nr_Cell2, nr_Cell4, v_DRBInfoList, -, -, cs_38508_RRCReconfiguration_CHO(-,v_MeasConfig_Step_9, v_ConditionalReconfiguration_r16_Step_9), -, -, -, false); //NoRefresh f_NR_SS_SystemIndCtrlConfig(nr_Cell4, cds_NR_SystemIndCtrl_RachPreamble(enable)); //@siclog "Step 11" siclog@ //SS adjusts the cell-specific reference signal level according to row "T2". f_NR_SetCellPowerList(v_CellPowerList_AtT2); //@siclog "Step 12" siclog@ //Check: Does the UE initiate a random access procedure by transmitting Preamble in NR Cell 4 within 10s? t_Watchdog.start; SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell4, cr_TimingInfo_Any, ?)); t_Watchdog.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 12"); f_NR_SS_SystemIndCtrlConfig(nr_Cell4, cds_NR_SystemIndCtrl_RachPreamble(disable)); //@siclog "Steps 13-14" siclog@ //Check:The SS transmits Random Access Response. //The UE transmits RRCReconfigurationComplete message in NR Cell 4. f_NR5GC_CHO_RRCReconfigurationComplete( nr_Cell2, nr_Cell4, -); //@siclog "Step 15" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.1-1 indicate that the UE is capable of exchanging IP data on DRB1 and NR Cell 4? f_NR5GC_CheckDataPath(nr_Cell4, f_NR5GC_GetPdnIndex(v_PDUSessionInfoList[0]), dedicatedBearer, v_DRBId, "Step 15"); //@siclog "Steps 16-17" siclog@ //The SS transmits an RRCReconfiguration message including MeasConfig and ConditionalReconfiguration for intra-frequency event A3 and A5 in NR cell 4. //The UE transmits an RRCReconfigurationComplete message in NR cell 4. v_MeasConfig_Step_16 := f_NR_GenerateMeasurementConfigNR({cs_MeasObjectId1(nr_Cell4 /*cell1 */)}, v_ReportConfigList, {cs_NR_MeasId_Config_id1_obj1_conf1, cs_NR_MeasId_Config_id2_obj1_conf2}); v_ConditionalReconfiguration_r16_Step_16 := cs_ConditionalReconfiguration_r16( -, cs_CondReconfigToAddModList_r16_2EntryCR_2EntryMI(1, tsc_NR_MeasId1, tsc_NR_MeasId2, f_NR_GenerateCondRRCReconfig_r16_Octetstring(nr_Cell1), 2, tsc_NR_MeasId1, tsc_NR_MeasId2, f_NR_GenerateCondRRCReconfig_r16_Octetstring(nr_Cell2))); f_NR5GC_CHO_RRCReconfiguration_SSConfig( nr_Cell4, nr_Cell1, v_DRBInfoList, -, -, cs_38508_RRCReconfiguration_CHO(-,v_MeasConfig_Step_16, v_ConditionalReconfiguration_r16_Step_16), -, -, -, false); //NoRefresh f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(enable)); //@siclog "Step 18" siclog@ //SS adjusts the cell-specific reference signal level according to row "T3". f_NR_SetCellPowerList(v_CellPowerList_AtT3); //@siclog "Step 19" siclog@ //Check: Does the UE initiate a random access procedure by transmitting Preamble in NR Cell 1 within 10s? t_Watchdog.start; SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo_Any, ?)); t_Watchdog.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 19"); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(disable)); //@siclog "Steps 20-21" siclog@ //The SS transmits Random Access Response. //The UE transmits RRCReconfigurationComplete message in NR Cell 1. f_NR5GC_CHO_RRCReconfigurationComplete( nr_Cell4, nr_Cell1, -); //@siclog "Step 22" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.1-1 indicate that the UE is capable of exchanging IP data on DRB1 and NR Cell 1? f_NR5GC_CheckDataPath(nr_Cell1, f_NR5GC_GetPdnIndex(v_PDUSessionInfoList[0]), dedicatedBearer, v_DRBId, "Step 22"); //@siclog "Step 23" siclog@ //SS adjusts the cell-specific reference signal level according to row "T1". f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 24" siclog@ //Check: Does the UE transmit RRCReconfigurationComplete message in NR Cell 2 within 10s? t_Watchdog.start; alt { []SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell2, cr_38508_RRCReconfigurationComplete)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 24, RRCReconfigurationComplete sent by UE "); } []t_Watchdog.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 24"); } }; }// End of fl_TC_8_1_4_4_1_TestBody

## Additional MCC160 implementation

| Function name | cs_CondReconfigToAddModList_r16_2EntryCR_2_1EntryMI |
| --- | --- |
| Reason for change | As per Table 8.1.4.4.1.3.3-10: CondReconfigToAddModList-r16 (Step 16, Table 8.1.4.4.1.3.3-6) we need to specify single measid under condExecutionCond-r16 for NR Cell 2 and two measid for NR Cell 1. |
| Summary of change | Added a new template to cater the need of adding single measid for NR Cell 2 and two measid for NR Cell 1. |
| TTCN module | RRC_Conditional_Handover_NR5GC.ttcn |
| MCC160 Comment | Accepted in principle, I renamed and modified the same template instead of adding new template |

// 2 Entry of CondReconfigId and 2 Entry of MeasId for CondReconfigId[1] and 1 Entry of MeasId for CondReconfigId[2]

template (value) CondReconfigToAddModList_r16 cs_CondReconfigToAddModList_r16_2EntryCR_2EntryMI_First_1Entry_Second( template (value) integer p_CondReconfigId1_r16,

template (value) integer p_MeasId1,

template (value) integer p_MeasId2,

template (value) octetstring p_CondRRCReconfig1_r16_Octetstring,

template (value) integer p_CondReconfigId2_r16,

template (value) integer p_MeasId3,

template (value) octetstring p_CondRRCReconfig2_r16_Octetstring):=

{/* @status    */

{

condReconfigId_r16                := p_CondReconfigId1_r16,

condExecutionCond_r16             := {p_MeasId1, p_MeasId2},

condRRCReconfig_r16               := p_CondRRCReconfig1_r16_Octetstring

},

{

condReconfigId_r16                := p_CondReconfigId2_r16,

condExecutionCond_r16             := {p_MeasId3},

condRRCReconfig_r16               := p_CondRRCReconfig2_r16_Octetstring

}

}

//Based to the comment from ROHDE & SCHWARZ, as per step 3 we need to enable and check the Rach indication for both NR Cell 2 and NR Cell 4.

| Function name | fl_TC_8_1_4_4_1_TestBody() |
| --- | --- |
| Reason for change | - As per test prose step 3 we need to enable and check the Rach indication for both NRCell 2 and NRCell 4. - Disable Rach indications after the reception of RRCReconfigurationComplete on NR Cell 4 at Step 14 for both NR Cell 2 and NR Cell 4 as it is no longer required for these two cells for further prose steps. - For NR Cell 1 Rach indication should only be disabled after we receive RRCReconfigurationComplete on NRCell 1 otherwise it will interfere RRCReconfigurationComplete reception while SS waits for disable confirmation. |
| Summary of change | - Enabled rach indication for both NRCell 2 and NRCell 4 before step 3 and added alt condition to ensure no rach on NR Cell 4 in addition to NR Cell 2 as per the current test prose checks requirement. - Added rach disable call for both NR Cell 2 and NR Cell 4 after Step 14. - Disabled the rach indication for NR Cell 1 after RRCReconfigurationComplete message reception on NR Cell1. |
| TTCN module | RRC_Conditional_Handover_NR5GC.ttcn |
| MCC160 Comment | - Accepted, implemented as below - Accepted, implemented as below - Accepted, implemented as below |

function fl_TC_8_1_4_4_1_TestBody() runs on NR5GC_PTC

{

…

//@siclog "Steps 1-2" siclog@

//The SS transmits an RRCReconfiguration message including MeasConfig and ConditionalReconfiguration to set NR Cell 2 and NR Cell 4 as target candidate cells and configure event A3 as trigger event in NR cell 1.

//The UE transmits an RRCReconfigurationComplete message in NR cell 1.

v_ReportConfigList    := {cs_NR_ReportConfigToAddMod_NR(tsc_NR_IdReportConfigId1, cs_38508_CondTriggerConfig(v_Rsrp)),

cs_NR_ReportConfigToAddMod_NR(tsc_NR_IdReportConfigId2, cs_38508_CondTriggerConfig_EventA5(v_RsrpThres1, v_RsrpThres2))};

v_MeasConfig_Step_1   :=  f_NR_GenerateMeasurementConfigNR({cs_MeasObjectId1(nr_Cell1)}, v_ReportConfigList, {cs_NR_MeasId_Config_id1_obj1_conf1}, -, -, -, -);

v_ConditionalReconfiguration_r16_Step_1 := cs_ConditionalReconfiguration_r16( -,

cs_CondReconfigToAddModList_r16_2EntryCR_1EntryMI(1, tsc_NR_MeasId1, f_NR_GenerateCondRRCReconfig_r16_Octetstring(nr_Cell2),

2, tsc_NR_MeasId1, f_NR_GenerateCondRRCReconfig_r16_Octetstring(nr_Cell4)));

f_NR5GC_CHO_RRCReconfiguration_SSConfig( nr_Cell1,

nr_Cell2,

v_DRBInfoList,

-,-,

cs_38508_RRCReconfiguration_CHO(-,v_MeasConfig_Step_1, v_ConditionalReconfiguration_r16_Step_1),

-, -, -, -); // fresh key according to Table 8.1.4.4.1.3.3-8

f_NR_SS_SystemIndCtrlConfig(nr_Cell2, cds_NR_SystemIndCtrl_RachPreamble(enable)); // @sic R5s220317 sic@

f_NR_SS_SystemIndCtrlConfig(nr_Cell4, cds_NR_SystemIndCtrl_RachPreamble(enable)); // @sic R5s220317 sic@

//@siclog "Step 3" siclog@

//Check: Does the UE initiate a random access procedure transmitting Preamble in NR Cell 2 or in NR cell 4 within 10s?

t_Watchdog.start;

alt {

[]SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell2, cr_TimingInfo_Any, ?))

{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 3, PRACH sent by UE on NR Cell 2");

}

[]SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell4, cr_TimingInfo_Any, ?)) // @sic R5s220317 sic@

{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 3, PRACH sent by UE on NR Cell 4");

}

[]t_Watchdog.timeout

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3");

}

};

//@siclog "Step 4" siclog@

//SS adjusts the cell-specific reference signal level according to row "T1"

f_NR_SetCellPowerList(v_CellPowerList_AtT1);

//@siclog "Step 5" siclog@

//Check: Does the UE initiate a random access procedure by transmitting Preamble in NR Cell 2 within 10s?

t_Watchdog.start;

SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell2, cr_TimingInfo_Any, ?));

t_Watchdog.stop;

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5");

//@siclog "Step 6-7" siclog@

//The SS transmits Random Access Response.

//The UE transmits RRCReconfigurationComplete message in NR Cell 2.

f_NR5GC_CHO_RRCReconfigurationComplete( nr_Cell1,

nr_Cell2,

-);

//@siclog "Steps 8" siclog@

//Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.1-1 indicate that the UE is capable of exchanging IP data on DRB1 and NR Cell 2?

v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession();

f_NR5GC_CheckDataPath(nr_Cell2,

f_NR5GC_GetPdnIndex(v_PDUSessionInfoList[0]),

dedicatedBearer,

v_DRBId,

"Step 8");

//@siclog "Steps 9-10" siclog@

//The SS transmits an RRCReconfiguration message including MeasConfig and ConditionalReconfiguration to set NR Cell 1 and NR Cell 4 as target candidate cells and configure event A5 as trigger event in NR cell 2.

//The UE transmits an RRCReconfigurationComplete message in NR cell 2.

v_MeasConfig_Step_9 := f_NR_GenerateMeasurementConfigNR({cs_MeasObjectId1(nr_Cell2)}, v_ReportConfigList, {cs_NR_MeasId_Config_id1_obj1_conf2}, -, -, nr_Cell2); // @sic R5s220317 sic@

v_ConditionalReconfiguration_r16_Step_9 := cs_ConditionalReconfiguration_r16( -,

cs_CondReconfigToAddModList_r16_2EntryCR_1EntryMI(1, tsc_NR_MeasId1, f_NR_GenerateCondRRCReconfig_r16_Octetstring(nr_Cell1),

2, tsc_NR_MeasId1,  f_NR_GenerateCondRRCReconfig_r16_Octetstring(nr_Cell4)));// @sic R5s220317 sic@

f_NR5GC_CHO_RRCReconfiguration_SSConfig( nr_Cell2, // @sic R5s220317 sic@

nr_Cell4, // @sic R5s220317 sic@

v_DRBInfoList,

-, -,

cs_38508_RRCReconfiguration_CHO(-,v_MeasConfig_Step_9, v_ConditionalReconfiguration_r16_Step_9),

-, -, -, -); //fresh key according to Table 8.1.4.4.1.3.3-8

//@siclog "Step 11" siclog@

//SS adjusts the cell-specific reference signal level according to row "T2".

f_NR_SetCellPowerList(v_CellPowerList_AtT2);

//@siclog "Step 12" siclog@

//Check: Does the UE initiate a random access procedure by transmitting Preamble in NR Cell 4 within 10s?

t_Watchdog.start;

SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell4, cr_TimingInfo_Any, ?));

t_Watchdog.stop;

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 12");

//@siclog "Steps 13-14" siclog@

//Check:The SS transmits Random Access Response.

//The UE transmits RRCReconfigurationComplete message in NR Cell 4.

f_NR5GC_CHO_RRCReconfigurationComplete( nr_Cell2,

nr_Cell4,

-);

f_NR_SS_SystemIndCtrlConfig(nr_Cell2, cds_NR_SystemIndCtrl_RachPreamble(disable)); // @sic R5s220317 sic@

f_NR_SS_SystemIndCtrlConfig(nr_Cell4, cds_NR_SystemIndCtrl_RachPreamble(disable)); // @sic R5s220317 sic@

//@siclog "Step 15" siclog@

//Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.1-1 indicate that the UE is capable of exchanging IP data on DRB1 and NR Cell 4?

f_NR5GC_CheckDataPath(nr_Cell4,

f_NR5GC_GetPdnIndex(v_PDUSessionInfoList[0]),

dedicatedBearer,

v_DRBId,

"Step 15");

//@siclog "Steps 16-17" siclog@

//The SS transmits an RRCReconfiguration message including MeasConfig and ConditionalReconfiguration for intra-frequency event A3 and A5 in NR cell 4.

//The UE transmits an RRCReconfigurationComplete message in NR cell 4.

v_MeasConfig_Step_16 := f_NR_GenerateMeasurementConfigNR({cs_MeasObjectId1(nr_Cell1)}, v_ReportConfigList, {cs_NR_MeasId_Config_id1_obj1_conf1, cs_NR_MeasId_Config_id2_obj1_conf2}, -, -, nr_Cell4); // @sic R5s220317 sic@

v_ConditionalReconfiguration_r16_Step_16 := cs_ConditionalReconfiguration_r16( -,

cs_CondReconfigToAddModList_r16_2EntryCR_2EntryMI_First_1Entry_Second(1, tsc_NR_MeasId1, tsc_NR_MeasId2, f_NR_GenerateCondRRCReconfig_r16_Octetstring(nr_Cell1),

2, tsc_NR_MeasId1,  f_NR_GenerateCondRRCReconfig_r16_Octetstring(nr_Cell2)));

f_NR5GC_CHO_RRCReconfiguration_SSConfig( nr_Cell4,

nr_Cell1,

v_DRBInfoList,

-, -,

cs_38508_RRCReconfiguration_CHO(-,v_MeasConfig_Step_16, v_ConditionalReconfiguration_r16_Step_16),

-, -, -, -); // fresh key according to Table 8.1.4.4.1.3.3-8

f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(enable)); // @sic R5s220317 sic@

//@siclog "Step 18" siclog@

//SS adjusts the cell-specific reference signal level according to row "T3".

f_NR_SetCellPowerList(v_CellPowerList_AtT3);

//@siclog "Step 19" siclog@

//Check: Does the UE initiate a random access procedure by transmitting Preamble in NR Cell 1 within 10s?

t_Watchdog.start;

SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo_Any, ?));

t_Watchdog.stop;

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 19");

//@siclog "Steps 20-21" siclog@

//The SS transmits Random Access Response.

//The UE transmits RRCReconfigurationComplete message in NR Cell 1.

f_NR5GC_CHO_RRCReconfigurationComplete( nr_Cell4,

nr_Cell1,

-);

f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(disable)); // @sic R5s220317 sic@

//@siclog "Step 22" siclog@

//Check: Does the test result of generic test procedure in TS 38.508-1 Table 4.9.1-1 indicate that the UE is capable of exchanging IP data on DRB1 and NR Cell 1?

f_NR5GC_CheckDataPath(nr_Cell1,

f_NR5GC_GetPdnIndex(v_PDUSessionInfoList[0]),

dedicatedBearer,

v_DRBId,

"Step 22");

//@siclog "Step 23" siclog@

//SS adjusts the cell-specific reference signal level according to row "T1".

f_NR_SetCellPowerList(v_CellPowerList_AtT1);

//@siclog "Step 24" siclog@

//Check: Does the UE transmit RRCReconfigurationComplete message in NR Cell 2 within 10s?

t_Watchdog.start;

alt {

[]SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell2, cr_38508_RRCReconfigurationComplete))

{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 24, RRCReconfigurationComplete sent by UE ");

}

[]t_Watchdog.timeout

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 24");

}

};

…

}// End of fl_TC_8_1_4_4_1_TestBody

//Based to the comment from Anritsu, MasterKeyUpdate Condition according to table 8.1.4.4.3.3.3-8 TS 38.508-1 [4] Table 4.8.1-1A with condition RBConfig_KeyChange should be added on Conditional HandOver Octate.

| Function name | f_NR_GenerateCondRRCReconfig_r16_Octetstring() |
| --- | --- |
| Reason for change | - MasterKeyUpdate Condition according to table 8.1.4.4.3.3.3-8 TS 38.508-1 [4] Table 4.8.1-1A with condition RBConfig_KeyChange should be added on Conditional HandOver Octate. - Change 3 and Change 4 imported from R5s220271. |
| Summary of change | - Added masterKeyUpdate template. - Please see below details |
| TTCN module | RRC_Conditional_Handover_NR5GC.ttcn |
| MCC160 Comment | - Accepted - Other implementation is done according to R5s220271 |

function f_NR_GenerateCondRRCReconfig_r16_Octetstring(NR_CellId_Type p_TargetCellId) runs on NR5GC_PTC return octetstring

{

var template (value) DL_DCCH_Message v_DL_DCCH_Message; // @sic R5s220271 sic@

var template (value) RRCReconfiguration v_CondRRCReconfig; // @sic R5s220271 sic@

var octetstring v_CondRRCReconfig_Octetstring;

v_DL_DCCH_Message := f_NR5GC_RRCReconfiguration_IntraNR_HO(p_TargetCellId, -, -, -, -, -, cs_MasterKeyUpdate(false, tsc_NR_38508_NextHopChainingCount)); // @sic R5s220271 R5s220375 sic@

v_CondRRCReconfig := v_DL_DCCH_Message.message_.c1.rrcReconfiguration; // @sic R5s220271 sic@

v_CondRRCReconfig_Octetstring := bit2oct(encvalue(v_CondRRCReconfig)); // String to be encoded

return v_CondRRCReconfig_Octetstring;

}

# Branches executed

The test case was executed on NR band n78 using NR ciphering nea2 and integrity algorithm nia2

# Execution Log Files

## Samsung SM-G996B

The Samsung SM-G996B UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Keysight/TC_8_1_4_4_1_LOG.html

- PICS/PIXIT parameter file:

Keysight/TC_8_1_4_4_1_PIXIT.xml

# References

| [1] | R5s220318 : NR5GC FR1 : Supporting information for addition of NR5GC multiayer test case 8.1.4.4.1 |
| --- | --- |