3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220332

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2400 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC test case 7.1.1.3.9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add NR test case 7.1.1.3.9 to the IWD_21wk49 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 7.1.1.3.9 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.9.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	4

2.1	f_TC_7_1_1_3_9_NR5GC	4

2.2	fl_TC_7_1_1_3_9_NR5GC_TestBody	5

2.3	f_TC_7_1_1_3_9_NR_TestBody	8

3	Branches executed	10

4	Execution Log Files	10

4.1	MediaTek MT6983 UE	10

5	References	11

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 7.1.1.3.9 which is part of the NR5GC and ENDC test suite in iWD_TTCN3-B2020-09_D21wk49.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	7.1.1.3.9.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D21wk49

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	MediaTek MT6983

Verification Status:	PASS

# Corrections required

## f_TC_7_1_1_3_9_NR5GC

| Function name | f_TC_7_1_1_3_9_NR5GC |
| --- | --- |
| Reason for change | MAC cannot be put in test mode until the signalling to communicate the change in the PUSCH_AggegationFactor has been sent to the UE at Step 0A |
| Summary of change | Removed the configuration to put MAC in test mode |
| TTCN module | MAC_NR5GC |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_7_1_1_3_9_NR5GC() runs on NR5GC_PTC { // Correct Handling of UL HARQ process / PUSCH Aggregation <<SKIPPED CODE>> //Configure on SS the DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1, TEST_LOOPModeA_ON, cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, -, //p_IpPduDelayTime -, -, //MAC_CellGroupConfig -, v_SS_Drb_ConfigList ); v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDL_UL)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); f_NR_TestBody_Set(true); fl_TC_7_1_1_3_9_NR5GC_TestBody(v_DRBId); <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_3_9_NR5GC() runs on NR5GC_PTC { // Correct Handling of UL HARQ process / PUSCH Aggregation <<SKIPPED CODE>> //Configure on SS the DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1, TEST_LOOPModeA_ON, cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, -, //p_IpPduDelayTime -, -, //MAC_CellGroupConfig -, v_SS_Drb_ConfigList ); /*REMOVED v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDL_UL)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList);*/ f_NR_TestBody_Set(true); fl_TC_7_1_1_3_9_NR5GC_TestBody(v_DRBId); <<SKIPPED CODE>>

## fl_TC_7_1_1_3_9_NR5GC_TestBody

| Function name | fl_TC_7_1_1_3_9_NR5GC_TestBody |
| --- | --- |
| Reason for change | - MAC should put test mode after the peer signalling to communicate change in the PUSCH_AggregationFactor is sent to the UE. - The peer signalling message only changes the PUSCH_AggregationFactor in the BWP_UplinkDedicated. However, the local end configuration chooses to also include the common configuration for RACH, PUSCH, PUCCH in BWP_UplinkCommon. This is not required to be sent again as nothing has changed in the BWP_UplinkCommon from the previous instance the local end configuration was done. |
| Summary of change | - MAC put in test mode after signalling message at Step 0A is sent - BWP_UplinkCommon is set to Omit. |
| TTCN module | MAC_NR5GC |
| MCC160 Comment | - Accepted, use p_NR_DRB_Id instead of v_DRBId - Accepted as improvement, we need only to set the common part to omit before reconfiguring SS. See MCC160 proposed implementation |

Before Change:

function fl_TC_7_1_1_3_9_NR5GC_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { <<SKIPPED CODE>> //@siclog "Steps 0A-0B" siclog@ //SS transmits in the indicated downlink assignment an NR RRCReconfiguration. (Note 1) //UE transmits NR RRCReconfigurationComplete message to the SS. (Note 2) //Configure pusch-AggregationFactor=n4 or n8 in UE and SS //Set pusch-AggregationFactor as per Table 7.1.1.3.9.3.3-3: PUSCH-Config (Table 7.1.1.3.9.3.3-2) if ((f_NR_CellInfo_GetIsFDD(nr_Cell1) and f_NR_CellInfo_GetSCS(nr_Cell1) == kHz15) or not(f_NR_CellInfo_GetIsFR1(nr_Cell1))) { v_NR_UplinkBWP.Dedicated.R15.pusch_Config.setup.pusch_AggregationFactor := n8; } else { v_NR_UplinkBWP.Dedicated.R15.pusch_Config.setup.pusch_AggregationFactor := n4; } f_NR_CellInfo_SetUplinkBWP(nr_Cell1, v_NR_UplinkBWP, tsc_NR_BWP_Id); v_CellGroupConfig := cs_38508_CellGroupConfig( tsc_NR_CellGroupId_MCG,-,-,-,-, cs_NR_SpCellConfigDef(omit, -, -, cs_NR_ServingCellConfigDef(-, -, -, cs_NR_UplinkConfigDef(v_NR_UplinkBWP.Dedicated.R15, omit)))); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); f_NR_SendRRCReconfiguration(nr_Cell1, -,-, v_V1530Ext); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_UplinkBWP_ConfigCommon_REQ(nr_Cell1, -, {v_NR_UplinkBWP})); f_TC_7_1_1_3_9_NR_TestBody(p_NR_DRB_Id); }

After Change:

function fl_TC_7_1_1_3_9_NR5GC_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { <<SKIPPED CODE>> //WA# var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var DRB_Identity v_DRBId; //@siclog "Steps 0A-0B" siclog@ //SS transmits in the indicated downlink assignment an NR RRCReconfiguration. (Note 1) //UE transmits NR RRCReconfigurationComplete message to the SS. (Note 2) //Configure pusch-AggregationFactor=n4 or n8 in UE and SS //Set pusch-AggregationFactor as per Table 7.1.1.3.9.3.3-3: PUSCH-Config (Table 7.1.1.3.9.3.3-2) if ((f_NR_CellInfo_GetIsFDD(nr_Cell1) and f_NR_CellInfo_GetSCS(nr_Cell1) == kHz15) or not(f_NR_CellInfo_GetIsFR1(nr_Cell1))) { v_NR_UplinkBWP.Dedicated.R15.pusch_Config.setup.pusch_AggregationFactor := n8; } else { v_NR_UplinkBWP.Dedicated.R15.pusch_Config.setup.pusch_AggregationFactor := n4; } v_NR_UplinkBWP.Common := omit; //WA# //REMOVED f_NR_CellInfo_SetUplinkBWP(nr_Cell1, v_NR_UplinkBWP, tsc_NR_BWP_Id); v_CellGroupConfig := cs_38508_CellGroupConfig( tsc_NR_CellGroupId_MCG,-,-,-,-, cs_NR_SpCellConfigDef(omit, -, -, cs_NR_ServingCellConfigDef(-, -, -, cs_NR_UplinkConfigDef(v_NR_UplinkBWP.Dedicated.R15, omit)))); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); f_NR_SendRRCReconfiguration(nr_Cell1, -,-, v_V1530Ext); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_UplinkBWP_ConfigCommon_REQ(nr_Cell1, -, {v_NR_UplinkBWP})); //WA# v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDL_UL)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); f_TC_7_1_1_3_9_NR_TestBody(p_NR_DRB_Id); }

MCC160 proposed implementation

function fl_TC_7_1_1_3_9_NR5GC_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC

{

//Correct Handling of UL HARQ process / PUSCH Aggregation

var template (value) CellGroupConfig v_CellGroupConfig;

var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id);

var template (value) RRCReconfiguration_v1530_IEs v_V1530Ext;

var template (value)  NR_RadioBearerList_Type v_SS_Drb_ConfigList;

//@siclog "Steps 0A-0B" siclog@

//SS transmits in the indicated downlink assignment an NR RRCReconfiguration. (Note 1)

//UE transmits NR RRCReconfigurationComplete message to the SS. (Note 2)

//Configure pusch-AggregationFactor=n4 or n8 in UE and SS

//Set pusch-AggregationFactor as per Table 7.1.1.3.9.3.3-3: PUSCH-Config (Table 7.1.1.3.9.3.3-2)

if ((f_NR_CellInfo_GetIsFDD(nr_Cell1) and f_NR_CellInfo_GetSCS(nr_Cell1) == kHz15) or not(f_NR_CellInfo_GetIsFR1(nr_Cell1))) {

v_NR_UplinkBWP.Dedicated.R15.pusch_Config.setup.pusch_AggregationFactor := n8;

} else {

v_NR_UplinkBWP.Dedicated.R15.pusch_Config.setup.pusch_AggregationFactor := n4;

}

f_NR_CellInfo_SetUplinkBWP(nr_Cell1, v_NR_UplinkBWP, tsc_NR_BWP_Id);

v_CellGroupConfig := cs_38508_CellGroupConfig( tsc_NR_CellGroupId_MCG,-,-,-,-,

cs_NR_SpCellConfigDef(omit, -, -,

cs_NR_ServingCellConfigDef(-, -, -,

cs_NR_UplinkConfigDef(v_NR_UplinkBWP.Dedicated.R15, omit))));

v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig)));

f_NR_SendRRCReconfiguration(nr_Cell1, -,-, v_V1530Ext);

v_NR_UplinkBWP.Common := omit; //@sic R5s220332 sic@

f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_UplinkBWP_ConfigCommon_REQ(nr_Cell1, -, {v_NR_UplinkBWP}));

// Configure MAC in TestMode - after signalling on SRBs

//@sic R5s220332 sic@

v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(p_NR_DRB_Id, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDL_UL)};

f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList);

f_TC_7_1_1_3_9_NR_TestBody(p_NR_DRB_Id);

}

## f_TC_7_1_1_3_9_NR_TestBody

| Function name | f_TC_7_1_1_3_9_NR_TestBody |
| --- | --- |
| Reason for change | - As mentioned in the previous change, BWP_UplinkCommon is not required to be configured again in the local end configuration, however, this information is not set back to the Database as the function f_NR_ResourceAllocation_UplinkGrant makes use of the BWP_UplinkCommon to check for the TimeDomainResourceAllocation. Hence, the pusch_AggregationFactor should be set to the appropriate value before starting Step 1. - The setting of the Poll Bit to 1 in the RLC PDU would make the UE send Ack along with the loop back PDU at Step 4. However, only 1 entry in the SubPDUList is expected to be present in the variable v_SDU_SubPDUListUL whereas the presence of the ACK would lead to two entries being reported back causing a template mismatch and TC failure. It is recommended to set the Poll Bit to 0 - The deactivion of the local SR handler should be done after Step 4 to capture any unwanted SRs. At the moment, it is being deactivated too early. |
| Summary of change | - PUSCH_AggregationFactor is set to the correct value - Poll Bit is set to 0 - SR Handler deactivated after Step 4. |
| TTCN module | MAC_TC_Common_NR |
| MCC160 Comment | 1. Not needed as per MCC160 proposal in change 2.1 2. Accepted 3. Accepted |

Before Change:

function f_TC_7_1_1_3_9_NR_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { <<SKIPPED CODE>> var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL; //The SS ignores scheduling requests and does not allocate any uplink grant. f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); //The SS gets prepared for SR indication and stops default UL grant allocation. f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(enable)); // PDCP SDU =36B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header=44B =352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_Poll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_36B))); v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; v_SDU_SubPDUListUL := { cr_NR_MAC_SDU_SubPDU(?, ?) }; v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL)}; <<SKIPPED CODE>> // @siclog "Step 3" siclog@ //The SS allocates an UL Grant for HARQ process 1, sufficient for one RLC SDU to be looped back in a slot n, and NDI indicates new transmission and DCI scheduling the PUSCH indicates rvID = 0. v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(384, v_NR_UplinkBWP); // 384 bits f_NR_OneULGrantTransmission(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, cs_HarqProcessConfig(1)); deactivate(v_IgnoreNRSR); //UE shall not transmit SR for retransmissions <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_3_9_NR_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { <<SKIPPED CODE>> var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var template (present) NR_MAC_PDUList_Type v_MAC_PDUListUL; //WA# if ((f_NR_CellInfo_GetIsFDD(nr_Cell1) and f_NR_CellInfo_GetSCS(nr_Cell1) == kHz15) or not(f_NR_CellInfo_GetIsFR1(nr_Cell1))) { v_NR_UplinkBWP.Dedicated.R15.pusch_Config.setup.pusch_AggregationFactor := n8; } else { v_NR_UplinkBWP.Dedicated.R15.pusch_Config.setup.pusch_AggregationFactor := n4; } //The SS ignores scheduling requests and does not allocate any uplink grant. f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); //The SS gets prepared for SR indication and stops default UL grant allocation. f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(enable)); // PDCP SDU =36B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header=44B =352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_36B))); v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; v_SDU_SubPDUListUL := { cr_NR_MAC_SDU_SubPDU(?, ?) }; v_MAC_PDUListUL := { cr_NR_UL_MAC_PDU(v_SDU_SubPDUListUL)}; <<SKIPPED CODE>> f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); //Stop SS reporting SR indications f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable)); deactivate(v_IgnoreNRSR); //WA# UE shall not transmit SR for retransmissions //@siclog "Step 5" siclog@ //SS transmits a MAC PDU containing an RLC STATUS PDU acknowledging the reception of the AMD PDU in step 4. f_NR_RLC_Status_MAC_PDU_Tx(tsc_LCID_SCG_Bearer, 1, p_NR_DRB_Id); }

# Branches executed

## This NR TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n78” using NEA0/NIA1  algorithms.

# Execution Log Files

## MediaTek MT6983 UE

The MediaTek MT6983 UE passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
TC_7_1_1_3_9_MTK.log

(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s220333: Supporting information for addition of NR5GC testcase 7.1.1.3.9 |
| --- | --- |