3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220372

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2411 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the reestablishment functions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN Implementation , value expected for nonCriticalExtension RRCReestablishmentComplete-v1610-IEs OPTIONAL is not in align with the Table 4.6.1-11: RRCReestablishmentComplete ( Spec 38.508 ). While running rel-15 reestablishment test cases with Rel16 UEs, UE transmits RRCReestablishmentComplete-v1610-IEs filled (not omit) ; TTCN expects this value to be “Omit” in Rel-15 test cases , and it is leading to unexpected test case failure due to message content mismatch. |  |  |  |  |  |  |  |  |
| Summary of change: |  | nonCriticalExtension RRCReestablishmentComplete-v1610-IEs is filled with Not Checked |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All Rel15 Reestablishment test cases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	2

1.1	Correction to the function f_NR5GC_RRCReestablishment_Def	2

1.2	Correction to the function f_NR5GC_RRCReestablishment_Steps3To7	4

1.3	Correction to the template UL_DCCH_Message cr_38508_RRCReestablishmentComplete	6

# Corrections required

## Correction to the function f_NR5GC_RRCReestablishment_Def

| Function name | f_NR5GC_RRCReestablishment_Def |
| --- | --- |
| Reason for change | In the current TTCN Implementation , value expected for nonCriticalExtension RRCReestablishmentComplete-v1610-IEs OPTIONAL is not in align with the Table 4.6.1-11: RRCReestablishmentComplete ( Spec 38.508 ). While running rel-15 reestablishment test cases with Rel16 UE builds , as expected UE is sending RRCReestablishmentComplete-v1610-IEs filled ; TTCN expects this value to be “Omit” in Rel-15 test cases , and it is leading to unexpected test case failure due to message content mismatch. |
| Summary of change | nonCriticalExtension RRCReestablishmentComplete-v1610-IEs is filled with Not Checked |
| TTCN module | ttcn\develop\Common\NR5GC\NR5GC_RRCSteps.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_NR5GC_RRCReestablishment_Def(NR_CellId_Type p_NR_SourceCellId, NR_CellId_Type p_NR_TargetCellId, template(present) ReestablishmentCause p_ReestablishmentCause := handoverFailure, NextHopChainingCount p_NextHopChainingCount := 0, template (omit) NR_SCellToAddModList_Type p_SCellConfig := omit, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value4, template(omit) charstring p_VerdictMsg := omit, template(omit) SpCellConfig.rlf_TimersAndConstants p_RlfTimersAndConstants := omit, template RRCReestablishmentComplete_v1610_IEs p_RRCReestablishmentComplete_v1610_IEs := omit, boolean p_ReceiveForRRCreconfigurationComplete := true ) runs on NR5GC_PTC { //@sic R5-206369 sic@ //Steps 1-2 f_NR5GC_RRCReestablishment_Steps1To2(p_NR_SourceCellId, p_NR_TargetCellId, p_ReestablishmentCause, p_RNTI_Value, p_VerdictMsg); //@sic R5-211391 sic@ //Steps 3-7 f_NR5GC_RRCReestablishment_Steps3To7(p_NR_SourceCellId, p_NR_TargetCellId, p_NextHopChainingCount, -, p_RRCReestablishmentComplete_v1610_IEs); //Steps 8-13 //p_SCellConfig is used only to send RRCReconfiguration to UE - The SCell in SS shall already be configured f_NR5GC_RRCReestablishment_Steps8To13(p_NR_SourceCellId, p_NR_TargetCellId, p_SCellConfig, p_RlfTimersAndConstants, -, p_ReceiveForRRCreconfigurationComplete); }

After Change:

function f_NR5GC_RRCReestablishment_Def(NR_CellId_Type p_NR_SourceCellId, NR_CellId_Type p_NR_TargetCellId, template(present) ReestablishmentCause p_ReestablishmentCause := handoverFailure, NextHopChainingCount p_NextHopChainingCount := 0, template (omit) NR_SCellToAddModList_Type p_SCellConfig := omit, RNTI_Value p_RNTI_Value := tsc_C_RNTI_Value4, template(omit) charstring p_VerdictMsg := omit, template(omit) SpCellConfig.rlf_TimersAndConstants p_RlfTimersAndConstants := omit, template RRCReestablishmentComplete_v1610_IEs p_RRCReestablishmentComplete_v1610_IEs := *, boolean p_ReceiveForRRCreconfigurationComplete := true ) runs on NR5GC_PTC { //@sic R5-206369 sic@ //Steps 1-2 f_NR5GC_RRCReestablishment_Steps1To2(p_NR_SourceCellId, p_NR_TargetCellId, p_ReestablishmentCause, p_RNTI_Value, p_VerdictMsg); //@sic R5-211391 sic@ //Steps 3-7 f_NR5GC_RRCReestablishment_Steps3To7(p_NR_SourceCellId, p_NR_TargetCellId, p_NextHopChainingCount, -, p_RRCReestablishmentComplete_v1610_IEs); //Steps 8-13 //p_SCellConfig is used only to send RRCReconfiguration to UE - The SCell in SS shall already be configured f_NR5GC_RRCReestablishment_Steps8To13(p_NR_SourceCellId, p_NR_TargetCellId, p_SCellConfig, p_RlfTimersAndConstants, -, p_ReceiveForRRCreconfigurationComplete); }

## Correction to the function f_NR5GC_RRCReestablishment_Steps3To7

| Function name | f_NR5GC_RRCReestablishment_Steps3To7 |
| --- | --- |
| Reason for change | In the current TTCN Implementation , value expected for nonCriticalExtension RRCReestablishmentComplete-v1610-IEs OPTIONAL is not in align with the Table 4.6.1-11: RRCReestablishmentComplete ( Spec 38.508 ). While running rel-15 reestablishment test cases with Rel16 UE builds , as expected UE is sending RRCReestablishmentComplete-v1610-IEs filled ; TTCN expects this value to be “Omit” in Rel-15 test cases , and it is leading to unexpected test case failure due to message content mismatch. |
| Summary of change | nonCriticalExtension RRCReestablishmentComplete-v1610-IEs is filled with Not Checked |
| TTCN module | ttcn-3/ttcn/develop/Common/NR5GC/ NR5GC_RRCSteps.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_NR5GC_RRCReestablishment_Steps3To7(NR_CellId_Type p_NR_SourceCellId, NR_CellId_Type p_NR_TargetCellId, NextHopChainingCount p_NextHopChainingCount := 0, template(omit) MacBearerRouting_Type p_MacBearerRoutingCA := omit, template RRCReestablishmentComplete_v1610_IEs p_RRCReestablishmentComplete_v1610_IEs := omit ) runs on NR_BASE_PTC { // @sic R5-206369 R5-213516: added p_MacBearerRoutingCA sic@ var IntegerList_Type v_DrbIdList := f_NR_GetActiveDRBs(p_NR_SourceCellId); // 3. Source Cell: If the Source Cell is the same as the Target Cell: // Reset SRBs and release DRBs // Stop periodic TA if (p_NR_SourceCellId == p_NR_TargetCellId) { f_NR_SS_SRBs_DRBs_Release(p_NR_SourceCellId, cs_TimingInfo_Now, v_DrbIdList); //@sic R5-211417 R5s211169 sic@ //WA#WI=MCC f_NR_SS_SRBs_DRBs_Config(p_NR_SourceCellId, cs_TimingInfo_Now, omit); //WA#WI=919319 //WA#WI=MCC // @sic R5-217830 sic@ } // 4. void // @sic R5-217830 sic@ // 5. Target Cell: Activate Security f_NR_SS_AS_ActivateSecurity_Current(p_NR_TargetCellId, p_NextHopChainingCount, -, true); // 6. Target : Send RRCReestablishment SRB.send (cas_NR_SRB1_RrcPdu_REQ(p_NR_TargetCellId, cs_TimingInfo_Now, cs_38508_RRCReestablishment_Def(tsc_NR_RRC_TI_Def, p_NextHopChainingCount), p_MacBearerRoutingCA)); // 7. Target Cell: Receive RRCReestablishmentComplete SRB.receive ( car_NR_SRB1_RrcPdu_IND ( p_NR_TargetCellId, cr_38508_RRCReestablishmentComplete(-, p_RRCReestablishmentComplete_v1610_IEs), -, p_MacBearerRoutingCA) ); }

After Change:

function f_NR5GC_RRCReestablishment_Steps3To7(NR_CellId_Type p_NR_SourceCellId, NR_CellId_Type p_NR_TargetCellId, NextHopChainingCount p_NextHopChainingCount := 0, template(omit) MacBearerRouting_Type p_MacBearerRoutingCA := omit, template RRCReestablishmentComplete_v1610_IEs p_RRCReestablishmentComplete_v1610_IEs := * //WA#WI=938144 ) runs on NR_BASE_PTC { // @sic R5-206369 R5-213516: added p_MacBearerRoutingCA sic@ var IntegerList_Type v_DrbIdList := f_NR_GetActiveDRBs(p_NR_SourceCellId); // 3. Source Cell: If the Source Cell is the same as the Target Cell: // Reset SRBs and release DRBs // Stop periodic TA if (p_NR_SourceCellId == p_NR_TargetCellId) { f_NR_SS_SRBs_DRBs_Release(p_NR_SourceCellId, cs_TimingInfo_Now, v_DrbIdList); //@sic R5-211417 R5s211169 sic@ //WA#WI=MCC f_NR_SS_SRBs_DRBs_Config(p_NR_SourceCellId, cs_TimingInfo_Now, omit); //WA#WI=919319 //WA#WI=MCC // @sic R5-217830 sic@ } // 4. void // @sic R5-217830 sic@ // 5. Target Cell: Activate Security f_NR_SS_AS_ActivateSecurity_Current(p_NR_TargetCellId, p_NextHopChainingCount, -, true); // 6. Target : Send RRCReestablishment SRB.send (cas_NR_SRB1_RrcPdu_REQ(p_NR_TargetCellId, cs_TimingInfo_Now, cs_38508_RRCReestablishment_Def(tsc_NR_RRC_TI_Def, p_NextHopChainingCount), p_MacBearerRoutingCA)); // 7. Target Cell: Receive RRCReestablishmentComplete SRB.receive ( car_NR_SRB1_RrcPdu_IND ( p_NR_TargetCellId, cr_38508_RRCReestablishmentComplete(-, p_RRCReestablishmentComplete_v1610_IEs), -, p_MacBearerRoutingCA) ); }

## Correction to the template UL_DCCH_Message cr_38508_RRCReestablishmentComplete

| Template name | UL_DCCH_Message |
| --- | --- |
| Reason for change | In the current TTCN Implementation , value expected for nonCriticalExtension RRCReestablishmentComplete-v1610-IEs OPTIONAL is not in align with the Table 4.6.1-11: RRCReestablishmentComplete ( Spec 38.508 ). While running rel-15 reestablishment test cases with Rel16 Ucr_38508_RRCReestablishmentCompleteE builds , as expected UE is sending RRCReestablishmentComplete-v1610-IEs filled ; TTCN expects this value to be “Omit” in Rel-15 test cases , and it is leading to unexpected test case failure due to message content mismatch. |
| Summary of change | nonCriticalExtension RRCReestablishmentComplete-v1610-IEs is filled with Not Checked |
| TTCN module | Common\NR5GC\NR5GC_RRC_Templates.ttcn |
| MCC160 Comment | Accepted |

Before Change:

template(present) UL_DCCH_Message cr_38508_RRCReestablishmentComplete(RRC_TransactionIdentifier p_RRC_TI := tsc_NR_RRC_TI_Def, template RRCReestablishmentComplete_v1610_IEs p_RRCReestablishmentComplete_v1610_IEs := omit ) := { //As per 38.508-1 Table 4.6.1-11: RRCReestablishmentComplete /* @status APPROVED (NR5GC) */ message_ := { c1 := { rrcReestablishmentComplete := { rrc_TransactionIdentifier := p_RRC_TI, criticalExtensions := { rrcReestablishmentComplete := { lateNonCriticalExtension := *, nonCriticalExtension := p_RRCReestablishmentComplete_v1610_IEs } } } } } };

After Change:

template(present) UL_DCCH_Message cr_38508_RRCReestablishmentComplete(RRC_TransactionIdentifier p_RRC_TI := tsc_NR_RRC_TI_Def, template RRCReestablishmentComplete_v1610_IEs p_RRCReestablishmentComplete_v1610_IEs := * ) := //WA#WI=938144 { //As per 38.508-1 Table 4.6.1-11: RRCReestablishmentComplete /* @status APPROVED (NR5GC) */ message_ := { c1 := { rrcReestablishmentComplete := { rrc_TransactionIdentifier := p_RRC_TI, criticalExtensions := { rrcReestablishmentComplete := { lateNonCriticalExtension := *, nonCriticalExtension := p_RRCReestablishmentComplete_v1610_IEs } } } } } };