3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220375

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2412 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC Rel-16 Mobility Enhancement test case 8.1.4.4.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2022-03-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add Rel-16 Mobility Enhancement test case 8.1.4.4.3 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to Rel-16 Mobility Enhancement test case 8.1.4.4.3 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.4.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	6

3	Branches executed	18

4	Execution Log Files	18

4.1	Mediatek MT6983	18

5	References	18

- Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 8.1.4.4.3 which is part of the NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

  - Verification Test Summary

Test Case: 	8.1.4.4.3

ATS Version:	iwd-TTCN3-B2020-09_D21wk49

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR,

UE used:	Mediatek MT6983

Verification Status:	PASS

- Corrections required

Change 2 and 3 of R5s220271 is also  required for the verification of this test case(8.1.4.4.3)

  - Change 1

| Function name | f_NR_GenerateCondRRCReconfig_r16_Octetstring() |
| --- | --- |
| Reason for change | MasterKeyUpdate Condition according to table 8.1.4.4.3.3.3-8 TS 38.523-1 with condition RBConfig_KeyChange should be added on Conditional HandOver Octate. |
| Summary of change | Added masterKeyUpdate template. |
| TTCN module | RRC_Conditional_Handover_NR5GC.ttcn |
| MCC160 Comment | - Accepted |

Before Change

function f_NR_GenerateCondRRCReconfig_r16_Octetstring(NR_CellId_Type p_TargetCellId) runs on NR5GC_PTC return octetstring { var template (value) DL_DCCH_Message v_CondRRCReconfig; var octetstring v_CondRRCReconfig_Octetstring; v_CondRRCReconfig := f_NR5GC_RRCReconfiguration_IntraNR_HO(p_TargetCellId); v_CondRRCReconfig_Octetstring := bit2oct(encvalue(v_CondRRCReconfig)); // String to be encoded return v_CondRRCReconfig_Octetstring; }

After Change

function f_NR_GenerateCondRRCReconfig_r16_Octetstring(NR_CellId_Type p_TargetCellId) runs on NR5GC_PTC return octetstring { //var template (value) DL_DCCH_Message v_CondRRCReconfig; var template (value) RRCReconfiguration v_CondRRCReconfig; var octetstring v_CondRRCReconfig_Octetstring; v_CondRRCReconfig := f_NR5GC_RRCReconfiguration_IntraNR_HO_Mdfd(p_TargetCellId, -, -, -, -, -, cs_MasterKeyUpdate(false, tsc_NR_38508_NextHopChainingCount)); v_CondRRCReconfig_Octetstring := bit2oct(encvalue(v_CondRRCReconfig)); // String to be encoded return v_CondRRCReconfig_Octetstring; }

  - Change 2

| Function name | function f_TC_8_1_4_4_3_NR5GC() |
| --- | --- |
| Reason for change | - At step 19, SRB2 has not been established, so Deregistration Request at postamble will be requested on SRB1 by UE. - At step 19 Postamble will happen on Cell 2. |
| Summary of change | - Added SRB1 on variables to be passed on postamble funtion. - Cell 2 updated on Postamble. |
| TTCN module | RRC_Conditional_Handover_NR5GC.ttcn |
| MCC160 Comment | - Not accepted since the test case shall end with STATE_IDLE_1A Please see clause 2.4 Additional MCC160 implementation - Accepted |

Before Change

function f_TC_8_1_4_4_3_NR5GC() runs on NR5GC_PTC { // Conditional handover / Failure f_NR5GC_Init(NR_1); f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -79, -79); // Power levels are FFS for FR2 f_NR_CellInfo_InitMaxReferencePower(nr_Cell2, -79, -79); // Power levels are FFS for FR2 f_NR_CellInfo_InitMaxReferencePower(nr_Cell4, -79, -79); // Power levels are FFS for FR2 //Configure TAC of NR Cell 2 to 2 according to Table 8.1.4.4.3.3.3-1 f_NR_CellInfo_SetTAC (nr_Cell2, int2bit(2,24)); //Create and configure cells f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell2); f_NR_CellConfig_Def(nr_Cell4); // Preamble: If pc_IP_Ping is set to TRUE then, the UE is in 5GS state 3N-A on NR Cell 1 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3. // Else, the UE is in 5GS state 3N-A and Test Loop Function (On) with UE test loop mode B on NR Cell 1 according to 38.508-1[4], clause 4.4A.2 Table 4.4A.2-3. f_NR5GC_Preamble (nr_Cell1, STATE_CONNECTED_3A, PING_Or_TEST_LOOPModeB_ON); //Set SS/PBCH EPRE levels acc. to row "T0" f_NR_SetCellPower (nr_Cell1, -85, -85); // Power levels are FFS for FR2 f_NR_SetCellPower (nr_Cell2, -113, -113); // Power levels are FFS for FR2 f_NR_SetCellPower (nr_Cell4, -113, -113); // Power levels are FFS for FR2 f_NR_TestBody_Set(true); fl_TC_8_1_4_4_3_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell4, STATE_CONNECTED_3A); } // End of f_TC_8_1_4_4_3_NR5GC

After Change

function f_TC_8_1_4_4_3_NR5GC() runs on NR5GC_PTC { // Conditional handover / Failure f_NR5GC_Init(NR_1); f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -79, -79); // Power levels are FFS for FR2 f_NR_CellInfo_InitMaxReferencePower(nr_Cell2, -79, -79); // Power levels are FFS for FR2 f_NR_CellInfo_InitMaxReferencePower(nr_Cell4, -79, -79); // Power levels are FFS for FR2 //Configure TAC of NR Cell 2 to 2 according to Table 8.1.4.4.3.3.3-1 f_NR_CellInfo_SetTAC (nr_Cell2, int2bit(2,24)); //Create and configure cells f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell2); f_NR_CellConfig_Def(nr_Cell4); // Preamble: If pc_IP_Ping is set to TRUE then, the UE is in 5GS state 3N-A on NR Cell 1 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3. // Else, the UE is in 5GS state 3N-A and Test Loop Function (On) with UE test loop mode B on NR Cell 1 according to 38.508-1[4], clause 4.4A.2 Table 4.4A.2-3. f_NR5GC_Preamble (nr_Cell1, STATE_CONNECTED_3A, PING_Or_TEST_LOOPModeB_ON); //Set SS/PBCH EPRE levels acc. to row "T0" f_NR_SetCellPower (nr_Cell1, -85, -85); // Power levels are FFS for FR2 f_NR_SetCellPower (nr_Cell2, -113, -113); // Power levels are FFS for FR2 f_NR_SetCellPower (nr_Cell4, -113, -113); // Power levels are FFS for FR2 f_NR_TestBody_Set(true); fl_TC_8_1_4_4_3_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell2, STATE_CONNECTED_3A, tsc_NR_RbId_SRB1); //Anritsu TTCN CR required for 8.1.4.4.3.NR5GC } // End of f_TC_8_1_4_4_3_NR5GC

  - Change 3

| Function name | function fl_TC_8_1_4_4_3_TestBody() |
| --- | --- |
| Reason for change | - Premable will not be detected without enabling system Indication for Rach at Step 4-5, Step 7, Step14-15. - RACH procedure should be enabled at Cell 2 at step 17 as Cell 2 was set to no RandomAccessresponse at step 4-5. - Step 19 will be performed at cell2. |
| Summary of change | - Added f_NR_SS_SystemIndCtrlConfig - Enabled RACH procedure on cell 2. - Updated Cell ID. |
| TTCN module | RRC_Conditional_Handover_NR5GC.ttcn |
| MCC160 Comment | - Not Accepted in principle. We do not need to detect PRACH preamble in steps 4/5 and 14/15 since it’s not part of the TP. But we need to ensure T304 expires before moving to step 6/16. So f_Delay(2.0) is added in Step4/5 and Step 14/15 to detect the PRACH preamble. Please see clause 2.4 Additional MCC160 implementation - Accepted - Accepted |

Before Change

function fl_TC_8_1_4_4_3_TestBody() runs on NR5GC_PTC { var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT3; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT4; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT5; //var NR_AbsoluteCellPower_Type v_CellPower_Serving_FR1; var NR_AbsoluteCellPower_Type v_CellPower_EntryCondition_FR1; var NR_AbsoluteCellPower_Type v_CellPower_Leaving_FR1; var NR_AbsoluteCellPower_Type v_CellPower_NonSuitable_FR1; var RSRP_Range v_Rsrp; var integer v_DRBId; var template (value) MeasConfig v_MeasConfig; var template (value) ConditionalReconfiguration_r16 v_ConditionalReconfiguration_r16_Step_1; var template (value) ConditionalReconfiguration_r16 v_ConditionalReconfiguration_r16_Step_11; var template (value) NR_DRBInfoList_Type v_DRBInfoList := f_NR5GC_MobileInfo_GetDRBInfoList(); var PDUSessionInfoList_Type v_PDUSessionInfoList; timer t_Watchdog := 10.0; v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); //Power levels of NR cell 1, NR cell 2 and NR cell 4 FR1 acc. Table 8.1.4.4.1.3.2-1, Table 8.1.4.4.1.3.2-2 FFS /* ¦ Cell 1 ¦ Cell 2 ¦ Cell 4 * ---------------------------------------------- * T0 ¦ -85 ¦ -113 ¦ -113 * ---------------------------------------------- * T1 ¦ -91 ¦ -79 ¦ -91 * ---------------------------------------------- * T2 ¦ -113 ¦ -113 ¦ -79 * ---------------------------------------------- * T3 ¦ -79 ¦ -91 ¦ -91 * ---------------------------------------------- * T4 ¦ -113 ¦ -113 ¦ -113 * ---------------------------------------------- * T5 ¦ -113 ¦ -79 ¦ -113 */ // v_CellPower_Serving_FR1 := -85; v_CellPower_Leaving_FR1 := -91; v_CellPower_EntryCondition_FR1 := -79; v_CellPower_NonSuitable_FR1 := -113; v_CellPowerList_AtT1 := {cs_NR_CellPower(nr_Cell1, v_CellPower_Leaving_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell2, v_CellPower_EntryCondition_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell4, v_CellPower_Leaving_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // FR2 is FFS v_CellPowerList_AtT2 := {cs_NR_CellPower(nr_Cell1, v_CellPower_NonSuitable_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell2, v_CellPower_NonSuitable_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell4, v_CellPower_EntryCondition_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // FR2 is FFS v_CellPowerList_AtT3 := {cs_NR_CellPower(nr_Cell1, v_CellPower_EntryCondition_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell2, v_CellPower_Leaving_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell4, v_CellPower_Leaving_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // FR2 is FFS v_CellPowerList_AtT4 := {cs_NR_CellPower(nr_Cell1, v_CellPower_NonSuitable_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell2, v_CellPower_NonSuitable_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell4, v_CellPower_NonSuitable_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // FR2 is FFS v_CellPowerList_AtT5 := {cs_NR_CellPower(nr_Cell1, v_CellPower_NonSuitable_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell2, v_CellPower_EntryCondition_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell4, v_CellPower_NonSuitable_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // FR2 is FFS if(f_NR_CellInfo_GetIsFR1(nr_Cell1)){ v_Rsrp := 2; } else{ v_Rsrp := 2; //FFS dummy value FR2 } //@siclog "Step 1" siclog@ //The SS transmits an RRCReconfiguration message including MeasConfig and ConditionalReconfiguration to set NR Cell 2 and NR Cell 4 as the target candidate cells. v_MeasConfig := f_NR_GenerateMeasurementConfigNR( {cs_MeasObjectId1(nr_Cell1)}, {cs_NR_ReportConfigCondTrigger(tsc_NR_IdReportConfigId1, v_Rsrp)}, {cs_NR_MeasId_Config_id1_obj1_conf1}, -,-,-,-); v_ConditionalReconfiguration_r16_Step_1 := cs_ConditionalReconfiguration_r16( -, cs_CondReconfigToAddModList_r16_2EntryCR_1EntryMI(1, tsc_NR_MeasId1, f_NR_GenerateCondRRCReconfig_r16_Octetstring(nr_Cell2), 2, tsc_NR_MeasId1, f_NR_GenerateCondRRCReconfig_r16_Octetstring(nr_Cell4))); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, cs_38508_RRCReconfiguration_CHO(-,v_MeasConfig, v_ConditionalReconfiguration_r16_Step_1))); //@siclog "Step 2" siclog@ //The UE transmits an RRCReconfigurationComplete message in NR Cell 1. SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete)); //@siclog "Step 3" siclog@ //SS adjusts the cell-specific reference signal level according to row "T1". f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Steps 4-5" siclog@ //- EXCEPTION: The steps 4 and 5 below are repeated for the duration of T304. //The UE attempts to perform the intra frequency handover using MAC Random Access Preamble on NR Cell 2. //The SS does not respond. // Updated f_NR_SS_SystemIndCtrlConfig to check RACH preamble f_NR_SS_RachProcedureConfig(nr_Cell2, cs_NR_RachProcedureConfig_NoResponse); SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell2, cr_TimingInfo_Any, ?)); // Updated f_NR_SS_SystemIndCtrlConfig to check RACH preamble //@siclog "Step 6" siclog@ //SS adjusts the cell-specific reference signal level according to row "T2". f_NR_SetCellPowerList(v_CellPowerList_AtT2); //Target cell configration for handover f_NR5GC_CHO_RRCReconfiguration_SSConfig( nr_Cell1, nr_Cell4, v_DRBInfoList, -,-,-,-,-,-,-); //@siclog "Step 7" siclog@ //Check: Does the UE initiate a random access procedure by transmitting Preamble in NR Cell 4 within 10s? t_Watchdog.start; // Updated f_NR_SS_SystemIndCtrlConfig to check RACH preamble SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell4, cr_TimingInfo_Any, ?)); t_Watchdog.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7"); // Updated f_NR_SS_SystemIndCtrlConfig to check RACH preamble //@siclog "Steps 8-9" siclog@ //The SS transmits Randzom Access Response. //The UE transmits RRCReconfigurationComplete message in NR Cell 4. f_NR5GC_CHO_RRCReconfigurationComplete( nr_Cell1, nr_Cell4, -); //@siclog "Step 10" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.1-1 indicate that the UE is capable of exchanging IP data on DRB1 and NR Cell 4? v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); f_NR5GC_CheckDataPath(nr_Cell4, f_NR5GC_GetPdnIndex(v_PDUSessionInfoList[0]), dedicatedBearer, v_DRBId, "Step 10"); //@siclog "Step 11" siclog@ //The SS transmits an RRCReconfiguration message including MeasConfig and ConditionalReconfiguration to set NR Cell 1 and NR Cell 2 as the target candidate cells. v_ConditionalReconfiguration_r16_Step_11 := cs_ConditionalReconfiguration_r16( -, cs_CondReconfigToAddModList_r16_2EntryCR_1EntryMI(1, tsc_NR_MeasId1, f_NR_GenerateCondRRCReconfig_r16_Octetstring(nr_Cell1), 2, tsc_NR_MeasId1, f_NR_GenerateCondRRCReconfig_r16_Octetstring(nr_Cell2))); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell4, cs_TimingInfo_Now, cs_38508_RRCReconfiguration_CHO(-,v_MeasConfig, v_ConditionalReconfiguration_r16_Step_11))); //@siclog "Step 12" siclog@ //The UE transmits an RRCReconfigurationComplete message in NR Cell 4. SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell4, cr_38508_RRCReconfigurationComplete)); //@siclog "Step 13" siclog@ //SS adjusts the cell-specific reference signal level according to row "T3". f_NR_SetCellPowerList(v_CellPowerList_AtT3); //@siclog "Steps 14-15" siclog@ //EXCEPTION: The steps 14 and 15 below are repeated for the duration of T304. //The UE attempts to perform the inter frequency handover using MAC Random Access Preamble on NR Cell 1. //The SS does not respond. // Updated f_NR_SS_SystemIndCtrlConfig to check RACH preamble f_NR_SS_RachProcedureConfig(nr_Cell1, cs_NR_RachProcedureConfig_NoResponse); SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo_Any, ?)); // Updated f_NR_SS_SystemIndCtrlConfig to check RACH preamble //@siclog "Step 16" siclog@ //SS adjusts the cell-specific reference signal level according to row "T4". f_NR_SetCellPowerList(v_CellPowerList_AtT4); //@siclog "Step 17" siclog@ //Wait 5s to ensure that T304 and T311 expire. (Note 1: The wait time is selected to cover T304 (1000ms) + T311 (1000ms) to ensure that UE goes to RRC_IDLE.) f_Delay(5.0); // Added f_NR_SS_RachProcedureConfig_Def to enable Rach procedure on cell 2 //@siclog "Step 18" siclog@ //SS adjusts the cell-specific reference signal level according to row "T5". f_NR_SetCellPowerList(v_CellPowerList_AtT5); //@siclog "Step 19" siclog@ //Check: Does the generic mobility registration updating procedure described in TS 38.508-1 [4] Table 4.9.5.2.2-1 is performed on NR Cell 2. f_NR5GC_MobilityRegistration(nr_Cell12, -,-,-,-, noRrcConnectionRelease); }// End of fl_TC_8_1_4_4_3_TestBody

After Change

function fl_TC_8_1_4_4_3_TestBody() runs on NR5GC_PTC { var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT3; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT4; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT5; //var NR_AbsoluteCellPower_Type v_CellPower_Serving_FR1; var NR_AbsoluteCellPower_Type v_CellPower_EntryCondition_FR1; var NR_AbsoluteCellPower_Type v_CellPower_Leaving_FR1; var NR_AbsoluteCellPower_Type v_CellPower_NonSuitable_FR1; var RSRP_Range v_Rsrp; var integer v_DRBId; var template (value) MeasConfig v_MeasConfig; var template (value) ConditionalReconfiguration_r16 v_ConditionalReconfiguration_r16_Step_1; var template (value) ConditionalReconfiguration_r16 v_ConditionalReconfiguration_r16_Step_11; var template (value) NR_DRBInfoList_Type v_DRBInfoList := f_NR5GC_MobileInfo_GetDRBInfoList(); var PDUSessionInfoList_Type v_PDUSessionInfoList; timer t_Watchdog := 10.0; v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); //Power levels of NR cell 1, NR cell 2 and NR cell 4 FR1 acc. Table 8.1.4.4.1.3.2-1, Table 8.1.4.4.1.3.2-2 FFS /* ¦ Cell 1 ¦ Cell 2 ¦ Cell 4 * ---------------------------------------------- * T0 ¦ -85 ¦ -113 ¦ -113 * ---------------------------------------------- * T1 ¦ -91 ¦ -79 ¦ -91 * ---------------------------------------------- * T2 ¦ -113 ¦ -113 ¦ -79 * ---------------------------------------------- * T3 ¦ -79 ¦ -91 ¦ -91 * ---------------------------------------------- * T4 ¦ -113 ¦ -113 ¦ -113 * ---------------------------------------------- * T5 ¦ -113 ¦ -79 ¦ -113 */ // v_CellPower_Serving_FR1 := -85; v_CellPower_Leaving_FR1 := -91; v_CellPower_EntryCondition_FR1 := -79; v_CellPower_NonSuitable_FR1 := -113; v_CellPowerList_AtT1 := {cs_NR_CellPower(nr_Cell1, v_CellPower_Leaving_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell2, v_CellPower_EntryCondition_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell4, v_CellPower_Leaving_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // FR2 is FFS v_CellPowerList_AtT2 := {cs_NR_CellPower(nr_Cell1, v_CellPower_NonSuitable_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell2, v_CellPower_NonSuitable_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell4, v_CellPower_EntryCondition_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // FR2 is FFS v_CellPowerList_AtT3 := {cs_NR_CellPower(nr_Cell1, v_CellPower_EntryCondition_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell2, v_CellPower_Leaving_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell4, v_CellPower_Leaving_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // FR2 is FFS v_CellPowerList_AtT4 := {cs_NR_CellPower(nr_Cell1, v_CellPower_NonSuitable_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell2, v_CellPower_NonSuitable_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell4, v_CellPower_NonSuitable_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // FR2 is FFS v_CellPowerList_AtT5 := {cs_NR_CellPower(nr_Cell1, v_CellPower_NonSuitable_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell2, v_CellPower_EntryCondition_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS cs_NR_CellPower(nr_Cell4, v_CellPower_NonSuitable_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // FR2 is FFS if(f_NR_CellInfo_GetIsFR1(nr_Cell1)){ v_Rsrp := 2; } else{ v_Rsrp := 2; //FFS dummy value FR2 } //@siclog "Step 1" siclog@ //The SS transmits an RRCReconfiguration message including MeasConfig and ConditionalReconfiguration to set NR Cell 2 and NR Cell 4 as the target candidate cells. v_MeasConfig := f_NR_GenerateMeasurementConfigNR( {cs_MeasObjectId1(nr_Cell1)}, {cs_NR_ReportConfigCondTrigger(tsc_NR_IdReportConfigId1, v_Rsrp)}, {cs_NR_MeasId_Config_id1_obj1_conf1}, -,-,-,-); v_ConditionalReconfiguration_r16_Step_1 := cs_ConditionalReconfiguration_r16( -, cs_CondReconfigToAddModList_r16_2EntryCR_1EntryMI(1, tsc_NR_MeasId1, f_NR_GenerateCondRRCReconfig_r16_Octetstring(nr_Cell2), 2, tsc_NR_MeasId1, f_NR_GenerateCondRRCReconfig_r16_Octetstring(nr_Cell4))); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, cs_38508_RRCReconfiguration_CHO(-,v_MeasConfig, v_ConditionalReconfiguration_r16_Step_1))); //@siclog "Step 2" siclog@ //The UE transmits an RRCReconfigurationComplete message in NR Cell 1. SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete)); //@siclog "Step 3" siclog@ //SS adjusts the cell-specific reference signal level according to row "T1". f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Steps 4-5" siclog@ //- EXCEPTION: The steps 4 and 5 below are repeated for the duration of T304. //The UE attempts to perform the intra frequency handover using MAC Random Access Preamble on NR Cell 2. //The SS does not respond. f_NR_SS_SystemIndCtrlConfig(nr_Cell2, cds_NR_SystemIndCtrl_RachPreamble(enable)); f_NR_SS_RachProcedureConfig(nr_Cell2, cs_NR_RachProcedureConfig_NoResponse); SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell2, cr_TimingInfo_Any, ?)); f_NR_SS_SystemIndCtrlConfig(nr_Cell2, cds_NR_SystemIndCtrl_RachPreamble(disable)); //@siclog "Step 6" siclog@ //SS adjusts the cell-specific reference signal level according to row "T2". f_NR_SetCellPowerList(v_CellPowerList_AtT2); //Target cell configration for handover f_NR5GC_CHO_RRCReconfiguration_SSConfig( nr_Cell1, nr_Cell4, v_DRBInfoList, -,-,-,-,-,-,-); //@siclog "Step 7" siclog@ //Check: Does the UE initiate a random access procedure by transmitting Preamble in NR Cell 4 within 10s? f_NR_SS_SystemIndCtrlConfig(nr_Cell4, cds_NR_SystemIndCtrl_RachPreamble(enable)); t_Watchdog.start; SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell4, cr_TimingInfo_Any, ?)); t_Watchdog.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7"); f_NR_SS_SystemIndCtrlConfig(nr_Cell4, cds_NR_SystemIndCtrl_RachPreamble(disable)); //@siclog "Steps 8-9" siclog@ //The SS transmits Randzom Access Response. //The UE transmits RRCReconfigurationComplete message in NR Cell 4. f_NR5GC_CHO_RRCReconfigurationComplete( nr_Cell1, nr_Cell4, -); //@siclog "Step 10" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.1-1 indicate that the UE is capable of exchanging IP data on DRB1 and NR Cell 4? v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); f_NR5GC_CheckDataPath(nr_Cell4, f_NR5GC_GetPdnIndex(v_PDUSessionInfoList[0]), dedicatedBearer, v_DRBId, "Step 10"); //@siclog "Step 11" siclog@ //The SS transmits an RRCReconfiguration message including MeasConfig and ConditionalReconfiguration to set NR Cell 1 and NR Cell 2 as the target candidate cells. v_ConditionalReconfiguration_r16_Step_11 := cs_ConditionalReconfiguration_r16( -, cs_CondReconfigToAddModList_r16_2EntryCR_1EntryMI(1, tsc_NR_MeasId1, f_NR_GenerateCondRRCReconfig_r16_Octetstring(nr_Cell1), 2, tsc_NR_MeasId1, f_NR_GenerateCondRRCReconfig_r16_Octetstring(nr_Cell2))); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell4, cs_TimingInfo_Now, cs_38508_RRCReconfiguration_CHO(-,v_MeasConfig, v_ConditionalReconfiguration_r16_Step_11))); //@siclog "Step 12" siclog@ //The UE transmits an RRCReconfigurationComplete message in NR Cell 4. SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell4, cr_38508_RRCReconfigurationComplete)); //@siclog "Step 13" siclog@ //SS adjusts the cell-specific reference signal level according to row "T3". f_NR_SetCellPowerList(v_CellPowerList_AtT3); //@siclog "Steps 14-15" siclog@ //EXCEPTION: The steps 14 and 15 below are repeated for the duration of T304. //The UE attempts to perform the inter frequency handover using MAC Random Access Preamble on NR Cell 1. //The SS does not respond. f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(enable)); f_NR_SS_RachProcedureConfig(nr_Cell1, cs_NR_RachProcedureConfig_NoResponse); SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo_Any, ?)); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(disable)); //@siclog "Step 16" siclog@ //SS adjusts the cell-specific reference signal level according to row "T4". f_NR_SetCellPowerList(v_CellPowerList_AtT4); //@siclog "Step 17" siclog@ //Wait 5s to ensure that T304 and T311 expire. (Note 1: The wait time is selected to cover T304 (1000ms) + T311 (1000ms) to ensure that UE goes to RRC_IDLE.) f_Delay(5.0); f_NR_SS_RachProcedureConfig_Def(nr_Cell2); //@siclog "Step 18" siclog@ //SS adjusts the cell-specific reference signal level according to row "T5". f_NR_SetCellPowerList(v_CellPowerList_AtT5); //@siclog "Step 19" siclog@ //Check: Does the generic mobility registration updating procedure described in TS 38.508-1 [4] Table 4.9.5.2.2-1 is performed on NR Cell 2. f_NR5GC_MobilityRegistration(nr_Cell2, -,-,-,-, noRrcConnectionRelease); }// End of fl_TC_8_1_4_4_3_TestBody

  - Additional MCC160 implementation

//Based to the comment from ROHDE & SCHWARZ, swap step 9 and 10 to delay stopping periodic TA in source cell until CHO successes

| Function name | f_NR5GC_CHO_RRCReconfigurationComplete() |
| --- | --- |
| Reason for change | - Reception of rrcreconfigurationComplete on Target Cell should be prior to step 9 where we configure UL Grant for source Cell as SR based because at times its interfere the reception of rrcReconfigurationComplete on Target cell. |
| Summary of change | - Moved the reception of RRCReconfigurationComplete on Target Cell prior to the UL Grant configuration of source cell. |
| TTCN module | RRC_Conditional_Handover_NR5GC.ttcn |
| MCC160 Comment | - Accepted. |

/*

* @desc      Function for generating RRCReconfigurationComplete and completing the handover procedure as following step 10 to step 14.

* @desc      Step 9. Target Cell:         Receive RRCReconfigurationComplete.

* @desc      Step 10. Source Cell:         Stop periodic TA.

* @desc      Step 11.Target Cell:         Start periodic TA.

* @desc      Step 12.Target Cell:         Inform the SS about completion of the HO (e.g. to trigger PDCP STATUS REPORT PDU).

* @desc      Step 13.Target Cell:         Re-configure RACH procedure as for initial access.

* @desc      Step 14.Source Cell:         Reset SRBs and release DRBs.

* @param     p_SourceCellId

* @param     p_TargetCellId              The cell in which the UE will actually perform Conditional handover.

* @param     p_RRCConnReconfigComplete (default value: cr_38508_RRCReconfigurationComplete)

* @status

*/

function f_NR5GC_CHO_RRCReconfigurationComplete( NR_CellId_Type p_SourceCellId,

NR_CellId_Type p_TargetCellId,

template (present) UL_DCCH_Message p_RRCConnReconfigComplete := cr_38508_RRCReconfigurationComplete

) runs on NR_BASE_PTC

{

var IntegerList_Type v_DrbIdList := f_NR_GetActiveDRBs(p_SourceCellId);  //List of DRBs to release in the Source cell

//Step 9.Target Cell:         Receive RRCReconfigurationComplete.

SRB.receive(car_NR_SRB1_RrcPdu_IND(p_TargetCellId, p_RRCConnReconfigComplete));

//Step 10. Source Cell:        Stop periodic TA.

//NOTE 1: Unless explicitly specified UL grant configuration keeps configured as per default at the source cell.

f_NR_ULGrantConfiguration_Common(p_SourceCellId, cs_TimingInfo_Now, -, cs_NR_UplinkTimeAlignment_Stop, cs_UL_GrantConfig_OnSR);

//Step 11.Target Cell:         Start periodic TA.

f_NR_ULGrantConfiguration_Common(p_TargetCellId, cs_TimingInfo_Now, -, cs_NR_UplinkTimeAlignment_Start, -);

//Step 12.Target Cell:         Inform the SS about completion of the HO (e.g. to trigger PDCP STATUS REPORT PDU).

f_NR_SS_PdcpHandoverCtrl(p_TargetCellId, cas_NR_PdcpHandoverComplete_REQ(p_TargetCellId));

//Step 13.Target Cell:         Re-configure RACH procedure as for initial access.

f_NR_SS_RachProcedureConfig_Def(p_TargetCellId);

//Step 14.Source Cell:         Reset SRBs and release DRBs.

f_NR_SS_SRBs_DRBs_Release(p_SourceCellId, cs_TimingInfo_Now, v_DrbIdList);

f_NR_SS_SRBs_DRBs_Config(p_SourceCellId, cs_TimingInfo_Now, omit);

}

//Based to the comment from ROHDE & SCHWARZ,

| Function name | f_TC_8_1_4_4_3_NR5GC() |
| --- | --- |
| Reason for change | - At Postamble UE will be in idle mode. |
| Summary of change | - Corrected the UE state as STATE_IDLE_1A. |
| TTCN module | RRC_Conditional_Handover_NR5GC.ttcn |
| MCC160 Comment | - Accepted |

function f_TC_8_1_4_4_3_NR5GC() runs on NR5GC_PTC { // Conditional handover / Failure f_NR5GC_Init(NR_1); f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -79, -79); // Power levels are FFS for FR2 f_NR_CellInfo_InitMaxReferencePower(nr_Cell2, -79, -79); // Power levels are FFS for FR2 f_NR_CellInfo_InitMaxReferencePower(nr_Cell4, -79, -79); // Power levels are FFS for FR2 //Configure TAC of NR Cell 2 to 2 according to Table 8.1.4.4.3.3.3-1 f_NR_CellInfo_SetTAC (nr_Cell2, int2bit(2,24)); //Create and configure cells f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell2); f_NR_CellConfig_Def(nr_Cell4); // Preamble: If pc_IP_Ping is set to TRUE then, the UE is in 5GS state 3N-A on NR Cell 1 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3. // Else, the UE is in 5GS state 3N-A and Test Loop Function (On) with UE test loop mode B on NR Cell 1 according to 38.508-1[4], clause 4.4A.2 Table 4.4A.2-3. f_NR5GC_Preamble (nr_Cell1, STATE_CONNECTED_3A, PING_Or_TEST_LOOPModeB_ON); //Set SS/PBCH EPRE levels acc. to row "T0" f_NR_SetCellPower (nr_Cell1, -85, -85); // Power levels are FFS for FR2 f_NR_SetCellPower (nr_Cell2, -113, -113); // Power levels are FFS for FR2 f_NR_SetCellPower (nr_Cell4, -113, -113); // Power levels are FFS for FR2 f_NR_TestBody_Set(true); fl_TC_8_1_4_4_3_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell2, STATE_IDLE_1A); // @sic R5s220375 sic } // End of f_TC_8_1_4_4_3_NR5GC

//Based on the comments below from ROHDE & SCHWARZ,

| Function name | fl_TC_8_1_4_4_3_TestBody() |
| --- | --- |
| Reason for change | - Rach reception for T304 timer is required as per Step4/5 and Step 14/15. - Also No RAR response for NR Cell 2 should be configured prior to power level change at T1. - Target Cell configuration for handover should be configured prior to step 6 T2 power level changes. - Rach indication for NR Cell4 for Step 7 is required. - Also No RAR response for NR Cell 1 should be configured prior to power level change at T3. - At step 20 incorrect NR Cell 12 is mentioned. Also, as per prose there is no requirement of noRrcConnectionRelease. |
| Summary of change | - Enabled Rach indication and NO RAR response for NR Cell 2 prior to Step 3. - Moved Target cell configuration prior to T2 power level change at Step 6. - Added Rach enabling prior to Step 6 for NR Cell4 and its disabling after the reception of rrcreconfigurationComplete message at Step 9. - Enabled Rach indication and NO RAR response for NR Cell 1 prior to Step 13. - Corrected NR cell as 2 instead of 12 at Step 20 and removed the need of calling with noRrcConnectionRelease. |
| TTCN module | RRC_Conditional_Handover_NR5GC.ttcn |
| MCC160 Comment | - No need to detect PRACH preamble in steps 4/5 and 14/15 but f_Delay(2.0) is added just as mentioned in Change 3-1. - Accepted - Accepted - Accepted - Accepted - Accepted |

function fl_TC_8_1_4_4_3_TestBody() runs on NR5GC_PTC

{

var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1;

var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2;

var template (value) NR_CellPowerList_Type v_CellPowerList_AtT3;

var template (value) NR_CellPowerList_Type v_CellPowerList_AtT4;

var template (value) NR_CellPowerList_Type v_CellPowerList_AtT5;

//var NR_AbsoluteCellPower_Type v_CellPower_Serving_FR1;

var NR_AbsoluteCellPower_Type v_CellPower_EntryCondition_FR1;

var NR_AbsoluteCellPower_Type v_CellPower_Leaving_FR1;

var NR_AbsoluteCellPower_Type v_CellPower_NonSuitable_FR1;

var RSRP_Range v_Rsrp;

var integer v_DRBId;

var template (value) MeasConfig v_MeasConfig;

var template (value) ConditionalReconfiguration_r16 v_ConditionalReconfiguration_r16_Step_1;

var template (value) ConditionalReconfiguration_r16 v_ConditionalReconfiguration_r16_Step_11;

var template (value) NR_DRBInfoList_Type v_DRBInfoList := f_NR5GC_MobileInfo_GetDRBInfoList();

var PDUSessionInfoList_Type v_PDUSessionInfoList;

timer t_Watchdog := 10.0;

v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList();

//Power levels of NR cell 1, NR cell 2 and NR cell 4 FR1 acc. Table 8.1.4.4.1.3.2-1, Table 8.1.4.4.1.3.2-2 FFS

/*         |   Cell 1   |   Cell 2  |   Cell 4

* ----------------------------------------------

*   T0    |    -85     |    -113   |    -113

* ----------------------------------------------

*   T1    |    -91     |    -79    |    -91

* ----------------------------------------------

*   T2    |    -113    |    -113   |    -79

* ----------------------------------------------

*   T3    |    -79     |    -91    |    -91

* ----------------------------------------------

*   T4    |    -113    |    -113   |    -113

* ----------------------------------------------

*   T5    |    -113    |    -79    |    -113

*/

//v_CellPower_Serving_FR1 := -85;

v_CellPower_Leaving_FR1 := -91;

v_CellPower_EntryCondition_FR1 := -79;

v_CellPower_NonSuitable_FR1 := -113;

v_CellPowerList_AtT1 := {cs_NR_CellPower(nr_Cell1, v_CellPower_Leaving_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE),  // FR2 is FFS

cs_NR_CellPower(nr_Cell2, v_CellPower_EntryCondition_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS

cs_NR_CellPower(nr_Cell4, v_CellPower_Leaving_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // FR2 is FFS

v_CellPowerList_AtT2 := {cs_NR_CellPower(nr_Cell1, v_CellPower_NonSuitable_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE),  // FR2 is FFS

cs_NR_CellPower(nr_Cell2, v_CellPower_NonSuitable_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), // FR2 is FFS

cs_NR_CellPower(nr_Cell4, v_CellPower_EntryCondition_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // FR2 is FFS

v_CellPowerList_AtT3 := {cs_NR_CellPower(nr_Cell1, v_CellPower_EntryCondition_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE),  // FR2 is FFS

cs_NR_CellPower(nr_Cell2, v_CellPower_Leaving_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE),  // FR2 is FFS

cs_NR_CellPower(nr_Cell4, v_CellPower_Leaving_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // FR2 is FFS

v_CellPowerList_AtT4 := {cs_NR_CellPower(nr_Cell1, v_CellPower_NonSuitable_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE),  // FR2 is FFS

cs_NR_CellPower(nr_Cell2, v_CellPower_NonSuitable_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE),  // FR2 is FFS

cs_NR_CellPower(nr_Cell4, v_CellPower_NonSuitable_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // FR2 is FFS

v_CellPowerList_AtT5 := {cs_NR_CellPower(nr_Cell1, v_CellPower_NonSuitable_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE),  // FR2 is FFS

cs_NR_CellPower(nr_Cell2, v_CellPower_EntryCondition_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE),  // FR2 is FFS

cs_NR_CellPower(nr_Cell4, v_CellPower_NonSuitable_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // FR2 is FFS

if(f_NR_CellInfo_GetIsFR1(nr_Cell1)){

v_Rsrp := 2;

}

else{

v_Rsrp := 2;          //FFS dummy value FR2

}

//@siclog "Step 1" siclog@

//The SS transmits an RRCReconfiguration message including MeasConfig and ConditionalReconfiguration to set NR Cell 2 and NR Cell 4 as the target candidate cells.

v_MeasConfig := f_NR_GenerateMeasurementConfigNR( {cs_MeasObjectId1(nr_Cell1)},

{cs_NR_ReportConfigCondTrigger(tsc_NR_IdReportConfigId1, v_Rsrp)},

{cs_NR_MeasId_Config_id1_obj1_conf1},

-,-,-,-);

v_ConditionalReconfiguration_r16_Step_1 := cs_ConditionalReconfiguration_r16( -,

cs_CondReconfigToAddModList_r16_2EntryCR_1EntryMI(1, tsc_NR_MeasId1, f_NR_GenerateCondRRCReconfig_r16_Octetstring(nr_Cell2),

2, tsc_NR_MeasId1, f_NR_GenerateCondRRCReconfig_r16_Octetstring(nr_Cell4)));

SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1,

cs_TimingInfo_Now,

cs_38508_RRCReconfiguration_CHO(-,v_MeasConfig, v_ConditionalReconfiguration_r16_Step_1)));

//@siclog "Step 2" siclog@

//The UE transmits an RRCReconfigurationComplete message in NR Cell 1.

SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete));

f_NR_SS_RachProcedureConfig(nr_Cell2, cs_NR_RachProcedureConfig_NoResponse); // @sic R5s220375 sic@

//@siclog "Step 3" siclog@

//SS adjusts the cell-specific reference signal level according to row "T1".

f_NR_SetCellPowerList(v_CellPowerList_AtT1);

//@siclog "Steps 4-5" siclog@

//- EXCEPTION: The steps 4 and 5 below are repeated for the duration of T304.

//The UE attempts to perform the intra frequency handover using MAC Random Access Preamble on NR Cell 2.

//The SS does not respond.

f_Delay(2.0); // make sure T304 expires // @sic R5s220375 sic@

//Target cell configration for handover // @sic R5s220375 sic@

f_NR5GC_CHO_RRCReconfiguration_SSConfig( nr_Cell1,

nr_Cell4,

v_DRBInfoList,

-,-,-,-,-,-,-);

f_NR_SS_SystemIndCtrlConfig(nr_Cell4, cds_NR_SystemIndCtrl_RachPreamble(enable)); // @sic R5s220375 sic@

//@siclog "Step 6" siclog@

//SS adjusts the cell-specific reference signal level according to row "T2".

f_NR_SetCellPowerList(v_CellPowerList_AtT2);

//@siclog "Step 7" siclog@

//Check: Does the UE initiate a random access procedure by transmitting Preamble in NR Cell 4 within 10s?

t_Watchdog.start;

SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell4, cr_TimingInfo_Any, ?));

t_Watchdog.stop;

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7");

//@siclog "Steps 8-9" siclog@

//The SS transmits Randzom Access Response.

//The UE transmits RRCReconfigurationComplete message in NR Cell 4.

f_NR5GC_CHO_RRCReconfigurationComplete( nr_Cell1,

nr_Cell4,

-);

f_NR_SS_SystemIndCtrlConfig(nr_Cell4, cds_NR_SystemIndCtrl_RachPreamble(disable)); // @sic R5s220375 sic@

//@siclog "Step 10" siclog@

//Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.1-1 indicate that the UE is capable of exchanging IP data on DRB1 and NR Cell 4?

v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession();

f_NR5GC_CheckDataPath(nr_Cell4,

f_NR5GC_GetPdnIndex(v_PDUSessionInfoList[0]),

dedicatedBearer,

v_DRBId,

"Step 10");

//@siclog "Step 11" siclog@

//The SS transmits an RRCReconfiguration message including MeasConfig and ConditionalReconfiguration to set NR Cell 1 and NR Cell 2 as the target candidate cells.

v_ConditionalReconfiguration_r16_Step_11 := cs_ConditionalReconfiguration_r16( -,

cs_CondReconfigToAddModList_r16_2EntryCR_1EntryMI(1, tsc_NR_MeasId1, f_NR_GenerateCondRRCReconfig_r16_Octetstring(nr_Cell1),

2, tsc_NR_MeasId1, f_NR_GenerateCondRRCReconfig_r16_Octetstring(nr_Cell2)));

SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell4,

cs_TimingInfo_Now,

cs_38508_RRCReconfiguration_CHO(-,v_MeasConfig, v_ConditionalReconfiguration_r16_Step_11)));

//@siclog "Step 12" siclog@

//The UE transmits an RRCReconfigurationComplete message in NR Cell 4.

SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell4, cr_38508_RRCReconfigurationComplete));

f_NR_SS_RachProcedureConfig(nr_Cell1, cs_NR_RachProcedureConfig_NoResponse); // @sic R5s220375 sic@

//@siclog "Step 13" siclog@

//SS adjusts the cell-specific reference signal level according to row "T3".

f_NR_SetCellPowerList(v_CellPowerList_AtT3);

//@siclog "Steps 14-15" siclog@

//EXCEPTION: The steps 14 and 15 below are repeated for the duration of T304.

//The UE attempts to perform the inter frequency handover using MAC Random Access Preamble on NR Cell 1.

//The SS does not respond.

f_Delay(2.0); // make sure T304 expires // @sic R5s220375 sic@

//@siclog "Step 16" siclog@

//SS adjusts the cell-specific reference signal level according to row "T4".

f_NR_SetCellPowerList(v_CellPowerList_AtT4);

//@siclog "Step 17" siclog@

//Wait 5s to ensure that T304 and T311 expire. (Note 1:  The wait time is selected to cover T304 (1000ms) + T311 (1000ms) to ensure that UE goes to RRC_IDLE.)

f_Delay(5.0);

//@siclog "Step 18" siclog@

//SS adjusts the cell-specific reference signal level according to row "T5".

f_NR_SetCellPowerList(v_CellPowerList_AtT5);

//@siclog "Step 19" siclog@

//Check: Does the generic mobility registration updating procedure described in TS 38.508-1 [4] Table 4.9.5.2.2-1 is performed on NR Cell 2.

f_NR5GC_MobilityRegistration(nr_Cell12,

-,-,-,-,-); // @sic R5s220375 sic@

}// End of fl_TC_8_1_4_4_3_TestBody

- Branches executed

On Anritsu, the test case was executed on NR band n78 using NR ciphering nea2 and integrity algorithm nia2.

- Execution Log Files

  - Mediatek MT6983

The Mediatek MT6983 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anritsu\ TC_8_1_4_4_3_LOG.xml

- PICS/PIXIT parameter file:

Anritsu\ TC_8_1_4_4_3_PIXIT.txt

- References

| [1] | R5s220376 - Supporting information for addition of NR5GC Rel-16 Mobility Enhancement test case 8.1.4.4.3 |
| --- | --- |