3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220421

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2422 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC MDT testcase 8.1.6.1.3.4 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-03-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add NR5GC test case 8.1.6.1.3.4 to the IWD_21wk49 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 8.1.6.1.3.4 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.3.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required to test case 8.1.6.1.3.4	3

2.1	f_TC_8_1_6_1_3_4_NR5GC	3

2.2	cr_NR_Rlf_Report	9

3	Branches executed	12

4	Execution Log Files	12

4.1	MediaTek MT6983	12

5	References	12

- Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 8.1.6.1.3.4 which is part of the NR5GC test suite in iWD_TTCN3-B2020-09_D21wk49.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

  - Verification Test Summary

Test Case: 	8.1.6.1.3.4.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D21wk49

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	MediaTek MT6983

Verification Status:	PASS

- Corrections required to test case 8.1.6.1.3.4

  - f_TC_8_1_6_1_3_4_NR5GC

| Function name | f_TC_8_1_6_1_3_4_NR5GC |
| --- | --- |
| Reason for change | - In this test case Cell 1 and Cell 2 are being configured, but in several places cell 11 and cell 12 are being used in the test body. - In the current implementation at step 1 reconfiguration to trigger HO is sent which is going to be unsuccesful due to target cell is powered down. Reconfiguration to trigger HO will be resulting in SS retransmissions while UE is doing Handover failure based procedure ; To stop this reconfiguration retransmission rb release and reset is required before changing the CRNTI ( otherwise this RRC Configuration retx will still get continued with updated CRNTI and interfere the reestablishment procedure). See R5s220201 - In UE Information response, the C-RNTI value expected is value when the handover failure has happened and not the latest value - In RAN5#94e R5-221580 was agreed and Step 9 UE Information response message was updated. This CR is implmented |
| Summary of change | - Updated the Cell reference to use cell 1 and cell 2 - Added new variable v_DrbIdList and called function f_NR_SS_SRBs_DRBs_Release and f_NR_SS_SRBs_DRBs_Config - In UE Information response used tsc_C_RNTI_Value1 - Based on R5-221580 the contents of nrFailedPCellId-r16, reestablishmentCellId-r16, rlf-Cause-r16, and ra-InformationCommon-r16 are updated |
| TTCN module | RRC_MDT_IntraNR_RLF_NR5GC |
| MCC160 Comment | - Accepted. - Accepted in principle but implemented differently as per R5-221467. - Accepted in principle but implemented differently. - Accepted. |

Before Change:

function f_TC_8_1_6_1_3_4_NR5GC() runs on NR5GC_PTC { // Radio Link Failure / Reporting at NR handover f_NR5GC_Init(NR_2); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell2); f_NR5GC_Preamble (nr_Cell1, STATE_CONNECTED_3A); f_NR_SetCellPower (nr_Cell12, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR2); f_NR_TestBody_Set(true); f_TC_8_1_6_1_3_4_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A);//@sic R5-216291 sic@ }// End of f_TC_8_1_6_1_3_4_NR5GC() //---------------------------------------------------------------------------- /* * @desc Test body function for TC 8.1.6.1.3.4 * @status */ function f_TC_8_1_6_1_3_4_TestBody() runs on NR5GC_PTC { var template (value) DL_DCCH_Message v_RRCReconfigurationHO; var PLMN_Identity v_PlmnID_NRCell1 := f_NR_CellInfo_GetPLMN (nr_Cell1); var CellIdentity v_CellIdentity_NRCell1 := f_NR_CellInfo_GetCellIdentity(nr_Cell1); var TrackingAreaCode v_TAC_NRCell1 := f_NR_CellInfo_GetTAC (nr_Cell1); var PLMN_Identity v_PlmnID_NRCell2 := f_NR_CellInfo_GetPLMN (nr_Cell2); var CellIdentity v_CellIdentity_NRCell2 := f_NR_CellInfo_GetCellIdentity(nr_Cell2); var TrackingAreaCode v_TAC_NRCell2 := f_NR_CellInfo_GetTAC (nr_Cell2); var template RRCReestablishmentComplete_v1610_IEs v_RRCReestablishmentComplete_v1610_IEs;// := cr_RRCReestablishmentComplete_v1610_IEs_RLF; // Reconfigure nr_Cell2 to no respond to any RA Preamble // Table 8.1.4.1.5.3.2-4: The UE attempts to perform the handover using MAC Random Access Preamble on NR Cell 2. SS does not respond. f_NR_SS_RachProcedureConfig(nr_Cell2, cs_NR_RachProcedureConfig_NoResponse); //@siclog "Step 1" siclog@ // The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform handover to NR Cell 2. // Note: failure case UE doesn't go to nr_Cell2 => the standard function need not be used v_RRCReconfigurationHO := f_NR5GC_RRCReconfiguration_IntraNR_HO(nr_Cell2); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, v_RRCReconfigurationHO)); //@siclog "Step 2" siclog@ //The SS changes the power level setting according to the row "T1". f_NR_SetCellPower (nr_Cell12, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@siclog "Step 3-6" siclog@ //Re-establishment procedure v_RRCReestablishmentComplete_v1610_IEs := cr_RRCReestablishmentComplete_v1610_IEs_RLF; f_NR5GC_RRCReestablishment_Def(nr_Cell1, nr_Cell1, -, 2,-,-,-,-,valueof(v_RRCReestablishmentComplete_v1610_IEs),false); //@siclog "Step 7" siclog@ //Re-establishment procedure //Check: Does the UE transmit an RRCReconfigurationtComplete message with rlf-InfoAvailable included? SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete( cr_RRCReconfigurationComplete_v1530(-, cr_RRCReconfigurationComplete_v1560(-,cr_RRCReconfigurationComplete_v1610_RLF))))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7"); //@siclog "Step 8" siclog@ //The SS transmits a UEInformationRequest message. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell11, -, cs_38508_UEInformationRequest(tsc_NR_RRC_TI_Def, -, -, -, -, true_))); //@siclog "Steps 9" siclog@ //Check: Does the UE transmit a UEInformationResponse message on NR Cell 1? SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell11, cr_38508_UEInformationResponse(tsc_NR_RRC_TI_Def,-,-,-,-,-, cr_NR_Rlf_Report(f_NR_CellInfo_GetRNTI(nr_Cell1),-, (0..127), (0..127), // rsrp, rsrq omit,// measResultNeighCells_r16 cr_PreviousPCellId_r16(v_PlmnID_NRCell1, v_CellIdentity_NRCell1, v_TAC_NRCell1),//PreviousPCellId_r16 cr_NR_RLF_Report_failedPCellId_cellGlobalId_r16(cr_CGI_Info_Logging_r16(v_PlmnID_NRCell2, v_CellIdentity_NRCell2, v_TAC_NRCell2)),// failedPCellId_r16 omit,omit,omit,*,?, hof, randomAccessProblem, //rlf-Cause-r16--check *,omit)))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 9"); //@siclog "Steps 10" siclog@ //void @sic R5-216291 sic@ }//End of f_TC_8_1_6_1_3_4_TestBody() //---

After Change:

module RRC_MDT_IntraNR_RLF_NR5GC { import from NR_Timing all; import from NR_SecuritySteps all; //WA#WI=908563 import from CommonAspDefs all; //WA#WI=908563 … function f_TC_8_1_6_1_3_4_TestBody() runs on NR5GC_PTC { var template (value) DL_DCCH_Message v_RRCReconfigurationHO; var PLMN_Identity v_PlmnID_NRCell1 := f_NR_CellInfo_GetPLMN (nr_Cell1); var CellIdentity v_CellIdentity_NRCell1 := f_NR_CellInfo_GetCellIdentity(nr_Cell1); var TrackingAreaCode v_TAC_NRCell1 := f_NR_CellInfo_GetTAC (nr_Cell1); var PLMN_Identity v_PlmnID_NRCell2 := f_NR_CellInfo_GetPLMN (nr_Cell2); var CellIdentity v_CellIdentity_NRCell2 := f_NR_CellInfo_GetCellIdentity(nr_Cell2); var TrackingAreaCode v_TAC_NRCell2 := f_NR_CellInfo_GetTAC (nr_Cell2); var template RRCReestablishmentComplete_v1610_IEs v_RRCReestablishmentComplete_v1610_IEs;// := cr_RRCReestablishmentComplete_v1610_IEs_RLF; var IntegerList_Type v_DrbIdList; // Reconfigure nr_Cell2 to no respond to any RA Preamble // Table 8.1.4.1.5.3.2-4: The UE attempts to perform the handover using MAC Random Access Preamble on NR Cell 2. SS does not respond. f_NR_SS_RachProcedureConfig(nr_Cell2, cs_NR_RachProcedureConfig_NoResponse); //@siclog "Step 1" siclog@ // The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform handover to NR Cell 2. // Note: failure case UE doesn't go to nr_Cell2 => the standard function need not be used v_RRCReconfigurationHO := f_NR5GC_RRCReconfiguration_IntraNR_HO(nr_Cell2); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, v_RRCReconfigurationHO)); //@siclog "Step 2" siclog@ //The SS changes the power level setting according to the row "T1". f_NR_SetCellPower (nr_Cell2, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //updated incorrect cell //@siclog "Step 3-6" siclog@ //Re-establishment procedure v_DrbIdList := f_NR_GetActiveDRBs(nr_Cell1); f_NR_SS_SRBs_DRBs_Release(nr_Cell1, -, v_DrbIdList, tsc_NoCnfReq); //@sic R5-211417 R5s211169 sic@ f_NR_SS_SRBs_DRBs_Config(nr_Cell1, -, omit, true); //WA#WI=908563 v_RRCReestablishmentComplete_v1610_IEs := cr_RRCReestablishmentComplete_v1610_IEs_RLF; f_NR5GC_RRCReestablishment_Def(nr_Cell1, nr_Cell1, -, 2,-,-,-,-,v_RRCReestablishmentComplete_v1610_IEs,false); //WA#WI=908563 removed value of //@siclog "Step 7" siclog@ //Re-establishment procedure //Check: Does the UE transmit an RRCReconfigurationtComplete message with rlf-InfoAvailable included? SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete( cr_RRCReconfigurationComplete_v1530(-, cr_RRCReconfigurationComplete_v1560(-,cr_RRCReconfigurationComplete_v1610_RLF))))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7"); //@siclog "Step 8" siclog@ //The SS transmits a UEInformationRequest message. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1,// incorrect cell -, cs_38508_UEInformationRequest(tsc_NR_RRC_TI_Def, -, -, -, -, true_))); //@siclog "Steps 9" siclog@ //Check: Does the UE transmit a UEInformationResponse message on NR Cell 1? SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, //WA#WI=908563 incorrect cell cr_38508_UEInformationResponse(tsc_NR_RRC_TI_Def,-,-,-,-,-, cr_NR_Rlf_Report(tsc_C_RNTI_Value1,-, //WA#WI=908563 crnti of cell 1 is updated in reestablishment proc f_NR_CellInfo_GetRNTI(nr_Cell1) (0..127), (0..127), // rsrp, rsrq omit,// measResultNeighCells_r16 cr_PreviousPCellId_r16(v_PlmnID_NRCell1, v_CellIdentity_NRCell1, v_TAC_NRCell1),//PreviousPCellId_r16 -, //failedPCellId_r16 any value omit,omit,cr_NR_CGI_Info_Logging_r16_Def (f_NR_CellInfo_GetCellIdentity(nr_Cell1)),*,?, //WA#WI=908563 added cr_NR_CGI_Info_Logging_r16_Def hof, -, //rlf-Cause-r16--check //WA#WI=908563 not checked *)))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 9"); //@siclog "Steps 10" siclog@ //void @sic R5-216291 sic@ }//End of f_TC_8_1_6_1_3_4_TestBody()

MCC160 Implementation:

function f_TC_8_1_6_1_3_4_NR5GC() runs on NR5GC_PTC { // Radio Link Failure / Reporting at NR handover f_NR5GC_Init(NR_2); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell2); f_NR5GC_Preamble (nr_Cell1, STATE_CONNECTED_3A); f_NR_SetCellPower (nr_Cell2, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR2); f_NR_TestBody_Set(true); f_TC_8_1_6_1_3_4_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A);//@sic R5-216291 sic@ }// End of f_TC_8_1_6_1_3_4_NR5GC() //---------------------------------------------------------------------------- /* * @desc Test body function for TC 8.1.6.1.3.4 * @status */ function f_TC_8_1_6_1_3_4_TestBody() runs on NR5GC_PTC { var template (value) DL_DCCH_Message v_RRCReconfigurationHO; var PLMN_Identity v_PlmnID_NRCell1 := f_NR_CellInfo_GetPLMN (nr_Cell1); var CellIdentity v_CellIdentity_NRCell1 := f_NR_CellInfo_GetCellIdentity(nr_Cell1); var TrackingAreaCode v_TAC_NRCell1 := f_NR_CellInfo_GetTAC (nr_Cell1); var PLMN_Identity v_PlmnID_NRCell2 := f_NR_CellInfo_GetPLMN (nr_Cell2); var CellIdentity v_CellIdentity_NRCell2 := f_NR_CellInfo_GetCellIdentity(nr_Cell2); var TrackingAreaCode v_TAC_NRCell2 := f_NR_CellInfo_GetTAC (nr_Cell2); var template RRCReestablishmentComplete_v1610_IEs v_RRCReestablishmentComplete_v1610_IEs;// := cr_RRCReestablishmentComplete_v1610_IEs_RLF; var IntegerList_Type v_DrbIdList; var RNTI_Value v_RNTI_Value := f_NR_CellInfo_GetRNTI(nr_Cell1); … //@siclog "Steps 9" siclog@ //Check: Does the UE transmit a UEInformationResponse message on NR Cell 1? SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, //WA#WI=908563 incorrect cell cr_38508_UEInformationResponse(tsc_NR_RRC_TI_Def,-,-,-,-,-, cr_NR_Rlf_Report(v_RNTI_Value,-, //WA#WI=908563 crnti of cell 1 is updated in reestablishment proc f_NR_CellInfo_GetRNTI(nr_Cell1) (0..127), (0..127), // rsrp, rsrq omit,// measResultNeighCells_r16 cr_PreviousPCellId_r16(v_PlmnID_NRCell1, v_CellIdentity_NRCell1, v_TAC_NRCell1),//PreviousPCellId_r16 -, omit,omit,cr_NR_CGI_Info_Logging_r16_Def (f_NR_CellInfo_GetCellIdentity(nr_Cell1)),*,?, cr_NR_CGI_Info_Logging_r16_Def hof, -, //rlf-Cause-r16- not checked *)))); }

  - cr_NR_Rlf_Report

| Function name | cr_NR_Rlf_Report |
| --- | --- |
| Reason for change | In agreed CR R5-221580 the contents of rlf-Cause-r16 is updated to accept any value |
| Summary of change | Based on R5-221580 the contents of rlf-Cause-r16 is updated to allow any value |
| TTCN module | NR5GC_MDT_Specific_Templates |
| MCC160 Comment | Accepted. |

Before Change:

template RLF_Report_r16 cr_NR_Rlf_Report (RNTI_Value p_C_RNTI_r16, B64_Type p_SsbRLMConfigBitmap_r16 := tsc_SsbRLMConfigBitmap_r16, template integer p_Rsrp, template integer p_Rsrq, template NR_RLF_Report_measResultNeighCells p_MeasResultNeighCells_r16 := *, template NR_RLF_Report_previousPCellId p_PreviousPCellId_r16 := *, template (present) NR_RLF_Report_failedPCellId p_FailedPCellId_r16 := ?, template NR_RLF_Report_reconnectCellId p_ReconnectCellId_r16 := *, template TimeUntilReconnection_r16 p_TimeUntilReconnection_r16 := *, template CGI_Info_Logging_r16 p_ReestablishmentCellId_r16 := *, template integer p_TimeConnFailure_r16 := *, template (present) TimeSinceFailure_r16 p_TimeSinceFailure_r16 := ?, NR_RLF_Report_connectionFailureType_r16 p_ConnectionFailureType_r16, NR_RLF_Report_rlf_Cause p_Rlf_Cause_r16, template LocationInfo_r16 p_LocationInfo_r16 := *, template RA_InformationCommon_r16 p_RA_InformationCommon_r16 := *, template (present) SSB_Index p_SSB_Index := tsc_NR_SSB_Index1 // ) := { nr_RLF_Report_r16 := { measResultLastServCell_r16 := { measResult_r16 := { cellResults_r16 := { resultsSSB_Cell_r16 := { rsrp := p_Rsrp, rsrq := p_Rsrq, sinr := * }, resultsCSI_RS_Cell_r16 := * }, rsIndexResults_r16 := { resultsSSB_Indexes_r16 := {cr_ResultsPerSSB_Index(p_SSB_Index)}, ssbRLMConfigBitmap_r16 := p_SsbRLMConfigBitmap_r16, resultsCSI_RS_Indexes_r16 := omit, csi_rsRLMConfigBitmap_r16 := omit } } }, measResultNeighCells_r16 := p_MeasResultNeighCells_r16, c_RNTI_r16 := p_C_RNTI_r16, previousPCellId_r16 := p_PreviousPCellId_r16, failedPCellId_r16 := p_FailedPCellId_r16, reconnectCellId_r16 := p_ReconnectCellId_r16, timeUntilReconnection_r16 := p_TimeUntilReconnection_r16, //@sic R5s211526 BASELINE MOVING 2021 sic@ reestablishmentCellId_r16 := p_ReestablishmentCellId_r16, timeConnFailure_r16 := p_TimeConnFailure_r16, timeSinceFailure_r16 := p_TimeSinceFailure_r16, connectionFailureType_r16 := p_ConnectionFailureType_r16, rlf_Cause_r16 := p_Rlf_Cause_r16, locationInfo_r16 := p_LocationInfo_r16, noSuitableCellFound_r16 := *, ra_InformationCommon_r16 := p_RA_InformationCommon_r16, csi_rsRLMConfigBitmap_v1650 := * //@sic R5s211526 BASELINE MOVING 2021 sic@ } };

After Change:

template RLF_Report_r16 cr_NR_Rlf_Report (RNTI_Value p_C_RNTI_r16, B64_Type p_SsbRLMConfigBitmap_r16 := tsc_SsbRLMConfigBitmap_r16, template integer p_Rsrp, template integer p_Rsrq, template NR_RLF_Report_measResultNeighCells p_MeasResultNeighCells_r16 := *, template NR_RLF_Report_previousPCellId p_PreviousPCellId_r16 := *, template (present) NR_RLF_Report_failedPCellId p_FailedPCellId_r16 := ?, template NR_RLF_Report_reconnectCellId p_ReconnectCellId_r16 := *, template TimeUntilReconnection_r16 p_TimeUntilReconnection_r16 := *, template CGI_Info_Logging_r16 p_ReestablishmentCellId_r16 := *, template integer p_TimeConnFailure_r16 := *, template (present) TimeSinceFailure_r16 p_TimeSinceFailure_r16 := ?, NR_RLF_Report_connectionFailureType_r16 p_ConnectionFailureType_r16, template (present) NR_RLF_Report_rlf_Cause p_Rlf_Cause_r16 :=?,// template LocationInfo_r16 p_LocationInfo_r16 := *, template RA_InformationCommon_r16 p_RA_InformationCommon_r16 := *, template (present) SSB_Index p_SSB_Index := tsc_NR_SSB_Index1 ) := { nr_RLF_Report_r16 := { measResultLastServCell_r16 := { measResult_r16 := { cellResults_r16 := { resultsSSB_Cell_r16 := { rsrp := p_Rsrp, rsrq := p_Rsrq, sinr := * }, resultsCSI_RS_Cell_r16 := * }, rsIndexResults_r16 := { resultsSSB_Indexes_r16 := {cr_ResultsPerSSB_Index(p_SSB_Index)}, //WA#WI=928034 changed to use parameter ssbRLMConfigBitmap_r16 := p_SsbRLMConfigBitmap_r16, resultsCSI_RS_Indexes_r16 := omit, csi_rsRLMConfigBitmap_r16 := omit } } }, measResultNeighCells_r16 := p_MeasResultNeighCells_r16, c_RNTI_r16 := p_C_RNTI_r16, previousPCellId_r16 := p_PreviousPCellId_r16, failedPCellId_r16 := p_FailedPCellId_r16, reconnectCellId_r16 := p_ReconnectCellId_r16, timeUntilReconnection_r16 := p_TimeUntilReconnection_r16, //@sic R5s211526 BASELINE MOVING 2021 sic@ reestablishmentCellId_r16 := p_ReestablishmentCellId_r16, timeConnFailure_r16 := p_TimeConnFailure_r16, timeSinceFailure_r16 := p_TimeSinceFailure_r16, connectionFailureType_r16 := p_ConnectionFailureType_r16, rlf_Cause_r16 := p_Rlf_Cause_r16, locationInfo_r16 := p_LocationInfo_r16, noSuitableCellFound_r16 := *, ra_InformationCommon_r16 := p_RA_InformationCommon_r16, csi_rsRLMConfigBitmap_v1650 := * //@sic R5s211526 BASELINE MOVING 2021 sic@ } };

2.3	Additional MCC160 implementation

1. MCC160 proposed implementation: split f_NR5GC_RRCReestablishment_Steps8To13

Before

function f_NR5GC_RRCReestablishment_Steps8To13(NR_CellId_Type                              p_NR_SourceCellId,

NR_CellId_Type                              p_NR_TargetCellId,

template(omit) SpCellConfig.rlf_TimersAndConstants p_RlfTimersAndConstants := omit,

template (omit) MacBearerRouting_Type       p_MacBearerRoutingCA := omit,

boolean p_ReceiveForRRCreconfigurationComplete := true

) runs on NR5GC_PTC

{ //@sic R5-206369 R5-213516: added p_MacBearerRoutingCA R5s220354: deleted pârameter p_SCellToAddModList sic@

var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_TargetCellId);

var IntegerList_Type v_DrbIdList;

var template (value) RadioBearerConfig v_RadioBearerConfig := f_NR5GC_GetExisting_RadioBearerConfig(true);

var template (value) CellGroupConfig v_CellGroupConfig := f_NR_GetCellGroupConfigREEST(p_NR_TargetCellId, -, p_RlfTimersAndConstants);

var template (value) NR_RadioBearerList_Type v_SS_RadioBearerConfig := f_NR5GC_GetRadioBearerList(f_NR5GC_MobileInfo_GetDRBInfoList());

// Step 8. Target Cell: Reconfigure UL DCCH DCI on USS

f_NR_SS_CommonCellConfig(p_NR_TargetCellId, cads_NR_SearchSpaceUlDciAssignment_REQ(p_NR_TargetCellId, cs_TimingInfo_Now, f_NR_SearchSpaceUlDciAssignment(v_NR_PhysicalParameters)));

// @sic R5-217830 sic@

// Step 8a. Target Cell

// If the Source Cell is different from the Target Cell

//   Configure UL grant "OnSR" and stop periodic TA

// Else

//   Stop periodic TA

if (p_NR_SourceCellId != p_NR_TargetCellId) {

f_NR_ULGrantConfiguration_Start(p_NR_TargetCellId, -, -, -, cs_NR_UplinkTimeAlignment_Stop);

} else {

f_NR_ULGrantConfiguration_Common(p_NR_SourceCellId, cs_TimingInfo_Now, -, cs_NR_UplinkTimeAlignment_Stop, cs_UL_GrantConfig_OnSR);

}

// Step 8b. Target Cell Reconfigure DRBs

// Step 9. Target Cell: Send RRCReconfiguration

// Step 10. Target Cell: Receive RRCReconfigurationComplete

// Set RadioBearerConfig as per Table 4.6.3-132: RadioBearerConfig cond REEST

v_RadioBearerConfig.srb_ToAddModList := {cs_NR_SRB_ToAddMod(tsc_SRB1), cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB2)}; //@sic R5-205334 R5s220198 R5-221420 sic@

f_ReconfigureDRBs(p_NR_TargetCellId, omit, v_RadioBearerConfig, v_CellGroupConfig, v_SS_RadioBearerConfig, -, -, p_ReceiveForRRCreconfigurationComplete, -, -, -, p_MacBearerRoutingCA);  //@sic R5-205336 sic@

// 11. Target Cell: Reconfigure DL DCCH/DTCH DCI on USS

//NOTE 2: The FollowOnFlag is set to true in the ASP reconfiguring the target Cell.

f_NR_SS_CommonCellConfig(p_NR_TargetCellId, cas_NR_SearchSpaceDlDciAssignment_REQ(p_NR_TargetCellId, cs_TimingInfo_Now, f_NR_SearchSpaceDlDciAssignment_USS(v_NR_PhysicalParameters), tsc_NoCnfReq, tsc_FollowOnFlag));

// 12. Target Cell: Re-configure RACH procedure as for initial access, enable TA transmissions.

//NOTE 3: The FollowOnFlag is set to true in the ASP reconfiguring the target PCell.

f_NR_SS_CommonCellConfig(p_NR_TargetCellId, cads_NR_RachProcedureConfig_REQ(p_NR_TargetCellId, cs_TimingInfo_Now, cs_NR_RachProcedureConfig_CBRA_Msg4Based(v_NR_PhysicalParameters, cs_NR_RachProcedureMsg4WithNoRrcMsg), tsc_NoCnfReq, tsc_FollowOnFlag));

f_NR_SS_CommonCellConfig(p_NR_TargetCellId, cas_NR_UplinkTimeAlignment_REQ(p_NR_TargetCellId, -, cs_NR_UplinkTimeAlignment_Start));

// 13. Source Cell: If the Source Cell is different from the Target Cell:

//  Release security.

//  Reset SRBs and release DRBs.

// (Re-)configure SRBs.

//  Stop periodic UL grant and periodic TA.

if (p_NR_SourceCellId != p_NR_TargetCellId) {

f_NR_SS_RRC_ReleaseSecurity(p_NR_SourceCellId, cs_TimingInfo_Now); //@sic R5s211165 sic@

v_DrbIdList := f_NR_GetActiveDRBs(p_NR_SourceCellId);  //@sic R5s220354 sic@

f_NR_SS_SRBs_DRBs_Release(p_NR_SourceCellId, cs_TimingInfo_Now, v_DrbIdList); //@sic R5-211417 R5s211165 R5s220176 sic@

f_NR_SS_SRBs_DRBs_Config(p_NR_SourceCellId, cs_TimingInfo_Now, omit, tsc_NoCnfReq, tsc_FollowOnFlag); //@sic R5s220176 sic@

f_NR_ULGrantConfiguration_Stop(p_NR_SourceCellId, -, cs_NR_UplinkTimeAlignment_Stop);

}

}

After

function f_NR5GC_RRCReestablishment_Steps8To13(NR_CellId_Type                              p_NR_SourceCellId,

NR_CellId_Type                              p_NR_TargetCellId,

template(omit) SpCellConfig.rlf_TimersAndConstants p_RlfTimersAndConstants := omit,

template (omit) MacBearerRouting_Type       p_MacBearerRoutingCA := omit,

boolean p_ReceiveForRRCreconfigurationComplete := true

) runs on NR5GC_PTC

{ //@sic R5-206369 R5-213516: added p_MacBearerRoutingCA R5s220354: deleted pârameter p_SCellToAddModList sic@

f_NR5GC_RRCReestablishment_Steps8To10( p_NR_SourceCellId, p_NR_TargetCellId, p_RlfTimersAndConstants, p_MacBearerRoutingCA, p_ReceiveForRRCreconfigurationComplete );

f_NR5GC_RRCReestablishment_Steps11To13(p_NR_SourceCellId, p_NR_TargetCellId );

}

//================================================================================================================

/*

* @desc      RRC Reestablishment sequence as per 38.523-3 cl 7.3.5.4 RRC connection re-establishment steps 8 to 10

* @param     p_NR_SourceCellId

* @param     p_NR_TargetCellId

* @param     p_MacBearerRoutingCA (default value: omit)

* @param     p_ReceiveForRRCreconfigurationComplete (default value: true)

* @status    APPROVED (NR5GC)

*/

function f_NR5GC_RRCReestablishment_Steps8To10(NR_CellId_Type                              p_NR_SourceCellId,

NR_CellId_Type                              p_NR_TargetCellId,

template(omit) SpCellConfig.rlf_TimersAndConstants p_RlfTimersAndConstants := omit,

template (omit) MacBearerRouting_Type       p_MacBearerRoutingCA := omit,

boolean p_ReceiveForRRCreconfigurationComplete := true

) runs on NR5GC_PTC

{ //@sic R5-206369 R5-213516: added p_MacBearerRoutingCA R5s220354: deleted parameter p_SCellToAddModList sic@

var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_TargetCellId);

var template (value) RadioBearerConfig v_RadioBearerConfig := f_NR5GC_GetExisting_RadioBearerConfig(true);

var template (value) CellGroupConfig v_CellGroupConfig := f_NR_GetCellGroupConfigREEST(p_NR_TargetCellId, -, p_RlfTimersAndConstants);

var template (value) NR_RadioBearerList_Type v_SS_RadioBearerConfig := f_NR5GC_GetRadioBearerList(f_NR5GC_MobileInfo_GetDRBInfoList());

// Step 8. Target Cell: Reconfigure UL DCCH DCI on USS

f_NR_SS_CommonCellConfig(p_NR_TargetCellId, cads_NR_SearchSpaceUlDciAssignment_REQ(p_NR_TargetCellId, cs_TimingInfo_Now, f_NR_SearchSpaceUlDciAssignment(v_NR_PhysicalParameters)));

// @sic R5-217830 sic@

// Step 8a. Target Cell

// If the Source Cell is different from the Target Cell

//   Configure UL grant "OnSR" and stop periodic TA

// Else

//   Stop periodic TA

if (p_NR_SourceCellId != p_NR_TargetCellId) {

f_NR_ULGrantConfiguration_Start(p_NR_TargetCellId, -, -, -, cs_NR_UplinkTimeAlignment_Stop);

} else {

f_NR_ULGrantConfiguration_Common(p_NR_SourceCellId, cs_TimingInfo_Now, -, cs_NR_UplinkTimeAlignment_Stop, cs_UL_GrantConfig_OnSR);

}

// Step 8b. Target Cell Reconfigure DRBs

// Step 9. Target Cell: Send RRCReconfiguration

// Step 10. Target Cell: Receive RRCReconfigurationComplete

// Set RadioBearerConfig as per Table 4.6.3-132: RadioBearerConfig cond REEST

v_RadioBearerConfig.srb_ToAddModList := {cs_NR_SRB_ToAddMod(tsc_SRB1), cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB2)}; //@sic R5-205334 R5s220198 R5-221420 sic@

f_ReconfigureDRBs(p_NR_TargetCellId, omit, v_RadioBearerConfig, v_CellGroupConfig, v_SS_RadioBearerConfig, -, -, p_ReceiveForRRCreconfigurationComplete, -, -, -, p_MacBearerRoutingCA);  //@sic R5-205336 sic@

}

/*

* @desc      RRC Reestablishment sequence as per 38.523-3 cl 7.3.5.4 RRC connection re-establishment steps 11 to 13

* @param     p_NR_SourceCellId

* @param     p_NR_TargetCellId

* @status    APPROVED (NR5GC)

*/

function f_NR5GC_RRCReestablishment_Steps11To13(NR_CellId_Type                             p_NR_SourceCellId,

NR_CellId_Type                              p_NR_TargetCellId

) runs on NR5GC_PTC

{ //@sic R5-206369 R5-213516: added p_MacBearerRoutingCA R5s220354: deleted parameter p_SCellToAddModList sic@

var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_TargetCellId);

var IntegerList_Type v_DrbIdList;

// 11. Target Cell: Reconfigure DL DCCH/DTCH DCI on USS

//NOTE 2: The FollowOnFlag is set to true in the ASP reconfiguring the target Cell.

f_NR_SS_CommonCellConfig(p_NR_TargetCellId, cas_NR_SearchSpaceDlDciAssignment_REQ(p_NR_TargetCellId, cs_TimingInfo_Now, f_NR_SearchSpaceDlDciAssignment_USS(v_NR_PhysicalParameters), tsc_NoCnfReq, tsc_FollowOnFlag));

// 12. Target Cell: Re-configure RACH procedure as for initial access, enable TA transmissions.

//NOTE 3: The FollowOnFlag is set to true in the ASP reconfiguring the target PCell.

f_NR_SS_CommonCellConfig(p_NR_TargetCellId, cads_NR_RachProcedureConfig_REQ(p_NR_TargetCellId, cs_TimingInfo_Now, cs_NR_RachProcedureConfig_CBRA_Msg4Based(v_NR_PhysicalParameters, cs_NR_RachProcedureMsg4WithNoRrcMsg), tsc_NoCnfReq, tsc_FollowOnFlag));

f_NR_SS_CommonCellConfig(p_NR_TargetCellId, cas_NR_UplinkTimeAlignment_REQ(p_NR_TargetCellId, -, cs_NR_UplinkTimeAlignment_Start));

// 13. Source Cell: If the Source Cell is different from the Target Cell:

//  Release security.

//  Reset SRBs and release DRBs.

// (Re-)configure SRBs.

//  Stop periodic UL grant and periodic TA.

if (p_NR_SourceCellId != p_NR_TargetCellId) {

f_NR_SS_RRC_ReleaseSecurity(p_NR_SourceCellId, cs_TimingInfo_Now); //@sic R5s211165 sic@

v_DrbIdList := f_NR_GetActiveDRBs(p_NR_SourceCellId);  //@sic R5s220354 sic@

f_NR_SS_SRBs_DRBs_Release(p_NR_SourceCellId, cs_TimingInfo_Now, v_DrbIdList); //@sic R5-211417 R5s211165 R5s220176 sic@

f_NR_SS_SRBs_DRBs_Config(p_NR_SourceCellId, cs_TimingInfo_Now, omit, tsc_NoCnfReq, tsc_FollowOnFlag); //@sic R5s220176 sic@

f_NR_ULGrantConfiguration_Stop(p_NR_SourceCellId, -, cs_NR_UplinkTimeAlignment_Stop);

}

}

2. MCC160 proposed implementation: To solve the issue that the RRCReconfigurationComplete may crush with the reestablishment procedure.

function f_TC_8_1_6_1_3_4_TestBody() runs on NR5GC_PTC

{

var template (value) DL_DCCH_Message v_RRCReconfigurationHO;

var PLMN_Identity v_PlmnID_NRCell1 := f_NR_CellInfo_GetPLMN (nr_Cell1);

var CellIdentity v_CellIdentity_NRCell1 := f_NR_CellInfo_GetCellIdentity(nr_Cell1);

var TrackingAreaCode v_TAC_NRCell1 := f_NR_CellInfo_GetTAC (nr_Cell1);

var PLMN_Identity v_PlmnID_NRCell2 := f_NR_CellInfo_GetPLMN (nr_Cell2);

var CellIdentity v_CellIdentity_NRCell2 := f_NR_CellInfo_GetCellIdentity(nr_Cell2);

var TrackingAreaCode v_TAC_NRCell2 := f_NR_CellInfo_GetTAC (nr_Cell2);

var template RRCReestablishmentComplete_v1610_IEs v_RRCReestablishmentComplete_v1610_IEs;// := cr_RRCReestablishmentComplete_v1610_IEs_RLF;

// Reconfigure nr_Cell2 to no respond to any RA Preamble

// Table 8.1.4.1.5.3.2-4: The UE attempts to perform the handover using MAC Random Access Preamble on NR Cell 2. SS does not respond.

f_NR_SS_RachProcedureConfig(nr_Cell2, cs_NR_RachProcedureConfig_NoResponse);

//@siclog "Step 1" siclog@

// The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform handover to NR Cell 2.

// Note: failure case UE doesn't go to nr_Cell2 => the standard function need not be used

v_RRCReconfigurationHO := f_NR5GC_RRCReconfiguration_IntraNR_HO(nr_Cell2);

SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, v_RRCReconfigurationHO));

//@siclog "Step 2" siclog@

//The SS changes the power level setting according to the row "T1".

f_NR_SetCellPower (nr_Cell12, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE);

//@siclog "Step 3-6" siclog@

//Re-establishment procedure

v_RRCReestablishmentComplete_v1610_IEs := cr_RRCReestablishmentComplete_v1610_IEs_RLF;

f_NR5GC_RRCReestablishment_Steps0To2(nr_Cell1, nr_Cell1); //@sic R5s220421 sic@

f_NR5GC_RRCReestablishment_Steps3To7(nr_Cell1, 2, -, v_RRCReestablishmentComplete_v1610_IEs); //@sic R5s220421 sic@

f_NR5GC_RRCReestablishment_Steps8To10(nr_Cell1, nr_Cell1,-, -, false); //@sic R5s220421 sic@

//@siclog "Step 7" siclog@

//Re-establishment procedure

//Check: Does the UE transmit an RRCReconfigurationtComplete message with rlf-InfoAvailable included?

SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete(

cr_RRCReconfigurationComplete_v1530(-,

cr_RRCReconfigurationComplete_v1560(-,cr_RRCReconfigurationComplete_v1610_RLF)))));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7");

f_NR5GC_RRCReestablishment_Steps11To13(nr_Cell1, nr_Cell1); //@sic R5s220421 sic@

- Branches executed

## This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n78” using NIA2 Integrity and NEA2 ciphering algorithms

- Execution Log Files

  - MediaTek MT6983

The MediaTek MT6983 UE passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
TC_8_1_6_1_3_4_NR5GC_MTK.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

- References

| [1] | R5s220422: Supporting information for addition of NR5GC MDT testcase 8.1.6.1.3.4 in FR1 |
| --- | --- |