3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220424

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2424 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC testcase 11.6.2 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-03-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add NR5GC test case 11.6.2 to the IWD_21wk49 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 11.6.2 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.6.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required to test case 11.6.2	3

2.1	fl_TC_11_6_2_Body	3

2.2	a_NR5GC_ReceivePDUSessionModificationComplete	9

2.3	f_ExtdProtocolConfigOptions_CheckPSDataOff	9

3	Branches executed	11

4	Execution Log Files	11

4.1	Qualcomm SM8450, SDX65 UE	11

5	References	11

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 11.6.2 which is part of the NR5GC test suite in iWD_TTCN3-B2020-09_D21wk49.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	11.6.2.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D21wk49

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	QC SM8450, SDX65 UE

Verification Status:	PASS

# Corrections required to test case 11.6.2

## fl_TC_11_6_2_Body

| Function name | fl_TC_11_6_2_Body |
| --- | --- |
| Reason for change | - In the current TTCN implementation, the following assignment is done : v_Pco := v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.pdu_Session_Modification_Request.extdProtocolConfigurationOptions;. This is incorrect, and needs to be changed. - In the current TTCN implementation the following assignment is done : v_MsgToSend.Msg.pdu_Session_Modification_Command.extdProtocolConfigurationOptions := v_Pco; This is incorrect, and needs to be corrected. - The current TTCn implementation expects the UE to send a PDU Session Modification Complete at Step 2, after which a coordination message is sent to IMS PTC to handle the re-REGISTER. Some UEs will send the re-REGISTER immediately after sending the PDU Session Modification Complete, and the coordination message delay could cause testcase to fail because the REGISTER wont be captured by TTCN correctly. This needs to be addressed. |
| Summary of change | - Instead of assigning v_Pco := v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.pdu_Session_Modification_Request.extdProtocolConfigurationOptions;, assigned v_Pco := v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Modification_Request.extdProtocolConfigurationOptions; - Instead of assigning v_MsgToSend.Msg.pdu_Session_Modification_Command.extdProtocolConfigurationOptions := v_Pco; assigned v_MsgToSend.PiggybackedPduList[0].Msg.pdu_Session_Modification_Command.extdProtocolConfigurationOptions := v_Pco; - Created a default handler function to capture the PDU Session Modification Complete, and activated this before sending the PDU Session Modification Command and deactivated this after Step 7 when the IMS PTC has sent a coordination message to NR PTC. |
| TTCN module | PS_Data_OFF_NR5GC.ttcn |
| MCC160 Comment | - Accepted - Accepted - Accepted in principle but implemented differently |

Before Change:

function fl_TC_11_6_2_Body() runs on NR5GC_PTC { var PDUSessionInfoList_Type v_PDUSessionInfo := f_NR5GC_MobileInfo_GetSessionInfoList(); //How many PDU sessions are established ? var O1_Type v_PDUSessionIdToModify := v_PDUSessionInfo[0].SessionId; var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB2; var template (value) NG_NAS_DL_Pdu_Type v_MsgToSend; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (omit) ExtdProtocolConfigOptions v_Pco := omit; var NAS_ProtocolConfigOptions_Type v_ProtocolConfigOptions; var integer i := 0; //@siclog "Steps 1" siclog@ //AT command to activate Data Off f_UT_TriggerPS_DataOff(UT, true); //@siclog "Steps 2" siclog@ // The UE transmits a PDU SESSION MODIFICATION REQ SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionModificationReq (v_GSM_MobilityInfo, v_PDUSessionInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu,v_PDUSessionIdToModify)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Incorrect PDU Session ID"); } v_Pco := v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.pdu_Session_Modification_Request.extdProtocolConfigurationOptions; //Check if the UE has set the Data Off status to activated if (f_ExtdProtocolConfigOptions_CheckPSDataOff(v_Pco)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); }else{ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2"); } if (isvalue(v_Pco.pco)) { v_ProtocolConfigOptions := valueof(v_Pco.pco); for (i := 0; i < lengthof(v_ProtocolConfigOptions); i := i + 1) { if (v_ProtocolConfigOptions[i].protocolID == '0017'O) { v_Pco.pco[i].protocolLength :='00'O; v_Pco.pco[i].content := omit; } } } //@siclog "Steps 3" siclog@ //Send PDU Session Modification Command v_MsgToSend := f_Get_NG_PDUSessionModificationCommand (v_PDUSessionIdToModify, v_GSM_MobilityInfo); v_MsgToSend.Msg.pdu_Session_Modification_Command.extdProtocolConfigurationOptions := v_Pco; SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, -, -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Steps 4" siclog@ //receive PDU Session Modification Complete SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionIdToModify)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2"); } //Now expect the re-register on IMS PTC f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); //@siclog "Steps 7" siclog@ //Receive the coordination to trigger a Video call f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); //Make the UE attempt IMS voice call. f_UT_RequestIMSVideoCall(UT, px_IMS_CalleeUri); //Postamble after receiving coordination from IMS PTC f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); } //End of test body of TC_11_6_2

After Change:

function fl_TC_11_6_2_Body() runs on NR5GC_PTC { var PDUSessionInfoList_Type v_PDUSessionInfo := f_NR5GC_MobileInfo_GetSessionInfoList(); //How many PDU sessions are established ? var O1_Type v_PDUSessionIdToModify := v_PDUSessionInfo[0].SessionId; var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB2; var template (value) NG_NAS_DL_Pdu_Type v_MsgToSend; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (omit) ExtdProtocolConfigOptions v_Pco := omit; var NAS_ProtocolConfigOptions_Type v_ProtocolConfigOptions; var default v_MyDefaultVar := null; //WA#11_6_2 var integer i := 0; //@siclog "Steps 1" siclog@ //AT command to activate Data Off f_UT_TriggerPS_DataOff(UT, true); //@siclog "Steps 2" siclog@ // The UE transmits a PDU SESSION MODIFICATION REQ SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionModificationReq (v_GSM_MobilityInfo, v_PDUSessionInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu,v_PDUSessionIdToModify)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Incorrect PDU Session ID"); } v_Pco := v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Modification_Request.extdProtocolConfigurationOptions; //WA#11_6_2 //WA#11_6_2 COMMENTED OUT v_Pco := v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.pdu_Session_Modification_Request.extdProtocolConfigurationOptions; //Check if the UE has set the Data Off status to activated if (f_ExtdProtocolConfigOptions_CheckPSDataOff(v_Pco)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); }else{ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2"); } if (isvalue(v_Pco.pco)) { v_ProtocolConfigOptions := valueof(v_Pco.pco); for (i := 0; i < lengthof(v_ProtocolConfigOptions); i := i + 1) { if (v_ProtocolConfigOptions[i].protocolID == '0017'O) { v_Pco.pco[i].protocolLength :='00'O; v_Pco.pco[i].content := omit; } } } //@siclog "Steps 3" siclog@ //Send PDU Session Modification Command v_MsgToSend := f_Get_NG_PDUSessionModificationCommand (v_PDUSessionIdToModify, v_GSM_MobilityInfo); v_MsgToSend.PiggybackedPduList[0].Msg.pdu_Session_Modification_Command.extdProtocolConfigurationOptions := v_Pco; //WA#11_6_2 v_MyDefaultVar := activate(a_NR5GC_ReceivePDUSessionModificationComplete(v_PDUSessionIdToModify)); //WA#11_6_2 //WA#11_6_2 COMMENTED OUT v_MsgToSend.Msg.pdu_Session_Modification_Command.extdProtocolConfigurationOptions := v_Pco; SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, -, -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Steps 4" siclog@ //receive PDU Session Modification Complete /* WA#11_6_2 COMMENTED OUT SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionIdToModify)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2"); }*/ //Now expect the re-register on IMS PTC f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); //@siclog "Steps 7" siclog@ //Receive the coordination to trigger a Video call f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); deactivate(v_MyDefaultVar); //WA#11_6_2 //Make the UE attempt IMS voice call. f_UT_RequestIMSVideoCall(UT, px_IMS_CalleeUri); //Postamble after receiving coordination from IMS PTC f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); } //End of test body of TC_11_6_2 MCC160 Implementation: function f_TC_11_6_2_NR5GC() runs on NR5GC_PTC { //Data Off / MO Video Call //var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); f_NR5GC_Init_NAS(NR_1); //Create and configure cells f_NR_CellConfig_Def(nr_Cell1); //Preamble //The UE is in 5GS state 3N-A f_NR5GC_Preamble (nr_Cell1, STATE_CONNECTED_3A); //Main test body fl_TC_11_6_x_Body(); // Finish f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); } // Created a new common function to be used in TCs 11.6.x function fl_TC_11_6_x_Body() runs on NR5GC_PTC { var PDUSessionInfoList_Type v_PDUSessionInfo := f_NR5GC_MobileInfo_GetSessionInfoList(); //How many PDU sessions are established ? var O1_Type v_PDUSessionIdToModify := v_PDUSessionInfo[0].SessionId; var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB2; var template (value) NG_NAS_DL_Pdu_Type v_MsgToSend; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (omit) ExtdProtocolConfigOptions v_Pco := omit; var NAS_ProtocolConfigOptions_Type v_ProtocolConfigOptions; var integer i := 0; //@siclog "Steps 1" siclog@ //AT command to activate Data Off f_UT_TriggerPS_DataOff(UT, true); //@siclog "Steps 2" siclog@ // The UE transmits a PDU SESSION MODIFICATION REQ SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionModificationReq (v_GSM_MobilityInfo, v_PDUSessionInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu,v_PDUSessionIdToModify)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Incorrect PDU Session ID"); } v_Pco := v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Modification_Request.extdProtocolConfigurationOptions; //Check if the UE has set the Data Off status to activated if (f_ExtdProtocolConfigOptions_CheckPSDataOff(v_Pco)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); }else{ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2"); } if (isvalue(v_Pco.pco)) { v_ProtocolConfigOptions := valueof(v_Pco.pco); for (i := 0; i < lengthof(v_ProtocolConfigOptions); i := i + 1) { if (v_ProtocolConfigOptions[i].protocolID == '0017'O) { v_Pco.pco[i].protocolLength :='00'O; v_Pco.pco[i].content := omit; } } } //@siclog "Steps 3" siclog@ //Send PDU Session Modification Command v_MsgToSend := f_Get_NG_PDUSessionModificationCommand (v_PDUSessionIdToModify, v_GSM_MobilityInfo); v_MsgToSend.PiggybackedPduList[0].Msg.pdu_Session_Modification_Command.extdProtocolConfigurationOptions := v_Pco; SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, -, -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Steps 4" siclog@ //receive PDU Session Modification Complete SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionIdToModify)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } //Postamble after receiving coordination from IMS PTC f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); } //End of test body of TC_11_6_x function f_TC_11_6_2_NR5GC_IMS1()runs on IMS_PTC { timer t_Wait := 30.0; fl_TC_11_6_x_Steps5and6_NR5GC_IMS1(); //Now trigger the Video call //f_IMS_IPCAN_SendCoOrdMsg(IPCAN); @sic R5s220424 sic@ //Now trigger the Video call f_UT_RequestIMSVideoCall(MMI, px_IMS_CalleeUri); //@sic R5s220424 sic@ //@siclog "Step 8" siclog@ t_Wait.start; alt { [] IMS_Server.receive(car_IMS_Invite_Request(cr_INVITE_Request(?, ?))) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8"); } []t_Wait.timeout{} } //Ready for postamble f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IpcanReleaseWithOptionalDeregistration(); } Created new common function to be used in TCs 11.6.x IMS function fl_TC_11_6_x_Steps5and6_NR5GC_IMS1()runs on IMS_PTC { f_IMS_PTC_Init(PDN_1); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_Registration(); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); //@siclog "Steps 5-6" siclog@ //Check: does the UE re-register with "+g.3gpp.ps-data-off" header field parameter set to "active"? //SS responds with 200 OK fl_IMS_ReRegistration_DataOff(); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 5"); }

## a_NR5GC_ReceivePDUSessionModificationComplete

| Function name | a_NR5GC_ReceivePDUSessionModificationComplete |
| --- | --- |
| Reason for change | Please see Change 2.1 Point 3 |
| Summary of change | New function |
| TTCN module | PS_Data_OFF_NR5GC.ttcn |
| MCC160 Comment | Not needed. See MCC160 Implementation above |

Before Change:

altstep a_NR5GC_ReceivePDUSessionModificationComplete(template (present) O1_Type p_PDU_SessionId) runs on NR_BASE_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; [] SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered)))-> value v_ReceivedAsp { if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedAsp.Signalling.Nas[0].Pdu, p_PDU_SessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2"); } } }

## f_ExtdProtocolConfigOptions_CheckPSDataOff

| Function name | f_ExtdProtocolConfigOptions_CheckPSDataOff |
| --- | --- |
| Reason for change | The current TTCN implementation looks for the Container ID content within EPCO to be set to ’01’H. However, the Prose clearly indicates that the Container ID content should be ’02’. This needs to be addressed. |
| Summary of change | In the function f_ExtdProtocolConfigOptions_CheckPSDataOff , replaced v_Result := match ('01'O, v_Content); with v_Result := match ('02'O, v_Content); Please see below. |
| TTCN module | NG_NASTemplateFunctions.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_ExtdProtocolConfigOptions_CheckPSDataOff(template (omit) ExtdProtocolConfigOptions p_Pco) return boolean { var template (omit) octetstring v_Content := f_ExtdProtocolConfigOptions_Get(p_Pco, '0017'O); var boolean v_Result := false; if (isvalue(v_Content)) { if (lengthof(v_Content) > 0) { v_Result := match ('01'O, v_Content);//WA#11_6_2 01 --> 02 } } return v_Result; }

After Change:

function f_ExtdProtocolConfigOptions_CheckPSDataOff(template (omit) ExtdProtocolConfigOptions p_Pco) return boolean { var template (omit) octetstring v_Content := f_ExtdProtocolConfigOptions_Get(p_Pco, '0017'O); var boolean v_Result := false; if (isvalue(v_Content)) { if (lengthof(v_Content) > 0) { v_Result := match ('02'O, v_Content);//WA#11_6_2 01 --> 02 } } return v_Result; }

# Branches executed

## This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n5” using NIA1 Integrity and NEA1 ciphering algorithms

# Execution Log Files

## Qualcomm SM8450, SDX65 UE

The QC SM8450, SDX65 UE passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
tc_11_6_2_Qualcomm.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s220425: Supporting information for addition of NR5GC testcase 11.6.2 in FR1 |
| --- | --- |