3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220473

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2449 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC RRC CA test cases 8.1.4.1.9.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-03-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The timer t_T304Expiry should be started as soon as the RRCRecofiguration message is sent to the UE. Currently the changing of cell power levels delays the start of the timer so it may still be running when T304 expires at the UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Move t_T304Expiry.start() to earlier point. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail these test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.1.9.1.NR5GC, 8.1.4.1.9.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_8_1_4_1_9_x_NR5GC_TestBody |
| --- | --- |
| Reason for change | The timer t_T304Expiry should be started as soon as the RRCRecofiguration message is sent to the UE. Currently the changing of cell power levels delays the start of the timer so it may still be running when T304 expires at the UE. |
| Summary of change | Move t_T304Expiry.start() to earlier point. |
| TTCN module | RRC_NR_CA_IntraNR_Handover_NR5GC |
| MCC160 Comment | Accepted. |

Before Change

function f_TC_8_1_4_1_9_x_NR5GC_TestBody(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell) runs on NR5GC_PTC { ...... //@siclog "Step 18" siclog@ //The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform handover to NR Cell 3/10 and to release SCell NR Cell3/10. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, v_RRCReconfigurationHO, crs_MacBearerRouting(nr_Cell1))); //@sic R5-213516 sic@ //Reconfigure SCell to normal cell v_DRBInfoList := f_NR5GC_MobileInfo_GetDRBInfoList(); v_RadioBearerList := f_NR5GC_GetRadioBearerList(v_DRBInfoList); f_NR_ReconfigSCellToNormalCell(p_SCell, v_RadioBearerList, -); //@siclog "Step 19" siclog@ //The SS changes NR Cell 1 and NR Cell 3/10 parameters according to the row "T4" in table 8.1.4.1.9.1.3.2-1/1A f_NR_SetCellPowerList(v_CellPowerList_AtT4); //Wait for duration of T304, so that UE detects RRCReconfigurationWithSync Error while SS doesn't respond to RACH on Scell //@sic R5s220354 sic@ t_T304Expiry.start(v_T304Min_1sec); t_T304Expiry.timeout; ...... }

After Change

function f_TC_8_1_4_1_9_x_NR5GC_TestBody(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell) runs on NR5GC_PTC { ...... //@siclog "Step 18" siclog@ //The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform handover to NR Cell 3/10 and to release SCell NR Cell3/10. SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, v_RRCReconfigurationHO, crs_MacBearerRouting(nr_Cell1))); //@sic R5-213516 sic@ t_T304Expiry.start(v_T304Min_1sec); //Reconfigure SCell to normal cell v_DRBInfoList := f_NR5GC_MobileInfo_GetDRBInfoList(); v_RadioBearerList := f_NR5GC_GetRadioBearerList(v_DRBInfoList); f_NR_ReconfigSCellToNormalCell(p_SCell, v_RadioBearerList, -); //@siclog "Step 19" siclog@ //The SS changes NR Cell 1 and NR Cell 3/10 parameters according to the row "T4" in table 8.1.4.1.9.1.3.2-1/1A f_NR_SetCellPowerList(v_CellPowerList_AtT4); //Wait for duration of T304, so that UE detects RRCReconfigurationWithSync Error while SS doesn't respond to RACH on Scell //@sic R5s220354 sic@ //MOVED t_T304Expiry.start(v_T304Min_1sec); t_T304Expiry.timeout; ...... }