3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220495

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2468 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC postamble function |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current postamble sequence handling allows for the UE to send a PDU SESSION RELEASE REQUEST following switch off, and will perform the PDU session release procedure by a call to f_NR5GC_PDUSessionRelease() before receiving DEREGISTRATION REQUEST. However the function f_NR5GC_PDUSessionRelease() will stop the IP handling for the PDN after completing the PDU session release signalling, and this can delay the return of the function by several seconds. If the UE sends DEREGISTRATION REQUEST during this time the test case can fail. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Activated a Default handler for DEREGISTRATION REQUEST so that it can be stored and later processed after finishing the PDU Session Release procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail NR5GC test cases during postamble |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Common function f_NR5GC_OptionalIMSDeregisterOnSwitchOff() |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_NR5GC_OptionalIMSDeregisterOnSwitchOff() |
| --- | --- |
| Reason for change | Current postamble sequence handling allows for the UE to send a PDU SESSION RELEASE REQUEST following switch off, and will perform the PDU session release procedure by a call to f_NR5GC_PDUSessionRelease() before receiving DEREGISTRATION REQUEST. However the function f_NR5GC_PDUSessionRelease() will stop the IP handling for the PDN after completing the PDU session release signalling, and this can delay the return of the function by several seconds. If the UE sends DEREGISTRATION REQUEST during this time the test case can fail. |
| Summary of change | Activated a Default handler for DEREGISTRATION REQUEST so that it can be stored and later processed after finishing the PDU Session Release procedure. Note : This implementation is in line with the MCC implementation of TTCN CR R5s220174 for the similar scenario at emergency call release. |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment | Accepted |

Before Change

function f_NR5GC_OptionalIMSDeregisterOnSwitchOff(NR_CellId_Type p_NR_CellId, NG_NAS_MSG_Indication_Type p_InitialMsg) runs on NR5GC_PTC return NG_NAS_MSG_Indication_Type { var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_MSG_Indication_Type v_DetachReq; var GSM_MobilityInfo_Type v_GSM_MobilityInfo; if (ischosen(p_InitialMsg.Pdu.Msg.service_Request)) { f_NR5GC_RRC_ConnectedState3N_Steps5_8(p_NR_CellId, p_InitialMsg); // Once in the connected mode, the UE won't send any NAS msg until it's completed IMS Deregistration SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered)))) -> value v_ReceivedAsp; if (ischosen(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.deregistration_RequestMO)) { // @sic R5-216151 sic@ v_DetachReq := v_ReceivedAsp.Signalling.Nas[0]; } else { // Not a Detach Req if (not f_Check_NG_PDUSessionReleaseReq (v_GSM_MobilityInfo, f_NR5GC_MobileInfo_GetSessionInfoList(), v_ReceivedAsp.Signalling.Nas[0].Pdu, ?)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Request Message Failed"); } f_NR5GC_PDUSessionRelease(p_NR_CellId, v_GSM_MobilityInfo.SessionId); // Now wait for deregister msg SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; v_DetachReq := v_ReceivedAsp.Signalling.Nas[0]; } } else { // The initial message not a Service Req so must have been a Detach Req v_DetachReq := p_InitialMsg; } return v_DetachReq; }

After Change

function f_NR5GC_OptionalIMSDeregisterOnSwitchOff(NR_CellId_Type p_NR_CellId, NG_NAS_MSG_Indication_Type p_InitialMsg) runs on NR5GC_PTC return NG_NAS_MSG_Indication_Type { var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_MSG_Indication_Type v_DetachReq; var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var default v_MyDefaultVar := null; if (ischosen(p_InitialMsg.Pdu.Msg.service_Request)) { f_NR5GC_RRC_ConnectedState3N_Steps5_8(p_NR_CellId, p_InitialMsg); // Once in the connected mode, the UE won't send any NAS msg until it's completed IMS Deregistration SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered)))) -> value v_ReceivedAsp; if (ischosen(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.deregistration_RequestMO)) { // @sic R5-216151 sic@ v_DetachReq := v_ReceivedAsp.Signalling.Nas[0]; } else { // Not a Detach Req if (not f_Check_NG_PDUSessionReleaseReq (v_GSM_MobilityInfo, f_NR5GC_MobileInfo_GetSessionInfoList(), v_ReceivedAsp.Signalling.Nas[0].Pdu, ?)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Request Message Failed"); } f_NR5GC_MsgInDefault_ResetStoreOfNASMessageReq(); v_MyDefaultVar := activate(a_NR5GC_AdditionalPDUSessionEstablishmentReq (p_NR_CellId)); f_NR5GC_PDUSessionRelease(p_NR_CellId, v_GSM_MobilityInfo.SessionId); deactivate(v_MyDefaultVar); if(f_NR5GC_MsgInDefault_GetNumOfNASMessageReq() > 0) { v_ReceivedAsp := f_NR5GC_MsgInDefault_GetOneNASMessageReq(1); } else { // Now wait for deregister msg SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; } v_DetachReq := v_ReceivedAsp.Signalling.Nas[0]; } } else { // The initial message not a Service Req so must have been a Detach Req v_DetachReq := p_InitialMsg; } return v_DetachReq; }