3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220501

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2473 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC eNS test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, eNS-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Incorrect IE length for allowed NSSAI list in 9.1.10.1 and rejected NSSAI list in 9.1.10.6 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct IE lengths |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail these test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.10.1.1NR5GC, 9.1.10.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_9_1_10_1_TestBody() |
| --- | --- |
| Reason for change | Allowed NSSAI list at step 12 contains 1 NSSAI with 2 octets, IE length is incorrectly set. |
| Summary of change | Correct IE length to 2 octets |
| TTCN module | NetworkSlicingEnh_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change

function fl_TC_9_1_10_1_TestBody() runs on NR5GC_PTC { … … … var template (value) NSSAI v_AllowedNSSAI:= { iei := '15'O, iel := '02'O, s_nssai := {cs_S_NSSAI(omit, '01'O, '01'O, omit)} }; var template (value) NSSAI v_AllowedNSSAI_Step12:= { // @sic R5-217939 sic@ iei := '15'O, // @sic R5-222009 sic@ iel := '01'O, s_nssai := {cs_S_NSSAI(omit, '01'O, '03'O, omit)} }; var template (value) NSSAI v_RequestedNSSAI:= { iei := '2F'O, iel := '02'O, s_nssai := {cs_S_NSSAI(omit, '01'O, '01'O, omit)} }; var template (value) NSSAI v_ConfiguredNSSAI := { iei := '31'O, iel := '04'O, s_nssai := {cs_S_NSSAI(omit, '01'O, '01'O, omit), cs_S_NSSAI(omit, '01'O, '02'O, omit)} }; … … … }

After Change

function fl_TC_9_1_10_1_TestBody() runs on NR5GC_PTC { … … … var template (value) NSSAI v_AllowedNSSAI:= { iei := '15'O, iel := '02'O, s_nssai := {cs_S_NSSAI(omit, '01'O, '01'O, omit)} }; var template (value) NSSAI v_AllowedNSSAI_Step12:= { // @sic R5-217939 sic@ iei := '15'O, // @sic R5-222009 sic@ iel := '02'O, s_nssai := {cs_S_NSSAI(omit, '01'O, '03'O, omit)} }; var template (value) NSSAI v_RequestedNSSAI:= { iei := '2F'O, iel := '02'O, s_nssai := {cs_S_NSSAI(omit, '01'O, '01'O, omit)} }; var template (value) NSSAI v_ConfiguredNSSAI := { iei := '31'O, iel := '04'O, s_nssai := {cs_S_NSSAI(omit, '01'O, '01'O, omit), cs_S_NSSAI(omit, '01'O, '02'O, omit)} }; … … … }

## Change 2

| Function name | fl_TC_9_1_10_6_Steps2To20 |
| --- | --- |
| Reason for change | Rejected NSSAI list contains 1 NSSAI with 5 octets, IE length is incorrectly set. |
| Summary of change | Corrected IE length to 5 octets |
| TTCN module | NetworkSlicingEnh_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change

function fl_TC_9_1_10_6_Steps2To20() runs on NR5GC_PTC { … … … var template (value) NSSAI v_AllowedNSSAI:= { iei := '15'O, iel := '02'O, s_nssai := {cs_S_NSSAI(omit, '01'O, '01'O, omit)} }; var template (value) NSSAI v_ConfiguredNSSAI := { iei := '31'O, iel := '04'O, s_nssai := {cs_S_NSSAI(omit, '01'O, '01'O, omit), cs_S_NSSAI(omit, '01'O, '02'O, omit)} }; var template (value) NSSAI v_PendingNSSAI := { iei := '39'O, iel := '02'O, // @sic R5s220222 sic@ s_nssai := {cs_S_NSSAI(omit, '01'O, '02'O, omit)} }; var template (value) RejectedNSSAI v_RejectedNSSAI := { iei := '11'O, iel := '02'O, rejectS_NSSAI := {cs_RejectedS_NSSAI('0100'B,'0010'B, '02'O, 'FFFFFF'O)} // @sic R5s220222 sic@ }; … … … }

After Change

function fl_TC_9_1_10_6_Steps2To20() runs on NR5GC_PTC { … … … var template (value) NSSAI v_AllowedNSSAI:= { iei := '15'O, iel := '02'O, s_nssai := {cs_S_NSSAI(omit, '01'O, '01'O, omit)} }; var template (value) NSSAI v_ConfiguredNSSAI := { iei := '31'O, iel := '04'O, s_nssai := {cs_S_NSSAI(omit, '01'O, '01'O, omit), cs_S_NSSAI(omit, '01'O, '02'O, omit)} }; var template (value) NSSAI v_PendingNSSAI := { iei := '39'O, iel := '02'O, // @sic R5s220222 sic@ s_nssai := {cs_S_NSSAI(omit, '01'O, '02'O, omit)} }; var template (value) RejectedNSSAI v_RejectedNSSAI := { iei := '11'O, iel := '05'O, rejectS_NSSAI := {cs_RejectedS_NSSAI('0100'B,'0010'B, '02'O, 'FFFFFF'O)} // @sic R5s220222 sic@ }; … … }