3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220502

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2474 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for 5GMM test case 9.1.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At step 14, while checking the UT and local time zone, the expected value for the AT response check is missing a ‘/’between the year and the month |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected the expected AT response by adding a ‘/’ between the year string and the the month (year/month). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.4.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_9_1_4_1_TestBody |
| --- | --- |
| Reason for change | At step 14, while checking the UT and local time zone, the expected value for the AT response check is missing a ‘/’between the year and the month |
| Summary of change | Corrected the expected AT response by adding a ‘/’ between the year string and the the month (year/month). |
| TTCN module | ConfigurationUpdate_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function fl_TC_9_1_4_1_TestBody() runs on NR5GC_PTC

{

…

…

…

//@siclog "Steps 14" siclog@

f_AT_ResponseCheck(v_AT_Response,

v_CurrentYearStrExt& "12/31,14:[0-9]#(2):[0-9]#(2)", // TZ GMT+1, dst (1 char) ignored, check date and hour, ignore mins, secs @sic R5s200903, R5-221451 sic@

"Step 14 (UT and local time zone)",

v_Expression_CTZR,

2);

f_AT_ResponseCheck(v_AT_Response,

"1", // DST=1

"Step 14  DST",

v_Expression_CTZR,

1);

v_AT_Response := f_UT_ChkOperatorName (UT, LONG_NAME);

f_AT_ResponseCheck (v_AT_Response,

v_FullNameString,

"Step 14 (long network name)");

v_AT_Response := f_UT_ChkOperatorName (UT, SHORT_NAME);

f_AT_ResponseCheck (v_AT_Response,

v_ShortNameString,

"Step 14 (short network name)");

v_AT_Response := f_UT_ChkTimeZone (UT);

f_AT_ResponseCheck(v_AT_Response,

"*[0-9]#(2)/[0-9]#(2)/[0-9]#(2),[0-9]#(2):[0-9]#(2):[0-9]#(2)\+04*", // only TZ to be checked

"Step 14 (local time zone)");

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14");

…

…

…

}

After Change:

function fl_TC_9_1_4_1_TestBody() runs on NR5GC_PTC

{

…

…

…

//@siclog "Steps 14" siclog@

f_AT_ResponseCheck(v_AT_Response,

v_CurrentYearStrExt& "/12/31,14:[0-9]#(2):[0-9]#(2)", // TZ GMT+1, dst (1 char) ignored, check date and hour, ignore mins, secs @sic R5s200903, R5-221451 sic@

"Step 14 (UT and local time zone)",

v_Expression_CTZR,

2);

f_AT_ResponseCheck(v_AT_Response,

"1", // DST=1

"Step 14  DST",

v_Expression_CTZR,

1);

v_AT_Response := f_UT_ChkOperatorName (UT, LONG_NAME);

f_AT_ResponseCheck (v_AT_Response,

v_FullNameString,

"Step 14 (long network name)");

v_AT_Response := f_UT_ChkOperatorName (UT, SHORT_NAME);

f_AT_ResponseCheck (v_AT_Response,

v_ShortNameString,

"Step 14 (short network name)");

v_AT_Response := f_UT_ChkTimeZone (UT);

f_AT_ResponseCheck(v_AT_Response,

"*[0-9]#(2)/[0-9]#(2)/[0-9]#(2),[0-9]#(2):[0-9]#(2):[0-9]#(2)\+04*", // only TZ to be checked

"Step 14 (local time zone)");

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14");

…

…

…

}