3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220524

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2479 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_NR5GC_EmergencyCallRelease |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation : function f_NR5GC_EmergencyCallRelease(NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Emergency_PDN); // @sic R5s210363 sic@ var GSM_MobilityInfo_Type v_PDUSession := valueof(v_GSM_MobilityInfo); // Must be there as previously established it var O1_Type v_PDUSessionId := v_PDUSession.SessionId; var boolean v_WaitForReq := false; timer t_Wait := 5.0; f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); // @sic R5s220039 sic@ t_Wait.start; alt { // @sic R5-216829 sic@ [v_WaitForReq] a_NR5GC_PDUSessionReleaseReq (p_NR_CellId, v_PDUSessionId) // @sic R5s211524 sic@ { t_Wait.stop; v_WaitForReq := false; repeat; } [] IMS[tsc_Index_IMS2].receive(cmr_IMS_IPCAN_Trigger) // @sic R5s220039 sic@ { t_Wait.stop } [] t_Wait.timeout // @sic R5s220039 sic@ { repeat } } f_NR5GC_PDUSessionRelease (p_NR_CellId, v_PDUSessionId, cs_GMM_GSM_Cause (omit, '00100100'B)); f_IP_RtpRtcpLoopback(IP, v_PDUSession.PdnIndex, tsc_IMS_MediaPort_M1, omit); // @sic R5s210363 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Emergency Call Release"); } Variable v_WaitForReq is set to FALSE, which results in the PDU Session Release REQ never being handled. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Initialized v_WaitForReq to true. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required	4

1.1	f_NR5GC_EmergencyCallRelease	4

# Corrections required

## f_NR5GC_EmergencyCallRelease

| Function name | f_NR5GC_EmergencyCallRelease |
| --- | --- |
| Reason for change | In the current TTCN implementation : function f_NR5GC_EmergencyCallRelease(NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Emergency_PDN); // @sic R5s210363 sic@ var GSM_MobilityInfo_Type v_PDUSession := valueof(v_GSM_MobilityInfo); // Must be there as previously established it var O1_Type v_PDUSessionId := v_PDUSession.SessionId; var boolean v_WaitForReq := false; timer t_Wait := 5.0; f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); // @sic R5s220039 sic@ t_Wait.start; alt { // @sic R5-216829 sic@ [v_WaitForReq] a_NR5GC_PDUSessionReleaseReq (p_NR_CellId, v_PDUSessionId) // @sic R5s211524 sic@ { t_Wait.stop; v_WaitForReq := false; repeat; } [] IMS[tsc_Index_IMS2].receive(cmr_IMS_IPCAN_Trigger) // @sic R5s220039 sic@ { t_Wait.stop } [] t_Wait.timeout // @sic R5s220039 sic@ { repeat } } f_NR5GC_PDUSessionRelease (p_NR_CellId, v_PDUSessionId, cs_GMM_GSM_Cause (omit, '00100100'B)); f_IP_RtpRtcpLoopback(IP, v_PDUSession.PdnIndex, tsc_IMS_MediaPort_M1, omit); // @sic R5s210363 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Emergency Call Release"); } Variable v_WaitForReq is set to FALSE, which results in the PDU Session Release REQ never being handled. This needs to be addressed. |
| Summary of change | Initialized v_WaitForReq to true. |
| TTCN module | NR5GC_IMSProcedures.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_NR5GC_EmergencyCallRelease(NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Emergency_PDN); // @sic R5s210363 sic@ var GSM_MobilityInfo_Type v_PDUSession := valueof(v_GSM_MobilityInfo); // Must be there as previously established it var O1_Type v_PDUSessionId := v_PDUSession.SessionId; var boolean v_WaitForReq := false; timer t_Wait := 5.0; f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); // @sic R5s220039 sic@ t_Wait.start; alt { // @sic R5-216829 sic@ [v_WaitForReq] a_NR5GC_PDUSessionReleaseReq (p_NR_CellId, v_PDUSessionId) // @sic R5s211524 sic@ { t_Wait.stop; v_WaitForReq := false; repeat; } [] IMS[tsc_Index_IMS2].receive(cmr_IMS_IPCAN_Trigger) // @sic R5s220039 sic@ { t_Wait.stop } [] t_Wait.timeout // @sic R5s220039 sic@ { repeat } } f_NR5GC_PDUSessionRelease (p_NR_CellId, v_PDUSessionId, cs_GMM_GSM_Cause (omit, '00100100'B)); f_IP_RtpRtcpLoopback(IP, v_PDUSession.PdnIndex, tsc_IMS_MediaPort_M1, omit); // @sic R5s210363 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Emergency Call Release"); }

After Change:

function f_NR5GC_EmergencyCallRelease(NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Emergency_PDN); // @sic R5s210363 sic@ var GSM_MobilityInfo_Type v_PDUSession := valueof(v_GSM_MobilityInfo); // Must be there as previously established it var O1_Type v_PDUSessionId := v_PDUSession.SessionId; var boolean v_WaitForReq := true; //WA#11_4_5 false; timer t_Wait := 5.0; f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); // @sic R5s220039 sic@ t_Wait.start; alt { // @sic R5-216829 sic@ [v_WaitForReq] a_NR5GC_PDUSessionReleaseReq (p_NR_CellId, v_PDUSessionId) // @sic R5s211524 sic@ { t_Wait.stop; v_WaitForReq := false; repeat; } [] IMS[tsc_Index_IMS2].receive(cmr_IMS_IPCAN_Trigger) // @sic R5s220039 sic@ { t_Wait.stop } [] t_Wait.timeout // @sic R5s220039 sic@ { repeat } } f_NR5GC_PDUSessionRelease (p_NR_CellId, v_PDUSessionId, cs_GMM_GSM_Cause (omit, '00100100'B)); f_IP_RtpRtcpLoopback(IP, v_PDUSession.PdnIndex, tsc_IMS_MediaPort_M1, omit); // @sic R5s210363 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Emergency Call Release"); }