3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220535

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2481 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of NR5GC R16 eNS test case 9.1.10.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD, Keysight |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, eNS-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add NSSAA test case 9.1.10.3 NSSAA / UE configuration update / Rejected NSSAI |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to Network slice-specific authentication and authorization test case 9.1.10.3 required for approval |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.10.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

3	Branches executed	19

4	Execution Log Files	19

4.1	Qualcomm SM8450 + SDX65	19

4.2	Samsung SM-G991B	19

5	References	19

- Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 9.1.10.3 which is part of the NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

Shaun Harry 
                                      shaun.harry@keysight.com

  - Verification Test Summary

Test Case: 	9.1.10.3

ATS Version:	iwd-TTCN3-B2020-09_D22wk12

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR
                                                Keysight 5G Protocol Conformance Toolset

UE used:	Qualcomm SM8450 + SDX65

Samsung SM-G991B.

Verification Status:	PASS

- Corrections required

  - Change 1

| Function name | f_TC_9_1_10_3_NR5GC() |
| --- | --- |
| Reason for change | - In the testbody, UE ends in STATE_IDLE. This will cause the UE to send Deregistration Request MO using SRB1 in the Postamble. However, TTCN expects this to happen in SRB2 (by default) |
| Summary of change | - Add 3rd input parameter to f_NR_Postamble to receive Deregistration Request MO in SRB1 |
| TTCN module | NetworkSlicingEnh_NR5GC.ttcn |
| MCC160 Comment | Not Needed. The function has already handled in idle state. |

Before Change:

function f_TC_9_1_10_3_NR5GC() runs on NR5GC_PTC { //NSSAA / Initial registration / Rejected NSSAI, pending NSSAI var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); f_NR5GC_Init_NAS(NR_1); //Create and configure cell f_NR_CellConfig_Def(v_NGC_NASCellA); f_NR5GC_Preamble (v_NGC_NASCellA, STATE_OFF_0B); f_NR_TestBody_Set(true); fl_TC_9_1_10_3_TestBody(); f_NR_TestBody_Set(false); //Finish f_NR_Postamble(v_NGC_NASCellA, STATE_IDLE_1A); }

After Change:

function f_TC_9_1_10_3_NR5GC() runs on NR5GC_PTC { //NSSAA / Initial registration / Rejected NSSAI, pending NSSAI var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); f_NR5GC_Init_NAS(NR_1); //Create and configure cell f_NR_CellConfig_Def(v_NGC_NASCellA); f_NR5GC_Preamble (v_NGC_NASCellA, STATE_OFF_0B); f_NR_TestBody_Set(true); fl_TC_9_1_10_3_TestBody(); f_NR_TestBody_Set(false); //Finish f_NR_Postamble(v_NGC_NASCellA, STATE_IDLE_1A, tsc_NR_RbId_SRB1); }

  - Change 2

| Function name | fl_TC_9_1_10_3_TestBody() |
| --- | --- |
| Reason for change | - Incorrect iel, s_nssai_length, s_nssai_sd values for rejectedNSSAI IEs used for step12 and step 33. - AllowedNSSAI in Registration Accept message in step 12 include sst1 when it is already part of rejectedNSSAI. Similarly at Step 33, sst1 is part of allowedNSSAI even though it should be pending, and sst2 is part of allowedNSSAI when it should be part of rejectedNSSAI. - The expected AT Reply when querying the list of rejected NSSAI in the UE is as shown below (TS 27.007) However, as per TS 24.501 ‘FFFFFF’ denotes no SD value associated with this sst. TTCN should be modified to expect an AT Reply with either ‘FFFFFF’ included or not. - At Step 18, since SRB2 has not been configured, deregistration procedure will happen in SRB1 - At Step 38a2, the parameter for USIM Removal must be set to ‘true’ so that TTCN doesn’t send an AT Command to the UE to do a switch off. - At Step53, TTCN expects PDUSessionEstablishment Request to come in on SRB2 even though SRB2 has not been configured at this point. Also, TTCN expects that the S-NSSAI IE is present but its IEI omitted. - At Step 54, PDUSessionEstablishmentReject is getting sent on SRB2 with an incorrect PTI. |
| Summary of change | - As per prose CR, s_nssai_sd was set to ‘FFFFFF’, and iel, s_nssai_length were corrected. - At Step 12 Registration Accept, S-NSSAI=1 is removed from the allowed NSSAI list, and At Step 33 S-NSSAI=2 is removed from the allowed NSSAI list. - At Step 21 and Step 36, modified TTCN to allow AT Reply even when it doesn’t include FFFFFF - Modified TTCN to perform deregistration using SRB1 at Step 18 - At Step 38a2, set 4th input parameter of f_NR_UE_DeRegisterOnSwitchOff() to ‘true’ - Modified TTCN to expect incoming UL NAS Transport on SRB1, and to expect S-NSSAI IEI to be present with value ‘22’O - Modified TTCN so that PDUSessionEstablishmentReject is sent on SRB1. Added a line extracting the PTI value sent by the UE in PDUSessionEstablishmentRequest, and used this value in the PDUSessionEstablishmentReject. |
| TTCN module | NetworkingSlicingEnh_NR5GC.ttcn |
| MCC160 Comment | For change 1,2, prose CR needed. 3.Rejected. According to TS24.501 clause 9.11.3.46, “SD” of “Rejected NSSAI” should be included with the cause value. At the same time, the value checked here is only the rejected content the prose specified at the previous steps. 4.Rejected. SRB2 has been configured. The same with change 6 and 7. 5.Accepted. 6.Rejected.The check function IE is “S_NSSAI_Type” type instead of “NSSAI” type, the IEI should be omit. 7.Change for PTI is accepted. |

Before Change:

function fl_TC_9_1_10_3_TestBody() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_CellsOnDifferentPLMN); var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NR_SRB_COMMON_IND v_ReceivedAsp; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var NG_NAS_UL_Message_Type v_GSMMsg; var GSM_MobilityInfo_Type v_GSM_MobilityInfo_s; var ProcedureTransactionIdentifier v_PTI := '00'O; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (v_NGC_NASCellA); var template (value) NG_MobileIdentity v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), v_GutiParams); var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC (v_NGC_NASCellA); var template (value) NG_TrackingAreaIdList v_TAIList := cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), {bit2oct(v_TAC)}); var template (value) RejectedNSSAI v_RejectedNSSAI := { iei := '11'O, iel := '05'O, rejectS_NSSAI := {cs_RejectedS_NSSAI('0001'B, '0010'B, '01'O, 'FFFFFF'O)}}; //@sic R5-222008 sic@ var template (value) NSSAI v_AllowedNSSAI:= { iei := '15'O, iel := '06'O, s_nssai := {cs_S_NSSAI(omit, '01'O, '01'O, omit), cs_S_NSSAI(omit, '01'O, '02'O, omit), cs_S_NSSAI(omit, '01'O, '03'O, omit)} }; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var template (value) NG_NAS_DL_Pdu_Type v_MToSend; var charstring v_AT_Response; var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)); var charstring v_NSSAI := f_ConvertS_NSSAI_ForATCommand (cs_S_NSSAI(omit, '01'O, '01'O, omit)); var template (value) NSSAI v_ConfiguredNSSAI := { iei := '31'O, iel := '02'O, s_nssai := {cs_S_NSSAI(omit, '01'O, '01'O, omit)} }; var template (value) NSSAI v_PendingNSSAI := { iei := '39'O, iel := '02'O, s_nssai := {cs_S_NSSAI(omit, '01'O, '01'O, omit)} }; var template (value) GPRS_Timer3 v_T3512 := cs_GPRS_Timer2_3_IEI('5E'O, '101'B, '00001'B); timer t_Wait := 5.0; //@siclog "Step 1" siclog@ f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 2" siclog@ //The UE transmits a REGISTRATION REQUEST message including NSSAA bit v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure,-,-,-,-,cdr_NG_GMM_Cap_NSSAA)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); }else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); } //@siclog "Step 3-11" siclog@ f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), TESTMode_OFF); //@siclog "Step 12" siclog@ //The SS transmits a REGISTRATION ACCEPT message including Rejected S-NSSAI=1 v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, v_AllowedNSSAI, //@sic R5-222008 sic@ v_RejectedNSSAI, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, '1'B); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Step 13" siclog@ f_NR5GC_Registration_Complete(v_NGC_NASCellA); //@siclog "Step 14a" siclog@ if (pc_noOf_PDUsSameConnection > 0 ){ f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, -, tsc_NR_RbId_SRB1); } //@siclog "Step 15" siclog@ //Check: Is S-NSSAI=1 in the Rejected NSSAI list with cause "S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization" associated with current PLMN using AT/MMI command (+C5GNSSAIRDP)? v_AT_Response := f_UT_ReadRejectedNSSAI(UT, f_ConvertPLMNtoString(v_PLMN)); if (match (v_AT_Response, f_Convert_ListOfRejectedNSSAI_ForATCommand(v_RejectedNSSAI))) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 15"); } //@siclog "Step 16" siclog@ //Cause the UE to request establishment of PDU session with S-NSSAI=1 f_UT_RequestPDUSession(UT, 1, -, -, -, v_NSSAI); //@siclog "Step 17" siclog@ //Check: Does the UE transmit a PDU SESSION ESTABLISHMENT REQUEST message within 5 seconds? t_Wait.start; //@sic R5-222008 sic@ alt{ []SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) { t_Wait.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 17"); } []t_Wait.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 17"); } } //@siclog "Step 18" siclog@ //Switch off procedure in RRC_CONNECTED specified in TS 38.508-1 subclause 4.9.6.3 is performed. f_NR_UE_DeRegisterOnSwitchOff(v_NGC_NASCellA, STATE_CONNECTED_3A); //@siclog "Step 19" siclog@ f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@sic R5-222008 sic@ //@siclog "Step 20" siclog@ f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 21" siclog@ //Check: Is S-NSSAI=1 in the Rejected NSSAI list with cause "S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization" associated with current PLMN using AT/MMI command (+C5GNSSAIRDP)? v_AT_Response := f_UT_ReadRejectedNSSAI(UT, f_ConvertPLMNtoString(v_PLMN)); if (match (v_AT_Response, f_Convert_ListOfRejectedNSSAI_ForATCommand(v_RejectedNSSAI))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 21"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 21"); } //@siclog "Step 22" siclog@ Prose CR required //The SS configures NGC Cell A as the Serving cell f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@siclog "Step 23" siclog@ Prose CR required //The UE transmits a REGISTRATION REQUEST message including NSSAA bit v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure,-,-,-,-,cdr_NG_GMM_Cap_NSSAA)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); }else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 23"); } //@siclog "Step 24-32" siclog@ Prose CR required f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), TESTMode_OFF); //@siclog "Step 33" siclog@ v_RejectedNSSAI := { iei := '11'O, iel := '05'O, rejectS_NSSAI := {cs_RejectedS_NSSAI('0001'B, '0010'B, '02'O, 'FFFFFF'O)}}; //@sic R5-222008 sic@ v_AllowedNSSAI:= { iei := '15'O, iel := '04'O, s_nssai := {cs_S_NSSAI(omit, '01'O, '01'O, omit), cs_S_NSSAI(omit, '01'O, '02'O, omit)} }; v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, v_AllowedNSSAI, v_RejectedNSSAI, v_ConfiguredNSSAI, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, '1'B, -, -, -, -, -, -, v_PendingNSSAI); //@sic R5-222008 sic@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Step 34" siclog@ f_NR5GC_Registration_Complete(v_NGC_NASCellA); //@siclog "Step 35a1" siclog@ if (pc_noOf_PDUsSameConnection > 0 ){ f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, -, tsc_NR_RbId_SRB1); } //@siclog "Step 36" siclog@ //Check: Is S-NSSAI=2 in the Rejected NSSAI list with cause "S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization" associated with current PLMN using AT/MMI command (+C5GNSSAIRDP)? v_AT_Response := f_UT_ReadRejectedNSSAI(UT, f_ConvertPLMNtoString(v_PLMN)); if (match (v_AT_Response, f_Convert_ListOfRejectedNSSAI_ForATCommand(v_RejectedNSSAI))) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 36"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 36"); } //@siclog "Step 37" siclog@ f_NR_RRCRelease(v_NGC_NASCellA); if(pc_USIM_Removal){ //@siclog "Step 38a1" siclog@ f_UT_USIM_Remove (UT); //@siclog "Step 38a2" siclog@ f_NR_UE_DeRegisterOnSwitchOff(v_NGC_NASCellA, STATE_IDLE_1A); //@siclog "Step 38a3" siclog@ f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@sic R5-222008 sic@ //@siclog "Step 38a4" siclog@ f_UT_SwitchOnUE(UT, true); //@siclog "Step 38a5" siclog@ //Check: Using AT/MMI command (+C5GNSSAIRDP) verify that S-NSSAI=2 is not in the Rejected NSSAI list associated with current PLMN v_AT_Response := f_UT_ReadRejectedNSSAI(UT, f_ConvertPLMNtoString(v_PLMN)); if (match (v_AT_Response, f_Convert_ListOfRejectedNSSAI_ForATCommand(v_RejectedNSSAI))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 38a5"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 38a5"); } //@siclog "Step 38a6" siclog@ //The SS configures NGC Cell A as the "Serving cell". f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); } else { //@siclog "Step 38b1" siclog@ f_NR_UE_DeRegisterOnSwitchOff(v_NGC_NASCellA, STATE_IDLE_1A); //@sic R5-222008 sic@ //@siclog "Step 38b2" siclog@ f_NR5GC_SwitchOnAndResetIMS(); //@sic R5-222008 sic@ } //@siclog "Step 39" siclog@ //The UE transmits a REGISTRATION REQUEST message including NSSAA bit v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure,-,-,-,-,cdr_NG_GMM_Cap_NSSAA)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); }else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 39"); } //@siclog "Step 40-48" siclog@ Prose CR required f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), TESTMode_OFF); //@siclog "Step 49" siclog@ //The SS transmits a REGISTRATION ACCEPT message not in including Pending S-NSSAI=1 v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, -, -, v_ConfiguredNSSAI, -, -, -, -, -, -, -, -, v_T3512, -, -, -, -, -, -, -, -, -, -, -, -, -, -, '0'B); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Step 50" siclog@ f_NR5GC_Registration_Complete(v_NGC_NASCellA); //@siclog "Step 51a1" siclog@ if (pc_noOf_PDUsSameConnection > 0 ){ f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, -, tsc_NR_RbId_SRB1); } //@siclog "Step 52" siclog@ f_UT_RequestPDUSession(UT, 1, -, -, -, v_NSSAI); //@siclog "Step 53" siclog@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; //@sic R5-222008 sic@ if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, ?,?,-,-, -,-,-,-, -,-,-,-, cs_S_NSSAI(omit, '01'O, '01'O, omit) )) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed"); } v_GSMMsg := v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg; v_GSM_MobilityInfo_s.SessionId := v_GSMMsg.pdu_Session_Establishment_Request.pduSessionId.sessionId; //@siclog "Step 54" siclog@ v_MToSend := f_Get_NG_PDUSessionEstablishmentReject(v_GSM_MobilityInfo_s.SessionId , v_PTI, cs_GMM_GSM_Cause (omit, '00011010'B)); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, - , -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MToSend))); //@sic R5-222008 sic@ //@siclog "Step 55" siclog@ f_NR_RRCRelease(v_NGC_NASCellA); //@sic R5-222008 sic@ } //End of fl_TC_9_1_10_3_Body

After Change:

function fl_TC_9_1_10_3_TestBody() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_CellsOnDifferentPLMN); var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NR_SRB_COMMON_IND v_ReceivedAsp; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var NG_NAS_UL_Message_Type v_GSMMsg; var GSM_MobilityInfo_Type v_GSM_MobilityInfo_s; var ProcedureTransactionIdentifier v_PTI := '00'O; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (v_NGC_NASCellA); var template (value) NG_MobileIdentity v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), v_GutiParams); var TrackingAreaCode v_TAC := f_NR_CellInfo_GetTAC (v_NGC_NASCellA); var template (value) NG_TrackingAreaIdList v_TAIList := cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), {bit2oct(v_TAC)}); var template (value) RejectedNSSAI v_RejectedNSSAI := { iei := '11'O, iel := '05'O, rejectS_NSSAI := {cs_RejectedS_NSSAI('0100'B, '0010'B, '01'O, 'FFFFFF'O)}}; //@sic R5-222008 sic@ var template (value) NSSAI v_AllowedNSSAI:= { iei := '15'O, iel := '06'O, s_nssai := {cs_S_NSSAI(omit, '01'O, '02'O, omit), cs_S_NSSAI(omit, '01'O, '03'O, omit), cs_S_NSSAI(omit, '01'O, '04'O, omit)} }; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var template (value) NG_NAS_DL_Pdu_Type v_MToSend; var charstring v_AT_Response; var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)); var charstring v_NSSAI := f_ConvertS_NSSAI_ForATCommand (cs_S_NSSAI(omit, '01'O, '01'O, omit)); var template (value) NSSAI v_ConfiguredNSSAI := { iei := '31'O, iel := '02'O, s_nssai := {cs_S_NSSAI(omit, '01'O, '01'O, omit)} }; var template (value) NSSAI v_PendingNSSAI := { iei := '39'O, iel := '02'O, s_nssai := {cs_S_NSSAI(omit, '01'O, '01'O, omit)} }; var template (value) GPRS_Timer3 v_T3512 := cs_GPRS_Timer2_3_IEI('5E'O, '101'B, '00001'B); timer t_Wait := 5.0; var charstring v_expected_Response; var charstring v_expected_Response2; //@siclog "Step 1" siclog@ f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 2" siclog@ //The UE transmits a REGISTRATION REQUEST message including NSSAA bit v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure,-,-,-,-,cdr_NG_GMM_Cap_NSSAA)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); }else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); } //@siclog "Step 3-11" siclog@ f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), TESTMode_OFF); //@siclog "Step 12" siclog@ //The SS transmits a REGISTRATION ACCEPT message including Rejected S-NSSAI=1 v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, v_AllowedNSSAI, //@sic R5-222008 sic@ v_RejectedNSSAI, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, '1'B); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Step 13" siclog@ f_NR5GC_Registration_Complete(v_NGC_NASCellA); //@siclog "Step 14a" siclog@ if (pc_noOf_PDUsSameConnection > 0 ){ f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, -, tsc_NR_RbId_SRB1); } //@siclog "Step 15" siclog@ //Check: Is S-NSSAI=1 in the Rejected NSSAI list with cause "S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization" associated with current PLMN using AT/MMI command (+C5GNSSAIRDP)? v_AT_Response := f_UT_ReadRejectedNSSAI(UT, f_ConvertPLMNtoString(v_PLMN)); v_expected_Response := f_Convert_ListOfRejectedNSSAI_ForATCommand(v_RejectedNSSAI); //contains sd value FFFFFF v_expected_Response2 := substr(v_expected_Response, 0,2) & substr(v_expected_Response,9,2); //does not contain sd value if (match (v_AT_Response, v_expected_Response)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); } else if (match (v_AT_Response, v_expected_Response2)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); }else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 15"); } //@siclog "Step 16" siclog@ //Cause the UE to request establishment of PDU session with S-NSSAI=1 f_UT_RequestPDUSession(UT, 1, -, -, -, v_NSSAI); //@siclog "Step 17" siclog@ //Check: Does the UE transmit a PDU SESSION ESTABLISHMENT REQUEST message within 5 seconds? t_Wait.start; //@sic R5-222008 sic@ alt{ []SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) { t_Wait.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 17"); } []t_Wait.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 17"); } } //@siclog "Step 18" siclog@ //Switch off procedure in RRC_CONNECTED specified in TS 38.508-1 subclause 4.9.6.3 is performed. f_NR_UE_DeRegisterOnSwitchOff(v_NGC_NASCellA, STATE_CONNECTED_3A, tsc_NR_RbId_SRB1); //@siclog "Step 19" siclog@ f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@sic R5-222008 sic@ //@siclog "Step 20" siclog@ f_NR5GC_SwitchOnAndResetIMS(); //@siclog "Step 21" siclog@ //Check: Is S-NSSAI=1 in the Rejected NSSAI list with cause "S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization" associated with current PLMN using AT/MMI command (+C5GNSSAIRDP)? v_AT_Response := f_UT_ReadRejectedNSSAI(UT, f_ConvertPLMNtoString(v_PLMN)); if (match (v_AT_Response, f_Convert_ListOfRejectedNSSAI_ForATCommand(v_RejectedNSSAI))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 21"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 21"); } //@siclog "Step 22" siclog@ Prose CR required //The SS configures NGC Cell A as the Serving cell f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@siclog "Step 23" siclog@ Prose CR required //The UE transmits a REGISTRATION REQUEST message including NSSAA bit v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure,-,-,-,-,cdr_NG_GMM_Cap_NSSAA)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); }else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 23"); } //@siclog "Step 24-32" siclog@ Prose CR required f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), TESTMode_OFF); //@siclog "Step 33" siclog@ v_RejectedNSSAI := { iei := '11'O, iel := '05'O, rejectS_NSSAI := {cs_RejectedS_NSSAI('0100'B, '0010'B, '02'O, 'FFFFFF'O)}}; //@sic R5-222008 sic@ v_AllowedNSSAI:= { iei := '15'O, iel := '04'O, s_nssai := {cs_S_NSSAI(omit, '01'O, '03'O, omit), cs_S_NSSAI(omit, '01'O, '04'O, omit)} }; v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, v_AllowedNSSAI, v_RejectedNSSAI, v_ConfiguredNSSAI, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, '1'B, -, -, -, -, -, -, v_PendingNSSAI); //@sic R5-222008 sic@ SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Step 34" siclog@ f_NR5GC_Registration_Complete(v_NGC_NASCellA); //@siclog "Step 35a1" siclog@ if (pc_noOf_PDUsSameConnection > 0 ){ f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, -, tsc_NR_RbId_SRB1); } //@siclog "Step 36" siclog@ //Check: Is S-NSSAI=2 in the Rejected NSSAI list with cause "S-NSSAI not available due to the failed or revoked network slice-specific authentication and authorization" associated with current PLMN using AT/MMI command (+C5GNSSAIRDP)? v_AT_Response := f_UT_ReadRejectedNSSAI(UT, f_ConvertPLMNtoString(v_PLMN)); v_expected_Response := f_Convert_ListOfRejectedNSSAI_ForATCommand(v_RejectedNSSAI); //contains sd value FFFFFF v_expected_Response2 := substr(v_expected_Response, 0,2) & substr(v_expected_Response,9,2); //does not contain sd value if (match (v_AT_Response, v_expected_Response)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 36"); } else if (match (v_AT_Response, v_expected_Response2)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 36"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 36"); } //@siclog "Step 37" siclog@ f_NR_RRCRelease(v_NGC_NASCellA); if(pc_USIM_Removal){ //@siclog "Step 38a1" siclog@ f_UT_USIM_Remove (UT); //@siclog "Step 38a2" siclog@ f_NR_UE_DeRegisterOnSwitchOff(v_NGC_NASCellA, STATE_IDLE_1A, -, true); //@siclog "Step 38a3" siclog@ f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@sic R5-222008 sic@ //@siclog "Step 38a4" siclog@ f_UT_SwitchOnUE(UT, true); //@siclog "Step 38a5" siclog@ //Check: Using AT/MMI command (+C5GNSSAIRDP) verify that S-NSSAI=2 is not in the Rejected NSSAI list associated with current PLMN v_AT_Response := f_UT_ReadRejectedNSSAI(UT, f_ConvertPLMNtoString(v_PLMN)); if (match (v_AT_Response, f_Convert_ListOfRejectedNSSAI_ForATCommand(v_RejectedNSSAI))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 38a5"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 38a5"); } //@siclog "Step 38a6" siclog@ //The SS configures NGC Cell A as the "Serving cell". f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); } else { //@siclog "Step 38b1" siclog@ f_NR_UE_DeRegisterOnSwitchOff(v_NGC_NASCellA, STATE_IDLE_1A); //@sic R5-222008 sic@ //@siclog "Step 38b2" siclog@ f_NR5GC_SwitchOnAndResetIMS(); //@sic R5-222008 sic@ } //@siclog "Step 39" siclog@ //The UE transmits a REGISTRATION REQUEST message including NSSAA bit v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure,-,-,-,-,cdr_NG_GMM_Cap_NSSAA)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); }else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 39"); } //@siclog "Step 40-48" siclog@ Prose CR required f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, v_NGC_NASCellA, v_ReceivedMsg, f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)), TESTMode_OFF); //@siclog "Step 49" siclog@ //The SS transmits a REGISTRATION ACCEPT message not in including Pending S-NSSAI=1 v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, -, -, v_ConfiguredNSSAI, -, -, -, -, -, -, -, -, v_T3512, -, -, -, -, -, -, -, -, -, -, -, -, -, -, '0'B); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Step 50" siclog@ f_NR5GC_Registration_Complete(v_NGC_NASCellA); //@siclog "Step 51a1" siclog@ if (pc_noOf_PDUsSameConnection > 0 ){ f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, -, tsc_NR_RbId_SRB1); } //@siclog "Step 52" siclog@ f_UT_RequestPDUSession(UT, 1, -, -, -, v_NSSAI); //@siclog "Step 53" siclog@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; //@sic R5-222008 sic@ if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, ?,?,-,-, -,-,-,-, -,-,-,-, cs_S_NSSAI('22'O, '01'O, '01'O, omit) )) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed"); } v_GSMMsg := v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg; v_GSM_MobilityInfo_s.SessionId := v_GSMMsg.pdu_Session_Establishment_Request.pduSessionId.sessionId; v_PTI := v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request.procedureTransactionIdentifier; //@siclog "Step 54" siclog@ v_MToSend := f_Get_NG_PDUSessionEstablishmentReject(v_GSM_MobilityInfo_s.SessionId , v_PTI, cs_GMM_GSM_Cause (omit, '00011010'B)); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MToSend))); //@sic R5-222008 sic@ //@siclog "Step 55" siclog@ f_NR_RRCRelease(v_NGC_NASCellA); //@sic R5-222008 sic@ } //End of fl_TC_9_1_10_3_Body

- Branches executed

On Anritsu with Qualcomm UE, the test case was executed on NR band n41 using NR ciphering nea2 and integrity algorithm nia2.

On Anritsu with Samsung UE, the test case was executed on NR band n78 using NR ciphering nea2 and integrity algorithm nia2.

On Keysight with Qualcomm UE, the test case was executed on NR band n41  using NR ciphering nea1 and integrity algorithm nia1.

- Execution Log Files

  - Qualcomm SM8450 + SDX65

The Qualcomm SM8450 + SDX65 5G UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR and Keysight 5G Protocol Conformance Toolset  . The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anrirtsu\Qualcomm\TC_9_1_10_3_LOG.xml

Keysight\Qualcomm\TC_9_1_10_3_LOG.html

- PICS/PIXIT parameter file:

Anrirtsu\ Qualcomm\TC_9_1_10_3_PIXIT.txt

Keysight\Qualcomm\ TC_9_1_10_3_PIXIT.xml

  - Samsung SM-G991B

The Samsung SM-G991B5G UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anrirtsu\Samsung\TC_9_1_10_3_LOG.xml

- PICS/PIXIT parameter file:

Anrirtsu\ Samsung\TC_9_1_10_3_PIXIT.txt

- References

| [1] | R5s220536 - NR5GC FR1 : Supporting information for addition of NR5GC R16 eNS test case 9.1.10.3 |
| --- | --- |