3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220548

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2487 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC ENDC test case 8.2.2.8.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test , 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TTCN Function f_ENDC_GetKeysActivateSecurity_KeNB() After calling f_NR_InitAS_KeyChaining_KeNB(), following variables (KRRCenc, KUPenc, KRRCint) will be updated in AS_Ciphering_S. Hence it is only required to update v_SecurityParams.AS_Ciphering with the new generated parameters. Not the entire AS_Ciphering_S. overwring AS_Ciphering with AS_Ciphering_S will result in ActiTimeList getting updated with wrong parameters (Including SRBs), which is not required. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Function f_ENDC_GetKeysActivateSecurity_KeNB() is modified to update AS_Ciphering only with KRRCenc, KUPenc, KRRCint from AS_Ciphering_S. Also use AS_Ciphering to activate security rather than AS_Ciphering_S. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.8.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_ENDC_GetKeysActivateSecurity_KeNB () |
| --- | --- |
| Reason for change | In TTCN Function f_ENDC_GetKeysActivateSecurity_KeNB() After calling f_NR_InitAS_KeyChaining_KeNB(), following variables (KRRCenc, KUPenc, KRRCint) will be updated in AS_Ciphering_S. Hence it is only required to update v_SecurityParams.AS_Ciphering with the new generated parameters. Not the entire AS_Ciphering_S. overwring AS_Ciphering with AS_Ciphering_S will result in ActiTimeList getting updated with wrong parameters (Including SRBs), which is not required. |
| Summary of change | Function f_ENDC_GetKeysActivateSecurity_KeNB() is modified to update AS_Ciphering only with KRRCenc, KUPenc, KRRCint from AS_Ciphering_S. Also use AS_Ciphering to activate security rather than AS_Ciphering_S. |
| TTCN module | NR_CommonFunctions_ENDC.ttcn |
| MCC160 Comment | Only need to use AS_Ciphering to activate security – the rest of AS_Ciphering and AS_Ciphering_S should be the same |

Before Change

function f_ENDC_GetKeysActivateSecurity_KeNB(NR_CellId_Type p_NR_CellId, EUTRA_NR_CO_ORD_PORT p_Port) runs on NR_BASE_PTC { var EUTRA_NR_CoOrd_Security_Type v_SecurityRx; var NR_SecurityParams_Type v_SecurityParams; v_SecurityParams := f_NR_Security_Get (); // get security from EUTRA v_SecurityRx := f_EUTRA_NR_WaitForCoOrd_Security (p_Port); v_SecurityParams.SK_Counter := v_SecurityRx.SK_Counter; v_SecurityParams.S_KGNB := v_SecurityRx.Key; //contains KeNB // get latest security keys v_SecurityParams := f_NR_InitAS_KeyChaining_KeNB (v_SecurityParams); // Make sure all keys are updated @sic R5w220012 sic@ v_SecurityParams.AS_Ciphering := v_SecurityParams.AS_Ciphering_S; v_SecurityParams.AS_Integrity := v_SecurityParams.AS_Integrity_S; f_NR_Security_Set(v_SecurityParams); // Configure PDCP security v_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetCurrent(p_NR_CellId, false); // @sic R5s210277 sic@ f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_NR_CellId, v_SecurityParams.AS_Integrity_S, v_SecurityParams.AS_Ciphering_S); }

After Change

function f_ENDC_GetKeysActivateSecurity_KeNB(NR_CellId_Type p_NR_CellId, EUTRA_NR_CO_ORD_PORT p_Port) runs on NR_BASE_PTC { var EUTRA_NR_CoOrd_Security_Type v_SecurityRx; var NR_SecurityParams_Type v_SecurityParams; v_SecurityParams := f_NR_Security_Get (); // get security from EUTRA v_SecurityRx := f_EUTRA_NR_WaitForCoOrd_Security (p_Port); v_SecurityParams.SK_Counter := v_SecurityRx.SK_Counter; v_SecurityParams.S_KGNB := v_SecurityRx.Key; //contains KeNB // get latest security keys v_SecurityParams := f_NR_InitAS_KeyChaining_KeNB (v_SecurityParams); // Make sure all keys are updated @sic R5w220012 sic@ v_SecurityParams.AS_Ciphering.KRRCenc := v_SecurityParams.AS_Ciphering_S.KRRCenc; v_SecurityParams.AS_Ciphering.KUPenc := v_SecurityParams.AS_Ciphering_S.KUPenc; v_SecurityParams.AS_Integrity.KRRCint := v_SecurityParams.AS_Integrity_S.KRRCint; //@82281 f_NR_Security_Set(v_SecurityParams); // Configure PDCP security v_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetCurrent(p_NR_CellId, false); // @sic R5s210277 sic@ f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_NR_CellId, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering); //@82281 }

#### MCC160 Implementation:

function f_ENDC_GetKeysActivateSecurity_KeNB(NR_CellId_Type p_NR_CellId,

EUTRA_NR_CO_ORD_PORT p_Port) runs on NR_BASE_PTC

{

var EUTRA_NR_CoOrd_Security_Type v_SecurityRx;

var NR_SecurityParams_Type v_SecurityParams;

v_SecurityParams := f_NR_Security_Get ();

// get security from EUTRA

v_SecurityRx := f_EUTRA_NR_WaitForCoOrd_Security (p_Port);

v_SecurityParams.SK_Counter := v_SecurityRx.SK_Counter;

v_SecurityParams.S_KGNB := v_SecurityRx.Key;  //contains KeNB

// get latest security keys

v_SecurityParams := f_NR_InitAS_KeyChaining_KeNB (v_SecurityParams);

// Make sure all keys are updated @sic R5w220012 sic@

v_SecurityParams.AS_Ciphering := v_SecurityParams.AS_Ciphering_S;

v_SecurityParams.AS_Integrity := v_SecurityParams.AS_Integrity_S;

f_NR_Security_Set(v_SecurityParams);

// Configure PDCP security

v_SecurityParams.AS_Ciphering.ActTimeList := f_NR_RRC_CipherActTime_GetCurrent(p_NR_CellId, false); // @sic R5s210277 sic@

f_NR_SS_RRC_EnableIntProt_CiphULandDL(p_NR_CellId,

v_SecurityParams.AS_Integrity,

v_SecurityParams.AS_Ciphering);

}