3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220550

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2489 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for SDAP test case 7.1.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Following implementation of R5-221435, the QoS rule configuration is still inconstent. The current configuration has: QoS rule #3, DQR 0, PF list #1 (match-all) QoS rule #4, DQR 1, PF list#3 This violates 24.501 6.2.5.1.1.2 since a rule which is not the default rule should not use match-all filter type |  |  |  |  |  |  |  |  |
| Summary of change: |  | Revert to using QoS rule#1 instead of #3, since this has DQR field as ‘1’ and is associated with a match-all packet filter. Then modify the DQR field of QoS rule#4 to ‘0’ to follow the restriction that onle one configured QoS rule can be set as the default. Note : An assoociated pose CR on 38.523-1 will be raised at RAN5#95. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.4.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_4_1_NR5GC() |
| --- | --- |
| Reason for change | Following implementation of R5-221435, the QoS rule configuration is still inconstent. The current configuration has: QoS rule #3, DQR 0, PF list #1 (match-all) QoS rule #4, DQR 1, PF list#3 This violates 24.501 6.2.5.1.1.2 since a rule which is not the default rule should not use match-all filter type |
| Summary of change | Revert to using QoS rule#1 instead of #3, since this has DQR field as ‘1’ and is associated with a match-all packet filter. Then modify the DQR field of QoS rule#4 to ‘0’ to follow the restriction that onle one configured QoS rule can be set as the default. Note : An assoociated pose CR on 38.523-1 will be raised at RAN5#95. |
| TTCN module | SDAP_NR5GC.ttcn |
| MCC160 Comment | Accepted, conditional to prose CR agreement. |

Before Change

function f_TC_7_1_4_1_NR5GC() runs on NR5GC_PTC {//SDAP Data Transfer and PDU Header Handling UL/DL ……………….. v_DRB_ToAddModList := {cs_38508_NRDRB_ToAddMod(cs_SDAP_Config_Sdap(v_PduSessionId, present_, present_, false, {1,2}), // @sic R5s191119 sic@ v_SecondDRB, cs_38508_PDCP_Config(ms300)), cs_38508_NRDRB_ToAddMod(cs_SDAP_Config_Sdap(v_PduSessionId, present_, present_, true, {5,6}), // @sic R5s191119 sic@ v_FirstDRB, cs_38508_PDCP_Config(ms300))}; v_QoS_Rules := cs_QoS_Rules ({f_BuildDefaultQoSRule("3"), f_BuildQoSRuleRemoteAccess("4"), f_BuildQoSRuleRemoteAccess("5"), f_BuildQoSRuleRemoteAccess("6")}); // @sic R5-215714, R5s211487, R5-221435 sic@ v_QoS_Rules.listofQoSRules[0].packetFilterList := cs_PacketFilterList_1MatchAll; //@sic R5s211487, R5-221435 sic@, Acc. Table 7.1.4.1.3.3-7 v_QoSFlowDescr := cs_QoSFlowDescr ({cs_QoS_Flow1, cs_QoS_Flow2, cs_QoS_Flow3, cs_QoS_Flow4}); // Start IP handling in order for the TTCN to handle IPv6 address allocation, // then stop it before the test body, so that TTCN can route the IP data v_PDN_Index := f_NR5GC_GetPdnIndex(v_GSM_MobilityInfo_Type); v_QosFlowId := cs_IP_QosFlowId(oct2int(v_GSM_MobilityInfo_Type.SessionId), 1); //IP handling will go through QFI 1, the QFI with the match-all filter @sic R5s200564, R5s201505, R5s211487 sic@ ……………. }

After Change

function f_TC_7_1_4_1_NR5GC() runs on NR5GC_PTC {//SDAP Data Transfer and PDU Header Handling UL/DL ……………….. v_DRB_ToAddModList := {cs_38508_NRDRB_ToAddMod(cs_SDAP_Config_Sdap(v_PduSessionId, present_, present_, false, {1,2}), // @sic R5s191119 sic@ v_SecondDRB, cs_38508_PDCP_Config(ms300)), cs_38508_NRDRB_ToAddMod(cs_SDAP_Config_Sdap(v_PduSessionId, present_, present_, true, {5,6}), // @sic R5s191119 sic@ v_FirstDRB, cs_38508_PDCP_Config(ms300))}; v_QoS_Rules := cs_QoS_Rules ({f_BuildDefaultQoSRule("1"), f_BuildQoSRuleRemoteAccess("4"), f_BuildQoSRuleRemoteAccess("5"), f_BuildQoSRuleRemoteAccess("6")}); // @sic R5-215714, R5s211487, R5-221435 sic@ //REMOVED v_QoS_Rules.listofQoSRules[0].packetFilterList := cs_PacketFilterList_1MatchAll; //@sic R5s211487, R5-221435 sic@, Acc. Table 7.1.4.1.3.3-7 v_QoS_Rules.listofQoSRules[1].dqrBit := '0'B; v_QoSFlowDescr := cs_QoSFlowDescr ({cs_QoS_Flow1, cs_QoS_Flow2, cs_QoS_Flow3, cs_QoS_Flow4}); // Start IP handling in order for the TTCN to handle IPv6 address allocation, // then stop it before the test body, so that TTCN can route the IP data v_PDN_Index := f_NR5GC_GetPdnIndex(v_GSM_MobilityInfo_Type); v_QosFlowId := cs_IP_QosFlowId(oct2int(v_GSM_MobilityInfo_Type.SessionId), 1); //IP handling will go through QFI 1, the QFI with the match-all filter @sic R5s200564, R5s201505, R5s211487 sic@ ……………. }