3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220557

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4670 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE PSM testcases 9.2.1.1.7c, 9.2.3.1.5b and 9.2.4.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI12_Test, MTCe-UEPCOP-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to TS 24.301 section 5.5.1.2.2, “If the UE supports PSM and requests the use of PSM, then the UE shall include the T3324 value IE with a requested timer value in the ATTACH REQUEST message. When the UE includes the T3324 value IE and the UE indicates support for extended periodic timer value in the MS network feature support IE, it may also include the T3412 extended value IE to request a particular T3412 value to be allocated.” And according to TS 24.301 section 5.5.3.2.2, “If the UE supports PSM and requests the use of PSM, the UE shall include the T3324 value IE with a requested timer value in the TRACKING AREA UPDATE REQUEST message. When the UE includes the T3324 value IE and the UE indicates support for extended periodic timer value in the MS network feature support IE, it may also include the T3412 extended value IE to request a particular T3412 value to be allocated.” Currently above scenarios are taken care only by testcase 9.2.3.1.1a, but not other PSM testcases |  |  |  |  |  |  |  |  |
| Summary of change: |  | Allocate timer T3412 value in Attach Accept and TrackingAreaUpdate Accept messages when UE requests it. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Testcases 9.2.1.1.7c, 9.2.3.1.5b and 9.2.4.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS36.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

2.2	Change 2	8

2.3	Change 3	12

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk12 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | fl_TC_9_2_1_1_7c_Body |
| --- | --- |
| Reason for change | According to TS 24.301 section 5.5.1.2.2, “If the UE supports PSM and requests the use of PSM, then the UE shall include the T3324 value IE with a requested timer value in the ATTACH REQUEST message. When the UE includes the T3324 value IE and the UE indicates support for extended periodic timer value in the MS network feature support IE, it may also include the T3412 extended value IE to request a particular T3412 value to be allocated.” Currently above scenario is taken care only by testcase 9.2.3.1.1a, but not 9.2.1.1.7c |
| Summary of change | Allocate timer T3412 value in Attach Accept message when UE requests it. |
| TTCN module | \LTE_A_R12\9_2\NAS_Attach_EOnly_R12.ttcn |
| MCC160 Comment | Accepted conditional to prose CR agreement at RAN5#95-e |

Before change:

function fl_TC_9_2_1_1_7c_Body() runs on EUTRA_PTC

{

var GutiParameters_Type v_GutiParams_CellA := f_EUTRA_CellInfo_GetGuti(eutra_CellA);

var NAS_PlmnId v_PLMN := f_Asn2Nas_PlmnId(v_GutiParams_CellA.PLMN_Identity);

var template AdditionalUpdateType v_AdditionalUpdateType := f_GetAdditionalUpdateType(EPS_Only);

var NAS_AttDetValue_Type v_AttachType :=f_GetEAttachType(EPS_Only);

var NAS_MSG_Indication_Type v_NasInd;

var EPS_BearerIdentity v_EPS_Bearer_Id :=  f_EUTRA_InitialRegistrationEpsBearerId(firstPDN);

var DRB_Identity v_DRB_Id := f_EUTRA_NB_EpsBearerAssociatedDRB(v_EPS_Bearer_Id);

var APNandPCOs_Type v_APNandPCOs;

var template (omit) ProtocolConfigOptions v_PcoFromUE := omit;

var template (value) ProtocolConfigOptions v_PcoToUE;

var SRB_COMMON_IND v_ReceivedAsp;

var ProcedureTransactionIdentifier v_EPS_TI := '00'O;

var NAS_Lac v_LacA := f_EUTRA_CellInfo_GetLocationAreaCode(eutra_CellA);

var template (omit) LocAreaId v_LAI := f_GetLAI(v_PLMN, v_LacA, EPS_Only);

//@siclog "Step 1" siclog@

//The UE is switched on

f_EUTRA_SwitchOnUEandStartIP(eutra_CellA);

//@siclog "Step 2" siclog@

//The UE transmits an RRCConnectionRequest message.

f_EUTRA_RRC_ConnectionRequest_Def(eutra_CellA); // @sic R5s180257 sic@

//@siclog "Step 3" siclog@

//The SS transmits an RRCConnectionSetup message

f_EUTRA_RRC_ConnectionSetup_Def(eutra_CellA);

//@siclog "Step 4" siclog@

//Check: Does the UE transmit an ATTACH REQUEST message including the T3324 IE set to two minutes?

v_NasInd := f_EUTRA_RRCConnectionSetupComplete_Def(eutra_CellA,

cr_508_RRCConnectionSetupComplete(tsc_RRC_TI_Def, ?, -, f_EUTRA_CellInfo_GetSelectedPlmnIndex(eutra_CellA)),

cr_NAS_IndicationWithPiggybacking(tsc_SHT_IntegrityProtected,

cdr_ATTACH_REQUEST_WithGuti_T3324(v_AttachType, v_AdditionalUpdateType, ?,

f_GutiParameters2MobileIdentity(omit, v_GutiParams_CellA), //GUTI1

cr_TAI( v_PLMN, tsc_TAC1),

cr_GPRS_Timer2_3('6A'O, tsc_GprsTimerUnit_1min, '00010'B)), //T3324 = 2mins

cr_508_PDN_CONNECTIVITY_REQUEST(tsc_PdnRequest_InitialAttach, ?)));

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 4");

v_APNandPCOs := f_EUTRA_InitialRegistration_Step5_13( eutra_CellA, STATE2_IDLEUPDATE, v_NasInd);

v_EPS_TI := v_NasInd.Pdu.PiggybackedPduList[0].Msg.pDN_CONNECTIVITY_REQUEST.procedureTransactionIdentifier; // Use this value in SS responses

// Get the PCO to be used later (TS 36.508 v.8.2.2 Table 4.7.3-6)

if (ispresent(v_APNandPCOs.protocolConfigurationOptions)){

v_PcoFromUE := v_APNandPCOs.protocolConfigurationOptions;

}

v_PcoToUE := f_GetDefaultProtocolConfigOptions(v_PcoFromUE);

//@siclog "Step 12" siclog@

//SS responds with ATTACH ACCEPT message including an Extended DRX parameters IE. The ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message is piggybacked in ATTACH ACCEPT message

f_EUTRA_RRCConnectionReconfiguration_SRB2_1AM_DRB_Est(eutra_CellA,

tsc_RRC_TI_Def,

cs_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered,

cs_508_ATTACH_ACCEPT( v_AttachType,

cs_TrackingAreaIdListDef_lv,

f_GutiParameters2MobileIdentity(tsc_IEI_Guti, v_GutiParams_CellA),

v_LAI,

omit,

omit,

f_GetAdditionalUpdateResult(v_NasInd.Pdu.Msg.aTTACH_REQUEST.addUpdateType,Combined),

-,

-,

-,

cs_GPRS_Timer2_3_IEI('6A'O, tsc_GprsTimerUnit_1min, '00010'B)),

cs_508_ActivateDefEpsBearerContextRequest(v_EPS_Bearer_Id,

v_EPS_TI,

v_APNandPCOs.accessPointName,

f_EUTRA_GetPDNAddress(f_CheckPCOforIPallocationViaNas(v_PcoToUE)),

-,

v_PcoToUE)),

v_DRB_Id);

//@siclog "Step 13" siclog@

//Check: Does the UE transmit an ATTACH COMPLETE message including an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message?

SRB.receive(car_SRB2_NasPdu_IND(eutra_CellA,

cr_NAS_IndicationWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered,

cr_508_ATTACH_COMPLETE,

cr_508_ActivateDefEpsBearerContextAccept(v_EPS_Bearer_Id))))  -> value v_ReceivedAsp;

// @siclog "13A1a1-13A3b1" siclog@

//Steps 16a1-18b1 of the generic procedure for UE registration specified in TS36.508 subclause 4.5.2.3 are performed.

v_NasInd := v_ReceivedAsp.Signalling.Nas[0];

f_EUTRA_InitialRegistration_Step16_18b1(eutra_CellA, -, -, v_NasInd);

//The SS shall wait for 1 minute and then execute the following steps before timer T3324 expires

f_Delay(60.0);

//@siclog "Step 14-24" siclog@

//Check: Does the UE accept the paging request?

//Steps 1-10, from Table 4.5.3.3-1 in TS 36.508, are performed

f_EUTRA_RbEst_Def(eutra_CellA);

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 14-24");

}

After change:

function fl_TC_9_2_1_1_7c_Body() runs on EUTRA_PTC

{

var GutiParameters_Type v_GutiParams_CellA := f_EUTRA_CellInfo_GetGuti(eutra_CellA);

var NAS_PlmnId v_PLMN := f_Asn2Nas_PlmnId(v_GutiParams_CellA.PLMN_Identity);

var template AdditionalUpdateType v_AdditionalUpdateType := f_GetAdditionalUpdateType(EPS_Only);

var NAS_AttDetValue_Type v_AttachType :=f_GetEAttachType(EPS_Only);

var NAS_MSG_Indication_Type v_NasInd;

var EPS_BearerIdentity v_EPS_Bearer_Id :=  f_EUTRA_InitialRegistrationEpsBearerId(firstPDN);

var DRB_Identity v_DRB_Id := f_EUTRA_NB_EpsBearerAssociatedDRB(v_EPS_Bearer_Id);

var APNandPCOs_Type v_APNandPCOs;

var template (omit) ProtocolConfigOptions v_PcoFromUE := omit;

var template (value) ProtocolConfigOptions v_PcoToUE;

var SRB_COMMON_IND v_ReceivedAsp;

var ProcedureTransactionIdentifier v_EPS_TI := '00'O;

var NAS_Lac v_LacA := f_EUTRA_CellInfo_GetLocationAreaCode(eutra_CellA);

var template (omit) LocAreaId v_LAI := f_GetLAI(v_PLMN, v_LacA, EPS_Only);

var template (omit) GPRS_Timer3 v_T3412ExtdValue := omit;

//@siclog "Step 1" siclog@

//The UE is switched on

f_EUTRA_SwitchOnUEandStartIP(eutra_CellA);

//@siclog "Step 2" siclog@

//The UE transmits an RRCConnectionRequest message.

f_EUTRA_RRC_ConnectionRequest_Def(eutra_CellA); // @sic R5s180257 sic@

//@siclog "Step 3" siclog@

//The SS transmits an RRCConnectionSetup message

f_EUTRA_RRC_ConnectionSetup_Def(eutra_CellA);

//@siclog "Step 4" siclog@

//Check: Does the UE transmit an ATTACH REQUEST message including the T3324 IE set to two minutes?

v_NasInd := f_EUTRA_RRCConnectionSetupComplete_Def(eutra_CellA,

cr_508_RRCConnectionSetupComplete(tsc_RRC_TI_Def, ?, -, f_EUTRA_CellInfo_GetSelectedPlmnIndex(eutra_CellA)),

cr_NAS_IndicationWithPiggybacking(tsc_SHT_IntegrityProtected,

cdr_ATTACH_REQUEST_WithGuti_T3324(v_AttachType, v_AdditionalUpdateType, ?,

f_GutiParameters2MobileIdentity(omit, v_GutiParams_CellA), //GUTI1

cr_TAI( v_PLMN, tsc_TAC1),

cr_GPRS_Timer2_3('6A'O, tsc_GprsTimerUnit_1min, '00010'B)), //T3324 = 2mins

cr_508_PDN_CONNECTIVITY_REQUEST(tsc_PdnRequest_InitialAttach, ?)));

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 4");

v_APNandPCOs := f_EUTRA_InitialRegistration_Step5_13( eutra_CellA, STATE2_IDLEUPDATE, v_NasInd);

v_EPS_TI := v_NasInd.Pdu.PiggybackedPduList[0].Msg.pDN_CONNECTIVITY_REQUEST.procedureTransactionIdentifier; // Use this value in SS responses

// Get the PCO to be used later (TS 36.508 v.8.2.2 Table 4.7.3-6)

if (ispresent(v_APNandPCOs.protocolConfigurationOptions)){

v_PcoFromUE := v_APNandPCOs.protocolConfigurationOptions;

}

v_PcoToUE := f_GetDefaultProtocolConfigOptions(v_PcoFromUE);

if (isvalue(v_NasInd.Pdu.Msg.aTTACH_REQUEST.t3412ExtdValue)) {

v_T3412ExtdValue := v_NasInd.Pdu.Msg.aTTACH_REQUEST.t3412ExtdValue;

}

//@siclog "Step 12" siclog@

//SS responds with ATTACH ACCEPT message including an Extended DRX parameters IE. The ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message is piggybacked in ATTACH ACCEPT message

f_EUTRA_RRCConnectionReconfiguration_SRB2_1AM_DRB_Est(eutra_CellA,

tsc_RRC_TI_Def,

cs_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered,

cs_508_ATTACH_ACCEPT( v_AttachType,

cs_TrackingAreaIdListDef_lv,

f_GutiParameters2MobileIdentity(tsc_IEI_Guti, v_GutiParams_CellA),

v_LAI,

omit,

omit,

f_GetAdditionalUpdateResult(v_NasInd.Pdu.Msg.aTTACH_REQUEST.addUpdateType,Combined),

-,

-,

v_T3412ExtdValue,

cs_GPRS_Timer2_3_IEI('6A'O, tsc_GprsTimerUnit_1min, '00010'B)),

cs_508_ActivateDefEpsBearerContextRequest(v_EPS_Bearer_Id,

v_EPS_TI,

v_APNandPCOs.accessPointName,

f_EUTRA_GetPDNAddress(f_CheckPCOforIPallocationViaNas(v_PcoToUE)),

-,

v_PcoToUE)),

v_DRB_Id);

//@siclog "Step 13" siclog@

//Check: Does the UE transmit an ATTACH COMPLETE message including an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message?

SRB.receive(car_SRB2_NasPdu_IND(eutra_CellA,

cr_NAS_IndicationWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered,

cr_508_ATTACH_COMPLETE,

cr_508_ActivateDefEpsBearerContextAccept(v_EPS_Bearer_Id))))  -> value v_ReceivedAsp;

// @siclog "13A1a1-13A3b1" siclog@

//Steps 16a1-18b1 of the generic procedure for UE registration specified in TS36.508 subclause 4.5.2.3 are performed.

v_NasInd := v_ReceivedAsp.Signalling.Nas[0];

f_EUTRA_InitialRegistration_Step16_18b1(eutra_CellA, -, -, v_NasInd);

//The SS shall wait for 1 minute and then execute the following steps before timer T3324 expires

f_Delay(60.0);

//@siclog "Step 14-24" siclog@

//Check: Does the UE accept the paging request?

//Steps 1-10, from Table 4.5.3.3-1 in TS 36.508, are performed

f_EUTRA_RbEst_Def(eutra_CellA);

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 14-24");

}

## 2.2 Change 2

| Function name | fl_TC_9_2_3_1_5b_Body |
| --- | --- |
| Reason for change | And according to TS 24.301 section 5.5.3.2.2, “If the UE supports PSM and requests the use of PSM, the UE shall include the T3324 value IE with a requested timer value in the TRACKING AREA UPDATE REQUEST message. When the UE includes the T3324 value IE and the UE indicates support for extended periodic timer value in the MS network feature support IE, it may also include the T3412 extended value IE to request a particular T3412 value to be allocated.” Currently above scenario is taken care only by testcase 9.2.3.1.1a, but not 9.2.3.1.5b |
| Summary of change | Allocate timer T3412 value in TrackingAreaUpdate Accept message when UE requests it. |
| TTCN module | \LTE_A_R12\9_2\NAS_Attach_EOnly_R12.ttcn |
| MCC160 Comment | Accepted conditional to prose CR agreement at RAN5#95-e |

Before change:

function fl_TC_9_2_3_1_5b_Body() runs on EUTRA_PTC

{ //@sic R5s160882 R5-168385 sic@

var GutiParameters_Type v_Guti1_Params := f_EUTRA_CellInfo_GetGuti (eutra_CellA);

var GutiParameters_Type v_Guti2_Params := f_EUTRA_CellInfo_GetGuti(eutra_CellB);

var GutiParameters_Type v_Guti3_Params := f_CreateGutiParamsWithUpdatedSTmsi(v_Guti1_Params);

var template (omit) EPS_BearerContextStatus v_EpsBearerCtxtStatus;

var template (omit) AdditionalUpdateResult v_AdditionalUpdateResult;

var B1_Type v_MS_NetworkFeatureSupport := '1'B ;

var NAS_PlmnId v_PLMN := f_Asn2Nas_PlmnId(v_Guti1_Params.PLMN_Identity);

var NAS_Lac v_LAC := f_EUTRA_CellInfo_GetLocationAreaCode(eutra_CellA);

var template (omit) LocAreaId v_LAI := f_GetLAI(v_PLMN, v_LAC, NORMAL);

var template AdditionalUpdateType v_AdditionalUpdateType := f_GetAdditionalUpdateType ();

var NAS_MSG_Indication_Type v_NasInd;

var float v_Wait4Mins := f_EUTRA_SetTimerToleranceMin(eutra_CellA, nasTimer, 240.0);

var template (value) GPRS_Timer2 v_T3324Value2Mins := cs_GPRS_Timer2_3_IEI('6A'O, tsc_GprsTimerUnit_1min, '00010'B); //T3324 value = 2 mins

var template (value) GPRS_Timer2 v_T3324Value4Mins := cs_GPRS_Timer2_3_IEI('6A'O, tsc_GprsTimerUnit_1min, '00100'B); //T3324 value = 4 mins

var template (value) GPRS_Timer2 v_T3412ValueExt4Mins := cs_GPRS_Timer2_3_IEI('5E'O, tsc_Gprs3TimerUnit_1min, '00100'B); //T3412 value = 4 mins @sic R5s170040 sic@

//@siclog "Step 1" siclog@

// The user requests PSM by MMI or by AT command. The requested value of T3324 is 2 minutes and T3412 extended value is 4 minutes.

f_UT_ConfigurePSM(UT, omit, v_T3412ValueExt4Mins, v_T3324Value2Mins);

//@siclog "Step 2" siclog@

// Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message?

v_NasInd := f_EUTRA_RRC_ConnEst_DefWithNas(eutra_CellA,

tsc_RRC_TI_Def,

cr_EstablishmentCause_Any,

cr_NAS_Indication(tsc_SHT_IntegrityProtected,

cdr_TAU_Request_OldGuti_MSNetworkFeatureSupport_T3324_T3412ExtValue(tsc_EpsUpdate_TaUpdate,

f_EUTRA_SecurityKSIasme_Get(),

-,

v_AdditionalUpdateType,

f_GutiParameters2MobileIdentity(omit, v_Guti1_Params),

cr_TAI_Any,

cr_MS_NetworkFeatureSupport('C'H, v_MS_NetworkFeatureSupport),

cr_GPRS_Timer2_3('6A'O, tsc_GprsTimerUnit_1min, '00010'B),

(cr_GPRS_Timer2_3('5E'O, tsc_Gprs3TimerUnit_30sec,'01000'B),

cr_GPRS_Timer2_3('5E'O, tsc_Gprs3TimerUnit_1min, '00100'B)))));

v_EpsBearerCtxtStatus := v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.epsBearerContextStatus;

v_AdditionalUpdateResult := f_GetAdditionalUpdateResult(v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.addUpdateType);

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 2");

//@siclog "Step 3" siclog@

// The SS transmits a TRACKING AREA UPDATE ACCEPT message.

SRB.send(cas_SRB1_NasPdu_REQ(eutra_CellA,

cs_TimingInfo_Now,

cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered,

cs_508_TAU_Accept_T3324_T3412ExtValue(tsc_EpsUpdate_TaUpdate,

f_GutiParameters2MobileIdentity(tsc_IEI_Guti, v_Guti2_Params),

cs_TrackingAreaIdListDef_tlv,

v_EpsBearerCtxtStatus,

v_LAI,

f_GetMSId (NORMAL),

v_AdditionalUpdateResult,

v_T3324Value2Mins,

v_T3412ValueExt4Mins))));

//@siclog "Step 4" siclog@

// The UE transmits a TRACKING AREA UPDATE COMPLETE message

SRB.receive(car_SRB1_NasPdu_IND(eutra_CellA,

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered,

cr_508_TAU_Complete)));

//@siclog "Step 5" siclog@

// The SS releases the RRC connection.

f_EUTRA_RRC_ConnectionRelease(eutra_CellA);

//@siclog "Step 6" siclog@

// The SS waits 4 minutes. (Expiry of T3412 extended value)

f_Delay(v_Wait4Mins);

//@siclog "Step 7" siclog@

// Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message?

v_NasInd := f_EUTRA_RRC_ConnEst_DefWithNas(eutra_CellA,

tsc_RRC_TI_Def,

cr_EstablishmentCause_Any,

cr_NAS_Indication(tsc_SHT_IntegrityProtected,

cdr_TAU_Request_OldGuti_MSNetworkFeatureSupport_T3324_T3412ExtValue(tsc_EpsUpdate_Periodic,

f_EUTRA_SecurityKSIasme_Get(),

-,

v_AdditionalUpdateType,

f_GutiParameters2MobileIdentity(omit, v_Guti2_Params),

cr_TAI_Any,

?,

cr_GPRS_Timer2_3('6A'O, tsc_GprsTimerUnit_1min, '00010'B),

?)));

v_EpsBearerCtxtStatus := v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.epsBearerContextStatus;

v_AdditionalUpdateResult := f_GetAdditionalUpdateResult(v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.addUpdateType);

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 7");

//@siclog "Step 8" siclog@

// The SS transmits a TRACKING AREA UPDATE ACCEPT message.

SRB.send(cas_SRB1_NasPdu_REQ(eutra_CellA,

cs_TimingInfo_Now,

cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered,

cs_508_TAU_Accept_T3324_T3412ExtValue(tsc_EpsUpdate_TaUpdate,

f_GutiParameters2MobileIdentity(tsc_IEI_Guti, v_Guti3_Params),

cs_TrackingAreaIdListDef_tlv,

v_EpsBearerCtxtStatus,

v_LAI,

f_GetMSId (NORMAL),

v_AdditionalUpdateResult,

v_T3324Value4Mins,

omit))));

//@siclog "Step 9" siclog@

// The UE transmits a TRACKING AREA UPDATE COMPLETE message

SRB.receive(car_SRB1_NasPdu_IND(eutra_CellA,

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered,

cr_508_TAU_Complete)));

f_EUTRA_CellInfo_SetGuti(eutra_CellA, v_Guti3_Params);

}

After change:

function fl_TC_9_2_3_1_5b_Body() runs on EUTRA_PTC

{ //@sic R5s160882 R5-168385 sic@

var GutiParameters_Type v_Guti1_Params := f_EUTRA_CellInfo_GetGuti (eutra_CellA);

var GutiParameters_Type v_Guti2_Params := f_EUTRA_CellInfo_GetGuti(eutra_CellB);

var GutiParameters_Type v_Guti3_Params := f_CreateGutiParamsWithUpdatedSTmsi(v_Guti1_Params);

var template (omit) EPS_BearerContextStatus v_EpsBearerCtxtStatus;

var template (omit) AdditionalUpdateResult v_AdditionalUpdateResult;

var B1_Type v_MS_NetworkFeatureSupport := '1'B ;

var NAS_PlmnId v_PLMN := f_Asn2Nas_PlmnId(v_Guti1_Params.PLMN_Identity);

var NAS_Lac v_LAC := f_EUTRA_CellInfo_GetLocationAreaCode(eutra_CellA);

var template (omit) LocAreaId v_LAI := f_GetLAI(v_PLMN, v_LAC, NORMAL);

var template AdditionalUpdateType v_AdditionalUpdateType := f_GetAdditionalUpdateType ();

var NAS_MSG_Indication_Type v_NasInd;

var float v_Wait4Mins := f_EUTRA_SetTimerToleranceMin(eutra_CellA, nasTimer, 240.0);

var template (value) GPRS_Timer2 v_T3324Value2Mins := cs_GPRS_Timer2_3_IEI('6A'O, tsc_GprsTimerUnit_1min, '00010'B); //T3324 value = 2 mins

var template (value) GPRS_Timer2 v_T3324Value4Mins := cs_GPRS_Timer2_3_IEI('6A'O, tsc_GprsTimerUnit_1min, '00100'B); //T3324 value = 4 mins

var template (value) GPRS_Timer2 v_T3412ValueExt4Mins := cs_GPRS_Timer2_3_IEI('5E'O, tsc_Gprs3TimerUnit_1min, '00100'B); //T3412 value = 4 mins @sic R5s170040 sic@

var template (omit) GPRS_Timer3 v_T3412ExtdValue := omit;

//@siclog "Step 1" siclog@

// The user requests PSM by MMI or by AT command. The requested value of T3324 is 2 minutes and T3412 extended value is 4 minutes.

f_UT_ConfigurePSM(UT, omit, v_T3412ValueExt4Mins, v_T3324Value2Mins);

//@siclog "Step 2" siclog@

// Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message?

v_NasInd := f_EUTRA_RRC_ConnEst_DefWithNas(eutra_CellA,

tsc_RRC_TI_Def,

cr_EstablishmentCause_Any,

cr_NAS_Indication(tsc_SHT_IntegrityProtected,

cdr_TAU_Request_OldGuti_MSNetworkFeatureSupport_T3324_T3412ExtValue(tsc_EpsUpdate_TaUpdate,

f_EUTRA_SecurityKSIasme_Get(),

-,

v_AdditionalUpdateType,

f_GutiParameters2MobileIdentity(omit, v_Guti1_Params),

cr_TAI_Any,

cr_MS_NetworkFeatureSupport('C'H, v_MS_NetworkFeatureSupport),

cr_GPRS_Timer2_3('6A'O, tsc_GprsTimerUnit_1min, '00010'B),

(cr_GPRS_Timer2_3('5E'O, tsc_Gprs3TimerUnit_30sec,'01000'B),

cr_GPRS_Timer2_3('5E'O, tsc_Gprs3TimerUnit_1min, '00100'B)))));

v_EpsBearerCtxtStatus := v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.epsBearerContextStatus;

v_AdditionalUpdateResult := f_GetAdditionalUpdateResult(v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.addUpdateType);

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 2");

//@siclog "Step 3" siclog@

// The SS transmits a TRACKING AREA UPDATE ACCEPT message.

SRB.send(cas_SRB1_NasPdu_REQ(eutra_CellA,

cs_TimingInfo_Now,

cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered,

cs_508_TAU_Accept_T3324_T3412ExtValue(tsc_EpsUpdate_TaUpdate,

f_GutiParameters2MobileIdentity(tsc_IEI_Guti, v_Guti2_Params),

cs_TrackingAreaIdListDef_tlv,

v_EpsBearerCtxtStatus,

v_LAI,

f_GetMSId (NORMAL),

v_AdditionalUpdateResult,

v_T3324Value2Mins,

v_T3412ValueExt4Mins))));

//@siclog "Step 4" siclog@

// The UE transmits a TRACKING AREA UPDATE COMPLETE message

SRB.receive(car_SRB1_NasPdu_IND(eutra_CellA,

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered,

cr_508_TAU_Complete)));

//@siclog "Step 5" siclog@

// The SS releases the RRC connection.

f_EUTRA_RRC_ConnectionRelease(eutra_CellA);

//@siclog "Step 6" siclog@

// The SS waits 4 minutes. (Expiry of T3412 extended value)

f_Delay(v_Wait4Mins);

//@siclog "Step 7" siclog@

// Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message?

v_NasInd := f_EUTRA_RRC_ConnEst_DefWithNas(eutra_CellA,

tsc_RRC_TI_Def,

cr_EstablishmentCause_Any,

cr_NAS_Indication(tsc_SHT_IntegrityProtected,

cdr_TAU_Request_OldGuti_MSNetworkFeatureSupport_T3324_T3412ExtValue(tsc_EpsUpdate_Periodic,

f_EUTRA_SecurityKSIasme_Get(),

-,

v_AdditionalUpdateType,

f_GutiParameters2MobileIdentity(omit, v_Guti2_Params),

cr_TAI_Any,

?,

cr_GPRS_Timer2_3('6A'O, tsc_GprsTimerUnit_1min, '00010'B),

?)));

v_EpsBearerCtxtStatus := v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.epsBearerContextStatus;

v_AdditionalUpdateResult := f_GetAdditionalUpdateResult(v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.addUpdateType);

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 7");

if (isvalue(v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.t3412ExtdValue)) {

v_T3412ExtdValue := v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.t3412ExtdValue;

}

//@siclog "Step 8" siclog@

// The SS transmits a TRACKING AREA UPDATE ACCEPT message.

SRB.send(cas_SRB1_NasPdu_REQ(eutra_CellA,

cs_TimingInfo_Now,

cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered,

cs_508_TAU_Accept_T3324_T3412ExtValue(tsc_EpsUpdate_TaUpdate,

f_GutiParameters2MobileIdentity(tsc_IEI_Guti, v_Guti3_Params),

cs_TrackingAreaIdListDef_tlv,

v_EpsBearerCtxtStatus,

v_LAI,

f_GetMSId (NORMAL),

v_AdditionalUpdateResult,

v_T3324Value4Mins,

v_T3412ExtdValue))));

//@siclog "Step 9" siclog@

// The UE transmits a TRACKING AREA UPDATE COMPLETE message

SRB.receive(car_SRB1_NasPdu_IND(eutra_CellA,

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered,

cr_508_TAU_Complete)));

f_EUTRA_CellInfo_SetGuti(eutra_CellA, v_Guti3_Params);

}

## 2.3 Change 3

| Function name | fl_TC_9_2_4_1_2_Body |
| --- | --- |
| Reason for change | According to TS 24.301 section 5.5.1.2.2, “If the UE supports PSM and requests the use of PSM, then the UE shall include the T3324 value IE with a requested timer value in the ATTACH REQUEST message. When the UE includes the T3324 value IE and the UE indicates support for extended periodic timer value in the MS network feature support IE, it may also include the T3412 extended value IE to request a particular T3412 value to be allocated.” And according to TS 24.301 section 5.5.3.2.2, “If the UE supports PSM and requests the use of PSM, the UE shall include the T3324 value IE with a requested timer value in the TRACKING AREA UPDATE REQUEST message. When the UE includes the T3324 value IE and the UE indicates support for extended periodic timer value in the MS network feature support IE, it may also include the T3412 extended value IE to request a particular T3412 value to be allocated.” Currently above scenario is taken care only by testcase 9.2.3.1.1a, but not 9.2.4.1.2 |
| Summary of change | Allocate timer T3412 value in Attach Accept and TrackingAreaUpdate Accept messages when UE requests it. Move template definition of ‘cs_508_TAU_Accept_T3324_T3412ExtValue’ from NAS_Attach_EOnly_R12.ttcn to EPS_NAS_Templates.ttcn |
| TTCN module | \LTE_A_PRO\9_2\NAS_Attach_EOnly_eDRX.ttcn |
| MCC160 Comment | Accepted conditional to prose CR agreement at RAN5#95-e |

Before change:

function fl_TC_9_2_4_1_2_Body() runs on EUTRA_PTC

{ // @sic R5s220413 sic@

var ProcedureTransactionIdentifier v_EPS_TI := '00'O;

var RRC_TransactionIdentifier v_RRC_TI := tsc_RRC_TI_Def;

var GutiParameters_Type v_GutiParams_CellA := f_EUTRA_CellInfo_GetGuti(eutra_CellA);

var GutiParameters_Type v_GutiParams_CellB := f_EUTRA_CellInfo_GetGuti(eutra_CellB);

var template (value) MobileIdentity v_Guti := f_GutiParameters2MobileIdentity(omit, v_GutiParams_CellA); // @sic R5s170325 sic@

var NAS_PlmnId v_PLMN := f_Asn2Nas_PlmnId(v_GutiParams_CellA.PLMN_Identity);

var NAS_PlmnId v_PLMN_B := f_Asn2Nas_PlmnId(v_GutiParams_CellB.PLMN_Identity);

var NAS_UL_Message_Type v_PdnConnectivityRequest;

var TrackingAreaCode v_TacB:= f_EUTRA_CellInfo_GetTAC(eutra_CellB);

var NAS_Lac v_LAC := f_EUTRA_CellInfo_GetLocationAreaCode(eutra_CellA);

var template (omit) LocAreaId v_LAI := f_GetLAI(v_PLMN, v_LAC, NORMAL);

var template (omit) MobileIdentity v_MSId := f_GetMSId (NORMAL);

var template AdditionalUpdateType v_AdditionalUpdateType := f_GetAdditionalUpdateType();

var template (omit) EPS_BearerContextStatus v_EpsBearerCtxtStatus;

var APNandPCOs_Type v_APNandPCOs;

var template (omit) ProtocolConfigOptions v_Pco := omit;

var NAS_AttDetValue_Type v_AttachType :=f_GetEAttachType(NORMAL);

var NAS_MSG_Indication_Type v_NasInd;

var SRB_COMMON_IND v_ReceivedAsp;

var EPS_BearerIdentity v_EPS_Bearer_Id := f_EUTRA_InitialRegistrationEpsBearerId(firstPDN); // @sic R5s170325 sic@

var DRB_Identity v_DRB_Id := f_EUTRA_NB_EpsBearerAssociatedDRB(v_EPS_Bearer_Id);

var integer v_PTWLength_Index := 1; // Paging Time Window - Duration in seconds (2.56)

var integer v_EDRXValue_Index := 3; //index on table - eDRX cycle length = 40.96sec - See Table 10.5.5.32/3GPP TS 24.008: Extended DRX parameters information element

var float v_TimerValue;

var template (value) CellPowerList_Type v_CellPowerList;

var template (value) GPRS_Timer2 v_T3324Value2Mins := cs_GPRS_Timer2_3_IEI('6A'O, tsc_GprsTimerUnit_1min, '00010'B);  //value = 2 mins

timer t_T3324 := f_EUTRA_SetTimerToleranceMax(eutra_CellB, nasTimer, 120.0);

//@siclog "Step 1" siclog@

//The UE is switched on

f_EUTRA_SwitchOnUEandStartIP(eutra_CellA);

//@siclog "Step 2" siclog@

//The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message and an Extended DRX parameters IE and T3324 IE set to two minutes

v_NasInd := f_EUTRA_RRC_ConnEst_DefWithNas(eutra_CellA,

tsc_RRC_TI_Def,

cr_EstablishmentCause_Any,

cr_NAS_IndicationWithPiggybacking(tsc_SHT_IntegrityProtected,

cdr_ATTACH_REQUEST_AttachType_ExtdDrx_T3324(v_AttachType,

v_AdditionalUpdateType,

v_Guti,

cr_TAI( v_PLMN, tsc_TAC1),

v_LAI,

v_T3324Value2Mins,

cr_ExtdDRXParamsAny), //T3324 value = 2 mins

cr_508_PDN_CONNECTIVITY_REQUEST(tsc_PdnRequest_InitialAttach, f_GetPdnType())));

//@siclog "Steps 3 - 11" siclog@

// Steps 5 to 13 of the generic test procedure in TS 36.508 Table 4.5.2.3-1 (Attach procedure) are performed on Cell A.

v_PdnConnectivityRequest := v_NasInd.Pdu.PiggybackedPduList[0].Msg;

v_EPS_TI := v_PdnConnectivityRequest.pDN_CONNECTIVITY_REQUEST.procedureTransactionIdentifier; // Use this value in SS responses

v_APNandPCOs := f_EUTRA_InitialRegistration_Step5_13(eutra_CellA, STATE2_IDLEUPDATE, v_NasInd);

if (isvalue(v_APNandPCOs.protocolConfigurationOptions)) {

v_Pco := v_APNandPCOs.protocolConfigurationOptions;

}

/////////////////////////////////////////////////////////////////////////////////////////////////////////

// EXCEPTION: In parallel to the event described in steps 13 below the generic procedure for IP        //

// address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing    //

// IP address allocation in the U-plane if requested by the UE.                                       //

////////////////////////////////////////////////////////////////////////////////////////////////////////

///////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

// EXCEPTION: IF PDN1_IMS (NOTE 1) then in parallel to the event described in steps 13 below the generic                 //

// procedure for IMS signalling in the Uplane specified in TS 36.508 subclause 4.5A.3 takes place if requested by the UE //

///////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

f_IP_ChangeEutraCell(IP, eutra_CellA); // @sic R5s170325 - R5s180239 Ch. 2 sic@

//@siclog "Steps 12" siclog@

// SS responds with ATTACH ACCEPT message including an Extended DRX parameters IE and the T3324 IE set to two minutes.

// The ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message is piggybacked in ATTACH ACCEPT message

f_EUTRA_RRCConnectionReconfiguration_SRB2_1AM_DRB_Est(eutra_CellA,  // @sic R5s170325 sic@

tsc_RRC_TI_Def,

cs_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered,

cs_508_ATTACH_ACCEPT(v_AttachType,

cs_TrackingAreaIdListDef_lv,

f_GutiParameters2MobileIdentity(tsc_IEI_Guti, v_GutiParams_CellA),

v_LAI,

v_MSId,

omit,

f_GetAdditionalUpdateResult(v_NasInd.Pdu.Msg.aTTACH_REQUEST.addUpdateType, NORMAL),

-,

-,

-,

v_T3324Value2Mins,

cs_EDRXParams('0001'B, '0011'B)),

cs_508_ActivateDefEpsBearerContextRequest(v_EPS_Bearer_Id,

v_EPS_TI,

v_APNandPCOs.accessPointName,

f_EUTRA_GetPDNAddress (f_CheckPCOforIPallocationViaNas (v_Pco), f_EUTRA_InitialRegistrationPdnIndex(firstPDN)),

-,

f_GetDefaultProtocolConfigOptions (v_Pco))),

v_DRB_Id);

//@siclog "Step 13" siclog@

//<=== SKIPPED ===>

v_EpsBearerCtxtStatus := v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.epsBearerContextStatus;

//@siclog "Step 26" siclog@

//The SS transmits a TRACKING AREA UPDATE ACCEPT message without an Extended DRX parameters IE but with the T3324 IE set to 2 minutes.

SRB.send(cas_SRB1_NasPdu_REQ(eutra_CellB,

cs_TimingInfo_Now,

cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered,

cs_TAU_Accept_ExtdDrx_T3324(f_GetEPSTAUType(NORMAL),

f_GutiParameters2MobileIdentity(tsc_IEI_Guti, v_GutiParams_CellB),

cds_TAIListNonConsecutive_tlv(v_PLMN_B, { bit2oct(v_TacB)}),

v_EpsBearerCtxtStatus,

f_EUTRA_GetCellLAI (eutra_CellB, NORMAL),

v_MSId,

f_GetAdditionalUpdateResult(v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.addUpdateType),

v_T3324Value2Mins))));

t_T3324.start;

//@siclog "Step 27" siclog@

//<=== SKIPPED ===>

}

After change:

function fl_TC_9_2_4_1_2_Body() runs on EUTRA_PTC

{ // @sic R5s220413 sic@

var ProcedureTransactionIdentifier v_EPS_TI := '00'O;

var RRC_TransactionIdentifier v_RRC_TI := tsc_RRC_TI_Def;

var GutiParameters_Type v_GutiParams_CellA := f_EUTRA_CellInfo_GetGuti(eutra_CellA);

var GutiParameters_Type v_GutiParams_CellB := f_EUTRA_CellInfo_GetGuti(eutra_CellB);

var template (value) MobileIdentity v_Guti := f_GutiParameters2MobileIdentity(omit, v_GutiParams_CellA); // @sic R5s170325 sic@

var NAS_PlmnId v_PLMN := f_Asn2Nas_PlmnId(v_GutiParams_CellA.PLMN_Identity);

var NAS_PlmnId v_PLMN_B := f_Asn2Nas_PlmnId(v_GutiParams_CellB.PLMN_Identity);

var NAS_UL_Message_Type v_PdnConnectivityRequest;

var TrackingAreaCode v_TacB:= f_EUTRA_CellInfo_GetTAC(eutra_CellB);

var NAS_Lac v_LAC := f_EUTRA_CellInfo_GetLocationAreaCode(eutra_CellA);

var template (omit) LocAreaId v_LAI := f_GetLAI(v_PLMN, v_LAC, NORMAL);

var template (omit) MobileIdentity v_MSId := f_GetMSId (NORMAL);

var template AdditionalUpdateType v_AdditionalUpdateType := f_GetAdditionalUpdateType();

var template (omit) EPS_BearerContextStatus v_EpsBearerCtxtStatus;

var APNandPCOs_Type v_APNandPCOs;

var template (omit) ProtocolConfigOptions v_Pco := omit;

var NAS_AttDetValue_Type v_AttachType :=f_GetEAttachType(NORMAL);

var NAS_MSG_Indication_Type v_NasInd;

var SRB_COMMON_IND v_ReceivedAsp;

var EPS_BearerIdentity v_EPS_Bearer_Id := f_EUTRA_InitialRegistrationEpsBearerId(firstPDN); // @sic R5s170325 sic@

var DRB_Identity v_DRB_Id := f_EUTRA_NB_EpsBearerAssociatedDRB(v_EPS_Bearer_Id);

var integer v_PTWLength_Index := 1; // Paging Time Window - Duration in seconds (2.56)

var integer v_EDRXValue_Index := 3; //index on table - eDRX cycle length = 40.96sec - See Table 10.5.5.32/3GPP TS 24.008: Extended DRX parameters information element

var float v_TimerValue;

var template (value) CellPowerList_Type v_CellPowerList;

var template (value) GPRS_Timer2 v_T3324Value2Mins := cs_GPRS_Timer2_3_IEI('6A'O, tsc_GprsTimerUnit_1min, '00010'B);  //value = 2 mins

var template (omit) GPRS_Timer3 v_T3412ExtdValue := omit;

timer t_T3324 := f_EUTRA_SetTimerToleranceMax(eutra_CellB, nasTimer, 120.0);

//@siclog "Step 1" siclog@

//The UE is switched on

f_EUTRA_SwitchOnUEandStartIP(eutra_CellA);

//@siclog "Step 2" siclog@

//The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message and an Extended DRX parameters IE and T3324 IE set to two minutes

v_NasInd := f_EUTRA_RRC_ConnEst_DefWithNas(eutra_CellA,

tsc_RRC_TI_Def,

cr_EstablishmentCause_Any,

cr_NAS_IndicationWithPiggybacking(tsc_SHT_IntegrityProtected,

cdr_ATTACH_REQUEST_AttachType_ExtdDrx_T3324(v_AttachType,

v_AdditionalUpdateType,

v_Guti,

cr_TAI( v_PLMN, tsc_TAC1),

v_LAI,

v_T3324Value2Mins,

cr_ExtdDRXParamsAny), //T3324 value = 2 mins

cr_508_PDN_CONNECTIVITY_REQUEST(tsc_PdnRequest_InitialAttach, f_GetPdnType())));

//@siclog "Steps 3 - 11" siclog@

// Steps 5 to 13 of the generic test procedure in TS 36.508 Table 4.5.2.3-1 (Attach procedure) are performed on Cell A.

v_PdnConnectivityRequest := v_NasInd.Pdu.PiggybackedPduList[0].Msg;

v_EPS_TI := v_PdnConnectivityRequest.pDN_CONNECTIVITY_REQUEST.procedureTransactionIdentifier; // Use this value in SS responses

v_APNandPCOs := f_EUTRA_InitialRegistration_Step5_13(eutra_CellA, STATE2_IDLEUPDATE, v_NasInd);

if (isvalue(v_APNandPCOs.protocolConfigurationOptions)) {

v_Pco := v_APNandPCOs.protocolConfigurationOptions;

}

if (isvalue(v_NasInd.Pdu.Msg.aTTACH_REQUEST.t3412ExtdValue)) {

v_T3412ExtdValue := v_NasInd.Pdu.Msg.aTTACH_REQUEST.t3412ExtdValue;

}

/////////////////////////////////////////////////////////////////////////////////////////////////////////

// EXCEPTION: In parallel to the event described in steps 13 below the generic procedure for IP        //

// address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing    //

// IP address allocation in the U-plane if requested by the UE.                                       //

////////////////////////////////////////////////////////////////////////////////////////////////////////

///////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

// EXCEPTION: IF PDN1_IMS (NOTE 1) then in parallel to the event described in steps 13 below the generic                 //

// procedure for IMS signalling in the Uplane specified in TS 36.508 subclause 4.5A.3 takes place if requested by the UE //

///////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

f_IP_ChangeEutraCell(IP, eutra_CellA); // @sic R5s170325 - R5s180239 Ch. 2 sic@

//@siclog "Steps 12" siclog@

// SS responds with ATTACH ACCEPT message including an Extended DRX parameters IE and the T3324 IE set to two minutes.

// The ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message is piggybacked in ATTACH ACCEPT message

f_EUTRA_RRCConnectionReconfiguration_SRB2_1AM_DRB_Est(eutra_CellA,  // @sic R5s170325 sic@

tsc_RRC_TI_Def,

cs_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered,

cs_508_ATTACH_ACCEPT(v_AttachType,

cs_TrackingAreaIdListDef_lv,

f_GutiParameters2MobileIdentity(tsc_IEI_Guti, v_GutiParams_CellA),

v_LAI,

v_MSId,

omit,

f_GetAdditionalUpdateResult(v_NasInd.Pdu.Msg.aTTACH_REQUEST.addUpdateType, NORMAL),

-,

-,

v_T3412ExtdValue,

v_T3324Value2Mins,

cs_EDRXParams('0001'B, '0011'B)),

cs_508_ActivateDefEpsBearerContextRequest(v_EPS_Bearer_Id,

v_EPS_TI,

v_APNandPCOs.accessPointName,

f_EUTRA_GetPDNAddress (f_CheckPCOforIPallocationViaNas (v_Pco), f_EUTRA_InitialRegistrationPdnIndex(firstPDN)),

-,

f_GetDefaultProtocolConfigOptions (v_Pco))),

v_DRB_Id);

//@siclog "Step 13" siclog@

//<=== SKIPPED ===>

v_EpsBearerCtxtStatus := v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.epsBearerContextStatus;

if (isvalue(v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.t3412ExtdValue)) {

v_T3412ExtdValue := v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.t3412ExtdValue;

}

//@siclog "Step 26" siclog@

//The SS transmits a TRACKING AREA UPDATE ACCEPT message without an Extended DRX parameters IE but with the T3324 IE set to 2 minutes.

SRB.send(cas_SRB1_NasPdu_REQ(eutra_CellB,

cs_TimingInfo_Now,

cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered,

cs_508_TAU_Accept_T3324_T3412ExtValue(f_GetEPSTAUType(NORMAL),

f_GutiParameters2MobileIdentity(tsc_IEI_Guti, v_GutiParams_CellB),

cds_TAIListNonConsecutive_tlv(v_PLMN_B, { bit2oct(v_TacB)}),

v_EpsBearerCtxtStatus,

f_EUTRA_GetCellLAI (eutra_CellB, NORMAL),

v_MSId,

f_GetAdditionalUpdateResult(v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.addUpdateType),

v_T3324Value2Mins,

v_T3412ExtdValue))));

t_T3324.start;

//@siclog "Step 27" siclog@

//<=== SKIPPED ===>

}