3GPP TSG-RAN5 Meeting #94-e	R5-22xxxx

# Online, ,  9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | - | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE PSM Testcases 9.2.1.1.7c, 9.2.3.1.5b and 9.2.4.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI12_Test, MTCe-UEPCOP-UEConTest, TEI13_Test |  |  |  |  |  |  | Date: |  | 2022-04-05 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to TS 24.301 section 5.5.1.2.2, “If the UE supports PSM and requests the use of PSM, then the UE shall include the T3324 value IE with a requested timer value in the ATTACH REQUEST message. When the UE includes the T3324 value IE and the UE indicates support for extended periodic timer value in the MS network feature support IE, it may also include the T3412 extended value IE to request a particular T3412 value to be allocated.” And according to TS 24.301 section 5.5.3.2.2, “If the UE supports PSM and requests the use of PSM, the UE shall include the T3324 value IE with a requested timer value in the TRACKING AREA UPDATE REQUEST message. When the UE includes the T3324 value IE and the UE indicates support for extended periodic timer value in the MS network feature support IE, it may also include the T3412 extended value IE to request a particular T3412 value to be allocated.” Currently above scenarios are taken care only by testcase 9.2.3.1.1a, but not other PSM testcases |  |  |  |  |  |  |  |  |
| Summary of change: |  | Allocate timer T3412 Extd Value in Attach Accept and TrackingAreaUpdate Accept messages when UE requests it. Similar to tables 9.2.3.1.1a.3.3-1 and 9.2.3.1.1a.3.3-2 (TS 36.523-1) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Testcases 9.2.1.1.7c, 9.2.3.1.5b and 9.2.4.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | Associated with TTCN CR: R5s220557 |  |  |  |  |  |  |  |  |

<start of change>

##### 9.2.1.1.7c	Attach / Success / PSM

9.2.1.1.7c.1	Test Purpose (TP)

(1)

with { the UE is switched-off with a valid USIM inserted and the UE is configured to attach with PSM }

ensure that {

when { UE is powered on }

then { the UE transmits an ATTACH REQUEST message including the T3324 IE set to a configured value }

}

(2)

with { the UE in IDLE mode }

ensure that {

when { UE receives a paging message before timer T3324 is expired }

then { the UE responds to the paging request }

}

9.2.1.1.7c.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.301, clauses 5.3.11, 5.5.1.2.2.

[TS 24.301, clause 5.3.11]

The UE can request the use of power saving mode (PSM) during an attach or tracking area updating procedure (see 3GPP TS 23.682 [11A] and 3GPP TS 23.401 [10]). The UE shall not request the use of PSM during:

-	an attach for emergency bearer services procedure;

-	an attach procedure for initiating a PDN connection for emergency bearer services with attach type not set to "EPS emergency attach";

-	a tracking area updating procedure for initiating a PDN connection for emergency bearer services; or

-	a tracking area updating procedure when the UE has a PDN connection established for emergency bearer services.

The network accepts the use of PSM by providing a specific value for timer T3324 when accepting the attach or tracking area updating procedure. The UE may use PSM only if the network has provided the T3324 value IE during the last attach or tracking area updating procedure with a value different from "deactivated".

NOTE:	Timer T3324 is specified in 3GPP TS 24.008 [13].

Upon expiry of the timer T3324 or if the T3324 value provided by the network is zero, the UE may deactivate the AS layer and activate PSM by entering the state EMM-REGISTERED.NO-CELL-AVAILABLE if:

a)	the UE is not attached for emergency bearer services;

b)	the UE has no PDN connection for emergency bearer services;

c)	the UE is in EMM-IDLE mode; and

d)	in the EMM-REGISTERED.NORMAL-SERVICE state.

If conditions a, b and c are fulfilled, but the UE is in a state other than EMM-REGISTERED.NORMAL-SERVICE when timer T3324 expires, the UE may activate PSM when the MS returns to state EMM-REGISTERED.NORMAL-SERVICE.

A UE that has already been allocated timer T3324 with a value different from "deactivated" and the timer T3324 has expired, may activate PSM if it receives an "Extended wait time" from lower layers.

If the UE is attached for emergency bearer services or has a PDN connection for emergency bearer services, the UE shall not activate PSM.

The UE may deactivate PSM at any time (e.g. for the transfer of mobile originated signalling or user data), by activating the AS layer before initiating the necessary EMM procedures. When PSM is activated all NAS timers are stopped and associated procedures aborted except for T3412, T3346 and T3396.

[TS 24.301, clause 5.5.1.2.2]

If the UE supports PSM and requests the use of PSM, then the UE shall include the T3324 value IE with a requested timer value in the ATTACH REQUEST message. When the UE includes the T3324 value IE and the UE indicates support for extended periodic timer value in the MS network feature support IE, it may also include the T3412 extended value IE to request a particular T3412 value to be allocated.

9.2.1.1.7c.3	Test description

9.2.1.1.7c.3.1	Pre-test conditions

System Simulator:

-	cell A.

UE:

- 	The UE is configured to initiate EPS attach;

-	The UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18].

-	The UE is configured to use Power Saving Mode

-	The UE is configured to use a specific value of T3324

Preamble:

-	The UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.1.1.7c.3.2	Test procedure sequence

Table 9.2.1.1.7c.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | The UE transmits an RRCConnectionRequest message. | - | - | - | - |
| 3 | The SS transmits an RRCConnectionSetup message. | - | - | - | - |
| 4 | Check: Does the UE transmit an ATTACH REQUEST message including the T3324 IE set to two minutes? | --> | ATTACH REQUEST | 1 | P |
| 5 -13Eb1 | Steps 5-18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
|  | The SS shall wait for 1 minute and then execute the following steps before timer T3324 expires. |  |  |  |  |
| 14 - 24 | Check: Does the UE accept the paging request? Steps 1-10, from Table 4.5.3.3-1 in TS 36.508, are performed. | - | - | 2 | P |

9.2.1.1.7c.3.3	Specific message contents

Table 9.2.1.1.7c.3.3-1: Void

Table 9.2.1.1.7c.3.3-2: Message ATTACH REQUEST (step 4, Table 9.2.1.1.7c.3.2-1)

| Derivation path: TS 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Old GUTI or IMSI | GUTI(belonging to PLMN with same MCC as stored in EFIMSI on the test USIM card) | GUTI copied from USIM Old and valid GUTI is included by the UE |  |
| ESM message container | PDN CONNECTIVITY REQUEST message to request PDN connectivity to the default PDN |  |  |
| Last visited registered TAI | TAI1 | GUTI copied from USIM If available, the last TAI is included by UE and will be used to establish a good list of TAIs in subsequent ATTACH ACCEPT message. |  |
| T3324 | ’0010 0010’B | 2 minutes |  |
| T3412 extended value | Not present |  |  |
|  | Any allowed value |  | IF the UE indicates support for extended periodic timer value in the MS network feature support IE |

Table 9.2.1.1.7c.3.3-4: Void

Table 9.2.1.1.7c.3.3-5: Message ATTACH ACCEPT (step 10, Table 9.2.1.1.7c.3.2-1)

| Derivation path: TS 36.508 table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| T3324 | T3324 timer value | Timer set to the same value as in table 9.2.1.1.7c.3.3-2 |  |
| T3412 extended value | Value received in ATTACH REQUEST | Included only if the IE 'T3412 extended value' was provided in the ATTACH REQUEST |  |

<skipped unmodified text>

##### 9.2.3.1.5b	Periodic tracking area update / Accepted / PSM / T3412 Extended Value

9.2.3.1.5b.1	Test Purpose (TP)

(1)

with { UE in state EMM-REGISTERED and EMM-IDLE mode}

ensure that {

when { PSM is activated }

then { UE send TRACKING AREA UPDATE REQUEST message with the T3324 value set to 2 minutes and T3412 extended value set to 4 minutes  }

}

(2)

with { UE in state EMM-REGISTERED and EMM-IDLE mode with timer T3412 extended value active }

ensure that {

when { timer T3412 extended value expires }

then { UE sends TRACKING AREA UPDATE REQUEST message with EPS update type = “Periodic updating” }

}

9.2.3.1.5b.2	Conformance requirements

[TS 24.301, clause 5.3.11]

The UE can request the use of power saving mode (PSM) during an attach or tracking area updating procedure (see 3GPP TS 23.682 [11A] and 3GPP TS 23.401 [10]). The UE shall not request the use of PSM during:

-	an attach for emergency bearer services procedure;

-	an attach procedure for initiating a PDN connection for emergency bearer services with attach type not set to "EPS emergency attach";

-	a tracking area updating procedure for initiating a PDN connection for emergency bearer services; or

-	a tracking area updating procedure when the UE has a PDN connection established for emergency bearer services.

The network accepts the use of PSM by providing a specific value for timer T3324 when accepting the attach or tracking area updating procedure. The UE may use PSM only if the network has provided the T3324 value IE during the last attach or tracking area updating procedure with a value different from "deactivated".

NOTE:	Timer T3324 is specified in 3GPP TS 24.008 [13].

Upon expiry of the timer T3324 or if the T3324 value provided by the network is zero, the UE may deactivate the AS layer and activate PSM by entering the state EMM-REGISTERED.NO-CELL-AVAILABLE if:

a)	the UE is not attached for emergency bearer services;

b)	the UE has no PDN connection for emergency bearer services;

c)	the UE is in EMM-IDLE mode; and

d)	in the EMM-REGISTERED.NORMAL-SERVICE state.

If conditions a, b and c are fulfilled, but the UE is in a state other than EMM-REGISTERED.NORMAL-SERVICE when timer T3324 expires, the UE may activate PSM when the MS returns to state EMM-REGISTERED.NORMAL-SERVICE.

A UE that has already been allocated timer T3324 with a value different from "deactivated" and the timer T3324 has expired, may activate PSM if it receives an "Extended wait time" from lower layers.

If the UE is attached for emergency bearer services or has a PDN connection for emergency bearer services, the UE shall not activate PSM.

The UE may deactivate PSM at any time (e.g. for the transfer of mobile originated signalling or user data), by activating the AS layer before initiating the necessary EMM procedures. When PSM is activated all NAS timers are stopped and associated procedures aborted except for T3412, T3346 and T3396.

[TS 24.301, clause 5.5.1.2.2]

If the UE supports PSM and requests the use of PSM, then the UE shall include the T3324 value IE with a requested timer value in the ATTACH REQUEST message. When the UE includes the T3324 value IE and the UE indicates support for extended periodic timer value in the MS network feature support IE, it may also include the T3412 extended value IE to request a particular T3412 value to be allocated.

9.2.3.1.5b.3	Test description

9.2.3.1.5b.3.1	Pre-test conditions

System Simulator:

-	cell A.

UE:

- 	the UE is configured to initiate EPS attach.

-	The UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on cell A using default message contents according to TS 36.508 [18].

-	The UE is configured to use Power Saving Mode

Preamble:

-	the UE is in state Registered, Idle Mode (state 2) on cell A according to TS 36.508 [18].

9.2.3.1.5b.3.2	Test procedure sequence

Table 9.2.3.1.5b.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The user requests PSM by MMI or by AT command. The requested value of T3324 is 2 minutes and T3412 extended value is 4 minutes. | - | - | - | - |
| 2 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message? | --> | TRACKING AREA UPDATE REQUEST | 1 | P |
| 3 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 4 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| 5 | The SS releases the RRC connection. |  |  | - | - |
| 6 | The SS waits 4 minutes. (Expiry of T3412 extended value) | - | - | - | - |
| 7 | Check: Does the UE transmit a TRACKING AREA UPDATE REQUEST message? | --> | TRACKING AREA UPDATE REQUEST | 2 | P |
| 8 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 9 | The UE transmits a TRACKING AREA UPDATE COMPLETE message? | --> | TRACKING AREA UPDATE COMPLETE | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA connected (E2_T3440) according to TS 36.508. | - | - | - | - |

9.2.3.1.5b.3.3	Specific message contents

Table 9.2.3.1.5b.3.3-1: Message TRACKING AREA UPDATE REQUEST (step 2, Table 9.2.3.1.5b.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type | 000 | TA updating |  |
| Old GUTI | GUTI-1 | ''Old GUTI is included by UE if valid, IMSI otherwise'' |  |
| T3324 value | ’0010 0010’B | 2 minutes |  |
| MS network feature support | 1 | MS supports the extended periodic timer in this domain |  |
| T3412 extended value | ‘1010 0100’B or ‘1000 1000’B | 4 minutes or 240 seconds |  |

Table 9.2.3.1.5b.3.3-2: Message TRACKING AREA UPDATE ACCEPT (step 3, Table 9.2.3.1.5b.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update result | 000 | "TA only" |  |
| GUTI | GUTI-2 |  |  |
| T3324 value | ’0010 0010’B | 2 minutes |  |
| T3412 extended value | ‘1010 0100’B or ‘1000 1000’B (same value as received in TAU REQUEST message at Step 2) | 4 minutes or 240 seconds |  |

Table 9.2.3.1.5b.3.3-3: Message TRACKING AREA UPDATE REQUEST (step 7, Table 9.2.3.1.5b.3.2-1)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update type | 011 | periodic updating' |  |
| Old GUTI | GUTI-2 | ''Old GUTI is included by UE if valid, IMSI otherwise'' |  |
| T3324 value | ’0010 0010’B | 2 minutes |  |
| T3412 extended value | Not present |  |  |
|  | Any allowed value |  | IF the UE indicates support for extended periodic timer value in the MS network feature support IE |

Table 9.2.3.1.5b.3.3-4: Message TRACKING AREA UPDATE ACCEPT (step 8, Table 9.2.3.1.5b.3.2-1)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| EPS update result | 000 | "TA only" |  |
| GUTI | GUTI-3 |  |  |
| T3324 value | ’0010 0100’B | 4 minutes |  |
| T3412 extended value | Value received in TRACKING AREA UPDATE REQUEST (Step 7) | Included only if the IE 'T3412 extended value' was provided in the TRACKING AREA UPDATE REQUEST |  |

<skipped unmodified text>

##### 9.2.4.1.2	Attach & Normal tracking area update Procedure / Success / With and without Idle eDRX and PSM parameters

9.2.4.1.2.1	Test Purpose (TP)

(1)

with { the UE has sent an ATTACH REQUEST message with the extended DRX parameters IE and the T3324 IE }

ensure that {

when { the UE receives the ATTACH ACCEPT message including the extended DRX and T3324 IE parameters }

then { the UE sends an ATTACH COMPLETE message }

}

(2)

with { the UE has applied extended DRX parameters received in the ATTACH ACCEPT message }

ensure that {

when { the UE receives a Paging message in a valid paging occasion in paging hyperframe as per idle eDRX }

then { the UE sends a SERVICE REQUEST message }

}

(3)

with { the UE has sent a TAU REQUEST message with the extended DRX parameters IE and the T3324 IE }

ensure that {

when { the UE receives the TAU ACCEPT message not including the extended DRX parameters but including the T3324 IE }

then { the UE sends a TAU COMPLETE message }

}

(4)

with { the UE has received a TAU ACCEPT message not including extended DRX parameters IE but including T3324 IE }

ensure that {

when { the UE receives a Paging message after T3324 timer expiry in a valid paging occasion as per normal DRX }

then { the UE does not send the SERVICE REQUEST message }

}

(5)

with { the UE has sent an ATTACH REQUEST message with the extended DRX parameters IE and the T3324 IE }

ensure that {

when { the UE receives the ATTACH ACCEPT message including the extended DRX parameters but not the T3324 IE }

then { the UE sends an ATTACH COMPLETE message }

}

(6)

with { the UE has received an ATTACH ACCEPT message including extended DRX parameters IE but not the T3324 IE }

ensure that {

when { the UE receives a Paging message in a valid paging occasion in paging hyperframe as per Idle eDRX }

then { the UE sends a SERVICE REQUEST message }

}

9.2.4.1.2.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 24.301 clauses 5.3.11, 5.3.12, 5.3.13, 5.5.1.2.2, 5.5.1.2.4, 5.5.3.2.2, 5.5.3.2.4 and 5.6.2.2.1.1

[TS 24.301 clause 5.3.11]

The UE can request the use of power saving mode (PSM) during an attach or tracking area updating procedure (see 3GPP TS 23.682 [11A] and 3GPP TS 23.401 [10]). The UE shall not request the use of PSM during:

-	an attach for emergency bearer services procedure;

-	an attach procedure for initiating a PDN connection for emergency bearer services with attach type not set to "EPS emergency attach";

-	a tracking area updating procedure for initiating a PDN connection for emergency bearer services; or

-	a tracking area updating procedure when the UE has a PDN connection established for emergency bearer services.

The network accepts the use of PSM by providing a specific value for timer T3324 when accepting the attach or tracking area updating procedure. The UE may use PSM only if the network has provided the T3324 value IE during the last attach or tracking area updating procedure with a value different from "deactivated".

[TS 24.301 clause 5.3.12]

The UE may request the use of extended idle-mode DRX cycle (eDRX) during an attach or tracking area updating procedure by including the extended DRX parameters IE (see 3GPP TS 23.682 [11A] and 3GPP TS 23.401 [10]). The UE shall not request the use of eDRX during:

…

The network accepts the request to use the eDRX by providing the extended DRX parameters IE when accepting the attach or the tracking area updating procedure. The UE shall use eDRX only if the network has provided the extended DRX parameters IE during the last attach or tracking area updating procedure.

NOTE:	If the UE wants to keep using eDRX, the UE includes the extended DRX parameters IE in each attach or tracking area updating procedure.

[TS 24.301 clause 5.3.13]

The UE can request the use of both PSM and eDRX during an attach or tracking area update procedure but it is up to the network to decide to enable none, one of them or both (see 3GPP TS 23.682 [11A] and 3GPP TS 23.401 [10]).

If the network accepts the use of both PSM (see subclause 5.3.11) and eDRX (see subclause 5.3.12), the extended DRX parameters IE provided to the UE should allow for multiple paging occasions before the active timer expires.

[TS 24.301 clause 5.5.1.2.2]

If the UE supports eDRX and requests the use of eDRX, the UE shall include the extended DRX parameters IE in the ATTACH REQUEST message.

[TS 24.301 clause 5.5.1.2.4]

The MME shall include the extended DRX parameters IE in the ATTACH ACCEPT message only if the extended DRX parameters IE was included in the ATTACH REQUEST message, and the MME supports and accepts the use of eDRX.

[TS 24.301 clause 5.5.3.2.2]

The UE in state EMM-REGISTERED shall initiate the tracking area updating procedure by sending a TRACKING AREA UPDATE REQUEST message to the MME,

- when the UE detects entering a tracking area that is not in the list of tracking areas that the UE previously registered in the MME;

...

u)	when the UE needs to request the use of eDRX or needs to stop the use of eDRX;

v)	when a change in the eDRX usage conditions at the UE requires different extended DRX parameters;

…

If the UE supports eDRX and requests the use of eDRX, the UE shall include the extended DRX parameters IE in the TRACKING AREA UPDATE REQUEST message.

[TS 24.301 clause 5.5.3.2.4]

The MME shall include the extended DRX parameters IE in the TRACKING AREA UPDATE ACCEPT message only if the extended DRX parameters IE was included in the TRACKING AREA UPDATE REQUEST message, and the MME supports and accepts the use of eDRX.

[TS 24.301 clause 5.6.2.2.1.1]

To initiate the procedure the EMM entity in the network requests the lower layer to start paging (see 3GPP TS 36.300 [20], 3GPP TS 36.413 [23]) and shall start the timer:

-	T3415 for this paging procedure, if the network accepted to use eDRX for the UE.

9.2.4.1.2.3	Test description

9.2.4.1.2.3.1	Pre-test conditions

System Simulator:

-	cell A (belongs to TAI-1, home PLMN) is set to ''Serving cell'';

-	cell B (belongs to TAI-2, home PLMN) is set to ''Non- Suitable cell'';

UE:

- 	the UE is configured to request the use of  eDRX  (in the ATTACH REQUEST and TRACKING AREA UPDATE REQUEST messages).

-	The UE is configured to use Power Saving Mode

-	The UE is configured to set T3324 as two minutes

Preamble:

-	the UE is in state Switched OFF (state 1) according to TS 36.508 [18].

9.2.4.1.2.3.2	Test procedure sequence

Table 9.2.4.1.2.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | The following messages are sent and shall be received on cell A. | - | - | - | - |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | The UE transmits an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message and an Extended DRX parameters IE and T3324 IE set to two minutes | --> | ATTACH REQUEST | - | - |
| 3-11 | Steps 5 to 13 of the generic test procedure in TS 36.508 Table 4.5.2.3-1 (Attach procedure) are performed on Cell A. | - | - | - | - |
| 12 | SS responds with ATTACH ACCEPT message including an Extended DRX parameters IE and the T3324 IE set to two minutes. The ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message is piggybacked in ATTACH ACCEPT message | <-- | ATTACH ACCEPT | - | - |
| - | EXCEPTION: In parallel to the event described in steps 13 below the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane if requested by the UE. | - | - | - | - |
| - | EXCEPTION: IF PDN1_IMS (NOTE 1) then in parallel to the event described in steps 13 below the generic procedure for IMS signalling in the Uplane specified in TS 36.508 subclause 4.5A.3 takes place if requested by the UE | - | - | - | - |
| 13 | Check: Does the UE transmit an ATTACH COMPLETE message including an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message? | --> | ATTACH COMPLETE | 1 | P |
| 14a1-16b1 | Steps 16a1-18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed | - | - | - | - |
| 17 | SS sends a Paging message to the UE in a valid PO within the PTW of the next upcoming UE’s PH as per Idle eDRX | <-- | Paging | - | - |
| 18 | Check: Does the UE transmit a SERVICE REQUEST message? | --> | SERVICE REQUEST | 2 | P |
| 19-22 | Steps 6 to 9 of the generic radio bearer establishment procedure (TS 36.508 4.5.3.3-1) are executed to successfully complete the service request procedure. | - | - | - | - |
| 23 | The SS releases the RRC connection. | - | - | - | - |
| 24 | SS sets the cell type of cell A to the "Non-Suitable cell". Set the cell type of cell B to the "Serving cell" | - | - | - | - |
| 25 | The UE transmits a TRACKING AREA UPDATE REQUEST message including an Extended DRX parameters IE and the T3324 IE set to two minutes | --> | TRACKING AREA UPDATE REQUEST | - | - |
| 26 | The SS transmits a TRACKING AREA UPDATE ACCEPT message without an Extended DRX parameters IE but with the T3324 IE set to 2 minutes. | <-- | TRACKING AREA UPDATE ACCEPT | - | - |
| 27 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | TRACKING AREA UPDATE COMPLETE | 3 | P |
| 28 | The SS releases the RRC connection. | - | - | - | - |
| 29 | When the T3324 timer expires the SS sends a Paging message including a matched identity in a valid PO as per normal DRX | <-- | Paging | - | - |
| 30 | Check: Does the UE respond to the Paging message? | - | - | 4 | F |
| 31 | If possible (see ICS) the UE is switched off or otherwise the power is removed | - | - | - | - |
| - | EXCEPTION: Step 32a1 describes behaviour that depends on the UE capability. | - | - | - | - |
| 32a1 | If pc_SwitchOnOff or pc_USIM_Removal then UE send DETACH REQUEST message. | --> | DETACH REQUEST | - | - |
| 33 | SS sets the cell type of cell A to the "Serving cell". Set the cell type of cell B to the "Non-Suitable cell". | - | - | - | - |
| 34 | The UE is switched on | - | - | - | - |
| 35 | The UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message and an Extended DRX parameters IE and the T3324 IE set to two minutes | --> | ATTACH REQUEST | - | - |
| 36-44 | Steps 5 to 13 of the generic test procedure in TS 36.508 Table 4.5.2.3-1 (Attach procedure) are performed on Cell A. | - | - | - | - |
| 45 | SS responds with ATTACH ACCEPT message including an Extended DRX parameters IE but without the T3324 IE. The ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message is piggybacked in ATTACH ACCEPT message. | <-- | ATTACH ACCEPT | - | - |
| 46 | Check: Does the UE transmit an ATTACH COMPLETE message including an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message? | --> | ATTACH COMPLETE | 5 | P |
| 47a1-49b1 | Steps 16a1-18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed | - | - | - | - |
| 50 | SS sends a Paging message to the UE in a valid PO within the PTW of the next upcoming UE’s PH as per Idle eDRX | <-- | Paging | - | - |
| 51 | Check: Does the UE transmit a SERVICE REQUEST message? | --> | SERVICE REQUEST | 6 | P |
| 52-55 | Steps 6 to 9 of the generic radio bearer establishment procedure (TS 36.508 4.5.3.3-1) are executed to successfully complete the service request procedure. | - | - | - | - |
| 56 | The SS releases the RRC connection. | - | - | - | - |
| NOTE 1: PDN1_IMS as defined in TS 36.508 subclause 4.5.2. |  |  |  |  |  |

9.2.4.1.2.3.3	Specific message contents

Table 9.2.4.1.2.3.3-1: SystemInformationBlockType1 for Cell A and Cell B

| Derivation path: 36.508 Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| hyperSFN-r13 | Current H-SFN Value |  |  |
| eDRX-Allowed-r13 | true |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.2.4.1.2.3.3-1A: SystemInformationBlockType1-BR-r13 for Cell A and Cell B when UE under test is CAT M1

| Derivation path: 36.508 Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| hyperSFN-r13 | Current H-SFN Value |  |  |
| eDRX-Allowed-r13 | true |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.2.4.1.2.3.3-2: Message ATTACH REQUEST (step 2)

| Derivation path: TS 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Extended DRX parameters |  |  |  |
| Paging Time Window | Any Value |  |  |
| eDRX value | Any Value |  |  |
| T3324 | ’00100010’B | 2 minutes |  |
| T3412 extended value | Not present |  |  |
|  | Any allowed value |  | IF the UE indicates support for extended periodic timer value in the MS network feature support IE |

Table 9.2.4.1.2.3.3-3: Message ATTACH ACCEPT (step 12)

| Derivation path: TS 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Extended DRX parameters |  |  |  |
| Paging Time Window | ‘0001’B | 2.56 second |  |
| eDRX value | ‘0011’B | 40.96 seconds |  |
| T3324 | ’00100010’B | 2 minutes |  |
| T3412 extended value | Value received in ATTACH REQUEST | Included only if the IE 'T3412 extended value' was provided in the ATTACH REQUEST |  |

Table 9.2.4.1.2.3.3-4: Message TRACKING AREA UPDATE REQUEST (step 25)

| Derivation path: 36.508 table 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Extended DRX parameters |  |  |  |
| Paging Time Window | Any Value |  |  |
| eDRX value | Any Value |  |  |
| T3324 value | ’00100010’B | 2 minutes |  |
| T3412 extended value | Not present |  |  |
|  | Any allowed value |  | IF the UE indicates support for extended periodic timer value in the MS network feature support IE |

Table 9.2.4.1.2.3.3-5: Message TRACKING AREA UPDATE ACCEPT (step 26)

| Derivation path: 36.508 table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| T3324 value | ’00100010’B | 2 minutes |  |
| T3412 extended value | Value received in TRACKING AREA UPDATE REQUEST | Included only if the IE 'T3412 extended value' was provided in the TRACKING AREA UPDATE REQUEST |  |

Table 9.2.4.1.2.3.3-6: Message ATTACH REQUEST (step 35)

| Derivation path: TS 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Extended DRX parameters |  |  |  |
| Paging Time Window | Any Value |  |  |
| eDRX value | Any Value |  |  |
| T3324 value | ’00100010’B | 2 minutes |  |

Table 9.2.4.1.2.3.3-7: Message ATTACH ACCEPT (step 45)

| Derivation path: TS 36.508 table 4.7.2-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Extended DRX parameters |  |  |  |
| Paging Time Window | ‘0001’B | 2.56 seconds |  |
| eDRX value | ‘0011’B | 40.96 seconds |  |
| T3324 | Not Present |  |  |

<endo