3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220575

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0818 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function fl_SIP_BuildReferToUri_RX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current implementation of fl_SIP_BuildReferToUri_RX will fail in case that ReferTo Uri includes more than “Replace” header. The inclusion only of “Replace” header is expected for most of the test cases unless for 8.36 which require the addition of “Require” header. This will cause test case 8.36 to fail as Require header is not expected and will provoke a mismatch when REFER message is received. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Allowed ReferTo Uri to include more headers than “Replace” in header list. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.36.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	3

1.1	f_UT_RequestIMSCall	3

# Corrections required

## Correction to function fl_SIP_BuildReferToUri_RX

| Function name | fl_SIP_BuildReferToUri_RX |
| --- | --- |
| Reason for change | Current implementation of fl_SIP_BuildReferToUri_RX will fail in case that ReferTo Uri includes more than “Replace” header. The inclusion only of “Replace” header is expected for most of the test cases unless for 8.36 which require the addition of “Require” header. This will cause test case 8.36 to fail as Require header is not expected and will provoke a mismatch when REFER message is received. |
| Summary of change | Allowed ReferTo Uri to include more headers than “Replace” in header list. |
| TTCN module | IMS\Common\IMS_Procedures_Conferencing.ttcn |
| MCC160 Comment | Accepted in principle: According to the prose the Refer-To URI may have a Replaces header and in addition optionally a Require header but there are no other headers. By handing over p_ReplacesHeaderIsRequired the expected headers can be fully described and matched in the receive statement; the explicit checks in a_IMS_REFER_ReceiveRequest regarding presence of Replaces and Require header are not needed anymore. MCC160 Implementation |

Before Change:

<<SKIPPED CODE>> function fl_SIP_BuildReferToUri_RX(charstring p_ReferToUriString, boolean p_ReplacesHeaderIsRequired) return template (present) SipUrl { var template (present) SipUrl v_ReferToUri := f_SIP_BuildSipUri_RX(p_ReferToUriString); var template (present) UserInfo v_UserInfo; var template (present) UriComponents v_UriComponents; var template SemicolonParam_List v_Params; if (p_ReplacesHeaderIsRequired) { if (ischosen(v_ReferToUri.components.tel)) { /* convert tel URI to SIP URI acc. to RFC 3261 clause 19.1.6: */ v_UserInfo := cr_UserInfo(v_ReferToUri.components.tel.subscriber, omit); v_Params := superset(cr_GenericParam_Common("user", cr_GenValueToken("phone"))); // @sic R5s160295; NOTE: per default f_SIP_BuildSipUri_RX initialises urlParameters with * sic@ v_UriComponents := cr_SipUriComponent(v_UserInfo, *, *); v_ReferToUri := cr_SipUrl_Common(c_sipScheme, v_UriComponents, v_Params); } v_ReferToUri.headers := { cr_GenericParam_Common("Replaces", cr_GenValueToken(?)) }; } return v_ReferToUri; } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function fl_SIP_BuildReferToUri_RX(charstring p_ReferToUriString, boolean p_ReplacesHeaderIsRequired) return template (present) SipUrl { var template (present) SipUrl v_ReferToUri := f_SIP_BuildSipUri_RX(p_ReferToUriString); var template (present) UserInfo v_UserInfo; var template (present) UriComponents v_UriComponents; var template SemicolonParam_List v_Params; if (p_ReplacesHeaderIsRequired) { if (ischosen(v_ReferToUri.components.tel)) { /* convert tel URI to SIP URI acc. to RFC 3261 clause 19.1.6: */ v_UserInfo := cr_UserInfo(v_ReferToUri.components.tel.subscriber, omit); v_Params := superset(cr_GenericParam_Common("user", cr_GenValueToken("phone"))); // @sic R5s160295; NOTE: per default f_SIP_BuildSipUri_RX initialises urlParameters with * sic@ v_UriComponents := cr_SipUriComponent(v_UserInfo, *, *); v_ReferToUri := cr_SipUrl_Common(c_sipScheme, v_UriComponents, v_Params); } v_ReferToUri.headers := superset ( cr_GenericParam_Common("Replaces", cr_GenValueToken(?)) ); } return v_ReferToUri; } <<SKIPPED CODE>>

MCC160 Implementation

function fl_SIP_BuildReferToUri_RX(charstring p_ReferToUriString, boolean p_ReplacesHeaderIsRequired, boolean p_RequireHeaderIsRequired) return template (present) SipUrl { var template (present) SipUrl v_ReferToUri := f_SIP_BuildSipUri_RX(p_ReferToUriString); var template (present) UserInfo v_UserInfo; var template (present) UriComponents v_UriComponents; var template SemicolonParam_List v_Params; if (p_ReplacesHeaderIsRequired) { if (ischosen(v_ReferToUri.components.tel)) { /* convert tel URI to SIP URI acc. to RFC 3261 clause 19.1.6: */ v_UserInfo := cr_UserInfo(v_ReferToUri.components.tel.subscriber, omit); v_Params := superset(cr_GenericParam_Common("user", cr_GenValueToken("phone"))); // @sic R5s160295; NOTE: per default f_SIP_BuildSipUri_RX initialises urlParameters with * sic@ v_UriComponents := cr_SipUriComponent(v_UserInfo, *, *); v_ReferToUri := cr_SipUrl_Common(c_sipScheme, v_UriComponents, v_Params); } v_ReferToUri.headers[0] := cr_GenericParam_Common("Replaces", cr_GenValueToken(?)); if (p_RequireHeaderIsRequired) { /* @sic R5s220575 sic@ */ v_ReferToUri.headers[1] := cr_GenericParam("Require", "replaces"); } } return v_ReferToUri; }

altstep a_IMS_REFER_ReceiveRequest(out IMS_DATA_REQ p_IMS_REFER_REQ, charstring p_RequestUriString, charstring p_ReferToUriString, template (omit) IMS_PTC_DialogIndex_Type p_ActiveSessionToBeReplaced, boolean p_ReferCreatingDialog := false, IMS_CallType_Type p_CallType := CONFERENCE_CALL) runs on IMS_PTC { var IMS_DATA_REQ v_IMS_REFER_REQ_ByRef; var template (present) SipUrl v_RequestUriRefer := f_SIP_BuildSipUri_RX(p_RequestUriString); var boolean v_ReplacesHeaderIsRequired := ispresent(p_ActiveSessionToBeReplaced); var boolean v_RequireHeaderIsRequired := v_ReplacesHeaderIsRequired and (p_CallType == NORMAL_CALL); var template (present) SipUrl v_ReferToUri := fl_SIP_BuildReferToUri_RX(p_ReferToUriString, v_ReplacesHeaderIsRequired, v_RequireHeaderIsRequired); /* @sic R5s150382 change 3; R5s160043, R5-160796 MCC160 implementation sic@ */ var template (present) MessageHeader v_MessageHeader_Refer := f_IMS_ReferRequest_MessageHeaderRX(v_RequestUriRefer, v_ReferToUri, p_ReferCreatingDialog, -, p_CallType); var SipUrl v_ReferToUrl; var GenericParam v_Replaces; var template (value) CallId v_CallId; var charstring v_FromTag; var charstring v_ToTag; var CharStringList_Type v_ReceivedDialogId; var template (present) CharStringList_Type v_ExpectedDialogId; [] a_IMS_ReceiveRequest(car_IMS_Refer_Request(cr_REFER_Request(v_RequestUriRefer, v_MessageHeader_Refer)), v_IMS_REFER_REQ_ByRef) { f_IMS_A_2_10_Refer_CommonChecks(v_IMS_REFER_REQ_ByRef, p_ReferCreatingDialog); if (not p_ReferCreatingDialog) { /* @sic R5s160045: CSeq value to be incremented and checked for in-dialog REFER sic@ */ f_IMS_RequestInDialog_IncrAndCheckCSeqValue(v_IMS_REFER_REQ_ByRef.Request.Refer.msgHeader); } v_ReferToUrl := v_IMS_REFER_REQ_ByRef.Request.Refer.msgHeader.referTo.nameAddr.addrSpec; if (v_ReplacesHeaderIsRequired) { /* @sic R5-153765, R5s150909 sic@ */ v_Replaces := valueof(f_SIP_AmpersandParamList_GetParam(v_ReferToUrl.headers, "Replaces")); // if there is no Replaces header, v_ReferToUri does not match in the receive statement v_CallId := f_IMS_PTC_ImsInfo_DialogGetCallId(p_ActiveSessionToBeReplaced); v_FromTag := valueof(f_IMS_PTC_ImsInfo_DialogGetRemoteTag(p_ActiveSessionToBeReplaced)); // @sic R5s150909 sic@ v_ToTag := valueof(f_IMS_PTC_ImsInfo_DialogGetLocalTag(p_ActiveSessionToBeReplaced)); // @sic R5s150909 sic@ v_ExpectedDialogId := { v_CallId.callid, permutation("to-tag=" & v_ToTag, "from-tag=" & v_FromTag, *) }; v_ReceivedDialogId := f_StringSplit(f_URL_Decoding(v_Replaces.paramValue.tokenOrHost), {";"}); // see RFC 3891 6.1 if (not match(v_ReceivedDialogId, v_ExpectedDialogId)) { f_IMS_SetVerdictFailOrInconc(__FILE__, __LINE__, "Invalid dialog id in Replaces header of Refer-To's SipUri"); } } p_IMS_REFER_REQ := v_IMS_REFER_REQ_ByRef; // return message by ref } }