3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220611

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0828 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IMS 5GS emergency call test case 10.10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add Emergency Call test case 10.10. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to Emergency Call test case 10.10 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.10.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 34-229-5 CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | R5-222276 is raised during the RAN5#95-e meeting |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	6

2.3	Change 3	7

2.4	Change 4	9

2.5	Change 5	12

3	Branches executed	13

4	Execution Log Files	13

4.1	Qualcomm SM 8450+SDX65 5G UE	13

5	References	13

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 10.10 which is part of the NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	10.10

ATS Version:	iwd-TTCN3-B2020-09_D22wk12

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR,

UE used:	Qualcomm SM 8450+SDX65 5G UE

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_10_10_NR5GC_IMS2 |
| --- | --- |
| Reason for change | - Resource reservation, PDU Session Modification, will happen during call stablishment. With current implementation it may happen that PDU Session Modification procedure has not finished before Call is released (BYE is sent from SS). - After the call is released it is necessary to inform IPCAN and also to handle coordination trigger sent by IPCAN – both done by f_IMS_CC_EndCall. - Another coordination trigger to IPCAN is needed to coordinate PDU session release. |
| Summary of change | - Added a delay of 5s to give some time for PDU Session Modification Complete to NR PTC. - Added f_IMS_CC_EndCall in f_TC_10_10_NR5GC_IMS2 after the call is released. - Added coordination trigger to IPCAN |
| TTCN module | IMS\NR5GC_10\IMS_NR5GC_EmergencyCallTestcases.ttcn |
| MCC160 Comment | Not needed. Alternative solution in R5s220531 |

Before Change

function f_TC_10_10_NR5GC_IMS2() runs on IMS_PTC { /* IMS implementation for PDN2 - Emergency call */ var IMS_DATA_REQ v_IMS_DATA_REQ; var REGISTER_Request v_RegisterRequest; var template (value) MessageHeader v_MessageHeader; var template (value) MessageBody v_MessageBody; var SipUrl v_ContactUrl; var boolean v_CondA7 := true; f_IMS_PTC_Init(PDN_2); f_IMS_IMS_WaitForTrigger(OtherPDN); f_IMS_TestBody_Set(true); // @siclog "Steps 1-8" siclog@ // Steps 1-8 of Table 4.9.11.2.2-1 of TS 38.508-1 are performed. // EXCEPTION: In parallel to the events described in steps 9-10 below the events specified in steps 1a1 of Table 4.9.11.2.2-2 of TS 38.508-1 [21] take place // @siclog "Step 9-10" siclog @ // Steps 9-10 of Table 4.9.11.2.2-1 of TS 38.508-1 are performed. f_IMS_CC_StartCall(IPCAN_EmergencyCall_NoRegistration); // 4.9.11.2.2-2 of TS 38.508-1 // EXCEPTION: In parallel to the events described in steps 11-19 below the events specified in 11-13 of Table 4.9.11.2.2-1 of TS 38.508-1 [21] are performed. // @siclog "Step 11" siclog@ // UE sends REGISTER(Step 1 of Annex A.3) v_IMS_DATA_REQ := f_IMS_REGISTER_InitialRequest(-, omit, -, -, -, v_CondA7); v_RegisterRequest := v_IMS_DATA_REQ.Request.Register; // Reconfig of UE client ports in case the UE uses UDP and different than default (5060) ports f_IMS_Register_SecurityInit(v_RegisterRequest); // @siclog "Step 12" siclog@ // SS sends 403 Forbidden // The SS responds with a failure. v_MessageHeader := f_IMS_RegisterResponse_403_MessageHeaderTX(v_RegisterRequest); v_MessageHeader.contentType := cs_ContentType(c_ims3gppApplication); v_MessageBody := cs_MessageBody_XML_Ims_3gpp(cs_Ims_3gpp_Emergency("anonymous-emergencycall")); IMS_Server.send(cas_IMS_DATA_RSP(f_IMS_RoutingInfo_ULtoDL(v_IMS_DATA_REQ.RoutingInfo), cs_Response(c_statusLine403, v_MessageHeader, v_MessageBody))); f_IMS_PTC_RegistrationInfo_Reset(); f_IMS_PTC_Reset(); f_IMS_PTC_SetRanType(NR); // Ran Type Condition A28 f_IMS_GlobalRelease(oldestProtectedContextOnly); // @siclog "Step 13" siclog@ // Check: Does the UE send a correctly composed INVITE request? // Condition A28 checked v_IMS_DATA_REQ := f_IMS_EmergencyCallSetup_NR5GC_A_6_Step1(A_2_1_A6); // Check for INVITE according to Table 10.10.3.3-2 f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 13"); //@siclog "Step 14 - 17" siclog@ //SS sends 100 Trying.(Step 2 of annex A.6) //SS sends 180 Ringing //SS sends 200 OK.(Step 4 of annex A.6) //UE sends ACK.(Step 5 of annex A.6) f_IMS_EmergencyCallSetup_NR5GC_A_6_Step2ToEnd(v_IMS_DATA_REQ); //@siclog "Steps 18-19" siclog@ // SS sends BYE.(Step 1 of annex A.8) //UE sends 200 OK.(Step 2 of annex A.8) v_ContactUrl := f_IMS_PTC_ImsInfo_GetContactUrl(dialog); f_IMS_CallReleaseMT(v_ContactUrl); f_IMS_TestBody_Set(false); f_IMS_IMS_SendCoOrdMsg(OtherPDN); f_IMS_TCP_CloseHandling(waitAndClose); }

After Change

function f_TC_10_10_NR5GC_IMS2() runs on IMS_PTC { /* IMS implementation for PDN2 - Emergency call */ var IMS_DATA_REQ v_IMS_DATA_REQ; var REGISTER_Request v_RegisterRequest; var template (value) MessageHeader v_MessageHeader; var template (value) MessageBody v_MessageBody; var SipUrl v_ContactUrl; var boolean v_CondA7 := true; f_IMS_PTC_Init(PDN_2); f_IMS_IMS_WaitForTrigger(OtherPDN); f_IMS_TestBody_Set(true); // @siclog "Steps 1-8" siclog@ // Steps 1-8 of Table 4.9.11.2.2-1 of TS 38.508-1 are performed. // EXCEPTION: In parallel to the events described in steps 9-10 below the events specified in steps 1a1 of Table 4.9.11.2.2-2 of TS 38.508-1 [21] take place // @siclog "Step 9-10" siclog @ // Steps 9-10 of Table 4.9.11.2.2-1 of TS 38.508-1 are performed. f_IMS_CC_StartCall(IPCAN_EmergencyCall_NoRegistration); // 4.9.11.2.2-2 of TS 38.508-1 // EXCEPTION: In parallel to the events described in steps 11-19 below the events specified in 11-13 of Table 4.9.11.2.2-1 of TS 38.508-1 [21] are performed. // @siclog "Step 11" siclog@ // UE sends REGISTER(Step 1 of Annex A.3) v_IMS_DATA_REQ := f_IMS_REGISTER_InitialRequest(-, omit, -, -, -, v_CondA7); v_RegisterRequest := v_IMS_DATA_REQ.Request.Register; // Reconfig of UE client ports in case the UE uses UDP and different than default (5060) ports f_IMS_Register_SecurityInit(v_RegisterRequest); // @siclog "Step 12" siclog@ // SS sends 403 Forbidden // The SS responds with a failure. v_MessageHeader := f_IMS_RegisterResponse_403_MessageHeaderTX(v_RegisterRequest); v_MessageHeader.contentType := cs_ContentType(c_ims3gppApplication); v_MessageBody := cs_MessageBody_XML_Ims_3gpp(cs_Ims_3gpp_Emergency("anonymous-emergencycall")); IMS_Server.send(cas_IMS_DATA_RSP(f_IMS_RoutingInfo_ULtoDL(v_IMS_DATA_REQ.RoutingInfo), cs_Response(c_statusLine403, v_MessageHeader, v_MessageBody))); f_IMS_PTC_RegistrationInfo_Reset(); f_IMS_PTC_Reset(); f_IMS_PTC_SetRanType(NR); // Ran Type Condition A28 f_IMS_GlobalRelease(oldestProtectedContextOnly); // @siclog "Step 13" siclog@ // Check: Does the UE send a correctly composed INVITE request? // Condition A28 checked v_IMS_DATA_REQ := f_IMS_EmergencyCallSetup_NR5GC_A_6_Step1(A_2_1_A6); // Check for INVITE according to Table 10.10.3.3-2 f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 13"); //@siclog "Step 14 - 17" siclog@ //SS sends 100 Trying.(Step 2 of annex A.6) //SS sends 180 Ringing //SS sends 200 OK.(Step 4 of annex A.6) //UE sends ACK.(Step 5 of annex A.6) f_IMS_EmergencyCallSetup_NR5GC_A_6_Step2ToEnd(v_IMS_DATA_REQ); f_Delay(5.0); //@siclog "Steps 18-19" siclog@ // SS sends BYE.(Step 1 of annex A.8) //UE sends 200 OK.(Step 2 of annex A.8) v_ContactUrl := f_IMS_PTC_ImsInfo_GetContactUrl(dialog); f_IMS_CallReleaseMT(v_ContactUrl); f_IMS_CC_EndCall(IPCAN_EmergencyCall_NormalService, IPCAN_SpeechAndEmergencyCall); f_IMS_TestBody_Set(false); // Inform NR that call is released (caught by f_NR5GC_EmergencyCallRelease) f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IMS_SendCoOrdMsg(OtherPDN); f_IMS_TCP_CloseHandling(waitAndClose); }

## Change 2

| Function name | f_TC_10_10_NR5GC_IMS1 |
| --- | --- |
| Reason for change | - The f_IMS_CC_Postamble (if called with TestProcedure parameter) is informing IPCAN to release PDU session, however that was already done from IMS2 and doing it again from IMS1 would lead to runtime error as there are no more PDU sessions. - Test purpose dictates that UE registers on V-PLMN and makes the emergency call there. |
| Summary of change | - f_IMS_CC_Postamble is not called with the TestProcedure parameter. - Added new test configuration “IPCAN_EmergencyCall_VPLMN” that uses NR_cell12 which by default is not HPLMN. This change requires also Change 4 and 5 in this document. Prose CR(R5-222276) is raised during RAN5#95-e. |
| TTCN module | IMS_NR5GC_EmergencyCallTestcases.ttcn |
| MCC160 Comment | - Alternative solution implemented: moved existing function call from IMS1 to IMS2 - Accepted in principle but instead added new TestProcedure_Type IPCAN_UseVPLMN. See Change 5 |

Before Change

function f_TC_10_10_NR5GC_IMS1() runs on IMS_PTC { // Emergency call without emergency registration / Failure of registration / Rejected by 403(Forbidden) var boolean v_GeolocationUsage := pc_IMS_Geolocation; //The UE is in test state 1N-A (TS 38.508-1) and registered to IMS. f_IMS_CC_Preamble(IPCAN_EmergencyCall, IMS_REGISTERED); // Trigger SS to execute location related functions to allow UE to get its position if (v_GeolocationUsage) { f_IMS_IPCAN_UpdateUELocationInformation(IPCAN); } f_IMS_IMS_SendCoOrdMsg(OtherPDN); f_IMS_IMS_WaitForTrigger(OtherPDN); // wait for trigger after IMS call release f_IMS_CC_Postamble(IPCAN_EmergencyCall_NoRegistration); }

After Change

function f_TC_10_10_NR5GC_IMS1() runs on IMS_PTC { // Emergency call without emergency registration / Failure of registration / Rejected by 403(Forbidden) var boolean v_GeolocationUsage := pc_IMS_Geolocation; //The UE is in test state 1N-A (TS 38.508-1) and registered to IMS. f_IMS_CC_Preamble(IPCAN_EmergencyCall_VPLMN, IMS_REGISTERED); // Trigger SS to execute location related functions to allow UE to get its position if (v_GeolocationUsage) { f_IMS_IPCAN_UpdateUELocationInformation(IPCAN); } f_IMS_IMS_SendCoOrdMsg(OtherPDN); f_IMS_IMS_WaitForTrigger(OtherPDN); // wait for trigger after IMS call release f_IMS_CC_Postamble(); }

MCC 160 Implementation

function f_TC_10_10_NR5GC_IMS1() runs on IMS_PTC

{ // Emergency call without emergency registration / Failure of registration / Rejected by 403(Forbidden)

var boolean v_GeolocationUsage := pc_IMS_Geolocation;

//The UE is in test state 1N-A (TS 38.508-1) and registered to IMS.

f_IMS_CC_Preamble(IPCAN_EmergencyCall, IMS_REGISTERED, -, -, -, -, IPCAN_UseVPLMN); //@sic R5s220611 sic@

// Trigger SS to execute location related functions to allow UE to get its position

if  (v_GeolocationUsage) {

f_IMS_IPCAN_UpdateUELocationInformation(IPCAN);

}

f_IMS_IMS_SendCoOrdMsg(OtherPDN);

}

/*

* @desc      REFERENCE TS 34.229-5 clause 10.10

* @status

*/

function f_TC_10_10_NR5GC_IMS2() runs on IMS_PTC

{ /* IMS implementation for PDN2 - Emergency call */

var IMS_DATA_REQ v_IMS_DATA_REQ;

var REGISTER_Request v_RegisterRequest;

var template (value) MessageHeader v_MessageHeader;

var template (value) MessageBody v_MessageBody;

var SipUrl v_ContactUrl;

var boolean v_CondA7 := true;

f_IMS_PTC_Init(PDN_2);

f_IMS_IMS_WaitForTrigger(OtherPDN);

f_IMS_TestBody_Set(true);

// @siclog "Steps 1-8" siclog@

// Steps 1-8 of Table 4.9.11.2.2-1 of TS 38.508-1 are performed.

// EXCEPTION: In parallel to the events described in steps 9-10 below the events specified in steps 1a1 of Table 4.9.11.2.2-2 of TS 38.508-1 [21] take place

// @siclog "Step 9-10" siclog @

// Steps 9-10 of Table 4.9.11.2.2-1 of TS 38.508-1 are performed.

f_IMS_CC_StartCall(IPCAN_EmergencyCall_NoRegistration); // 4.9.11.2.2-2 of TS 38.508-1

// EXCEPTION: In parallel to the events described in steps 11-19 below the events specified in 11-13 of Table 4.9.11.2.2-1 of TS 38.508-1 [21] are performed.

// @siclog "Step 11" siclog@

// UE sends REGISTER(Step 1 of Annex A.3)

v_IMS_DATA_REQ := f_IMS_REGISTER_InitialRequest(-, omit, -, -, -, v_CondA7);

v_RegisterRequest := v_IMS_DATA_REQ.Request.Register;

// Reconfig of UE client ports in case the UE uses UDP and different than default (5060) ports

f_IMS_Register_SecurityInit(v_RegisterRequest);

// @siclog "Step 12" siclog@

// SS sends 403 Forbidden

// The SS responds with a failure.

v_MessageHeader := f_IMS_RegisterResponse_403_MessageHeaderTX(v_RegisterRequest);

v_MessageHeader.contentType := cs_ContentType(c_ims3gppApplication);

v_MessageBody := cs_MessageBody_XML_Ims_3gpp(cs_Ims_3gpp_Emergency("anonymous-emergencycall"));

IMS_Server.send(cas_IMS_DATA_RSP(f_IMS_RoutingInfo_ULtoDL(v_IMS_DATA_REQ.RoutingInfo), cs_Response(c_statusLine403, v_MessageHeader, v_MessageBody)));

f_IMS_PTC_SetRanType(NR); // Ran Type Condition A28

f_IMS_GlobalRelease(keepUnprotected);  // @sic R5s220609 - Additional changes: keepUnprotected instead of oldestProtectedContextOnly sic@

// @siclog "Step 13" siclog@

// Check: Does the UE send a correctly composed INVITE request?

// Condition A28 checked

v_IMS_DATA_REQ := f_IMS_EmergencyCallSetup_NR5GC_A_6_Step1(A_2_1_A6);

// Check for INVITE according to Table 10.10.3.3-2

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 13");

//@siclog "Step 14 - 17" siclog@

//SS sends 100 Trying.(Step 2 of annex A.6)

//SS sends 180 Ringing

//SS sends 200 OK.(Step 4 of annex A.6)

//UE sends ACK.(Step 5 of annex A.6)

f_IMS_EmergencyCallSetup_NR5GC_A_6_Step2ToEnd(v_IMS_DATA_REQ);

//@siclog "Steps 18-19" siclog@

// SS sends BYE.(Step 1 of annex A.8)

//UE sends 200 OK.(Step 2 of annex A.8)

v_ContactUrl := f_IMS_PTC_ImsInfo_GetContactUrl(dialog);

f_IMS_CallReleaseMT(v_ContactUrl);

f_IMS_TestBody_Set(false);

f_IMS_CC_Postamble(IPCAN_EmergencyCall_NoRegistration); // @sic R5s220611sic@

}

## Change 3

| Function name | fl_IP_IMS_Server_SendDL |
| --- | --- |
| Reason for change | In scenarios where IMS security establishment failed but SIP message needs to be sent in DL nonetheless (emergency call with failed/rejected 401 procedure, e.g. tests 10.9 and 10.10) it is needed to assign UE/NW addresses for routing. |
| Summary of change | Added routing addresses into fl_IP_IMS_Server_SendDL also when only unprotected context is present. |
| TTCN module | IP_PTC\IP_PTC_IMS_Handler.ttcn |
| MCC160 Comment | Not needed. Alternative solution in R5s220609 |

Before Change

function fl_IP_IMS_Server_SendDL(inout IMS_Server_Type p_ImsServer, IMS_RoutingInfo_Type p_RoutingInfo, IMS_RequestResponse_Type p_RequestResponse, bitstring p_Data) runs on IP_PTC { … if (isvalue(p_RoutingInfo.NW_Address) and isvalue(p_RoutingInfo.UE_Address)) { v_NW_Address := p_RoutingInfo.NW_Address; v_UE_Address := p_RoutingInfo.UE_Address; } else if (ispresent(p_ImsServer.RegistrationAddress)) { /* @sic R5s160735: fl_ImsServer_UeIsRegistered removed sic@ */ v_NW_Address := p_ImsServer.RegistrationAddress.NW; v_UE_Address := p_ImsServer.RegistrationAddress.UE; } else { FatalError(__FILE__, __LINE__, "missing IP addresses"); } …

After Change

function fl_IP_IMS_Server_SendDL(inout IMS_Server_Type p_ImsServer, IMS_RoutingInfo_Type p_RoutingInfo, IMS_RequestResponse_Type p_RequestResponse, bitstring p_Data) runs on IP_PTC { … if (isvalue(p_RoutingInfo.NW_Address) and isvalue(p_RoutingInfo.UE_Address)) { v_NW_Address := p_RoutingInfo.NW_Address; v_UE_Address := p_RoutingInfo.UE_Address; } else if (ispresent(p_ImsServer.RegistrationAddress)) { /* @sic R5s160735: fl_ImsServer_UeIsRegistered removed sic@ */ v_NW_Address := p_ImsServer.RegistrationAddress.NW; v_UE_Address := p_ImsServer.RegistrationAddress.UE; } else if (ispresent(p_ImsServer.UnprotectedContext.TcpServer)) { v_NW_Address := p_ImsServer.UnprotectedContext.TcpServer.Local.IpAddr; v_UE_Address := p_ImsServer.UnprotectedContext.TcpServer.Remote.IpAddr; } else { FatalError(__FILE__, __LINE__, "missing IP addresses"); } …

## Change 4

| Function name | f_NR5GC_IPCAN_MainLoop |
| --- | --- |
| Reason for change | To accommodate Change 2 bullet 2 |
| Summary of change | Added new test configuration “IPCAN_EmergencyCall_VPLMN” that uses NR_cell12 which by default is not HPLMN. |
| TTCN module | PART3_IPCAN\IPCAN_NR5GC.ttcn |
| MCC160 Comment | Accepted in principle. Similar changes also made to IPCAN_EUTRA. |

Before Change

function f_NR5GC_IPCAN_MainLoop() runs on NR5GC_PTC { var NR_CellId_Type v_CellId := tsc_IMS_NR5GC_CellId; var IMS_IPCAN_Coordination_MSG v_IMS_IPCAN_Coordination_MSG; var IMS_IPCAN_Command_Type v_Command; var IMS_TestConfiguration_Type v_TestConfiguration; var TriggerResult_Type v_StartProcedureResult; var NR_SRB_COMMON_IND v_ReceivedAsp; var integer v_PdnIndex; var boolean v_ConnectionEstablished := false; var boolean v_IPCAN_Registered := false; // @sic R5s211495 sic@ var boolean v_Done := false; while (not v_Done) { alt { [] IMS[tsc_Index_IMS1].receive(cmr_IMS_IPCAN_Command) -> value v_IMS_IPCAN_Coordination_MSG { v_PdnIndex := tsc_Index_IMS1; } [] IMS[tsc_Index_IMS2].receive(cmr_IMS_IPCAN_Command) -> value v_IMS_IPCAN_Coordination_MSG { v_PdnIndex := tsc_Index_IMS2; } // @sic R5s211495 sic@ [v_ConnectionEstablished]SRB.receive(car_NR_SRB_NasPdu_IND(v_CellId, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered)))) -> value v_ReceivedAsp { v_IPCAN_Registered := f_NR5GC_IPCAN_AfterIMSDeregister(v_CellId, v_ReceivedAsp); repeat; } } v_Command := v_IMS_IPCAN_Coordination_MSG.IMS_IPCAN_Command; select (v_Command.Name) { case (IPCAN_INIT) { f_NR5GC_IPCAN_Init(IMS[v_PdnIndex], v_CellId); v_TestConfiguration := v_Command.TestConfiguration; } case (IPCAN_PROVIDELOCATIONINFORMATION) { f_NR5GC_IPCAN_LocationInformation(IMS[v_PdnIndex], v_CellId); } case (IPCAN_STARTPROCEDURE) { v_StartProcedureResult := f_NR5GC_IPCAN_StartProcedure(IMS[v_PdnIndex], v_CellId, v_Command.TestProcedure, v_ConnectionEstablished); v_ConnectionEstablished := true; if ( v_Command.TestProcedure == IPCAN_InitialRegistration) { // @sic R5s211495 sic@ v_IPCAN_Registered := true; } } case (IPCAN_ENDPROCEDURE) { if (v_TestConfiguration == IPCAN_SpeechAndEmergencyCall and isvalue(v_Command.TestConfiguration)) { // @sic R5-216220 sic@ // Just end the procedure as there's 2 calls ongoing, don't release the connection f_NR5GC_IPCAN_EndCall(IMS[v_PdnIndex], v_CellId, valueof(v_Command.TestConfiguration), v_Command.TestProcedure, v_StartProcedureResult); } else { f_NR5GC_IPCAN_ConnectionRelease(IMS[v_PdnIndex], v_CellId, v_Command.TestProcedure, v_StartProcedureResult); /* => RRC connection is released */ v_ConnectionEstablished := false; } } case (IPCAN_RELEASE) { f_NR5GC_IPCAN_Release(IMS[v_PdnIndex], v_CellId, v_IPCAN_Registered); // @sic R5s211495 sic@ v_Done := true; } case (IPCAN_QUERY) { /* @sic R5s130133 additional changes: handling of queries from the IMS PTC sic@ */ IMS[v_PdnIndex].send(cms_IPCAN_IMS_QueryResponse(f_NR_IMS_IpcanInfo())); } case else { FatalError(__FILE__, __LINE__, "invalid command"); } } } }

After Change

const NR_CellId_Type tsc_IMS_NR5GC_CellId_VPLMN := nr_Cell12; … function f_NR5GC_IPCAN_MainLoop() runs on NR5GC_PTC { var NR_CellId_Type v_CellId := tsc_IMS_NR5GC_CellId; var IMS_IPCAN_Coordination_MSG v_IMS_IPCAN_Coordination_MSG; var IMS_IPCAN_Command_Type v_Command; var IMS_TestConfiguration_Type v_TestConfiguration; var TriggerResult_Type v_StartProcedureResult; var NR_SRB_COMMON_IND v_ReceivedAsp; var integer v_PdnIndex; var boolean v_ConnectionEstablished := false; var boolean v_IPCAN_Registered := false; // @sic R5s211495 sic@ var boolean v_Done := false; while (not v_Done) { alt { [] IMS[tsc_Index_IMS1].receive(cmr_IMS_IPCAN_Command) -> value v_IMS_IPCAN_Coordination_MSG { v_PdnIndex := tsc_Index_IMS1; } [] IMS[tsc_Index_IMS2].receive(cmr_IMS_IPCAN_Command) -> value v_IMS_IPCAN_Coordination_MSG { v_PdnIndex := tsc_Index_IMS2; } // @sic R5s211495 sic@ [v_ConnectionEstablished]SRB.receive(car_NR_SRB_NasPdu_IND(v_CellId, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered)))) -> value v_ReceivedAsp { v_IPCAN_Registered := f_NR5GC_IPCAN_AfterIMSDeregister(v_CellId, v_ReceivedAsp); repeat; } } v_Command := v_IMS_IPCAN_Coordination_MSG.IMS_IPCAN_Command; select (v_Command.Name) { case (IPCAN_INIT) { v_TestConfiguration := v_Command.TestConfiguration; if (v_Command.TestConfiguration == IPCAN_EmergencyCall_VPLMN) {v_CellId := tsc_IMS_NR5GC_CellId_VPLMN} f_NR5GC_IPCAN_Init(IMS[v_PdnIndex], v_CellId); } case (IPCAN_PROVIDELOCATIONINFORMATION) { f_NR5GC_IPCAN_LocationInformation(IMS[v_PdnIndex], v_CellId); } case (IPCAN_STARTPROCEDURE) { v_StartProcedureResult := f_NR5GC_IPCAN_StartProcedure(IMS[v_PdnIndex], v_CellId, v_Command.TestProcedure, v_ConnectionEstablished); v_ConnectionEstablished := true; if ( v_Command.TestProcedure == IPCAN_InitialRegistration) { // @sic R5s211495 sic@ v_IPCAN_Registered := true; } } case (IPCAN_ENDPROCEDURE) { if (v_TestConfiguration == IPCAN_SpeechAndEmergencyCall and isvalue(v_Command.TestConfiguration)) { // @sic R5-216220 sic@ // Just end the procedure as there's 2 calls ongoing, don't release the connection f_NR5GC_IPCAN_EndCall(IMS[v_PdnIndex], v_CellId, valueof(v_Command.TestConfiguration), v_Command.TestProcedure, v_StartProcedureResult); } else { f_NR5GC_IPCAN_ConnectionRelease(IMS[v_PdnIndex], v_CellId, v_Command.TestProcedure, v_StartProcedureResult); /* => RRC connection is released */ v_ConnectionEstablished := false; } } case (IPCAN_RELEASE) { f_NR5GC_IPCAN_Release(IMS[v_PdnIndex], v_CellId, v_IPCAN_Registered); // @sic R5s211495 sic@ v_Done := true; } case (IPCAN_QUERY) { /* @sic R5s130133 additional changes: handling of queries from the IMS PTC sic@ */ IMS[v_PdnIndex].send(cms_IPCAN_IMS_QueryResponse(f_NR_IMS_IpcanInfo())); } case else { FatalError(__FILE__, __LINE__, "invalid command"); } } } }

## Change 5

| Type name | IMS_TestConfiguration_Type |
| --- | --- |
| Reason for change | To accommodate Change 2 bullet 2 |
| Summary of change | Added new test configuration “IPCAN_EmergencyCall_VPLMN” that uses NR_cell12 which by default is not HPLMN. |
| TTCN module | IMS_PTC_CoordMsg.ttcn |
| MCC160 Comment | Accepted in principle, but instead added new TestProcedure_Type IPCAN_UseVPLMN |

Before Change

type enumerated IMS_TestConfiguration_Type { /* @status APPROVED (ENDC, IMS, IMS_IRAT, LTE, LTE_A_IRAT, LTE_A_PRO, LTE_A_R10_R11, LTE_A_R12, LTE_IRAT, NBIOT, NR5GC, NR5GC_IRAT, POS, UTRAN) */ IPCAN_SignallingOnly, /* EUTRA: default DRB is used only */ IPCAN_SpeechCall, /* EUTRA: one dedicated UM bearer; for normal speech calls and emergency call for limited services */ IPCAN_VideoCall, /* EUTRA: two dedicated UM bearers */ IPCAN_EmergencyCall, /* EUTRA: second default bearer (AM) and one dedicated UM bearer */ IPCAN_SpeechAndEmergencyCall, /* EUTRA: default bearer + dedicated bearer for normal speech call and another default bearer + dedicated bearer for emergency call */ IPCAN_XCAP, /* EUTRA: second default bearer (AM) for second PDN used for XCAP signalling */ IPCAN_DataCall /* MCData: one dedicated UM bearer */ };

After Change

type enumerated IMS_TestConfiguration_Type { /* @status APPROVED (ENDC, IMS, IMS_IRAT, LTE, LTE_A_IRAT, LTE_A_PRO, LTE_A_R10_R11, LTE_A_R12, LTE_IRAT, NBIOT, NR5GC, NR5GC_IRAT, POS, UTRAN) */ IPCAN_SignallingOnly, /* EUTRA: default DRB is used only */ IPCAN_SpeechCall, /* EUTRA: one dedicated UM bearer; for normal speech calls and emergency call for limited services */ IPCAN_VideoCall, /* EUTRA: two dedicated UM bearers */ IPCAN_EmergencyCall, /* EUTRA: second default bearer (AM) and one dedicated UM bearer */ IPCAN_SpeechAndEmergencyCall, /* EUTRA: default bearer + dedicated bearer for normal speech call and another default bearer + dedicated bearer for emergency call */ IPCAN_XCAP, /* EUTRA: second default bearer (AM) for second PDN used for XCAP signalling */ IPCAN_DataCall, /* MCData: one dedicated UM bearer */ IPCAN_EmergencyCall_VPLMN };

#### MCC160 Implementation:

type enumerated IMS_TestProcedure_Type {      /* @status    APPROVED (ENDC, IMS, IMS_IRAT, LTE, LTE_A_IRAT, LTE_A_PRO, LTE_A_R10_R11, LTE_A_R12, LTE_IRAT, NBIOT, NR5GC, NR5GC_IRAT, POS, UTRAN) */

IPCAN_InitialRegistration,                  /* EUTRA/EPS signalling acc. to 36.508 cl. 4.5.2.3 without RRC Connection Release at the end of the procedure

* NOTE: As working assumption the UE does IMS REGISTRATION automatically after RRC/NAS registration */

IPCAN_EmergencyCall_NormalService,          /* EUTRA/EPS signalling acc. to 36.508 cl. 4.5A.4.3 */

IPCAN_EmergencyCall_NoRegistration,         /* EUTRA/EPS signalling acc. to 36.508 cl. 4.5A.4.3, but with Call Release according to 34.229-1 C.32a */

IPCAN_EmergencyCall_LimitedService,         /* EUTRA/EPS signalling acc. to 36.508 cl. 4.5A.5.3 */

IPCAN_MO_SpeechCall,                        /* EUTRA/EPS signalling acc. to 36.508 cl. 4.5A.6.3 */

IPCAN_MT_SpeechCall,                        /* EUTRA/EPS signalling acc. to 36.508 cl. 4.5A.7.3 */

IPCAN_MO_VideoCall,                         /* EUTRA/EPS signalling acc. to 36.508 cl. 4.5A.8.3 */

IPCAN_MT_VideoCall,                         /* EUTRA/EPS signalling acc. to 36.508 cl. 4.5A.9.3 */

IPCAN_MO_AddVideo,                          /* EUTRA/EPS signalling acc. to 36.508 cl. 4.5A.11.3 */

IPCAN_MT_AddVideo,                          /* EUTRA/EPS signalling acc. to 36.508 cl. 4.5A.12.3 */

IPCAN_ReleaseVideo,                         /* EUTRA/EPS signalling acc. to 36.508 cl. 4.5A.15.3 */

IPCAN_MO_IMS_Signalling,                    /* EUTRA/EPS signalling acc. to 36.508 cl. 4.5.3.3 with m = n = 0; used e.g. for MT SMS test case 18.2 */

IPCAN_MT_IMS_Signalling,                    /* EUTRA/EPS signalling acc. to 36.508 cl. 4.5.3.3 Steps 3 to 9 with m = n = 0; used e.g. for MT SMS test case 18.1 */

IPCAN_XCAP_Signalling,                      /* EUTRA/EPS signalling acc. to 36.508 cl. 4.5A.14 */

IPCAN_MO_DataCall,                          /* for MCData test cases */

IPCAN_MT_DataCall,                          /* for MCData test cases */

IPCAN_AdditionalIPAddress,                  /* CR 213400 */

IPCAN_Deregistration,                       /* @sic R5s211495 sic@ */

IPCAN_UseVPLMN

template (value) IMS_IPCAN_Coordination_MSG cms_IMS_IPCAN_InitReq(IMS_TestConfiguration_Type p_TestConfiguration, template (omit) IMS_TestProcedure_Type p_TestProcedure)    := cms_IMS_IPCAN_Command(IPCAN_INIT, p_TestConfiguration, p_TestProcedure);          /* @status    APPROVED (IMS)  */

function f_IMS_IPCAN_Init(IMS_IPCAN_CO_ORD_PORT p_Port,

IMS_TestConfiguration_Type p_TestConfiguration,

template (omit) IMS_TestProcedure_Type p_TestProcedure := omit

)

{ /* p_TestConfiguration specifies the DRBs needed by the test cases so that IPCAN can do pre-configuration of DRBs accordingly */

p_Port.send(cms_IMS_IPCAN_InitReq(p_TestConfiguration, p_TestProcedure)); // @sic R5s220611 sic@

alt {

[] p_Port.receive(cmr_IPCAN_IMS_Response) {}

[] a_IMS_IPCAN_Default(p_Port) {}

}

}

function f_IMS_CC_Preamble(IMS_TestConfiguration_Type p_TestConfiguration,

IMS_InitState_Type p_InitState,

template (omit) IMS_SecurityScheme_Type p_SecurityScheme := omit,

template (omit) IPCAN_RAN_Type p_RanType := omit,

boolean p_SigCompIsActive := false,

integer p_NumOfIPAddressesToAllocate := 1,

template (omit) IMS_TestProcedure_Type p_TestProcedure := omit

) runs on IMS_PTC

{ /* p_TestProcedure  ..  major test procedure of the test case (=> number/type of DRBs to be configured at SS

* p_InitState      ..  IDLE => intitial registration including IMS registration is done; RRC connection is released

*                      NULL => intitial registration is started at the IPCAN but IMS registration shall be controled by the test case */

/* @sic R5s130133 additional changes: XRES length shall be queried by the IMS PTC i.e. is not returned by f_IMS_IPCAN_StartProcedure_Preamble anymore sic@ */

/* @sic R5s130347: local GuardTimer sic@ */

/* @sic R5s130756 additional changes: p_IsGIBA replaced by p_SecurityScheme sic@ */

timer t_GuardTimer := tsc_GuardTimePreamble;

t_GuardTimer.start;

f_IMS_PTC_Init(PDN_1, -, p_SecurityScheme, p_RanType);      /* local initialisation */

f_IMS_IPCAN_Init(IPCAN, p_TestConfiguration, p_TestProcedure); /* initialisation of IPCAN:

- RAN type is selected by IPCAN PTC acc. to the PIXITs

- cell is created acc. to the RAN type and PIXITs

- Test procedure is handed over as for e.g. EUTRA we need to know in advance which DRBs need to be (pre-)configured at the SS

-                                      or if need to use a different cell to default (Cell 1) @sic R5s220611 sic@

p_TestcaseName is present If the test case requires a specific IPCAN initialisation e.g. for cell barring */

if (p_NumOfIPAddressesToAllocate != 1) { // @sic R5-213400 sic@

f_IMS_IPCAN_StartProcedure(IPCAN, IPCAN_AdditionalIPAddress);

}

if (p_InitState == IMS_REGISTERED) {

f_IMS_IPCAN_StartProcedure(IPCAN, IPCAN_InitialRegistration);

f_IMS_Registration(-, -, p_SigCompIsActive);

f_IMS_CC_ReleaseConnection(IPCAN_InitialRegistration);

}

t_GuardTimer.stop;

}

function f_NR5GC_IPCAN_MainLoop() runs on NR5GC_PTC

{

var NR_CellId_Type v_CellId := tsc_IMS_NR5GC_CellId;

var IMS_IPCAN_Coordination_MSG v_IMS_IPCAN_Coordination_MSG;

var IMS_IPCAN_Command_Type v_Command;

var IMS_TestConfiguration_Type v_TestConfiguration;

var TriggerResult_Type v_StartProcedureResult;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var integer v_PdnIndex;

var boolean v_ConnectionEstablished := false;

var boolean v_IPCAN_Registered := false;   // @sic R5s211495 sic@

var boolean v_Done := false;

while (not v_Done) {

alt {

[] IMS[tsc_Index_IMS1].receive(cmr_IMS_IPCAN_Command) -> value v_IMS_IPCAN_Coordination_MSG {

v_PdnIndex := tsc_Index_IMS1;

}

[] IMS[tsc_Index_IMS2].receive(cmr_IMS_IPCAN_Command) -> value v_IMS_IPCAN_Coordination_MSG {

v_PdnIndex := tsc_Index_IMS2;

}

// @sic R5s211495 sic@

[v_ConnectionEstablished]SRB.receive(car_NR_SRB_NasPdu_IND(v_CellId, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered)))) -> value v_ReceivedAsp {

v_IPCAN_Registered := f_NR5GC_IPCAN_AfterIMSDeregister(v_CellId, v_ReceivedAsp);

repeat;

}

}

v_Command := v_IMS_IPCAN_Coordination_MSG.IMS_IPCAN_Command;

select (v_Command.Name) {

case (IPCAN_INIT) {

if (isvalue(v_Command.TestProcedure)) {

if (v_Command.TestProcedure == IPCAN_UseVPLMN) {

v_CellId := tsc_IMS_NR5GC_CellId_VPLMN;

}

}

f_NR5GC_IPCAN_Init(IMS[v_PdnIndex], v_CellId);

v_TestConfiguration := v_Command.TestConfiguration;

}

case (IPCAN_PROVIDELOCATIONINFORMATION) {

...

function f_EUTRA_IPCAN_MainLoop() runs on EUTRA_PTC

{

var EUTRA_CellId_Type v_CellId := tsc_IMS_EUTRA_CellId;

var IMS_IPCAN_Coordination_MSG v_IMS_IPCAN_Coordination_MSG;

var IMS_IPCAN_Command_Type v_Command;

var integer v_PdnIndex;

var boolean v_DedicatedBearersEstablished := false;  // will be set to true e.g. after successful call establishment (note: there are test cases with unsuccessful call establishment)

var boolean v_ConnectionEstablished := false;        /* @sic R5s140490 change 7 MCC160 implementation sic@ */

var boolean v_Done := false;

var integer v_NextAvailableEpsBearerId := hex2int(tsc_EpsDedicatedBearerId);  // the next EPS bearer available is to be used as default for IMS Normal Emg Call

var IMS_TestConfiguration_Type v_TestConfiguration;

while (not v_Done) {

alt {

[] IMS[tsc_Index_IMS1].receive(cmr_IMS_IPCAN_Command) -> value v_IMS_IPCAN_Coordination_MSG {

v_PdnIndex := tsc_Index_IMS1;

}

[] IMS[tsc_Index_IMS2].receive(cmr_IMS_IPCAN_Command) -> value v_IMS_IPCAN_Coordination_MSG {

v_PdnIndex := tsc_Index_IMS2;

}

}

v_Command := v_IMS_IPCAN_Coordination_MSG.IMS_IPCAN_Command;

select (v_Command.Name) {

case (IPCAN_INIT) {

if (isvalue(v_Command.TestProcedure)) {

if (v_Command.TestProcedure == IPCAN_UseVPLMN) {

v_CellId := tsc_IMS_EUTRA_CellId_VPLMN;

}

}

f_EUTRA_IPCAN_Init(IMS[v_PdnIndex], v_CellId);

v_TestConfiguration := v_Command.TestConfiguration;  /* @sic R5s160247 sic@ */

}

case (IPCAN_PROVIDELOCATIONINFORMATION) {              /* @sic R5-162025 - Cleanup of 34.229-3 Annex H sic@ */

...

# Branches executed

On Anritsu, the test case was executed on NR band n41 using NR ciphering nea1 and integrity algorithm nia1.

# Execution Log Files

## Qualcomm SM 8450+SDX65 5G UE

The Qualcomm SDX UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_10_10_LOG.xml

- PICS/PIXIT parameter file:

TC_10_10_PIXIT.txt

# References

| [1] | R5s220612 - Supporting information for addition of IMS 5GS emergency call test case 10.10 |
| --- | --- |