3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220616

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0829 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to template cs_HttpResponse |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-04-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The last parameter of cs_HttpResponse template is an 'omit' value by default: template (value) HttpResponse_Type cs_HttpResponse(template (value) HttpStatusLine_Type p_HttpStatusLine, ... template (omit) charstring p_TextBody := omit) := which is then assigned to both message body field options: ... messageBody := p_TextBody, messageBodyUnion := { // @sic R5w220006 sic@ text := p_TextBody } While HttpRequest_Type/HttpResponse_Type allow 'omit' to be assigned to the fields directly: type record HttpRequest_Type { ... charstring messageBody optional, /* e.g. XCAP Message */ // !!!! TO BE REMOVED AFTER GRACE PERIOD !!!! HttpMessageBody messageBodyUnion optional } The "text" field or any other variant field in HttpMessageBody union cannot be assigned 'omit': type union HttpMessageBody { charstring text, octetstring binary, MIME_Message mime }; |  |  |  |  |  |  |  |  |
| Summary of change: |  | Created a new function to assign a valid content to messageBodyUnion. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Legacy and NR IMS XCAP test cases. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required	4

1.1	Correction to cs_HttpResponse template	4

1.2	f_getValidHttpMessageBody_Text function	5

# Corrections required

## Correction to cs_HttpResponse template

| Template name | cs_HttpResponse |
| --- | --- |
| Reason for change | The "text" field or any other variant field in HttpMessageBody union cannot be assigned 'omit': |
| Summary of change | Usage of newly created f_getValidHttpMessageBody_Text auxiliar function to assign a valid content to messageBodyUnion. |
| TTCN module | Common\HTTP\HTTP_CommonTemplates.ttcn |
| MCC160 Comment | Accepted in principle: 'messageBody' will be removed from HttpResponse_Type and HttpRequest_Type due to expiry of grace period at 22wk23 delivery (see R5w220025) "template (omit) charstring p_TextBody" will be replaced by "template (omit) HttpMessageBody p_MessageBody" |

Before Change:

<<SKIPPED CODE>> template (value) HttpResponse_Type cs_HttpResponse(template (value) HttpStatusLine_Type p_HttpStatusLine, template (omit) SIP_ETag p_HttpETag := omit, template (omit) Server p_Server := omit, template (omit) WwwAuthenticate p_WwwAuthenticate := omit, template (omit) AuthenticationInfo p_AuthenticationInfo := omit, template (omit) Expires p_Expires := omit, template (omit) ContentType p_ContentType := omit, template (omit) charstring p_TextBody := omit) := { /* @status APPROVED (IMS) */ statusLine := p_HttpStatusLine, date := cs_HttpDate, // @sic R5-151795 sic@ eTag := p_HttpETag, // @sic R5-151795 sic@ server := p_Server, // @sic R5-151795 sic@ wwwauthenticate := p_WwwAuthenticate, authenticationInfo := p_AuthenticationInfo, contentType := p_ContentType, location := omit, // @sic R5w180314 sic@ contentLength := f_HttpContentLength(p_TextBody), // @sic R5-151795 sic@ expires := p_Expires, cacheControl := omit, pragma := omit, messageBody := p_TextBody, messageBodyUnion := { // @sic R5w220006 sic@ text := p_TextBody } }; <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) HttpResponse_Type cs_HttpResponse(template (value) HttpStatusLine_Type p_HttpStatusLine, template (omit) SIP_ETag p_HttpETag := omit, template (omit) Server p_Server := omit, template (omit) WwwAuthenticate p_WwwAuthenticate := omit, template (omit) AuthenticationInfo p_AuthenticationInfo := omit, template (omit) Expires p_Expires := omit, template (omit) ContentType p_ContentType := omit, template (omit) charstring p_TextBody := omit) := { /* @status APPROVED (IMS) */ statusLine := p_HttpStatusLine, date := cs_HttpDate, // @sic R5-151795 sic@ eTag := p_HttpETag, // @sic R5-151795 sic@ server := p_Server, // @sic R5-151795 sic@ wwwauthenticate := p_WwwAuthenticate, authenticationInfo := p_AuthenticationInfo, contentType := p_ContentType, location := omit, // @sic R5w180314 sic@ contentLength := f_HttpContentLength(p_TextBody), // @sic R5-151795 sic@ expires := p_Expires, cacheControl := omit, pragma := omit, messageBody := p_TextBody, messageBodyUnion := f_getValidHttpMessageBody_Text(p_TextBody) // @sic R5w220006 sic@ }; <<SKIPPED CODE>>

## f_getValidHttpMessageBody_Text function

| Function name | f_getValidHttpMessageBody_Text |
| --- | --- |
| Reason for change | The "text" field or any other variant field in HttpMessageBody union cannot be assigned 'omit': |
| Summary of change | New function |
| TTCN module | Common\HTTP\HTTP_CommonTemplates.ttcn |
| MCC160 Comment | Not needed |

<<SKIPPED CODE>> function f_getValidHttpMessageBody_Text(template (omit) charstring p_TextBody := omit) return template HttpMessageBody { if(isvalue(p_TextBody)) { return HttpMessageBody:{text:=p_TextBody}; } return omit; } <<SKIPPED CODE>>