3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220673

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2531 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR test case 7.1.1.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-05-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In current TTCN Implementation , 1. At Step 0AA, The SS transmits an updated system information which includes prach_ConfigurationIndex = (94) configuration for TDD .Prach is expected to be received in the subframe = 4/9 ( every odd SFN ) ; If Prach is received in Subframe =9 and slot = 1 , no issue is observed ; But If RACH is received in subframe = 4 & Slot = 1 , RAR Tx can be scheduled on sf = 0 slot = 0 (or) sf = 5 slot = 0 . Considering the interference due to SSB block transmission sf = 0 slot = 0 can not be used for RAR transmission . Sending Rar at sf = 5 slot = 0 will make the Rar window to expire (sl20 is the rar window configured) - similar topic is already discussed in R5s220280 2. At Step 0A, SS transmits an RRC configuration message to configure Specific parameters (reconfigurationWithSync) ; On the reception of this Reconfiguration message conformant UE will trigger Rach procedure – refer 5.3.5.5.2 Reconfiguration with sync from 38.331 (start synchronising to the DL of the target SpCell) At this step , UE is already in connected state and contention is expected to be resolved with Ul grant Format_0_1 ( CRNTI based ). To perform CBRA With UL grant at this stage , test case is missing the corresponding configurations ( this change is specific to 7.1.1.1.2.NR5GC version ) |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Rar window is to be increased from sl20 sl40 - CBRA with UL grant configurations are given to SS before step 0A before sending RRC configuration message to configure Specific parameters (reconfigurationWithSync) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.1.2.ENDC, 7.1.1.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | A Prose CR will be raised. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required	3

1.1	Correction to the template cs_RACH_ConfigGeneric71112_FR1_TDD	3

1.2	Correction to the function f_TC_7_1_1_1_2_NR5GC	4

# Corrections required

## Correction to the template cs_RACH_ConfigGeneric71112_FR1_TDD

| Template name | f_NR5GC_508RRC_IntraNR_CA_HO_InterCell_Common |
| --- | --- |
| Reason for change | At Step 0AA, The SS transmits an updated system information which includes prach_ConfigurationIndex = (94) configuration for TDD .Prach is expected to be received in the subframe = 4/9 ( every odd SFN ) ; If Prach is received in Subframe =9 and slot = 1 , no issue is observed ; But If RACH is received in subframe = 4 & Slot = 1 , RAR Tx can be scheduled on sf = 0 slot = 0 (or) sf = 5 slot = 0 . Considering the interference due to SSB block transmission sf = 0 slot = 0 can not be used for RAR transmission . Sending Rar at sf = 5 slot = 0 will make the Rar window to expire (sl20 is the rar window configured) - similar topic is already discussed in R5s220280 |
| Summary of change | Rar window is to be increased from sl20 sl40 |
| TTCN module | MAC_TC_Common_NR.ttcn |
| MCC160 Comment | Accepted as improvement |

Before Change:

template (value) RACH_ConfigGeneric cs_RACH_ConfigGeneric71112_FR1_TDD := cs_NR_RACH_ConfigGenericDef(94,-,-,-,-104,n10,dB2,sl20); /* @status APPROVED (ENDC, NR5GC) */

After Change:

template (value) RACH_ConfigGeneric cs_RACH_ConfigGeneric71112_FR1_TDD := cs_NR_RACH_ConfigGenericDef(94,-,-,-,-104,n10,dB2,sl40); /* @status APPROVED (ENDC, NR5GC) */ //WA#WI=957784

## Correction to the function f_TC_7_1_1_1_2_NR5GC

| Function name | f_TC_7_1_1_1_2_NR5GC |
| --- | --- |
| Reason for change | At Step 0A, SS transmits an RRC configuration message to configure Specific parameters (reconfigurationWithSync) ; On the reception of this Reconfiguration message conformant UE will trigger Rach procedure – refer 5.3.5.5.2 Reconfiguration with sync from 38.331 (start synchronising to the DL of the target SpCell) At this step , UE is already in connected state and contention is expected to be resolved with Ul grant Format_0_1 ( CRNTI based ). To perform CBRA With UL grant at this stage , test case is missing the corresponding configurations ( this change is specific to 7.1.1.1.2.NR5GC version ) |
| Summary of change | CBRA with UL grant configurations are given to SS before step 0A before sending RRC configuration message to configure Specific parameters (reconfigurationWithSync) |
| TTCN module | MAC_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_7_1_1_1_2_NR5GC () runs on NR5GC_PTC { << SKIPPED CODE >> v_SpCellConfig := f_NR_GetReconfigurationWithSyncDef_MCG(nr_Cell1); //@sic R5s200043 sic@ v_NR_UplinkBWP_List := f_NR_CellInfo_GetUplinkBWP_List(nr_Cell1); v_SpCellConfig.reconfigurationWithSync.t304 := ms2000; v_SpCellConfig.reconfigurationWithSync.rach_ConfigDedicated := omit; //@sic R5s190842 sic@ v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.rach_ConfigGeneric := v_RACH_ConfigGeneric; v_NR_UplinkBWP_List[0].Common.R15.rach_ConfigCommon.setup.rach_ConfigGeneric := valueof(v_RACH_ConfigGeneric); v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.totalNumberOfRA_Preambles := 42; v_NR_UplinkBWP_List[0].Common.R15.rach_ConfigCommon.setup.totalNumberOfRA_Preambles := 42; v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.ssb_perRACH_OccasionAndCB_PreamblesPerSSB := { one := n32 }; v_NR_UplinkBWP_List[0].Common.R15.rach_ConfigCommon.setup.ssb_perRACH_OccasionAndCB_PreamblesPerSSB := { one := n32 }; v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.groupBconfigured := cs_GroupBConfigured_Def; v_NR_UplinkBWP_List[0].Common.R15.rach_ConfigCommon.setup.groupBconfigured := valueof(cs_GroupBConfigured_Def); v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.ra_ContentionResolutionTimer := sf48; v_NR_UplinkBWP_List[0].Common.R15.rach_ConfigCommon.setup.ra_ContentionResolutionTimer := sf48; v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.downlinkConfigCommon.initialDownlinkBWP.pdcch_ConfigCommon.setup.searchSpaceSIB1 := omit; //@sic Step 0AA //Update the same in Sys info //@sic R5s200883 sic@ f_NR_CellInfo_SetUplinkConfigCommon(nr_Cell1, v_RACH_ConfigGeneric, 42, v_Ssb_perRACH_OccasionAndCB_PreamblesPerSSB, cs_GroupBConfigured_Def, sf48); f_NR_CellInfo_SetUplinkBWP_List(nr_Cell1, v_NR_UplinkBWP_List); f_NR_SS_ConfigureSysinfo(nr_Cell1, - , f_NR_InitialiseBcchToPdschConfig(nr_Cell1)); v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG , omit, omit, omit, omit, v_SpCellConfig //SpCellConfig ); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); // Send message to UE <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_1_2_NR5GC () runs on NR5GC_PTC { << SKIPPED CODE >> v_SpCellConfig := f_NR_GetReconfigurationWithSyncDef_MCG(nr_Cell1); //@sic R5s200043 sic@ v_NR_UplinkBWP_List := f_NR_CellInfo_GetUplinkBWP_List(nr_Cell1); v_SpCellConfig.reconfigurationWithSync.t304 := ms2000; v_SpCellConfig.reconfigurationWithSync.rach_ConfigDedicated := omit; //@sic R5s190842 sic@ v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.rach_ConfigGeneric := v_RACH_ConfigGeneric; v_NR_UplinkBWP_List[0].Common.R15.rach_ConfigCommon.setup.rach_ConfigGeneric := valueof(v_RACH_ConfigGeneric); v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.totalNumberOfRA_Preambles := 42; v_NR_UplinkBWP_List[0].Common.R15.rach_ConfigCommon.setup.totalNumberOfRA_Preambles := 42; v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.ssb_perRACH_OccasionAndCB_PreamblesPerSSB := { one := n32 }; v_NR_UplinkBWP_List[0].Common.R15.rach_ConfigCommon.setup.ssb_perRACH_OccasionAndCB_PreamblesPerSSB := { one := n32 }; v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.groupBconfigured := cs_GroupBConfigured_Def; v_NR_UplinkBWP_List[0].Common.R15.rach_ConfigCommon.setup.groupBconfigured := valueof(cs_GroupBConfigured_Def); v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.uplinkConfigCommon.initialUplinkBWP.rach_ConfigCommon.setup.ra_ContentionResolutionTimer := sf48; v_NR_UplinkBWP_List[0].Common.R15.rach_ConfigCommon.setup.ra_ContentionResolutionTimer := sf48; v_SpCellConfig.reconfigurationWithSync.spCellConfigCommon.downlinkConfigCommon.initialDownlinkBWP.pdcch_ConfigCommon.setup.searchSpaceSIB1 := omit; //@sic Step 0AA //Update the same in Sys info //@sic R5s200883 sic@ f_NR_CellInfo_SetUplinkConfigCommon(nr_Cell1, v_RACH_ConfigGeneric, 42, v_Ssb_perRACH_OccasionAndCB_PreamblesPerSSB, cs_GroupBConfigured_Def, sf48); f_NR_CellInfo_SetUplinkBWP_List(nr_Cell1, v_NR_UplinkBWP_List); f_NR_SS_ConfigureSysinfo(nr_Cell1, - , f_NR_InitialiseBcchToPdschConfig(nr_Cell1)); v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG , omit, omit, omit, omit, v_SpCellConfig //SpCellConfig ); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); // Send message to UE // Configure SS to handle CBRA with UL Grant as a result of Reconfiguration with Sync v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); //WA#WI=957784 v_NR_RachProcedure := cs_NR_RachProcedureConfig_71112_2(v_NR_PhysicalParameters); //WA#WI=957784 f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedure); //WA#WI=957784 <<SKIPPED CODE>>