3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220688

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-4 | CR | 0162 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of LTE POS testcase 7.2.2.2.5s |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2022-05-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add LTE POS test case 7.2.2.2.5s to the IWD_22wk12 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to LTE POS test case 7.2.2.2.5s required for agreement. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.2.2.5s |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1 Correction to f_TC_7_2_2_2_5s_EUTRA	3

3	Branches executed	4

4	Execution Log Files	4

4.1	Mediatek MT6879	4

5	References	5

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of LTE POS test case 7.2.2.2.5s which is part of the POS test suite in iWD_TTCN3-B2020-09_D22wk12.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	7.2.2.2.5s

ATS Version:	iwd-TTCN3-B2020-09_D22wk12

System Simulator used:	Rohde & Schwarz CMW500

UE used:	Mediatek MT6879

Verification Status:	PASS

# Corrections required

## 2.1 Correction to f_TC_7_2_2_2_5s_EUTRA

| Function name | f_TC_7_2_2_2_5s_EUTRA |
| --- | --- |
| Reason for change | The function f_TC_7_2_2_2_5s_EUTRA does not allow to decode correctly the LPP message part of multiplePositioningProtocolPDUs. Current implementation inputs an empty string to f_POS_LppMessage_Decode, causing a failure during the decoding of the LPP message. |
| Summary of change | Remove the v_LPP_Msg_Length and the v_Octetstring3_1, v_Octetstring3_2 and v_Octetstring3_3. Pass the content of the multiplePositioningProtocolPDUs directly to the function f_POS_LppMessage_Decode |
| TTCN module | EUTRA_Positioning_Testcases.ttcn |
| MCC160 Comment | Accepted. Also implemented in fl_TC_7_2_2_1_TestBody, fl_TC_7_2_2_2_GNSS_TestBody and f_TC_7_2_2_2_6s_EUTRA. |

Before

<<SKIPPED CODE>>

if (lengthof(v_Components[0].invoke.parameter.lcs_MOLR.multiplePositioningProtocolPDUs) >= 1) {

//@sic R5s170177 sic@: Remove type and length before passing string to decoder

v_LPP_Msg_Length := oct2int(substr(v_Components[0].invoke.parameter.lcs_MOLR.multiplePositioningProtocolPDUs[0], 1, 1)); //Get lenght and convert to integer

v_Octetstring3_1 := substr(v_Components[0].invoke.parameter.lcs_MOLR.multiplePositioningProtocolPDUs[0], 2, v_LPP_Msg_Length);// First possible LPP message

v_LppMessage1 := f_POS_LppMessage_Decode (v_Octetstring3_1);

v_LppMsg1Present := true;

if (lengthof(v_Components[0].invoke.parameter.lcs_MOLR.multiplePositioningProtocolPDUs) >= 2) {

//@sic R5s170177 sic@: Remove type and length before passing string to decoder

v_LPP_Msg_Length := oct2int(substr(v_Components[0].invoke.parameter.lcs_MOLR.multiplePositioningProtocolPDUs[1], 1, 1)); //Get lenght and convert to integer

v_Octetstring3_2:= substr(v_Components[0].invoke.parameter.lcs_MOLR.multiplePositioningProtocolPDUs[1], 2, v_LPP_Msg_Length);// Second possible LPP message

v_LppMessage2 := f_POS_LppMessage_Decode (v_Octetstring3_2);

v_LppMsg2Present := true;

if (lengthof(v_Components[0].invoke.parameter.lcs_MOLR.multiplePositioningProtocolPDUs) >= 3) {

//@sic R5s170177 sic@: Remove type and length before passing string to decoder

v_LPP_Msg_Length := oct2int(substr(v_Components[0].invoke.parameter.lcs_MOLR.multiplePositioningProtocolPDUs[2], 1, 1)); //Get lenght and convert to integer

v_Octetstring3_3:= substr(v_Components[0].invoke.parameter.lcs_MOLR.multiplePositioningProtocolPDUs[2], 2, v_LPP_Msg_Length);// Third possible LPP message

v_LppMessage3 := f_POS_LppMessage_Decode (v_Octetstring3_3);

v_LppMsg3Present := true;

}

}

}

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

if (lengthof(v_Components[0].invoke.parameter.lcs_MOLR.multiplePositioningProtocolPDUs) >= 1) {

//@sic R5s170177 sic@: Remove type and length before passing string to decoder

v_LppMessage1 := f_POS_LppMessage_Decode (v_Components[0].invoke.parameter.lcs_MOLR.multiplePositioningProtocolPDUs[0]);

v_LppMsg1Present := true;

if (lengthof(v_Components[0].invoke.parameter.lcs_MOLR.multiplePositioningProtocolPDUs) >= 2) {

v_LppMessage2 := f_POS_LppMessage_Decode v_Components[0].invoke.parameter.lcs_MOLR.multiplePositioningProtocolPDUs[1]);

v_LppMsg2Present := true;

if (lengthof(v_Components[0].invoke.parameter.lcs_MOLR.multiplePositioningProtocolPDUs) >= 3) {

//@sic R5s170177 sic@: Remove type and length before passing string to decoder

v_LppMessage3 := f_POS_LppMessage_Decode v_Components[0].invoke.parameter.lcs_MOLR.multiplePositioningProtocolPDUs[2]);

v_LppMsg3Present := true;

}

}

}    <<SKIPPED CODE>>

# Branches executed

## This LTE POS testcase was executed in Eutra Band 4 with ciphering algorithm eea1 and integrity algorithm eia1.

# Execution Log Files

## Mediatek MT6879

The MediaTek MT6897 UE passed this test case on Rohde & Schwarz CMW500 Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
tc_7_2_2_2_5s_MTK.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s220689: Supporting information for addition of LTE POS testcase 7.2.2.2.5s. |
| --- | --- |