3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220707

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2554 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR test case 6.4.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-05-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to TS24.501 cl4.5.1 NOTE 2: 5GMM specific procedures initiated by NAS in 5GMM-CONNECTED mode are not subject to access control, e.g. a registration procedure after PS handover will not be prevented by access control (see subclause 5.5). According to TS38.331 resumeCause Provides the resume cause for the RRC connection resume request as provided by the upper layers or RRC. The network is not expected to reject an RRCResumeRequest due to unknown cause value being used by the UE. To check the ResumeCause value in step2 is not necessary. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Do not check the ResumeCause value in step2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1 Corrections required	2

1.1 Correction to f_TC_6_4_1_2_NR5GC_TestBody	2

# Corrections required

## Correction to f_TC_6_4_1_2_NR5GC_TestBody

| Function name | f_TC_6_4_1_2_NR5GC_TestBody |
| --- | --- |
| Reason for change | According to TS24.501 cl4.5.1 NOTE 2: 5GMM specific procedures initiated by NAS in 5GMM-CONNECTED mode are not subject to access control, e.g. a registration procedure after PS handover will not be prevented by access control (see subclause 5.5). According to TS38.331 resumeCause Provides the resume cause for the RRC connection resume request as provided by the upper layers or RRC. The network is not expected to reject an RRCResumeRequest due to unknown cause value being used by the UE. To check the ResumeCause value in step2 is not necessary. |
| Summary of change | Do not check the ResumeCause value in step2 |
| TTCN module | Inactive_PLMNSelection_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_6_4_1_2_NR5GC_TestBody() runs on NR5GC_PTC {//@sic R5s200638 R5-204545 R5-213163 R5s210892 sic@ var template (value) NR_CellPowerList_Type v_CellPowerList_T0; // Power levels on FR2 are FFS var template (value) NR_CellPowerList_Type v_CellPowerList_T1; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var float v_TimerValue1 := 60.0; timer t_WaitTime; v_CellPowerList_T0 := { cs_NR_CellPower (nr_Cell1, tsc_NR_SuitableInterFreqCellSSS_EPRE_FR1, tsc_NR_SuitableInterFreqCellSSS_EPRE_FR2), // FR2 FFS cs_NR_CellPower (nr_Cell12, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2), cs_NR_CellPower (nr_Cell13, -78, -78) }; v_CellPowerList_T1 := { cs_NR_CellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower (nr_Cell12, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower (nr_Cell13, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE) }; // @siclog "Step 1" siclog@ // SS adjusts cell levels according to row T0 of table 6.4.1.2.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_T0); // @siclog "Step 2" siclog@ // Check: Does the UE transmit an RRCResumeRequest message on NR Cell 12? f_NR_RRC_ResumeRequest_Def(nr_Cell12,-,mo_Signalling); // @sic R5s200892 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); // @siclog "Steps 3-6" siclog@ // Steps 2 to 5 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 with condition MOBILITY are performed // Note 1: The REGISTRATION REQUEST is accepted with the PLMN of NR Cell 1 listed as an Equivalent PLMN f_NR_ULGrantConfiguration_Start(nr_Cell12); // @sic R5s210042 sic@ f_NR_RRCSetup_Def (nr_Cell12); ... }

After Change:

function f_TC_6_4_1_2_NR5GC_TestBody() runs on NR5GC_PTC {//@sic R5s200638 R5-204545 R5-213163 R5s210892 sic@ var template (value) NR_CellPowerList_Type v_CellPowerList_T0; // Power levels on FR2 are FFS var template (value) NR_CellPowerList_Type v_CellPowerList_T1; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var float v_TimerValue1 := 60.0; timer t_WaitTime; v_CellPowerList_T0 := { cs_NR_CellPower (nr_Cell1, tsc_NR_SuitableInterFreqCellSSS_EPRE_FR1, tsc_NR_SuitableInterFreqCellSSS_EPRE_FR2), // FR2 FFS cs_NR_CellPower (nr_Cell12, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2), cs_NR_CellPower (nr_Cell13, -78, -78) }; v_CellPowerList_T1 := { cs_NR_CellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower (nr_Cell12, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower (nr_Cell13, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE) }; // @siclog "Step 1" siclog@ // SS adjusts cell levels according to row T0 of table 6.4.1.2.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_T0); // @siclog "Step 2" siclog@ // Check: Does the UE transmit an RRCResumeRequest message on NR Cell 12? f_NR_RRC_ResumeRequest_Def(nr_Cell12,-,?); // @sic R5s200892 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); // @siclog "Steps 3-6" siclog@ // Steps 2 to 5 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 with condition MOBILITY are performed // Note 1: The REGISTRATION REQUEST is accepted with the PLMN of NR Cell 1 listed as an Equivalent PLMN f_NR_ULGrantConfiguration_Start(nr_Cell12); // @sic R5s210042 sic@ f_NR_RRCSetup_Def (nr_Cell12); ... }