3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220712

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2556 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of NR5GC Logged MDT test case 8.1.6.1.3.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2022-05-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For nrFailedPCellId-r16 at step 12 on UEInformationResponse, prose has given option Either pci-arfcn-r16 or cellGlobalId-r16 corresponding to NR Cell 1 from Table 8.1.6.1.3.1.3.3-10: UEInformationResponse (step 12, Table 8.1.6.1.3.1.3.2-3). But current TTCN implementation will fail the case if UE send pci-arfcn-r16 as TTCN will not able to check cellGlobalId-r16 is available or not. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Check below for the changes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.3.1.NR5GC, 8.1.6.1.3.2.NR5GC, 8.1.6.1.3.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change 1	2

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D22wk12 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | function f_TC_8_1_6_1_3_Common () |
| --- | --- |
| Reason for change | For nrFailedPCellId-r16 at step 12 on UEInformationResponse, prose has given option Either pci-arfcn-r16 or cellGlobalId-r16 corresponding to NR Cell 1 from Table 8.1.6.1.3.1.3.3-10: UEInformationResponse (step 12, Table 8.1.6.1.3.1.3.2-3). But current TTCN implementation will fail the case if UE send pci-arfcn-r16 as TTCN will not able to check cellGlobalId-r16 is available or not. |
| Summary of change | Check below for the changes |
| TTCN module | RRC_MDT_IntraNR_RLF_NR5GC.ttcn |
| MCC160 Comment | Implemented with a minor deviation in var names |

Before Change:

function f_TC_8_1_6_1_3_Common(NR_CellId_Type p_NeighbourCell, boolean p_CheckLocationInfo := false) runs on NR5GC_PTC {//@sic R5s211638, R5-217921 sic@ var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2; var NR_AbsoluteCellPower_Type v_CellPower_A3_Entry_FR1; var NR_AbsoluteCellPower_Type v_CellPower_A3_Entry_FR2; var RSRP_Range v_RsrpThres; var template (value) MeasObjectParamsList_Type v_MeasuredObjectList; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasIdToAddModList v_MeasIdConfigList; var template (omit) MeasConfig v_MeasConfig; var PhysCellId v_PhysCellIdServing; var PhysCellId v_PhysCellIdNeighbour; var ARFCN_ValueNR v_ARFCN_Neighbour; var template (present) UL_DCCH_Message v_UEInformationResponse; var DeltaValues_Type v_Deltas := f_NR_MobileInfo_GetPrimaryDeltaValue(); //@sic R5-214550 sic@ var template (omit) MeasGapConfig v_MeasGapConfig := omit; var CellIdentity v_CellIdentity_Serving; var CellIdentity v_CellIdentity_Neighbour; var NR_SRB_COMMON_IND v_ReceivedMsg; var RLF_Report_r16 v_ReceivedReport; //Table 8.1.6.1.3.1.3.2-1/2 v_CellPower_A3_Entry_FR1 := -82; v_CellPower_A3_Entry_FR2 := tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR2; //@sic R5-220053 sic@ v_CellPowerList_AtT0 := {cs_NR_CellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2), cs_NR_CellPower(p_NeighbourCell, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR2)}; v_CellPowerList_AtT1 := {cs_NR_CellPower(p_NeighbourCell, v_CellPower_A3_Entry_FR1, v_CellPower_A3_Entry_FR2)}; v_CellPowerList_AtT2 := {cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE)}; f_NR_CellInfo_InitMaxReferencePower(p_NeighbourCell, v_CellPower_A3_Entry_FR1, v_CellPower_A3_Entry_FR2); //Create and configure cells f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(p_NeighbourCell); //Preamble //The UE is in state 3N-A as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1. f_NR5GC_Preamble (nr_Cell1, STATE_CONNECTED_3A); if(p_CheckLocationInfo) {/*The UE's positioning engine (e.g. standalone GNSS receiver) should be provided with any necessary stimulus to allow it to provide the position. *This shall be done by use of the test function Update UE Location Information defined in TS 38.509 [6], if supported by the UE according to pc_UpdateUE_LocationInformation */ f_NR_ProvideUE_LocationInformation(nr_Cell1, RRC_CONNECTED); } f_NR_TestBody_Set(true); f_NR_SetCellPowerList(v_CellPowerList_AtT0); //Table 8.1.6.1.3.1.3.3-2¦3¦4 v_RsrpThres := 2; //@sic R5-216289 sic@ if (p_NeighbourCell == nr_Cell3) //@sic R5s211638 sic@ { if (not f_NR_CellInfo_GetIsFR1(p_NeighbourCell)) { v_RsrpThres := v_RsrpThres + v_Deltas.DeltaNRf2 - v_Deltas.DeltaNRf1; //@sic R5-214550 sic@ } v_MeasuredObjectList := {cs_MeasObjectId1(nr_Cell1), cs_MeasObjectId2(p_NeighbourCell)}; v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj2_conf1}; v_MeasGapConfig := cs_38508_MeasGapConfig_UE(f_NR_GetGapConfig_InterFreqOdd(nr_Cell1, p_NeighbourCell)); } else { v_MeasuredObjectList := {cs_MeasObjectId1(nr_Cell1)}; v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj1_conf1}; } v_ReportConfigList := {cs_NR_ReportConfigEventA3(tsc_NR_IdReportConfigId1, v_RsrpThres, r1, -, -, cs_NR_MeasReportQuantity_Rsrp)}; //@sic R5-217921 sic@ v_MeasConfig :=f_NR_GenerateMeasurementConfigNR(v_MeasuredObjectList, v_ReportConfigList, v_MeasIdConfigList, v_MeasGapConfig); // @sic R5s211638 sic@ //@siclog "Steps 1-2" siclog@ // The SS transmits an RRCReconfiguration message including MeasConfig to setup NR measurement and reporting for intra-frequency event A3 (measId 1). //The UE transmits an RRCReconfigurationComplete message. f_NR_SendRRCReconfiguration_MeasNR(nr_Cell1, v_MeasConfig); //@siclog "Step 3" siclog@ //The SS changes NR Cell 2 parameters according to the row "T1" in Table 8.1.6.1.3.1.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 4" siclog@ //The UE transmit a MeasurementReport message to report event A3 (measId 1) with the measured RSRP value for NR Cell 2. v_PhysCellIdServing := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1); v_PhysCellIdNeighbour := f_NR_CellInfo_GetPhysicalCellId(p_NeighbourCell); f_NR_ReceiveMeasurementReports(nr_Cell1, v_PhysCellIdServing, tsc_NR_MeasId1, -, -, cr_NR_MeasResults_MeasResultNeighCells_NR({cr_NR_MeasResultNR_CellResults(v_PhysCellIdNeighbour, cr_NR_MeasQuantityResults_Rsrp)})); //@siclog "Step 5" siclog@ //The SS changes NR Cell 1 parameter according to the row "T2" in Table 8.1.6.1.3.1.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT2); //@siclog "Steps 6-10" siclog@ //The UE transmits an RRCReestablishmentRequest message on neighblur cell. //The SS transmits an RRCReestablishment message. //The UE transmits an RRCReestablishmentComplete message. //The SS transmits an RRCReconfiguration message to establish SRB2 and DRB. //The UE transmits an RRCReconfigurationtComplete message. //The SS transmits an RRCReestablishment message. //The UE transmits an RRCReestablishmentComplete message. f_NR5GC_RRCReestablishment_SON_MDT(nr_Cell1, p_NeighbourCell, rlf_InfoAvailable); //@sic R5s211638 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Test " & testcasename() & " Step 9: RRCReestablishmentComplete message with rlf-InfoAvailable included"); //@siclog "Step 11" siclog@ //The SS transmits a UEInformationRequest message. SRB.send(cas_NR_SRB_RrcPdu_REQ(p_NeighbourCell, tsc_NR_RbId_SRB1, -, //cs_TimingInfo_Now cs_NR_UEInformationRequest_RLF)); // condition RLF //@siclog "Step 12" siclog@ //Check: Does the UE transmit a UEInformationResponse message on NR Cell 2? v_ARFCN_Neighbour := f_NR_CellInfo_GetFrequencySSB(p_NeighbourCell); v_CellIdentity_Serving := f_NR_CellInfo_GetCellIdentity(nr_Cell1); v_CellIdentity_Neighbour := f_NR_CellInfo_GetCellIdentity(p_NeighbourCell); v_UEInformationResponse := cr_UEInformationResponse_r16_RLF(v_PhysCellIdNeighbour, v_ARFCN_Neighbour, v_CellIdentity_Neighbour, f_NR_CellInfo_GetRNTI(nr_Cell1)); // @sic R5s211638 sic@ if(p_CheckLocationInfo) { v_UEInformationResponse.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.rlf_Report_r16.nr_RLF_Report_r16.locationInfo_r16 := ?; //@sic R5-217923 sic@ } SRB.receive(car_NR_SRB_RrcPdu_IND(p_NeighbourCell, tsc_NR_RbId_SRB1, v_UEInformationResponse)) -> value v_ReceivedMsg; // @sic R5s211638 sic@ v_ReceivedReport := v_ReceivedMsg.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.rlf_Report_r16; if( v_ReceivedReport.nr_RLF_Report_r16.failedPCellId_r16.nrFailedPCellId_r16.cellGlobalId_r16.cellIdentity_r16 == v_CellIdentity_Serving or //@sic R5-217921 sic@ v_ReceivedReport.nr_RLF_Report_r16.failedPCellId_r16.nrFailedPCellId_r16.pci_arfcn_r16.physCellId_r16 == v_PhysCellIdServing) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Test " & testcasename() & " Step 12"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Test " & testcasename() & " Step 12: wrong failed PCell ID "); } //@siclog "Step 13" siclog@ //Void @sic R5-216289 sic@ f_NR_TestBody_Set(false); // Finish f_NR_Postamble(p_NeighbourCell, STATE_CONNECTED_3A); // @sic R5-216289 sic@ } // end of f_TC_8_1_6_1_3_Common

After Change:

function f_TC_8_1_6_1_3_Common(NR_CellId_Type p_NeighbourCell, boolean p_CheckLocationInfo := false) runs on NR5GC_PTC {//@sic R5s211638, R5-217921 sic@ var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2; var NR_AbsoluteCellPower_Type v_CellPower_A3_Entry_FR1; var NR_AbsoluteCellPower_Type v_CellPower_A3_Entry_FR2; var RSRP_Range v_RsrpThres; var template (value) MeasObjectParamsList_Type v_MeasuredObjectList; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasIdToAddModList v_MeasIdConfigList; var template (omit) MeasConfig v_MeasConfig; var PhysCellId v_PhysCellIdServing_ref; var PhysCellId v_PhysCellIdServing; var PhysCellId v_PhysCellIdNeighbour; var ARFCN_ValueNR v_ARFCN_Neighbour; var template (present) UL_DCCH_Message v_UEInformationResponse; var DeltaValues_Type v_Deltas := f_NR_MobileInfo_GetPrimaryDeltaValue(); //@sic R5-214550 sic@ var template (omit) MeasGapConfig v_MeasGapConfig := omit; var CellIdentity v_CellIdentity_Serving_ref; var CellIdentity v_CellIdentity_Serving; var CellIdentity v_CellIdentity_Neighbour; var NR_SRB_COMMON_IND v_ReceivedMsg; var RLF_Report_r16 v_ReceivedReport; //Table 8.1.6.1.3.1.3.2-1/2 v_CellPower_A3_Entry_FR1 := -82; v_CellPower_A3_Entry_FR2 := tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR2; //@sic R5-220053 sic@ v_CellPowerList_AtT0 := {cs_NR_CellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2), cs_NR_CellPower(p_NeighbourCell, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR2)}; v_CellPowerList_AtT1 := {cs_NR_CellPower(p_NeighbourCell, v_CellPower_A3_Entry_FR1, v_CellPower_A3_Entry_FR2)}; v_CellPowerList_AtT2 := {cs_NR_CellPower(nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE)}; f_NR_CellInfo_InitMaxReferencePower(p_NeighbourCell, v_CellPower_A3_Entry_FR1, v_CellPower_A3_Entry_FR2); //Create and configure cells f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(p_NeighbourCell); //Preamble //The UE is in state 3N-A as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1. f_NR5GC_Preamble (nr_Cell1, STATE_CONNECTED_3A); if(p_CheckLocationInfo) {/*The UE's positioning engine (e.g. standalone GNSS receiver) should be provided with any necessary stimulus to allow it to provide the position. *This shall be done by use of the test function Update UE Location Information defined in TS 38.509 [6], if supported by the UE according to pc_UpdateUE_LocationInformation */ f_NR_ProvideUE_LocationInformation(nr_Cell1, RRC_CONNECTED); } f_NR_TestBody_Set(true); f_NR_SetCellPowerList(v_CellPowerList_AtT0); //Table 8.1.6.1.3.1.3.3-2¦3¦4 v_RsrpThres := 2; //@sic R5-216289 sic@ if (p_NeighbourCell == nr_Cell3) //@sic R5s211638 sic@ { if (not f_NR_CellInfo_GetIsFR1(p_NeighbourCell)) { v_RsrpThres := v_RsrpThres + v_Deltas.DeltaNRf2 - v_Deltas.DeltaNRf1; //@sic R5-214550 sic@ } v_MeasuredObjectList := {cs_MeasObjectId1(nr_Cell1), cs_MeasObjectId2(p_NeighbourCell)}; v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj2_conf1}; v_MeasGapConfig := cs_38508_MeasGapConfig_UE(f_NR_GetGapConfig_InterFreqOdd(nr_Cell1, p_NeighbourCell)); } else { v_MeasuredObjectList := {cs_MeasObjectId1(nr_Cell1)}; v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj1_conf1}; } v_ReportConfigList := {cs_NR_ReportConfigEventA3(tsc_NR_IdReportConfigId1, v_RsrpThres, r1, -, -, cs_NR_MeasReportQuantity_Rsrp)}; //@sic R5-217921 sic@ v_MeasConfig :=f_NR_GenerateMeasurementConfigNR(v_MeasuredObjectList, v_ReportConfigList, v_MeasIdConfigList, v_MeasGapConfig); // @sic R5s211638 sic@ //@siclog "Steps 1-2" siclog@ // The SS transmits an RRCReconfiguration message including MeasConfig to setup NR measurement and reporting for intra-frequency event A3 (measId 1). //The UE transmits an RRCReconfigurationComplete message. f_NR_SendRRCReconfiguration_MeasNR(nr_Cell1, v_MeasConfig); //@siclog "Step 3" siclog@ //The SS changes NR Cell 2 parameters according to the row "T1" in Table 8.1.6.1.3.1.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 4" siclog@ //The UE transmit a MeasurementReport message to report event A3 (measId 1) with the measured RSRP value for NR Cell 2. v_PhysCellIdServing := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1); v_PhysCellIdNeighbour := f_NR_CellInfo_GetPhysicalCellId(p_NeighbourCell); f_NR_ReceiveMeasurementReports(nr_Cell1, v_PhysCellIdServing, tsc_NR_MeasId1, -, -, cr_NR_MeasResults_MeasResultNeighCells_NR({cr_NR_MeasResultNR_CellResults(v_PhysCellIdNeighbour, cr_NR_MeasQuantityResults_Rsrp)})); //@siclog "Step 5" siclog@ //The SS changes NR Cell 1 parameter according to the row "T2" in Table 8.1.6.1.3.1.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT2); //@siclog "Steps 6-10" siclog@ //The UE transmits an RRCReestablishmentRequest message on neighblur cell. //The SS transmits an RRCReestablishment message. //The UE transmits an RRCReestablishmentComplete message. //The SS transmits an RRCReconfiguration message to establish SRB2 and DRB. //The UE transmits an RRCReconfigurationtComplete message. //The SS transmits an RRCReestablishment message. //The UE transmits an RRCReestablishmentComplete message. f_NR5GC_RRCReestablishment_SON_MDT(nr_Cell1, p_NeighbourCell, rlf_InfoAvailable); //@sic R5s211638 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Test " & testcasename() & " Step 9: RRCReestablishmentComplete message with rlf-InfoAvailable included"); //@siclog "Step 11" siclog@ //The SS transmits a UEInformationRequest message. SRB.send(cas_NR_SRB_RrcPdu_REQ(p_NeighbourCell, tsc_NR_RbId_SRB1, -, //cs_TimingInfo_Now cs_NR_UEInformationRequest_RLF)); // condition RLF //@siclog "Step 12" siclog@ //Check: Does the UE transmit a UEInformationResponse message on NR Cell 2? v_ARFCN_Neighbour := f_NR_CellInfo_GetFrequencySSB(p_NeighbourCell); v_CellIdentity_Serving := f_NR_CellInfo_GetCellIdentity(nr_Cell1); v_CellIdentity_Neighbour := f_NR_CellInfo_GetCellIdentity(p_NeighbourCell); v_UEInformationResponse := cr_UEInformationResponse_r16_RLF(v_PhysCellIdNeighbour, v_ARFCN_Neighbour, v_CellIdentity_Neighbour, f_NR_CellInfo_GetRNTI(nr_Cell1)); // @sic R5s211638 sic@ if(p_CheckLocationInfo) { v_UEInformationResponse.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.rlf_Report_r16.nr_RLF_Report_r16.locationInfo_r16 := ?; //@sic R5-217923 sic@ } SRB.receive(car_NR_SRB_RrcPdu_IND(p_NeighbourCell, tsc_NR_RbId_SRB1, v_UEInformationResponse)) -> value v_ReceivedMsg; // @sic R5s211638 sic@ v_ReceivedReport := v_ReceivedMsg.Signalling.Rrc.Dcch.message_.messageClassExtension.c2.ueInformationResponse_r16.criticalExtensions.ueInformationResponse_r16.rlf_Report_r16; if( isvalue(v_ReceivedReport.nr_RLF_Report_r16.failedPCellId_r16.nrFailedPCellId_r16.cellGlobalId_r16.cellIdentity_r16)) {v_CellIdentity_Serving_ref := v_ReceivedReport.nr_RLF_Report_r16.failedPCellId_r16.nrFailedPCellId_r16.cellGlobalId_r16.cellIdentity_r16;} //@sic R5-217921 sic@ if(isvalue(v_ReceivedReport.nr_RLF_Report_r16.failedPCellId_r16.nrFailedPCellId_r16.pci_arfcn_r16.physCellId_r16)) {v_PhysCellIdServing_ref := v_ReceivedReport.nr_RLF_Report_r16.failedPCellId_r16.nrFailedPCellId_r16.pci_arfcn_r16.physCellId_r16;} if((v_PhysCellIdServing_ref == v_PhysCellIdServing) or (v_CellIdentity_Serving_ref == v_CellIdentity_Serving_ref)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Test " & testcasename() & " Step 12"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Test " & testcasename() & " Step 12: wrong failed PCell ID "); } //@siclog "Step 13" siclog@ //Void @sic R5-216289 sic@ f_NR_TestBody_Set(false); // Finish f_NR_Postamble(p_NeighbourCell, STATE_CONNECTED_3A); // @sic R5-216289 sic@ } // end of f_TC_8_1_6_1_3_Common