3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220765

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-4 | CR | 0163 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_TC_7_3_4_2_7s_EUTRA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2022-06-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to 3GPP TS 37.571-2, Table 7.3.4.2.3.3-9 , at Step 4b1, one of agnssProvideLocationInformation or otdoaProvideLocationInformation shall be present in the LPP Provide Location Information message. Current implementation for function f_TC_7_3_4_2_7s_EUTRA does not follow the behavior described above. Instead, it requires both agnssProvideLocationInformation and otdoaProvideLocationInformation to be present, otherwise it throws a fail verdict. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the implementation of function f_TC_7_3_4_2_7s_EUTRA to be spec compliant as described above |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.4.2.7s |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1.1 Correction to f_TC_7_3_4_2_7s_EUTRA	2

## 1.1 Correction to f_TC_7_3_4_2_7s_EUTRA

| Function name | f_TC_7_3_4_2_7s_EUTRA |
| --- | --- |
| Reason for change | According to 3GPP TS 37.571-2, Table 7.3.4.2.3.3-9 , at Step 4b1, one of agnssProvideLocationInformation or otdoaProvideLocationInformation shall be present in the LPP Provide Location Information message. Current implementation for function f_TC_7_3_4_2_7s_EUTRA does not follow the behavior described above. Instead, it requires both agnssProvideLocationInformation and otdoaProvideLocationInformation to be present, otherwise it throws a fail verdict. |
| Summary of change | Change the or to and in the if statement that checks the LPP Provide location information content. |
| TTCN module | EUTRA_Positioning_Testcases.ttcn |
| MCC160 Comment | Accepted |

Before

<<SKIPPED CODE>>

//@siclog "Step 4b1" siclog@

else if (match (v_LppMessage, cr_LppProvideLocationInformation_NoError_EarlyFix(tsc_POS_TransactionNumber))) {

if (not(isvalue (v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.a_gnss_ProvideLocationInformation)) or

not(isvalue (v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.otdoa_ProvideLocationInformation))) {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "Neither A-gnss Provide Location Information nor OTDOA Provide Location Information are present");

}

//@siclog "Step 4b2" siclog@

//SS sends LPP ACKNOWLEDGEMENT if requested by the UE at step 4

f_POS_SendLppAck(eutra_Cell1, tsc_POS_RoutingId, v_LppMessage);

//@siclog "Step 4b3" siclog@

//Receive LPP PROVIDE LOCATION INFORMATION

SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1,

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered,

cr_UL_GENERIC_NAS_TRANSPORT(tsc_LPP_MessageContainer,

cr_RoutingInfo (tsc_POS_RoutingId))))) //@sic R5s140739 sic@

-> value v_ReceivedAsp;

v_Octetstring := v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_GENERIC_NAS_TRANSPORT.genericMessageContainer.genericMsgContainerValue;

v_LppMessage := f_POS_LppMessage_DecodeAndMatch (v_Octetstring, (cr_LppProvideLocationInformation_NoError(tsc_POS_TransactionNumber), cr_LppProvideLocationInformation_UEA (tsc_POS_TransactionNumber))); //@sic R5-152149 sic@

if ((isvalue (v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.a_gnss_ProvideLocationInformation)) and

(isvalue (v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.otdoa_ProvideLocationInformation))) {

v_GNSS_MeasurementList := v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.a_gnss_ProvideLocationInformation.gnss_SignalMeasurementInformation.gnss_MeasurementList;  //@sic R5-198966 sic@

f_POS_CheckGnssMeasurementList(v_GNSS_MeasurementList);//@sic R5-198966, R5s210633 sic@

if (isvalue (v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.otdoa_ProvideLocationInformation.otdoaSignalMeasurementInformation)) { //@sic R5-144626 sic@

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 7.3.4.2 Step 4b3");

}

else {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "OTDOA Location Information present but SignalMeasurementInformation missing");  //@sic R5-144626 sic@

}

}

else {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "A-GNSS or OTDOA Location Information not present");

}

//@siclog "Step 4b4" siclog@

//SS sends LPP ACKNOWLEDGEMENT if requested by the UE at step 4

f_POS_SendLppAck(eutra_Cell1, tsc_POS_RoutingId, v_LppMessage);

}

// Postamble

f_EUTRA_TestBody_Set(false);

f_EUTRA_Postamble(eutra_Cell1, E2_CONNECTED);//@sic R5-103875 sic@

f_POS_StopPositioningSimulatorEUTRA (v_GNSSList);

} <<SKIPPED CODE>>

After

<<SKIPPED CODE>>

//@siclog "Step 4b1" siclog@

else if (match (v_LppMessage, cr_LppProvideLocationInformation_NoError_EarlyFix(tsc_POS_TransactionNumber))) {

if (not(isvalue (v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.a_gnss_ProvideLocationInformation)) and

not(isvalue (v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.otdoa_ProvideLocationInformation))) {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "Neither A-gnss Provide Location Information nor OTDOA Provide Location Information are present");

}

//@siclog "Step 4b2" siclog@

//SS sends LPP ACKNOWLEDGEMENT if requested by the UE at step 4

f_POS_SendLppAck(eutra_Cell1, tsc_POS_RoutingId, v_LppMessage);

//@siclog "Step 4b3" siclog@

//Receive LPP PROVIDE LOCATION INFORMATION

SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1,

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered,

cr_UL_GENERIC_NAS_TRANSPORT(tsc_LPP_MessageContainer,

cr_RoutingInfo (tsc_POS_RoutingId))))) //@sic R5s140739 sic@

-> value v_ReceivedAsp;

v_Octetstring := v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_GENERIC_NAS_TRANSPORT.genericMessageContainer.genericMsgContainerValue;

v_LppMessage := f_POS_LppMessage_DecodeAndMatch (v_Octetstring, (cr_LppProvideLocationInformation_NoError(tsc_POS_TransactionNumber), cr_LppProvideLocationInformation_UEA (tsc_POS_TransactionNumber))); //@sic R5-152149 sic@

if ((isvalue (v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.a_gnss_ProvideLocationInformation)) and

(isvalue (v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.otdoa_ProvideLocationInformation))) {

v_GNSS_MeasurementList := v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.a_gnss_ProvideLocationInformation.gnss_SignalMeasurementInformation.gnss_MeasurementList;  //@sic R5-198966 sic@

f_POS_CheckGnssMeasurementList(v_GNSS_MeasurementList);//@sic R5-198966, R5s210633 sic@

if (isvalue (v_LppMessage.lpp_MessageBody.c1.provideLocationInformation.criticalExtensions.c1.provideLocationInformation_r9.otdoa_ProvideLocationInformation.otdoaSignalMeasurementInformation)) { //@sic R5-144626 sic@

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 7.3.4.2 Step 4b3");

}

else {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "OTDOA Location Information present but SignalMeasurementInformation missing");  //@sic R5-144626 sic@

}

}

else {

f_EUTRA_SetVerdictFailOrInconc (__FILE__, __LINE__, "A-GNSS or OTDOA Location Information not present");

}

//@siclog "Step 4b4" siclog@

//SS sends LPP ACKNOWLEDGEMENT if requested by the UE at step 4

f_POS_SendLppAck(eutra_Cell1, tsc_POS_RoutingId, v_LppMessage);

}

// Postamble

f_EUTRA_TestBody_Set(false);

f_EUTRA_Postamble(eutra_Cell1, E2_CONNECTED);//@sic R5-103875 sic@

f_POS_StopPositioningSimulatorEUTRA (v_GNSSList);

}

<<SKIPPED CODE>>