3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220768

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2588 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of NR5GC Rel-16 CAG test case 6.5.2.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN_Vertical_LAN-UEConTest |  |  |  |  |  | Date: |  |  | 2022-06-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add NR5GC Rel-16 CAG test case 6.5.2.4 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC test case 6.5.2.4 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1 Change 1	3

2.2  Change 2	6

2.3 Change 3	8

2.4 change 4	9

2.5	Change 5	11

3	Branches executed	12

4	Execution Log Files	12

4.1	Qualcomm Snapdragon X70	12

5	References	12

- Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 6.5.2.4 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

  - Verification Test Summary

Test Case: 	6.5.2.4

ATS Version:	iwd-TTCN3-B2020-09_D22wk23

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	Qualcomm Snapdragon X70

Verification Status:	PASS

- Corrections required

2.1 Change 1

| Function name | f_TC_6_5_2_4_NR5GC() |
| --- | --- |
| Reason for change | - In the current TTCN implementatio, the template cdr_NG_GMM_Cap_CAG is passed to function fl_NR5GC_RRC_IdleSteps4_20_withCAGInfoList. This results in checking of the CAG support in the 5GMM capability in the first REGISTRATIONREQUEST, which will not contain the (non-cleartext) IE 5GMM Capability at all. - NR cells 3 and 6 are not initialised with the highest cell power level used in the test case - In order for the UE to do a reselection on a cell belonging to a different PLMN, the target PLMN should be part of the equivalent PLMN list in the source PLMN cell. |
| Summary of change | - Removed capability checking template ‘cdr_NG_GMM_Cap_CAG’ from the function call - Initialized the cells using function f_NR_CellInfo_InitMaxReferencePower() - Added PLMN2 as equivalent PLMN in the REGISTRATION ACCEPT on NR Cell 1 in the Preamble Note : An associated prose CR on 38.523-1 will be raised at RAN5#96 |
| TTCN module | CAG_Selection_NR5GC.ttcn |
| MCC160 Comment | - Accepted and implemented - Not Needed, already implemented in iwd-22wk23. - Accepted conditional to prose CR agreement at RAN5#96-e |

Before Change

function f_TC_6_5_2_4_NR5GC() runs on NR5GC_PTC { // CAG / cell reselection / Within allowed CAG / non-CAG cell to CAG cell var boolean v_IsFR1 := true; var ReselectionThreshold v_ThreshX_HighP; var template(value) InterFreqCarrierFreqList v_InterFreqCarrierFreqList_Cell; var template(value) SSB_MTC v_SSB_MTC; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; f_NR_TestBody_Set(true); // System information combination NR-4 as defined in TS 38.508-1 Table 4.4.3.1.2-1 is used in NR cells. f_NR5GC_Init(NR_4); v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell3); … … … // Make the pre-test condition satisfied v_ReceivedMsg := f_NR5GC_Preamble_Steps1_4 (nr_Cell1); //The UE is registered on NR Cell 1 using the procedure described in TS 38.508-1 [4] clause 4.5.2.2 except that the REGISTRATION ACCEPT message includes CAG information list with CAG-ID 1 and CAG-ID 2. fl_NR5GC_RRC_IdleSteps4_20_withCAGInfoList(nr_Cell1, v_ReceivedMsg, Initial_NoSecurity, cdr_NG_GMM_Cap_CAG, cs_CAGInfoList_twoEntries(cs_CAGInfo_oneID(f_NR_Asn2Nas_PlmnId (f_NR_CellInfo_GetPLMN (nr_Cell3)),'0'B, int2oct(1,4)),cs_CAGInfo_oneID(f_NR_Asn2Nas_PlmnId (f_NR_CellInfo_GetPLMN (nr_Cell6)),'0'B, int2oct(2,4)))); //f_NR_TestBody_Set(true); fl_TC_6_5_2_4_TestBody(); f_NR_TestBody_Set(false); // Finish f_NR_Postamble(nr_Cell6, STATE_IDLE_1A); }

After Change

function f_TC_6_5_2_4_NR5GC() runs on NR5GC_PTC { // CAG / cell reselection / Within allowed CAG / non-CAG cell to CAG cell var boolean v_IsFR1 := true; var ReselectionThreshold v_ThreshX_HighP; var template(value) InterFreqCarrierFreqList v_InterFreqCarrierFreqList_Cell; var template(value) SSB_MTC v_SSB_MTC; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; f_NR_TestBody_Set(true); // System information combination NR-4 as defined in TS 38.508-1 Table 4.4.3.1.2-1 is used in NR cells. f_NR5GC_Init(NR_4); f_NR_CellInfo_InitMaxReferencePower(nr_Cell3, -78, -78); f_NR_CellInfo_InitMaxReferencePower(nr_Cell6, -78, -78); v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell3); … … … // Make the pre-test condition satisfied v_ReceivedMsg := f_NR5GC_Preamble_Steps1_4 (nr_Cell1); //The UE is registered on NR Cell 1 using the procedure described in TS 38.508-1 [4] clause 4.5.2.2 except that the REGISTRATION ACCEPT message includes CAG information list with CAG-ID 1 and CAG-ID 2. fl_NR5GC_RRC_IdleSteps4_20_withCAGInfoList(nr_Cell1, v_ReceivedMsg, Initial_NoSecurity, -, cs_CAGInfoList_twoEntries(cs_CAGInfo_oneID(f_NR_Asn2Nas_PlmnId (f_NR_CellInfo_GetPLMN (nr_Cell3)),'0'B, int2oct(1,4)),cs_CAGInfo_oneID(f_NR_Asn2Nas_PlmnId (f_NR_CellInfo_GetPLMN (nr_Cell6)),'0'B, int2oct(2,4))), cs_PLMN_List_1PLMN(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell3))) //PLMN2 as EPLMN ); //f_NR_TestBody_Set(true); fl_TC_6_5_2_4_TestBody(); f_NR_TestBody_Set(false); // Finish f_NR_Postamble(nr_Cell6, STATE_IDLE_1A); }

2.2  Change 2

| Function name | fl_NR5GC_RRC_IdleSteps4_20_withCAGInfoList() |
| --- | --- |
| Reason for change | In order for the UE to do a reselection on a cell belonging to a different PLMN, the target PLMN should be part of the equivalent PLMN list in the source PLMN cell. |
| Summary of change | See change 2.3.3 |
| TTCN module | New parameter for equivalent PLMN list, see change 1 |
| MCC160 Comment | Accepted conditional to prose CR agreement at RAN5#96-e |

Before Change

function fl_NR5GC_RRC_IdleSteps4_20_withCAGInfoList(NR_CellId_Type p_NR_CellId, NG_NAS_MSG_Indication_Type p_ReceivedMsg, TypeOfRegistration_Type p_RegType := Initial_Secure, template NG_GMM_Cap p_GMM_Cap := *, template (omit) CAGInfoList p_CAGInfoList := omit) runs on NR5GC_PTC { … … … //Registration Accept v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, f_NR5GC_GetMobileIdGUTI (p_NR_CellId), cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (p_NR_CellId)), {bit2oct(f_NR_CellInfo_GetTAC (p_NR_CellId))}), -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, p_CAGInfoList); SRB.send(cas_NR_SRB_NasPdu_REQ(p_NR_CellId, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //Registration Complete f_NR5GC_Registration_Complete(p_NR_CellId); f_NR5GC_RRC_Idle_Steps16_20(p_NR_CellId,-,-,rrcConnectionRelease); //@sic Rs220599 sic@ }

After Change

function fl_NR5GC_RRC_IdleSteps4_20_withCAGInfoList(NR_CellId_Type p_NR_CellId, NG_NAS_MSG_Indication_Type p_ReceivedMsg, TypeOfRegistration_Type p_RegType := Initial_Secure, template NG_GMM_Cap p_GMM_Cap := *, template (omit) CAGInfoList p_CAGInfoList := omit, template (omit) PLMN_List p_EPLMNs := omit) runs on NR5GC_PTC { … … … //Registration Accept v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, f_NR5GC_GetMobileIdGUTI (p_NR_CellId), cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (p_NR_CellId)), {bit2oct(f_NR_CellInfo_GetTAC (p_NR_CellId))}), p_EPLMNs, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, p_CAGInfoList); SRB.send(cas_NR_SRB_NasPdu_REQ(p_NR_CellId, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //Registration Complete f_NR5GC_Registration_Complete(p_NR_CellId); f_NR5GC_RRC_Idle_Steps16_20(p_NR_CellId,-,-,rrcConnectionRelease); //@sic Rs220599 sic@ }

2.3 Change 3

| Function name | cs_CAGInfoList_twoEntries |
| --- | --- |
| Reason for change | The length value is incorrect since each CAG info list entry is fixed at 9 octets |
| Summary of change | Corrected the length value |
| TTCN module | CAG_Selection_NR5GC.ttcn |
| MCC160 Comment | Accepted and implemented |

Before Change

template (value) CAGInfoList cs_CAGInfoList_twoEntries(template(value) CAG_Info p_CAGInfo1, template(value) CAG_Info p_CAGInfo2) := { /* @status */ /* 24.501 cl. 9.11.3.18A */ iei := '75'O, iel := '0022'O, // @sic R5-220532 sic@ listOfCAGInfos := {p_CAGInfo1,p_CAGInfo2} };

After Change

template (value) CAGInfoList cs_CAGInfoList_twoEntries(template(value) CAG_Info p_CAGInfo1, template(value) CAG_Info p_CAGInfo2) := { /* @status */ /* 24.501 cl. 9.11.3.18A */ iei := '75'O, iel := '0012'O, // @sic R5-220532 sic@ listOfCAGInfos := {p_CAGInfo1,p_CAGInfo2} };

2.4 change 4

| Function name | f_NR_CellInfo_SetPLMN |
| --- | --- |
| Reason for change | The function f_NR_CellInfo_SetPLMN() does not consider whether CAG ID or NID should be set along with the PLMN when NPN is used, and incorrectly only sets NID for all cases |
| Summary of change | Added checks based on whether snpn_r16 or pni_npn_r16 is used in SIB1, and correct ID is initialized |
| TTCN module | NR_CellInfo.ttcn |
| MCC160 Comment | Not Needed, CAG ID has already been set in function f_NR_CellInfo_SetSIB1_SetPNI_NPN_IdentityInfoList. |

Before Change

function f_NR_CellInfo_SetPLMN(NR_CellId_Type p_NR_CellId, PLMN_Identity p_PLMN_Identity, integer p_PLMN_Entry1 := 0, integer p_PLMN_Entry2 := 0) runs on NR_BASE_PTC { var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); v_NR_CellInfo.NAS_Parameters.PLMN_Identity := p_PLMN_Identity; if (v_NR_CellInfo.Sysinfo.NR_SysinfoCombination != MibOnly) { //@sic R5s211526 BASELINE MOVING 2021 sic@ v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.cellAccessRelatedInfo.plmn_IdentityInfoList[p_PLMN_Entry1].plmn_IdentityList[p_PLMN_Entry2] := p_PLMN_Identity; // SNPN if (isvalue(v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.cellAccessRelatedInfo.npn_IdentityInfoList_r16)){ //Handle one SNPN entry by default v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.cellAccessRelatedInfo.npn_IdentityInfoList_r16[0].npn_IdentityList_r16[0].snpn_r16.plmn_Identity_r16 := valueof(p_PLMN_Identity); } } f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); }

After Change

function f_NR_CellInfo_SetPLMN(NR_CellId_Type p_NR_CellId, PLMN_Identity p_PLMN_Identity, integer p_PLMN_Entry1 := 0, integer p_PLMN_Entry2 := 0) runs on NR_BASE_PTC { var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); v_NR_CellInfo.NAS_Parameters.PLMN_Identity := p_PLMN_Identity; if (v_NR_CellInfo.Sysinfo.NR_SysinfoCombination != MibOnly) { //@sic R5s211526 BASELINE MOVING 2021 sic@ v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.cellAccessRelatedInfo.plmn_IdentityInfoList[p_PLMN_Entry1].plmn_IdentityList[p_PLMN_Entry2] := p_PLMN_Identity; // SNPN if (isvalue(v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.cellAccessRelatedInfo.npn_IdentityInfoList_r16)){ //Handle one SNPN entry by default if(isvalue(v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.cellAccessRelatedInfo.npn_IdentityInfoList_r16[0].npn_IdentityList_r16[0].pni_npn_r16)) { v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.cellAccessRelatedInfo.npn_IdentityInfoList_r16[0].npn_IdentityList_r16[0].pni_npn_r16.plmn_Identity_r16 := valueof(p_PLMN_Identity); v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.cellAccessRelatedInfo.npn_IdentityInfoList_r16[0].npn_IdentityList_r16[0].pni_npn_r16.cag_IdentityList_r16 := {f_NR_InitCAGId(p_NR_CellId)}; } else { v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.cellAccessRelatedInfo.npn_IdentityInfoList_r16[0].npn_IdentityList_r16[0].snpn_r16.plmn_Identity_r16 := valueof(p_PLMN_Identity); v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.cellAccessRelatedInfo.npn_IdentityInfoList_r16[0].npn_IdentityList_r16[0].snpn_r16.nid_List_r16 := {f_NR_InitNDI(p_NR_CellId)}; } } } f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); }

  - Change 5

| Function name | New function f_NR_InitCAGId() |
| --- | --- |
| Reason for change | The function f_NR_CellInfo_SetPLMN() does not consider whether CAG ID or NID should be set along with the PLMN when NPN is used, and incorrectly only sets NID for all cases |
| Summary of change | Initialise CAG ID, see change 4 |
| TTCN module | NR_CellInfo.ttcn |
| MCC160 Comment | Not Needed.Please see comment on change 4. |

New function:

function f_NR_InitCAGId( NR_CellId_Type p_NR_CellId ) return CAG_IdentityInfo_r16 { var CAG_IdentityInfo_r16 v_CAGID_r16; v_CAGID_r16.manualCAGselectionAllowed_r16 := true_; select (p_NR_CellId) { case(nr_Cell2) { v_CAGID_r16.cag_Identity_r16 := int2bit(1,32)} case(nr_Cell3) { v_CAGID_r16.cag_Identity_r16 := int2bit(1,32)} case(nr_Cell4) { v_CAGID_r16.cag_Identity_r16 := int2bit(2,32)} case(nr_Cell6) { v_CAGID_r16.cag_Identity_r16 := int2bit(2,32)} case else {FatalError(__FILE__, __LINE__, "Invalid NR CellId");} } return v_CAGID_r16; }

- Branches executed

The test case was executed on NR band n41 using NR ciphering nea2 and integrity algorithm nia2

- Execution Log Files

  - Qualcomm Snapdragon X70

The Qualcomm Snapdragon X70 UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Keysight/TC_6_2_5_4_LOG.html

- PICS/PIXIT parameter file:

Keysight/TC_6_2_5_4_PIXIT.xml

- References

| [1] | R5s220769 : NR5GC FR1 : Supporting information for addition of NR5GC Rel-16 CAG test case 6.5.2.4 |
| --- | --- |