3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220791

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2605 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to EPS FB test case 11.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-06-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - According to 36.508 4.7.2-4 the EPS mobile identity is any allowed value - There is coordination between EUTRA_Multi5GS_PTC and IMS component missing in steps 24A-24C |  |  |  |  |  |  |  |  |
| Summary of change: |  | - EPS mobile identity is any allowed value - Added coordination trigger |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change 1	2

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | fl_TAUReject_AttachWithoutN26 |
| --- | --- |
| Reason for change | According to 36.508 4.7.2-4 the EPS mobile identity is any allowed value. |
| Summary of change | EPS mobile identity is any allowed value |
| TTCN module | EPS_Fallback_NR5GC_EUTRA.ttcn |
| MCC160 Comment | Accepted and implemented under R5-225449 |

Before Change

function fl_TAUReject_AttachWithoutN26 (boolean p_UseMappedGUTI := false) runs on EUTRA_Multi5GS_PTC { var SRB_COMMON_IND v_ReceivedAsp; var NAS_MSG_Indication_Type v_NasInd; var template (present) NAS_MSG_Indication_Type v_NAS_Expected; var EUTRA_NR_CoOrd_GUTI_Type v_CoOrd_Guti; var template (value) MobileIdentity v_MappedGUTI; var template (value) MobileIdentity v_MobileId; var GutiParameters_Type v_GutiParams; var template AdditionalUpdateType v_AdditionalUpdateType; var NAS_PlmnId v_PLMN; var TrackingAreaCode v_Tac; var template TrackingAreaId v_TAI:= omit; var template (present) NAS_KsiValue v_KsiValue; var EUTRA_NR_PduSessionInfoList_Type v_PduSessionInfoList; var template (omit) integer v_SessionId; v_CoOrd_Guti := f_EUTRA_NR_WaitForCoOrd_GutiParams(NR); v_MappedGUTI := f_EUTRA38_NR_Convert5G_CoOrdToEPS_GUTI (v_CoOrd_Guti, omit); v_PduSessionInfoList := f_EUTRA38_WaitForPDUSessionInfoList(eutra_Cell1); // @sic R5s210149 sic@ // Tell IMS PTC the RAT has changed f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IPCAN_IMS_QueryResponse(f_EUTRA_IMS_IpcanInfo())); v_AdditionalUpdateType := f_GetAdditionalUpdateType (NORMAL); v_KsiValue := f_EUTRA_SecurityKSIasme_Get(); v_GutiParams:= f_EUTRA_CellInfo_GetGuti(eutra_Cell1); v_PLMN := f_Asn2Nas_PlmnId(v_GutiParams.PLMN_Identity); v_Tac := f_EUTRA_CellInfo_GetTAC(eutra_Cell1); v_TAI := cr_TAI(v_PLMN, bit2oct(v_Tac)); v_MobileId := f_GutiParameters2MobileIdentity(omit, v_GutiParams); // @sic R5s211706 sic@ // @sic R5-212453, R5-223431 sic@ if (p_UseMappedGUTI) { // @sic R5-223431 sic@ 11.1.2. specifies to use a mapped GUTI v_MobileId := v_MappedGUTI; } // @siclog "Step 7 - 8" siclog@ v_NAS_Expected := (cr_NAS_Indication(tsc_SHT_IntegrityProtected, cr_TAU_Request_N1ToS1(f_GetEPSTAUType(NORMAL), v_KsiValue, cr_DRXparameter_Any ('5C'O) ifpresent, v_AdditionalUpdateType, v_MappedGUTI, v_TAI, tsc_EpsUpdate_Active, -, -, omit, cr_UeRadioCap ifpresent)), cr_NAS_IndicationWithPiggybacking(tsc_SHT_IntegrityProtected, cdr_ATTACH_REQUEST_N1ToS1(f_GetEAttachType(NORMAL), v_AdditionalUpdateType, v_KsiValue, v_MobileId, v_TAI), cr_PDN_CONNECTIVITY_REQUEST_N1ToS1(tsc_PdnRequest_Handover, f_GetPdnType()))); // @siclog "Step 8a1/8b1" siclog@ v_NasInd := f_EUTRA_RRC_ConnEst_DefWithNas(eutra_Cell1, tsc_RRC_TI_Def, ?, v_NAS_Expected); if (ischosen (v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST)) { // @siclog "Step 8b2" siclog@ SRB.send(cas_SRB1_NasPdu_REQ(eutra_Cell1, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_NoSecurityProtection, cs_508_TRACKING_AREA_UPDATE_REJECT(tsc_EmmCause_UEIdentityNotDerived)))); // @siclog "Step 8b3" siclog@ SRB.receive(car_SRB1_NasPdu_IND(eutra_Cell1, cr_NAS_IndicationWithPiggybacking((tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected), cdr_ATTACH_REQUEST_IMSIonly(f_GetEAttachType(NORMAL), v_AdditionalUpdateType, v_TAI), cr_PDN_CONNECTIVITY_REQUEST_N1ToS1(tsc_PdnRequest_Handover, f_GetPdnType())))) -> value v_ReceivedAsp; v_NasInd := v_ReceivedAsp.Signalling.Nas[0]; } // @siclog "Step 9 - 20" siclog@ v_SessionId := f_ProtocolConfigOptions_GetSessionId(v_NasInd.Pdu.PiggybackedPduList[0].Msg.pDN_CONNECTIVITY_REQUEST.protocolConfigurationOptions); if (not isvalue (v_SessionId)) { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Id not present in PCO"); } f_EUTRA38_MultiPDN_InitialRegistration_Step5_16 (eutra_Cell1, TESTMode_OFF, v_NasInd, -, v_PduSessionInfoList, true); // @sic R5s210149, R5s220177 sic@ }

After Change

function fl_TAUReject_AttachWithoutN26 (boolean p_UseMappedGUTI := false) runs on EUTRA_Multi5GS_PTC { var SRB_COMMON_IND v_ReceivedAsp; var NAS_MSG_Indication_Type v_NasInd; var template (present) NAS_MSG_Indication_Type v_NAS_Expected; var EUTRA_NR_CoOrd_GUTI_Type v_CoOrd_Guti; var template (value) MobileIdentity v_MappedGUTI; var template (value) MobileIdentity v_MobileId; var GutiParameters_Type v_GutiParams; var template AdditionalUpdateType v_AdditionalUpdateType; var NAS_PlmnId v_PLMN; var TrackingAreaCode v_Tac; var template TrackingAreaId v_TAI:= omit; var template (present) NAS_KsiValue v_KsiValue; var EUTRA_NR_PduSessionInfoList_Type v_PduSessionInfoList; var template (omit) integer v_SessionId; v_CoOrd_Guti := f_EUTRA_NR_WaitForCoOrd_GutiParams(NR); v_MappedGUTI := f_EUTRA38_NR_Convert5G_CoOrdToEPS_GUTI (v_CoOrd_Guti, omit); v_PduSessionInfoList := f_EUTRA38_WaitForPDUSessionInfoList(eutra_Cell1); // @sic R5s210149 sic@ // Tell IMS PTC the RAT has changed f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IPCAN_IMS_QueryResponse(f_EUTRA_IMS_IpcanInfo())); v_AdditionalUpdateType := f_GetAdditionalUpdateType (NORMAL); v_KsiValue := f_EUTRA_SecurityKSIasme_Get(); v_GutiParams:= f_EUTRA_CellInfo_GetGuti(eutra_Cell1); v_PLMN := f_Asn2Nas_PlmnId(v_GutiParams.PLMN_Identity); v_Tac := f_EUTRA_CellInfo_GetTAC(eutra_Cell1); v_TAI := cr_TAI(v_PLMN, bit2oct(v_Tac)); v_MobileId := f_GutiParameters2MobileIdentity(omit, v_GutiParams); // @sic R5s211706 sic@ // @sic R5-212453, R5-223431 sic@ if (p_UseMappedGUTI) { // @sic R5-223431 sic@ 11.1.2. specifies to use a mapped GUTI v_MobileId := v_MappedGUTI; } // @siclog "Step 7 - 8" siclog@ v_NAS_Expected := (cr_NAS_Indication(tsc_SHT_IntegrityProtected, cr_TAU_Request_N1ToS1(f_GetEPSTAUType(NORMAL), v_KsiValue, cr_DRXparameter_Any ('5C'O) ifpresent, v_AdditionalUpdateType, v_MappedGUTI, v_TAI, tsc_EpsUpdate_Active, -, -, omit, cr_UeRadioCap ifpresent)), cr_NAS_IndicationWithPiggybacking(tsc_SHT_IntegrityProtected, cdr_ATTACH_REQUEST_N1ToS1(f_GetEAttachType(NORMAL), v_AdditionalUpdateType, v_KsiValue, ?, v_TAI), cr_PDN_CONNECTIVITY_REQUEST_N1ToS1(tsc_PdnRequest_Handover, f_GetPdnType()))); // @siclog "Step 8a1/8b1" siclog@ v_NasInd := f_EUTRA_RRC_ConnEst_DefWithNas(eutra_Cell1, tsc_RRC_TI_Def, ?, v_NAS_Expected); if (ischosen (v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST)) { // @siclog "Step 8b2" siclog@ SRB.send(cas_SRB1_NasPdu_REQ(eutra_Cell1, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_NoSecurityProtection, cs_508_TRACKING_AREA_UPDATE_REJECT(tsc_EmmCause_UEIdentityNotDerived)))); // @siclog "Step 8b3" siclog@ SRB.receive(car_SRB1_NasPdu_IND(eutra_Cell1, cr_NAS_IndicationWithPiggybacking((tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected), cdr_ATTACH_REQUEST_IMSIonly(f_GetEAttachType(NORMAL), v_AdditionalUpdateType, v_TAI), cr_PDN_CONNECTIVITY_REQUEST_N1ToS1(tsc_PdnRequest_Handover, f_GetPdnType())))) -> value v_ReceivedAsp; v_NasInd := v_ReceivedAsp.Signalling.Nas[0]; } // @siclog "Step 9 - 20" siclog@ v_SessionId := f_ProtocolConfigOptions_GetSessionId(v_NasInd.Pdu.PiggybackedPduList[0].Msg.pDN_CONNECTIVITY_REQUEST.protocolConfigurationOptions); if (not isvalue (v_SessionId)) { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Id not present in PCO"); } f_EUTRA38_MultiPDN_InitialRegistration_Step5_16 (eutra_Cell1, TESTMode_OFF, v_NasInd, -, v_PduSessionInfoList, true); // @sic R5s210149, R5s220177 sic@ }

## Change 2

| Function name | fl_TC_11_1_2_NR5GC_EUTRA_TestBody |
| --- | --- |
| Reason for change | There is coordination between EUTRA_Multi5GS_PTC and IMS component missing in steps 24A-24C |
| Summary of change | Added coordination trigger. |
| TTCN module | EPS_Fallback_NR5GC_EUTRA.ttcn |
| MCC160 Comment | Alternative solution implemented: removed the wait for the 2nd instance of this message from f_TC_11_1_2_NR5GC_IMS1 in the IMS PTC |

Before Change

function fl_TC_11_1_2_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC // @sic R5-204399 sic@ { var integer v_EPSBearerId; // Send EUTRA frequency to NR and wait for NR info f_EUTRA_NR_SendSysinfoCoOrd(NR, f_EUTRA38_IRAT_ListOfFrequencies({eutra_Cell1})); f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Change Power Level"); // @sic R5-216190 sic@ // @siclog "Step 5AA" siclog@ f_EUTRA38_SetCellPower(eutra_Cell1, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE); fl_TAUReject_AttachWithoutN26 (true); // @sic R5-223431 sic@ // @siclog "Parallel Behaviour" siclog@ f_EUTRA38_MultiPDN_EstablishAdditionalPDNs (eutra_Cell1, (pc_noOf_PDNsSameConnection + pc_noOf_PDNsNewConnection), false, tsc_SRB2, omit, '010'B, cr_AccessPointName(f_DomainName_Encode(pc_APN_ID_Internet))); // @sic R5-216188 sic@ // @siclog "Step 24A - 24C" siclog@ // @sic R5-223430 sic@ v_EPSBearerId := f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(eutra_Cell1); // @sic R5s220448 sic@ // Test body finishes on IMS PTC f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_EUTRA_DeactivateEPS_BearerContext (eutra_Cell1, int2hex(v_EPSBearerId, 1)); // @sic R5s220448 sic@ f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); }

After Change

function fl_TC_11_1_2_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC // @sic R5-204399 sic@ { var integer v_EPSBearerId; // Send EUTRA frequency to NR and wait for NR info f_EUTRA_NR_SendSysinfoCoOrd(NR, f_EUTRA38_IRAT_ListOfFrequencies({eutra_Cell1})); f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Change Power Level"); // @sic R5-216190 sic@ // @siclog "Step 5AA" siclog@ f_EUTRA38_SetCellPower(eutra_Cell1, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE); fl_TAUReject_AttachWithoutN26 (true); // @sic R5-223431 sic@ // @siclog "Parallel Behaviour" siclog@ f_EUTRA38_MultiPDN_EstablishAdditionalPDNs (eutra_Cell1, (pc_noOf_PDNsSameConnection + pc_noOf_PDNsNewConnection), false, tsc_SRB2, omit, '010'B, cr_AccessPointName(f_DomainName_Encode(pc_APN_ID_Internet))); // @sic R5-216188 sic@ // @siclog "Step 24A - 24C" siclog@ // @sic R5-223430 sic@ v_EPSBearerId := f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(eutra_Cell1); // @sic R5s220448 sic@ f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IPCAN_IMS_QueryResponse(f_EUTRA_IMS_IpcanInfo())); // Test body finishes on IMS PTC f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_EUTRA_DeactivateEPS_BearerContext (eutra_Cell1, int2hex(v_EPSBearerId, 1)); // @sic R5s220448 sic@ f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); }