3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220793

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2607 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 7.1.1.3.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-06-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to Prose CR R5-222111 raised and agreed at RAN5 95-e, pre test condition is to use DCI_0_0 in NR Serving cell configuration. Hence, corresponding TTCN changes should be done in the Test Body as well to use Short DCI to reflect the Prose CR |  |  |  |  |  |  |  |  |
| Summary of change: |  | Short DCI related TTCN changes are implemented |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.3.NR5GC, 7.1.1.3.3.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required for NR testcase 7.1.1.3.3	2

2.1	Correction to f_TC_7_1_1_3_3_NR_TestBody	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required for NR testcase 7.1.1.3.3

## Correction to f_TC_7_1_1_3_3_NR_TestBody

| Function name | f_TC_7_1_1_3_3_NR_TestBody () |
| --- | --- |
| Reason for change | According to Prose CR R5-222111 raised and agreed at RAN5 95-e, pre test condition is to use DCI_0_0 in NR Serving cell configuration. Hence, corresponding TTCN changes should be done in the Test Body to use Short DCI to reflect the Prose CR |
| Summary of change | Short DCI related TTCN changes are implemented |
| TTCN module | MAC_TC_Common_NR |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_7_1_1_3_3_NR_TestBody(DRB_Identity p_NR_DRB_Id1, DRB_Identity p_NR_DRB_Id2) runs on NR_BASE_PTC { var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); //@sic R5s201617 sic@ var NR_ResourceAllocation_Type v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(208, v_NR_UplinkBWP); var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var NR_SYSTEM_IND v_Received_NR_SysInd; var integer v_SR_Count := 0; var integer v_Duration; var boolean v_Flag := false; var boolean v_PRACH_Flag := false; var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedure := cs_NR_RachProcedureConfig_CBRA_CrntiBased(v_NR_PhysicalParameters); //@sic R5s200747 sic@ var SubFrameTiming_Type v_TimingDRB; var SubFrameTiming_Type v_TimingSysInd; <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_3_3_NR_TestBody(DRB_Identity p_NR_DRB_Id1, DRB_Identity p_NR_DRB_Id2) runs on NR_BASE_PTC { var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); //@sic R5s201617 sic@ var NR_ResourceAllocation_Type v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(208, v_NR_UplinkBWP, -, formats0_0_And_1_0); //WA# var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var NR_SYSTEM_IND v_Received_NR_SysInd; var integer v_SR_Count := 0; var integer v_Duration; var boolean v_Flag := false; var boolean v_PRACH_Flag := false; var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedure := cs_NR_RachProcedureConfig_CBRA_CrntiBased(v_NR_PhysicalParameters,-,-,-,-,cs_NR_DciFormat_0_0_Params); //WA# //@sic R5s200747 sic@ var SubFrameTiming_Type v_TimingDRB; var SubFrameTiming_Type v_TimingSysInd; <<SKIPPED CODE>>