3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220807

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2618 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for function f_NR5GC_RRC_Idle_Steps16_20() |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-06-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The parameter p_ReleaseConnection is currently not passed on to the function f_NR5GC_PDUSessionEstablishment(). Note : Because of this the parameter p_ConnectionWillBeReleased in f_NR5GC_PDUSessionEstablishment() will always default to rrcConnectionRelease, which means the function f_NR_DelayForUserPlaneSignalling() is always called so the 10s timer t_WaitForUPlaneSignalling is started. For e.g. 34.229-5 test cases where the test case will starte executing on IMS PTC as soon as IMS signalling begins, this timer will block IPCAN requests (e.g.IPCAN_Release) for 10s. This timer should not be started in the case where the calling function (e.g. f_NR5GC_IPCAN_IdleUpdated()) specifies “noRrcConnectionRelease” |  |  |  |  |  |  |  |  |
| Summary of change: |  | Pass the parameter p_ReleaseConnection through to _NR5GC_PDUSessionEstablishment() |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | f_NR5GC_RRC_Idle_Steps16_20() |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk23 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## 2.1 Change 1

| Function name | f_NR5GC_RRC_Idle_Steps16_20() |
| --- | --- |
| Reason for change | The parameter p_ReleaseConnection is currently not passed on to the function f_NR5GC_PDUSessionEstablishment(). Note : Because of this the parameter p_ConnectionWillBeReleased in f_NR5GC_PDUSessionEstablishment() will always default to rrcConnectionRelease, which means the function f_NR_DelayForUserPlaneSignalling() is always called so the 10s timer t_WaitForUPlaneSignalling is started. For e.g. 34.229-5 test cases where the test case will starte executing on IMS PTC as soon as IMS signalling begins, this timer will block IPCAN requests (e.g.IPCAN_Release) for 10s. This timer should not be started in the case where the calling function (e.g. f_NR5GC_IPCAN_IdleUpdated()) specifies “noRrcConnectionRelease” |
| Summary of change | Pass the parameter p_ReleaseConnection through to _NR5GC_PDUSessionEstablishment() |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment | Accept that IPCAN (34.229-5) TCs do not need to wait for user plane signalling, but to avoid impact in 38.523-1 TCs, implemented alternative solution: modified to use parameter p_NormalIMSOperation instead and called this from f_NR5GC_IPCAN_IdleUpdated |

Before Change

function f_NR5GC_RRC_Idle_Steps16_20(NR_CellId_Type p_NR_CellId, integer p_ExpectedNumberOfNewPDUSessions := pc_noOf_PDUsSameConnection, // @sic R5s201358 sic@, NR_TESTMODE_STATE_Type p_TestMode := TESTMode_OFF, RRCConnectionReleaseRequired_Type p_ReleaseConnection := noRrcConnectionRelease, template (value) UE_TestLoopModeA_NR_LB_Setup_Type p_UE_TestLoopModeA_NR_LB_Setup := cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O, template (omit) SDAP_Config p_SDAP_Config := omit, template S_NSSAI_Type p_ExpectedS_NSSAI := *, template DNN p_ExpectedDNN := *, template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B), template (omit) DNN p_DNN := omit, template (omit) QoS_Rules p_QoS_Rules := omit, template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O), template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI, // @sic R5s190919 sic@ template (omit) QoSFlowDescr p_QoSFlowDescr := omit, template (present) B4_Type p_ServiceType := '0000'B) runs on NR5GC_PTC { //Steps 16 - 19a1 // No SRB2 so must be on SRB1 if (p_ExpectedNumberOfNewPDUSessions < 0 ) { // @sic R5-194797, R5-199075, R5-204329 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "3GPP RAN5 5GS test specifications do not support UEs configured with pc_noOf_PDUsSameConnection // @sic R5s201358 sic@ < 0"); } f_NR5GC_PDUSessionEstablishment (p_NR_CellId, p_ExpectedNumberOfNewPDUSessions, tsc_NR_RbId_SRB1, p_SDAP_Config, // @sic R5s210134 sic@ p_ExpectedS_NSSAI, p_ExpectedDNN, p_SSC_Mode, p_DNN, p_QoS_Rules, p_Session_AMBR, p_S_NSSAI, p_QoSFlowDescr); …………………………. }

After Change

function f_NR5GC_RRC_Idle_Steps16_20(NR_CellId_Type p_NR_CellId, integer p_ExpectedNumberOfNewPDUSessions := pc_noOf_PDUsSameConnection, // @sic R5s201358 sic@, NR_TESTMODE_STATE_Type p_TestMode := TESTMode_OFF, RRCConnectionReleaseRequired_Type p_ReleaseConnection := noRrcConnectionRelease, template (value) UE_TestLoopModeA_NR_LB_Setup_Type p_UE_TestLoopModeA_NR_LB_Setup := cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O, template (omit) SDAP_Config p_SDAP_Config := omit, template S_NSSAI_Type p_ExpectedS_NSSAI := *, template DNN p_ExpectedDNN := *, template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B), template (omit) DNN p_DNN := omit, template (omit) QoS_Rules p_QoS_Rules := omit, template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O), template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI, // @sic R5s190919 sic@ template (omit) QoSFlowDescr p_QoSFlowDescr := omit, template (present) B4_Type p_ServiceType := '0000'B) runs on NR5GC_PTC { //Steps 16 - 19a1 // No SRB2 so must be on SRB1 if (p_ExpectedNumberOfNewPDUSessions < 0 ) { // @sic R5-194797, R5-199075, R5-204329 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "3GPP RAN5 5GS test specifications do not support UEs configured with pc_noOf_PDUsSameConnection // @sic R5s201358 sic@ < 0"); } f_NR5GC_PDUSessionEstablishment (p_NR_CellId, p_ExpectedNumberOfNewPDUSessions, tsc_NR_RbId_SRB1, p_SDAP_Config, // @sic R5s210134 sic@ p_ExpectedS_NSSAI, p_ExpectedDNN, p_SSC_Mode, p_DNN, p_QoS_Rules, p_Session_AMBR, p_S_NSSAI, p_QoSFlowDescr,-,-,-,-,-,-,-,-,-,-,-,-,-,-,-,-,-,-,-,-,-,-,p_ReleaseConnection); ………………….. }

#### MCC160 Implementation

function f_NR5GC_IPCAN_IdleUpdated(NR_CellId_Type p_NR_CellId, TypeOfRegistration_Type p_RegType := Initial_Secure) runs on NR5GC_PTC

{

f_NR5GC_Registration(p_NR_CellId, -, p_RegType);

f_NR5GC_RRC_Idle_Steps16_20 (p_NR_CellId,

-,

-,

noRrcConnectionRelease,

-, -, -, -, -, -, -, -, -, -, -, -,

false);

}

function f_NR5GC_RRC_Idle_Steps16_20(NR_CellId_Type p_NR_CellId,

integer p_ExpectedNumberOfNewPDUSessions := pc_noOf_PDUsSameConnection,  // @sic R5s201358 sic@,

NR_TESTMODE_STATE_Type p_TestMode := TESTMode_OFF,

RRCConnectionReleaseRequired_Type p_ReleaseConnection := noRrcConnectionRelease,

template (value) UE_TestLoopModeA_NR_LB_Setup_Type p_UE_TestLoopModeA_NR_LB_Setup := cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling,

UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O,

template (omit) SDAP_Config p_SDAP_Config := omit,

template S_NSSAI_Type p_ExpectedS_NSSAI := *,

template DNN p_ExpectedDNN := *,

template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B),

template (omit) DNN p_DNN := omit,

template (omit) QoS_Rules p_QoS_Rules := omit,

template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O),

template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI, // @sic R5s190919 sic@

template (omit) QoSFlowDescr p_QoSFlowDescr := omit,

template (present) B4_Type p_ServiceType := '0000'B,

boolean p_NormalIMSOperation := true)  runs on NR5GC_PTC

{

//Steps 16 - 19a1

// No SRB2 so must be on SRB1

if (p_ExpectedNumberOfNewPDUSessions < 0 ) { // @sic R5-194797, R5-199075, R5-204329 sic@

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "3GPP RAN5 5GS test specifications do not support UEs configured with pc_noOf_PDUsSameConnection  // @sic R5s201358 sic@ < 0");

}

f_NR5GC_PDUSessionEstablishment (p_NR_CellId,

p_ExpectedNumberOfNewPDUSessions,

tsc_NR_RbId_SRB1,

p_SDAP_Config, // @sic R5s210134 sic@

p_ExpectedS_NSSAI,

p_ExpectedDNN,

p_SSC_Mode,

p_DNN,

p_QoS_Rules,

p_Session_AMBR,

p_S_NSSAI,

p_QoSFlowDescr,

-, -, -, -, -, -, -, -, -, -, -, -, -, -, -,

-, -, -, -, -, -, -,

p_NormalIMSOperation);

//Step 20a1

if (p_ReleaseConnection == rrcConnectionRelease or pc_noOf_PDUsNewConnection >0) {// @sic R5-199075, R5s200489 sic@

// Release RRC to enter RRC_IDLE

f_NR_RRCRelease(p_NR_CellId);

} else { // @sic R5s200489 sic@

// Step 19Aa1 - 19Aa2

select (p_TestMode) { // @sic R5-194797 sic@

case (TEST_LOOPModeA_ON) {

f_NR5GC_CloseUE_TestLoopModeA (p_NR_CellId, p_UE_TestLoopModeA_NR_LB_Setup);

}

case (TEST_LOOPModeB_ON) {

f_NR5GC_CloseUE_TestLoopModeB (p_NR_CellId, p_IpPduDelayTime);

}

case (PING_Or_TEST_LOOPModeB_ON) {

if (not pc_IP_Ping) {

f_NR5GC_CloseUE_TestLoopModeB (p_NR_CellId, p_IpPduDelayTime);

}

}

}

}

if (pc_noOf_PDUsNewConnection >0 ){ // @sic R5s200026 sic@

// @sic R5-199075 sic@

f_NR5GC_RRC_Idle_Extension(p_NR_CellId,

p_TestMode,

p_ReleaseConnection,

p_UE_TestLoopModeA_NR_LB_Setup,

p_IpPduDelayTime,

-,

p_ExpectedS_NSSAI,

p_ExpectedDNN,

p_SSC_Mode,

p_DNN,

p_QoS_Rules,

p_Session_AMBR,

p_S_NSSAI,

p_QoSFlowDescr,

p_ServiceType); // @sic R5s210189, R5s210608 sic@

}

}

function f_NR5GC_PDUSessionEstablishment(NR_CellId_Type p_NR_CellId,

integer p_ExpectedNumberOfNewPDUSessions := pc_noOf_PDUsSameConnection,  // @sic R5s201358 sic@,

NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB2,

template (omit) SDAP_Config p_SDAP_Config := omit,

template S_NSSAI_Type p_ExpectedS_NSSAI := *,

template DNN p_ExpectedDNN := *,

template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B),

template (omit) DNN p_DNN := omit,

template (omit) QoS_Rules p_QoS_Rules := omit,

template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O),

template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI, // @sic R5s190919 sic@

template (omit) QoSFlowDescr p_QoSFlowDescr := omit,

template (present) O1_Type p_PDU_SessionId := ?,

template ProcedureTransactionIdentifier p_PTI := omit,

template (omit) PDU_SessionType p_PDU_SessionType := omit,

template SSC_Mode p_ExpectedSSC_Mode := *, // @sic R5s190290 sic@

template NG_UE_SM_Cap p_UECap := *,

template MaxNumPacketFilters p_MaxNumPacketFilters := *,

template IntegrityProtMaxDataRate p_IntegrityProtMaxDataRate := ?,

template AlwaysOnPDUSessionReq p_AlwaysOnPDUSessionReq := *,

template SM_PDU_DN_RequestContainer p_PDUReq := *,

template ExtdProtocolConfigOptions p_ExtdPCO := *,

template NG_PDU_SessionId p_OldPDU_SessionId := *,

template NG_Request_Type p_RequestType := cr_NG_Request_Type('001'B),

template AdditionalInformation p_AdditionalInfo := *,

template (omit) GMM_GSM_Cause p_Cause := omit,

template (omit) GPRS_Timer p_RQTimer := omit,

template (omit) EAP_Message p_EAP := omit,

template (omit) GPRS_Timer3 p_BackOff := omit,

boolean p_PDUSessionAuthenticationRequired := false,

template (omit) charstring p_TriggerWithThisAPN := omit,

boolean p_FailWhenK_LessN := true,

template (omit) charstring p_BuildQosFlow := omit,

template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduInfoFromLTE := omit, // @sic R5-198184, R5-199075 sic@

boolean p_NormalIMSOperation := true, // @sic R5s210515 sic@

integer p_EstReqAlreadyInDefault := 0 ) runs on NR5GC_PTC // @sic R5s210515 sic@

{

var SRBs_AlreadyConfigured v_SRBs := SRB1_2;

var NR_RadioBearerId_Type v_SRB := p_SRB;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var integer v_K_NumOfReqsProcessed := 0;

var integer v_L_NumOfReqsReceivedInParallel := 0;

var boolean v_EndLoop := false;

var boolean v_RxdMsg;

var boolean v_IMS_Configured := false;

var PDU_PDN_DNN_Type v_PDUType;

var DelayForUserPlaneSignalling_Type v_WaitForUserplaneSignalling := noDelay; // Default: no IP signalling possible

var default v_MyDefaultVar := null;

timer t_Wait := 8.0; // @sic R5-206288 sic@ 8s is less than T3540

if (p_EstReqAlreadyInDefault == 0){ // @sic R5s210520 sic@

// Clear store of msgs received in parallel

f_NR5GC_MsgInDefault_ResetStoreOfNASMessageReq(); // @sic R5s220174 sic@

}

// Then activate store for any new msgs received

v_MyDefaultVar := activate(a_NR5GC_AdditionalPDUSessionEstablishmentReq(p_NR_CellId));

while ((v_K_NumOfReqsProcessed < p_ExpectedNumberOfNewPDUSessions) and not v_EndLoop) { // @sic R5-199075, R5-204329 sic@

v_RxdMsg := false;

t_Wait.start; // @sic R5-204329 sic@

if (f_NR5GC_MsgInDefault_GetNumOfNASMessageReq() > v_L_NumOfReqsReceivedInParallel) { // @sic R5s220174 sic@

// If another msg has been recieved in parallel since the last time we looked

v_L_NumOfReqsReceivedInParallel := v_L_NumOfReqsReceivedInParallel + 1;

if (v_L_NumOfReqsReceivedInParallel > p_ExpectedNumberOfNewPDUSessions){ // @sic R5-204329 sic@

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "More PDU Session Establishment Messages Received Than Expected");

}

v_ReceivedAsp := f_NR5GC_MsgInDefault_GetOneNASMessageReq(v_L_NumOfReqsReceivedInParallel); // @sic R5s220174 sic@

v_RxdMsg := true;

t_Wait.stop;      // @sic R5s210520 sic@

} else { // no outstanding msgs, so must wait for next PDU Establishment

alt {

[] SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp

{ // Stop timer and continue with while loop

t_Wait.stop;

v_RxdMsg := true;

}

[] t_Wait.timeout

{

if (p_FailWhenK_LessN) { // @sic R5-204329 sic@

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Not enough PDU Session Establishment Messages Received");

} else {

v_EndLoop := true;

}

}

}

}

if (v_K_NumOfReqsProcessed > 0) { // @sic R5s200751 sic@

if (isvalue(p_TriggerWithThisAPN)) { // @sic R5-199075  sic@

f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, valueof(p_TriggerWithThisAPN)); // @sic R5-204439 sic@

}

}

if (v_RxdMsg) { // @sic R5-204329 sic@ this will be false if either just triggered a new PDN, or about to end the loop

// Update K now as latest msg is about to be processed

v_K_NumOfReqsProcessed := v_K_NumOfReqsProcessed + 1;

if (v_SRB == tsc_NR_RbId_SRB1) { // @sic R5-198923 sic@

v_SRBs := SRB1;

}

v_PDUType := f_NR5GC_PDUSessionEstablishment_InnerLoop(p_NR_CellId,

v_ReceivedAsp,

v_SRBs,

p_SDAP_Config,

p_ExpectedS_NSSAI,

p_ExpectedDNN,

p_SSC_Mode,

p_DNN,

p_QoS_Rules,

p_Session_AMBR,

p_S_NSSAI,

p_QoSFlowDescr,

p_PDU_SessionId,

p_PTI,

p_PDU_SessionType,

p_ExpectedSSC_Mode,

p_UECap,

p_MaxNumPacketFilters,

p_IntegrityProtMaxDataRate,

p_AlwaysOnPDUSessionReq,

p_PDUReq,

p_ExtdPCO,

p_OldPDU_SessionId,

p_RequestType,

p_AdditionalInfo,

p_Cause,

p_RQTimer,

p_EAP,

p_BackOff,

p_PDUSessionAuthenticationRequired,

p_BuildQosFlow,

p_PduInfoFromLTE); // @sic R5-198184 sic@

if (v_SRB == tsc_NR_RbId_SRB1) { // SRB2 will be configured in the 1st instance (if it hasn't already)

v_SRB := tsc_NR_RbId_SRB2;

v_SRBs := SRB1_2;  // @sic R5-198923 sic@

}

v_IMS_Configured := v_IMS_Configured or (match (IMS_DNN, v_PDUType));    // @sic R5-206296 sic@

}

}

if (p_NormalIMSOperation) { // @sic R5s210515 sic@

/* Need to delay for IP/IMS signalling if RRCRelease is about to be sent

if set to noRrcConnectionRelease, assume the connection will be up long enough

to complete IMS signalling without waiting here */

// Wait for IP signalling @sic R5-204329, R5-206296 sic@

if (v_IMS_Configured) {

v_WaitForUserplaneSignalling := waitForIMS;

}  else { // currently all PDUs require IP signalling

v_WaitForUserplaneSignalling := dontWaitForIMS;

}

f_NR_DelayForUserPlaneSignalling(v_WaitForUserplaneSignalling);  // @sic R5-206296 sic@

}

// all expected msgs have been processed, so deactivate the extra default

deactivate(v_MyDefaultVar);

}