3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220817

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2627 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC Test case 11.3.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-06-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The f_IMS_MOCallSetup_NR5GC_A_4_X_Step1 function already hadles reception of the INVITE so when calling also f_IMS_MOCallSetup_ReceiveINVITE_Common it leads to the test waiting for another INVITE to be received. - Since this test is not an early contention test it only needs to start UL grant instead of calling the entire f_NR_RRCSetup_Def function in a preparation for the new conneciton. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - fl_IMS_NR5GC_MOSpeechCall_Step1 is no more calling the f_IMS_MOCallSetup_ReceiveINVITE_Common twice. - Replaced f_NR_RRCSetup_Def with f_NR_ULGrantConfiguration_Start before the step 18 starts. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.2.1.NR5GC, 11.3.1.NR5GC, 11.4.1.NR5GC, 11.4.7.NR5GC, 11.4.8.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change 1	2

2.2	Change 2	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | fl_IMS_NR5GC_MOSpeechCall_Step1 |
| --- | --- |
| Reason for change | The f_IMS_MOCallSetup_NR5GC_A_4_X_Step1 function already hadles reception of the INVITE so when calling also f_IMS_MOCallSetup_ReceiveINVITE_Common it leads to the test waiting for another INVITE to be received. |
| Summary of change | fl_IMS_NR5GC_MOSpeechCall_Step1 is no more calling the f_IMS_MOCallSetup_ReceiveINVITE_Common twice. |
| TTCN module | IMS_NR5GC_Functions.ttcn |
| MCC160 Comment | Accepted |

Before Change

function fl_IMS_NR5GC_MOSpeechCall_Step1(template SipUrl p_OtherCalleeUri := omit) runs on IMS_PTC return IMS_InviteRequestWithSdp_Type { if (pc_Preconditions) { // @sic R5-222077: Additional checks for A.4.2 (no 'precondition' option tag in Supported header field, no precondition attributes) sic@ f_IMS_MOCallSetup_NR5GC_A_4_1_Step1(p_OtherCalleeUri); } else { f_IMS_MOCallSetup_NR5GC_A_4_2_Step1(p_OtherCalleeUri); } return f_IMS_MOCallSetup_ReceiveINVITE_Common(p_OtherCalleeUri, -, tsc_IMS_SDP_AudioCodecList_EVS_WBAMR_AMR, pc_Preconditions); // @sic R5s10603 sic@ }

After Change

function fl_IMS_NR5GC_MOSpeechCall_Step1(template SipUrl p_OtherCalleeUri := omit) runs on IMS_PTC return IMS_InviteRequestWithSdp_Type { if (pc_Preconditions) { // @sic R5-222077: Additional checks for A.4.2 (no 'precondition' option tag in Supported header field, no precondition attributes) sic@ return f_IMS_MOCallSetup_ReceiveINVITE_Common(p_OtherCalleeUri, -, tsc_IMS_SDP_AudioCodecList_EVS_WBAMR_AMR, true); } else { return f_IMS_MOCallSetup_NR5GC_A_4_2_Step1(p_OtherCalleeUri); } }

## Change 2

| Function name | fl_TC_11_3_1_TestBody |
| --- | --- |
| Reason for change | Since this test is not an early contention test it only needs to start UL grant instead of calling the entire f_NR_RRCSetup_Def function. |
| Summary of change | Replaced f_NR_RRCSetup_Def with f_NR_ULGrantConfiguration_Start before the step 18 starts. |
| TTCN module | UAC_NR5GC.ttcn |
| MCC160 Comment | Accepted but implemented differently. UL Grant also started in step 34Aa1 |

Before Change

function fl_TC_11_3_1_TestBody() runs on NR5GC_PTC { … //@siclog "Steps 18-28" siclog@ //Step 3 to step 13 from test procedure for IMS MO speech call establishment as described in TS 38.508-1 [4] Table 4.9.15.2.2-1 take place, // v_ReceivedMsg := f_NR5GC_RRCIdleToConnected_Steps2_4(nr_Cell1, '0001'B, mo_VoiceCall); //v_GSM_MobilityInfo := f_NR5GC_SpeechCall_Steps2_8(nr_Cell1, mo_VoiceCall, '0001'B); // @sic R5s210037 sic@ // send RRC Connection Setup on SRB0 (CCCH) and switch on UL grant assignments: f_NR_RRCSetup_Def (nr_Cell1); // UE shall establish SRB1 acc. to 38.331 cl. 5.3.3.1 // UE enters RRC_CONNECTED state // receive RRC Connection Setup Complete with piggy-backed NAS message on SRB1 (DCCH): v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected ); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), f_NG_S_TMSI2MobileIdentity(omit, f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1)), '0001'B)) { // @sic R5s200654 subject to prose CR approval sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); v_GSM_MobilityInfo:= f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), IMS_DNN); // done on IMS PTC // Wait until need to add voice QoS f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_NR5GC_ModifyPDUSession_AddVoice(nr_Cell1, valueof(v_GSM_MobilityInfo)); if (not pc_Preconditions) { // @sic R5s220579 sic@ f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Voice Call Established"); // Call now established f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); //@siclog "Steps 29-31" siclog@

After Change

function fl_TC_11_3_1_TestBody() runs on NR5GC_PTC { … //@siclog "Steps 18-28" siclog@ //Step 3 to step 13 from test procedure for IMS MO speech call establishment as described in TS 38.508-1 [4] Table 4.9.15.2.2-1 take place, // v_ReceivedMsg := f_NR5GC_RRCIdleToConnected_Steps2_4(nr_Cell1, '0001'B, mo_VoiceCall); //v_GSM_MobilityInfo := f_NR5GC_SpeechCall_Steps2_8(nr_Cell1, mo_VoiceCall, '0001'B); // @sic R5s210037 sic@ // send RRC Connection Setup on SRB0 (CCCH) and switch on UL grant assignments: f_NR_ULGrantConfiguration_Start(nr_Cell1); // UE shall establish SRB1 acc. to 38.331 cl. 5.3.3.1 // UE enters RRC_CONNECTED state // receive RRC Connection Setup Complete with piggy-backed NAS message on SRB1 (DCCH): v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected ); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), f_NG_S_TMSI2MobileIdentity(omit, f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1)), '0001'B)) { // @sic R5s200654 subject to prose CR approval sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); v_GSM_MobilityInfo:= f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), IMS_DNN); // done on IMS PTC // Wait until need to add voice QoS f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_NR5GC_ModifyPDUSession_AddVoice(nr_Cell1, valueof(v_GSM_MobilityInfo)); if (not pc_Preconditions) { // @sic R5s220579 sic@ f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Voice Call Established"); // Call now established f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); //@siclog "Steps 29-31" siclog@

MCC160 Implementation

function fl_TC_11_3_1_TestBody() runs on NR5GC_PTC

{

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo;

var float v_TimerValue := 21.0; //@sic R5-221459 sic@

timer t_WaitTime;

//@sic R5s211658 sic@

f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, //@sic R5-221459 sic@

cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_1Entry(4,1),

-, cs_UAC_BarringInfoSetList_1Entry(p00, s16, '0000000'B)));

f_NR_ModifySysinfo(nr_Cell1);

f_NR_WaitModificationPeriods(nr_Cell1);

// @sic R5-216192 sic@

//@siclog "Step 13" siclog@

//Make the UE attempt a MMTel voice call.

f_UT_RequestIMSCall(UT);

//@siclog "Step 14" siclog@

//Check: Does the UE transmit RRCSetupRequest message including mo-VoiceCall with within 21 s?

t_WaitTime.start(v_TimerValue);

alt {

[] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(mo_VoiceCall,?)))

{

t_WaitTime.stop;

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 14");

}

[] t_WaitTime.timeout

{

}

}

//@siclog "Step 15" siclog@

//SS changes SIB1 according to Table 11.3.1.3.3-1 and sends Short Message on PDCCH using P-RNTI. Wait for the new system information to take effect and T390 expire. (Note 1)

f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1,

cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_1Entry(6,1),

-, cs_UAC_BarringInfoSetList_1Entry(p00, s16, '0000000'B)));

f_NR_ModifySysinfo(nr_Cell1); //Modify Sysinfo in SS

//@sic R5-221459 sic@

//@siclog "Step 15A" siclog@

t_WaitTime.start(20.0);

alt {

[] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest(mo_VoiceCall,?)))

{

t_WaitTime.stop;

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15Aa1");

f_NR_ULGrantConfiguration_Start(nr_Cell1); // @sic R5s220817 sic@

}

[] t_WaitTime.timeout

{

//@siclog "Step 15Ab2 - 15Ab3" siclog@

//Make the UE attempt another MO MTSI MMTEL Voice session.

//Check: Does the UE send NR RRCSetupRequest with EstablishmentCause set to 'mo-VoiceCall'?

f_UT_RequestIMSCall(UT);

f_NR_RRC_SetupRequest_Def(nr_Cell1, mo_VoiceCall);

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15Ab3");

}

}

//@siclog "Steps 16-17" siclog@

// void

//@siclog "Steps 18-28" siclog@

//Step 3 to step 13 from test procedure for IMS MO speech call establishment as described in TS 38.508-1 [4] Table 4.9.15.2.2-1 take place,:

//Send RRC Connection Setup on SRB0 (CCCH): done automatically by SS in Msg4 of the RA procedure

v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1,

cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def),

tsc_SHT_IntegrityProtected

);

if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), f_NG_S_TMSI2MobileIdentity(omit, f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1)), '0001'B)) {  // @sic R5s200654 subject to prose CR approval sic@

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed");

}

f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg);

v_GSM_MobilityInfo:= f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), IMS_DNN);

// done on IMS PTC

// Wait until need to add voice QoS

f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]);

f_NR5GC_ModifyPDUSession_AddVoice(nr_Cell1, valueof(v_GSM_MobilityInfo));

if (not pc_Preconditions) {  // @sic R5s220579 sic@

f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]);

}

f_NR_PreliminaryPass(__FILE__, __LINE__, "Voice Call Established");

// Call now established

f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]);

//@siclog "Steps 29-31" siclog@

//Step 1 to step 5 from test procedure for test procedure for IMS MO speech call release as described in TS 38.508-1 [4] Table 4.9.17.2.2-1 take place

f_NR5GC_CallRelease(nr_Cell1, IPCAN_SpeechCall);

//@siclog "Step 31A" siclog@

//The SS transmits an RRCRelease message to release RRC connection and move the UE to RRC_IDLE.

f_NR_RRCRelease(nr_Cell1); //@sic R5-216192 sic@

//@siclog "Step 32" siclog@

//AT command to make the UE attempt to send SMS over IP

f_UT_IMS_TriggerSMS(UT);

//@siclog "Step 33" siclog@

//Check: Does the UE transmit an RRCSetupRequest message with 'establishmentCause' set to 'mo-SMS'?

// @sic R5-210356 sic@

t_WaitTime.start(v_TimerValue);

alt {

[] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, mo_SMS))) //@sic R5-221459 sic@

{

t_WaitTime.stop;

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 33");

}

[] t_WaitTime.timeout {}

}

//@siclog "Step 34" siclog@

//SS changes SIB1 according to Table 11.3.1.3.3-1 and sends Short Message on PDCCH using P-RNTI. Wait for the new system information to take effect and T390 expire. (Note 1)

f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, omit);

f_NR_ModifySysinfo(nr_Cell1);

//@siclog "Step 34A" siclog@

t_WaitTime.start(20.0);

alt {

//@siclog "Step 34Aa1" siclog@

//Check: Does the UE send NR RRCSetupRequest with EstablishmentCause set to 'mo-SMS'?

[] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest( mo_SMS,?)))

{

t_WaitTime.stop;

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 34Aa1");

f_NR_ULGrantConfiguration_Start(nr_Cell1); // @sic R5s220817 sic@

}

}

[] t_WaitTime.timeout

{

//@siclog "Step 34Ab2" siclog@

//AT command to make the UE attempt to send another SMS over IP

f_UT_IMS_TriggerSMS(UT);

f_NR_RRC_SetupRequest_Def(nr_Cell1, mo_SMS);

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 34Ab3");

}

}

//@siclog "Steps 36-43" siclog@

//Steps 3-16 from generic procedure IMS MO SMS in 5GS as described in TS 38.508-1 [4] Table 4.9.19.2.2-1 take place.

// Send RRC Connection Setup on SRB0 (CCCH): done automatically by SS in Msg4 of the RA procedure

v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1,

cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def),

tsc_SHT_IntegrityProtected

);

if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), f_NG_S_TMSI2MobileIdentity(omit, f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1)), '0001'B)) {  // @sic R5s200654 subject to prose CR approval sic@

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed");

}

f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); //@sic R5s211658 sic@

f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg);

f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]);

}