3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220824

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2632 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC RRC test cases 8.1.4.1.5 and 8.1.4.1.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-06-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At step 4 (and step 10 of 8.1.4.1.5) the UE may send the Reestablishment Req before the expiry of T304 timer as soon as the Cell 2 is turned off at T2 time instance. This could fail the test case. The reestablishment req sent by the UE at this step should be because of T304 timer expiry, and not because of radio link failure on cell 2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Make cell 2 “suitableIntraFreq” level (-94dbM) instead of “OFF”. This way, the UE will always find NR Cell 2 as suitable cell, but will ignore it because of the T304 expiry. Also, the UE will always prefer NR Cell 1 because that is still at serving cell power. This ensures that the reestablishment request sent by the UE at this step is because of T304 timer expiry, and not because of radio link failure on cell 2. Note : An associated prose CR on 38.523-1 will be raised at RAN5#96 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail these test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.1.5.NR5GC, 8.1.4.1.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1 Change 1	3

2.2 Change 2	5

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk23 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## 2.1 Change 1

| Function name | fl_TC_8_1_4_1_5_TestBody() |
| --- | --- |
| Reason for change | At steps 4 and 10, the UE may send the Reestablishment Req before the expiry of T304 timer as soon as the Cell 2 is turned off at T2 time instance. This could fail the test case. The reestablishment req sent by the UE at this step should be because of T304 timer expiry, and not because of radio link failure on cell 2. |
| Summary of change | Make cell 2 “suitableIntraFreq” level (-94dbM) instead of “OFF”. This way, the UE will always find NR Cell 2 as suitable cell, but will ignore it because of the T304 expiry. Also, the UE will always prefer NR Cell 1 because that is still at serving cell power. This ensures that the reestablishment request sent by the UE at this step is because of T304 timer expiry, and not because of radio link failure on cell 2. Note : An associated prose CR on 38.523-1 will be raised at RAN5#96 |
| TTCN module | RRC_IntraNR_Handover_NR5GC.ttcn |
| MCC160 Comment | Accepted conditional to prose CR agreement on 38.523-1 at RAN5#96-e. |

Before Change

function fl_TC_8_1_4_1_5_TestBody () runs on NR5GC_PTC { var template (value) DL_DCCH_Message v_RRCReconfigurationHO; //Power level as per Table 8.1.4.1.5.3.2-1 var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1 := { cs_NR_CellPower(nr_Cell2, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2)}; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2 := { cs_NR_CellPower(nr_Cell2, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE)}; // Reconfigure nr_Cell2 to no respond to any RA Preamble // Table 8.1.4.1.5.3.2-4: The UE attempts to perform the handover using MAC Random Access Preamble on NR Cell 2. SS does not respond. f_NR_SS_RachProcedureConfig(nr_Cell2, cs_NR_RachProcedureConfig_NoResponse); //@siclog "Step 1" siclog@ // The SS changes the power level setting according to the row "T1". f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 2" siclog@ // The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform handover to NR Cell 2. // Note: failure case UE doesn't go to nr_Cell2 => the standard function need not be used v_RRCReconfigurationHO := f_NR5GC_RRCReconfiguration_IntraNR_HO(nr_Cell2); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, v_RRCReconfigurationHO)); //@sic R5s220176 sic@ …………. }

After Change

function fl_TC_8_1_4_1_5_TestBody () runs on NR5GC_PTC { var template (value) DL_DCCH_Message v_RRCReconfigurationHO; //Power level as per Table 8.1.4.1.5.3.2-1 var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1 := { cs_NR_CellPower(nr_Cell2, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2)}; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2 := { cs_NR_CellPower(nr_Cell2, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1)}; // Reconfigure nr_Cell2 to no respond to any RA Preamble // Table 8.1.4.1.5.3.2-4: The UE attempts to perform the handover using MAC Random Access Preamble on NR Cell 2. SS does not respond. f_NR_SS_RachProcedureConfig(nr_Cell2, cs_NR_RachProcedureConfig_NoResponse); //@siclog "Step 1" siclog@ // The SS changes the power level setting according to the row "T1". f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 2" siclog@ // The SS transmits an RRCReconfiguration message on NR Cell 1 to order the UE to perform handover to NR Cell 2. // Note: failure case UE doesn't go to nr_Cell2 => the standard function need not be used v_RRCReconfigurationHO := f_NR5GC_RRCReconfiguration_IntraNR_HO(nr_Cell2); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, cs_TimingInfo_Now, v_RRCReconfigurationHO)); //@sic R5s220176 sic@ …………. }

## 2.2 Change 2

| Function name | fl_TC_8_1_4_1_6_TestBody() |
| --- | --- |
| Reason for change | At step 4 the UE may send the Reestablishment Req before the expiry of T304 timer as soon as the Cell 2 is turned off at T2 time instance. This could fail the test case. The reestablishment req sent by the UE at this step should be because of T304 timer expiry, and not because of radio link failure on cell 2. |
| Summary of change | Make cell 2 “suitableIntraFreq” level (-94dbM) instead of “OFF”. This way, the UE will always find NR Cell 2 as suitable cell, but will ignore it because of the T304 expiry. Also, the UE will always prefer NR Cell 1 because that is still at serving cell power. This ensures that the reestablishment request sent by the UE at this step is because of T304 timer expiry, and not because of radio link failure on cell 2. Note : An associated prose CR on 38.523-1 will be raised at RAN5#96 |
| TTCN module | RRC_IntraNR_Handover_NR5GC.ttcn |
| MCC160 Comment | Accepted conditional to prose CR agreement on 38.523-1 at RAN5#96-e. |

Before Change

function fl_TC_8_1_4_1_6_TestBody () runs on NR5GC_PTC { var RNTI_Value v_RNTI_Value := f_NR_CellInfo_GetRNTI(nr_Cell1); var PhysCellId v_PhysCellId := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1); var ShortMAC_I v_ShortMAC_I; var template (value) DL_DCCH_Message v_RRCReconfigurationHO; var NextHopChainingCount v_NextHopChainingCount; var float v_T304Min := f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, 1.0); //T304 is ms1000 according to 38.508-1 Table 4.6.3-19. var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1 := { cs_NR_CellPower(nr_Cell2, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) }; //As per Table 8.1.4.1.6.3.2-1 var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2 := { cs_NR_CellPower(nr_Cell2, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE) }; //As per Table 8.1.4.1.6.3.2-1 var IntegerList_Type v_DrbIdList := f_NR_GetActiveDRBs(nr_Cell1); //@sic R5s201221 sic@ var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); var NG_NAS_MSG_Indication_Type v_NasInd; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; timer t_T304Min; ………….. }

After Change

function fl_TC_8_1_4_1_6_TestBody () runs on NR5GC_PTC { var RNTI_Value v_RNTI_Value := f_NR_CellInfo_GetRNTI(nr_Cell1); var PhysCellId v_PhysCellId := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1); var ShortMAC_I v_ShortMAC_I; var template (value) DL_DCCH_Message v_RRCReconfigurationHO; var NextHopChainingCount v_NextHopChainingCount; var float v_T304Min := f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, 1.0); //T304 is ms1000 according to 38.508-1 Table 4.6.3-19. var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1 := { cs_NR_CellPower(nr_Cell2, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) }; //As per Table 8.1.4.1.6.3.2-1 var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2 := { cs_NR_CellPower(nr_Cell2, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1) }; //As per Table 8.1.4.1.6.3.2-1 var IntegerList_Type v_DrbIdList := f_NR_GetActiveDRBs(nr_Cell1); //@sic R5s201221 sic@ var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); var NG_NAS_MSG_Indication_Type v_NasInd; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; timer t_T304Min; …………… }