3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220826

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2634 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR MAC test case 7.1.1.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-06-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For FR2 operation, there is conflict between the SR configuration and TDD UL_DL pattern on cell 2, the SR slot is configured as a DL slot. Due to this SS cannot detect any SR during test case postamble. The current TTCN has TDD-DL-UL-Configure as DDFUU, which is not in line with prose Table 7.1.1.1.1.3.3-9: TDD-UL-DL-ConfigCommon.as DDDFU for FR2. The current TTCN also has SchedulingRequestResourceConfig periodicityAndOffset sf80:=5, which is not inli ne with prose (there is no testcase specific configuration, so the configuration sf80:=9 as per 38.508-1 should apply), for both FR1 and FR2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update TDD-DL-UL-Configure and SchedulingRequestResourceConfig for cell 2 to be the same as for 7.1.1.1.1a |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail these test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.1.1.NR5GC, 7.1.1.1.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk23 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

# Change 1

| Function name | f_TC_7_1_1_1_1_NR5GC |
| --- | --- |
| Reason for change | For FR2 operation, there is conflict between the SR configuration and TDD UL_DL pattern on cell 2, the SR slot is configured as a DL slot. Due to this SS cannot detect any SR during test case postamble. The current TTCN has TDD-DL-UL-Configure as DDFUU, which is not in line with prose Table 7.1.1.1.1.3.3-9: TDD-UL-DL-ConfigCommon.as DDDFU for FR2. The current TTCN also has SchedulingRequestResourceConfig periodicityAndOffset sf80:=5, which is not inli ne with prose (there is no testcase specific configuration, so the configuration sf80:=9 as per 38.508-1 should apply), for both FR1 and FR2. |
| Summary of change | Update TDD-DL-UL-Configure and SchedulingRequestResourceConfig for cell 2 to be the same as for 7.1.1.1.1a |
| TTCN module | MAC_NR5GC |
| MCC160 Comment | Accepted. The FR2 else branch is missing in shown changes, but added same as 7.1.1.1.1a else //FR2 { v_TDD_UL_DL_ConfigCommon :=cs_TDD_UL_DL_ConfigCommonPattern2(v_SubcarrierSpacing,cs_TDD_UL_DL_Pattern1_FR2,cs_TDD_UL_DL_Pattern2_FR2); } |

Before Change

function f_TC_7_1_1_1_1_NR5GC() runs on NR5GC_PTC { //Correct selection of RACH parameters / Random access preamble and PRACH resource explicitly signalled to the UE by PDCCH Order / contention free random access procedure var NR_PhysicalParameters_Type v_NR_PhysicalParameters ; var template (value) NR_RachProcedureConfig_Type v_NR_RachProcedure ; var template (value) DL_DCCH_Message v_RRCReconfiguration; var template (value) ReconfigurationWithSync.rach_ConfigDedicated v_RACH_ConfigDedicated ; var boolean v_RBConfig_KeyChange := false; //@sic R5-190795 sic@ var NextHopChainingCount v_NCC:= tsc_NR_38508_NextHopChainingCount; var template (value) NR_DRBInfoList_Type v_DRBInfoList ; //@sic R5-198886 sic@ var NR_DuplexMode_Type v_NR_DuplexMode ; var template (value) TDD_UL_DL_ConfigCommon v_TDD_UL_DL_ConfigCommon; var NR_Prach_RootSequenceIndex_Type v_Prach_RootSequenceIndex; var boolean v_IsFR1 := false; var SubcarrierSpacing v_SubcarrierSpacing; var NR_UplinkBWP_Type v_NR_UplinkBWP; //@sic R5s200059 sic@ //Init Cell parameters f_NR5GC_Init(NR_1); f_NR_CellInfo_SetMAC_CellGroupConfigL2(nr_Cell2); //@sic R5s210315 sic@ //@sic R5-198886 sic@ v_NR_DuplexMode := f_NR_CellInfo_GetDuplexMode( nr_Cell2 ); v_SubcarrierSpacing := f_NR_CellInfo_GetSCS( nr_Cell2 ); v_IsFR1 := f_NR_CellInfo_GetIsFR1( nr_Cell2 ); if (v_IsFR1) // FR1 { v_Prach_RootSequenceIndex := {l839 := 0}; f_NR_CellInfo_SetSIB1_Prach_RootSequenceIndex(nr_Cell2, v_Prach_RootSequenceIndex); //@sic R5s200059 sic@ f_NR_CellInfo_SetPRACH_ConfigurationIndex(nr_Cell2,14); f_NR_CellInfo_SetZeroCorrelationZoneConfig(nr_Cell2,12); } if (ischosen(v_NR_DuplexMode.TDD)) { if(v_IsFR1) { v_TDD_UL_DL_ConfigCommon :=cs_TDD_UL_DL_ConfigCommonPattern2(v_SubcarrierSpacing,cs_TDD_UL_DL_Pattern1_FR1,cs_TDD_UL_DL_Pattern2_FR1); } else { v_TDD_UL_DL_ConfigCommon :=cs_38508_TDD_UL_DL_ConfigCommon1(v_SubcarrierSpacing,cs_TDD_UL_DL_Pattern1_FR2); } f_NR_CellInfo_SetTDD_UL_DL_ConfigCommon(nr_Cell2,v_TDD_UL_DL_ConfigCommon); //@sic R5s200059 sic@ v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell2, tsc_NR_BWP_Id); v_NR_UplinkBWP.Dedicated.R15.pucch_Config.setup.schedulingRequestResourceToAddModList[0] := valueof(cs_SchedulingRequestResourceConfig_Mac(v_SubcarrierSpacing)); f_NR_CellInfo_SetUplinkBWP(nr_Cell2, v_NR_UplinkBWP, tsc_NR_BWP_Id); }; //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Create NR Cell 2 //@sic R5s190795 sic@ //v_RACH_ConfigDedicated := cs_NR_RACH_ConfigDedicated_Uplink(cs_38508_RACH_ConfigDedicated(f_NR_CellInfo_GetRACH_ConfigGeneric(nr_Cell2,tsc_NR_BWP_Id), tsc_PreambleIndex2, f_NR_CellInfo_GetSSB_Index(nr_Cell2))); v_RACH_ConfigDedicated := cs_NR_RACH_ConfigDedicated_Uplink(f_NR_GetRACHConfigDedicated_Def( nr_Cell2, tsc_PreambleIndex2)); f_NR_CellInfo_SetRACH_ConfigDedicated (nr_Cell2,v_RACH_ConfigDedicated.uplink); v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell2); v_NR_RachProcedure := cs_NR_RachProcedureConfig_71111(v_NR_PhysicalParameters); f_NR_CellConfig_Def(nr_Cell2); // Get NR capabilities sent during EUTRA preamble f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A); //@sic R5-198888 sic@ ……. }

After Change

function f_TC_7_1_1_1_1_NR5GC() runs on NR5GC_PTC { //Correct selection of RACH parameters / Random access preamble and PRACH resource explicitly signalled to the UE by PDCCH Order / contention free random access procedure var NR_PhysicalParameters_Type v_NR_PhysicalParameters ; var template (value) NR_RachProcedureConfig_Type v_NR_RachProcedure ; var template (value) DL_DCCH_Message v_RRCReconfiguration; var template (value) ReconfigurationWithSync.rach_ConfigDedicated v_RACH_ConfigDedicated ; var boolean v_RBConfig_KeyChange := false; //@sic R5-190795 sic@ var NextHopChainingCount v_NCC:= tsc_NR_38508_NextHopChainingCount; var template (value) NR_DRBInfoList_Type v_DRBInfoList ; //@sic R5-198886 sic@ var NR_DuplexMode_Type v_NR_DuplexMode ; var template (value) TDD_UL_DL_ConfigCommon v_TDD_UL_DL_ConfigCommon; var NR_Prach_RootSequenceIndex_Type v_Prach_RootSequenceIndex; var boolean v_IsFR1 := false; var SubcarrierSpacing v_SubcarrierSpacing; var NR_UplinkBWP_Type v_NR_UplinkBWP; //@sic R5s200059 sic@ //Init Cell parameters f_NR5GC_Init(NR_1); f_NR_CellInfo_SetMAC_CellGroupConfigL2(nr_Cell2); //@sic R5s210315 sic@ //@sic R5-198886 sic@ v_NR_DuplexMode := f_NR_CellInfo_GetDuplexMode( nr_Cell2 ); v_SubcarrierSpacing := f_NR_CellInfo_GetSCS( nr_Cell2 ); v_IsFR1 := f_NR_CellInfo_GetIsFR1( nr_Cell2 ); if (v_IsFR1) // FR1 { v_Prach_RootSequenceIndex := {l839 := 0}; f_NR_CellInfo_SetSIB1_Prach_RootSequenceIndex(nr_Cell2, v_Prach_RootSequenceIndex); //@sic R5s200059 sic@ f_NR_CellInfo_SetPRACH_ConfigurationIndex(nr_Cell2,14); f_NR_CellInfo_SetZeroCorrelationZoneConfig(nr_Cell2,12); } if (ischosen(v_NR_DuplexMode.TDD)) { if(v_IsFR1) { if(v_SubcarrierSpacing == kHz30){ v_TDD_UL_DL_ConfigCommon :=cs_TDD_UL_DL_ConfigCommonPattern2(v_SubcarrierSpacing,cs_TDD_UL_DL_Pattern1_FR1,cs_TDD_UL_DL_Pattern2_FR1); }else if(v_SubcarrierSpacing == kHz15){ v_TDD_UL_DL_ConfigCommon :=cs_TDD_UL_DL_ConfigCommonPattern2(v_SubcarrierSpacing,cs_TDD_UL_DL_Pattern1_FR1_SCS15, cs_TDD_UL_DL_Pattern2_FR1_SCS15); }else{ FatalError(__FILE__, __LINE__, "SCS value not handled in TTCN"); } f_NR_CellInfo_SetTDD_UL_DL_ConfigCommon(nr_Cell2,v_TDD_UL_DL_ConfigCommon); } //@sic R5s200059 sic@ v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell2, tsc_NR_BWP_Id); v_NR_UplinkBWP.Dedicated.R15.pucch_Config.setup.schedulingRequestResourceToAddModList[0] := valueof(cs_SchedulingRequestResourceConfig_Mac_71111a(v_SubcarrierSpacing)); f_NR_CellInfo_SetUplinkBWP(nr_Cell2, v_NR_UplinkBWP, tsc_NR_BWP_Id); }; //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Create NR Cell 2 //@sic R5s190795 sic@ //v_RACH_ConfigDedicated := cs_NR_RACH_ConfigDedicated_Uplink(cs_38508_RACH_ConfigDedicated(f_NR_CellInfo_GetRACH_ConfigGeneric(nr_Cell2,tsc_NR_BWP_Id), tsc_PreambleIndex2, f_NR_CellInfo_GetSSB_Index(nr_Cell2))); v_RACH_ConfigDedicated := cs_NR_RACH_ConfigDedicated_Uplink(f_NR_GetRACHConfigDedicated_Def( nr_Cell2, tsc_PreambleIndex2)); f_NR_CellInfo_SetRACH_ConfigDedicated (nr_Cell2,v_RACH_ConfigDedicated.uplink); v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell2); v_NR_RachProcedure := cs_NR_RachProcedureConfig_71111(v_NR_PhysicalParameters); f_NR_CellConfig_Def(nr_Cell2); // Get NR capabilities sent during EUTRA preamble f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A); //@sic R5-198888 sic@ …… }