3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220841

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0845 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR XCAP test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-06-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | XCAP test cases, depending on UE, as said in test case preamble: - The steps 0a and 0b of Annex A.21 have been executed - During these procedures the UE may request a DNS server address via NAS signalling and as parallel behaviour the UE may resolve the IP address of the XCAP server via DNS. Current implementation does not contemplate the inclusion of DNS server addresses if requested by UE in PDU Session Establishment Request, this will not allow the UE to resolve the IP address of the XCAP server and not able to start the HTTP transaction. This can be solved implementing the inclusion of DNS server addresses in PDU Session Complete if requested by the UE. Note: the solution proposed, in order to avoid not required DNS queries which may populate the logs, aims only to add DNS address information in PCO for internet PDU. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Included DNS server address in PDU Session Complete if requested by UE in PDU Session Establishment Request. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | NR XCAP test cases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	4

2.1	Correction to function f_Get_NG_PDUSessionEstablishmentAccept	4

2.2	Correction to function f_NG_GetDefaultExtdProtocolConfigOptions	4

2.3	Correction to function f_NG_NAS_GetExtdProtocolConfigOptionList	6

2.4	Correction to function fl_AddrServerArray_GetAddrServer	7

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to function f_Get_NG_PDUSessionEstablishmentAccept

| Function name | f_Get_NG_PDUSessionEstablishmentAccept |
| --- | --- |
| Reason for change | In order to avoid not required DNS queries which may populate the logs, solution aims only to add DNS address information in PCO for internet PDU. |
| Summary of change | Added input PDUType input parameter to function to limit the DNS contents to be added to internet PDU. |
| TTCN module | Common\NG_NAS\NG_NASTemplateFunctions.ttcn |
| MCC160 Comment | This requires a prose CR. Decision postponed until outcome of prose CR |

Before Change:

<<SKIPPED CODE>> if (isvalue(p_ExtdPCO)) { v_ExtdPCO := p_ExtdPCO; } else { if (isvalue(p_ExtraPDN_Address)) { // @sic R5-213400, R5-217830 sic@ v_PDN_Addresses[1] := valueof(p_ExtraPDN_Address); } v_ExtdPCO := f_NG_GetDefaultExtdProtocolConfigOptions (p_GSM_MobilityInfo.PCO, v_PDN_Addresses); // @sic R5s200643, R5-213400 sic@ } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> if (isvalue(p_ExtdPCO)) { v_ExtdPCO := p_ExtdPCO; } else { if (isvalue(p_ExtraPDN_Address)) { // @sic R5-213400, R5-217830 sic@ v_PDN_Addresses[1] := valueof(p_ExtraPDN_Address); } v_ExtdPCO := f_NG_GetDefaultExtdProtocolConfigOptions (p_GSM_MobilityInfo.PCO, v_PDN_Addresses, p_GSM_MobilityInfo.PDUType); // @sic R5s200643, R5-213400 sic@ } <<SKIPPED CODE>>

## Correction to function f_NG_GetDefaultExtdProtocolConfigOptions

| Function name | f_NG_GetDefaultExtdProtocolConfigOptions |
| --- | --- |
| Reason for change | In order to avoid not required DNS queries which may populate the logs, solution aims only to add DNS address information in PCO for internet PDU. |
| Summary of change | Added input PDUType input parameter to function to limit the DNS contents to be added to internet PDU. |
| TTCN module | Common\NG_NAS\NG_NASTemplateFunctions.ttcn |
| MCC160 Comment | This requires a prose CR. Decision postponed until outcome of prose CR |

Before Change:

<<SKIPPED CODE>> function f_NG_GetDefaultExtdProtocolConfigOptions(template (omit) ExtdProtocolConfigOptions p_Pco, ListOfPDN_AddressInfo_Type p_PDN_AddressInfo) return template (omit) ExtdProtocolConfigOptions { var template (omit) NAS_ExtdProtocolConfigOptions_Type v_ProtocolContainerList := omit; var ExtdProtocolConfigOptions v_ProtocolConfigOptionsRX; if (isvalue(p_Pco)) { v_ProtocolConfigOptionsRX := valueof(p_Pco); if (ispresent(p_Pco.pco)) { v_ProtocolContainerList := f_NG_NAS_GetExtdProtocolConfigOptionList(v_ProtocolConfigOptionsRX.pco, p_PDN_AddressInfo); // @sic R5s200643 sic@ } } return f_NG_NAS_ExtdProtocolConfigOptionsTX(v_ProtocolContainerList); } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function f_NG_GetDefaultExtdProtocolConfigOptions(template (omit) ExtdProtocolConfigOptions p_Pco, ListOfPDN_AddressInfo_Type p_PDN_AddressInfo, PDU_PDN_DNN_Type p_PDUType) return template (omit) ExtdProtocolConfigOptions { var template (omit) NAS_ExtdProtocolConfigOptions_Type v_ProtocolContainerList := omit; var ExtdProtocolConfigOptions v_ProtocolConfigOptionsRX; if (isvalue(p_Pco)) { v_ProtocolConfigOptionsRX := valueof(p_Pco); if (ispresent(p_Pco.pco)) { v_ProtocolContainerList := f_NG_NAS_GetExtdProtocolConfigOptionList(v_ProtocolConfigOptionsRX.pco, p_PDN_AddressInfo, p_PDUType); // @sic R5s200643 sic@ } } return f_NG_NAS_ExtdProtocolConfigOptionsTX(v_ProtocolContainerList); } <<SKIPPED CODE>>

## Correction to function f_NG_NAS_GetExtdProtocolConfigOptionList

| Function name | f_NG_NAS_GetExtdProtocolConfigOptionList |
| --- | --- |
| Reason for change | Current implementation does not contemplate the inclusion of DNS server addresses if requested by UE in PDU Session Establishment Request, this will not allow the UE to resolve the IP address of the XCAP server and not able to start the HTTP transaction. |
| Summary of change | Included DNS address related contents in extended PCO to be sent in PDU Session Complete. |
| TTCN module | Common\NG_NAS\NG_NASTemplateFunctions.ttcn |
| MCC160 Comment | This requires a prose CR. Decision postponed until outcome of prose CR |

Before Change:

<<SKIPPED CODE>> function f_NG_NAS_GetExtdProtocolConfigOptionList(NAS_ExtdProtocolConfigOptions_Type p_ConfigOptionsRX, ListOfPDN_AddressInfo_Type p_PDN_AddressInfo) return template (omit) NAS_ExtdProtocolConfigOptions_Type { var template (omit) NAS_ExtdProtocolConfigOptions_Type v_ProtocolContainerList := omit; var integer v_PcoCnt := 0; var O2_Type v_ProtocolId; var octetstring v_Contents; var integer i; var integer j; for (j := 0; j < lengthof(p_PDN_AddressInfo); j := j + 1) { // @sic R5-213400 sic@ for (i := 0; i < lengthof(p_ConfigOptionsRX); i := i + 1) { v_ProtocolId := p_ConfigOptionsRX[i].protocolID; v_Contents := ''O; select (v_ProtocolId) { // See 24.008 Table 10.5.154 case ('0001'O) { // P-CSCF IPv6 address v_Contents := f_Convert_IPv6Addr2OctString(p_PDN_AddressInfo[j].PCSCF_IPAddressIPv6); // @sic R5s200643 sic@ } case ('000C'O) { // P-CSCF IPv4 address v_Contents := f_Convert_IPv4Addr2OctString(p_PDN_AddressInfo[j].PCSCF_IPAddressIPv4); // @sic R5s200643 sic@ } } if (lengthof(v_Contents) > 0) { v_ProtocolContainerList[v_PcoCnt] := cs_ProtocolContainer(v_ProtocolId, v_Contents); v_PcoCnt := v_PcoCnt + 1; } } } return v_ProtocolContainerList; } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function f_NG_NAS_GetExtdProtocolConfigOptionList(NAS_ExtdProtocolConfigOptions_Type p_ConfigOptionsRX, ListOfPDN_AddressInfo_Type p_PDN_AddressInfo, PDU_PDN_DNN_Type p_PDUType) return template (omit) NAS_ExtdProtocolConfigOptions_Type { var template (omit) NAS_ExtdProtocolConfigOptions_Type v_ProtocolContainerList := omit; var integer v_PcoCnt := 0; var O2_Type v_ProtocolId; var octetstring v_Contents; var boolean v_DNS_OverInternet := (p_PDUType == Internet_DNN) and f_GetTestcaseAttrib_XCAP(testcasename()); var integer i; var integer j; for (j := 0; j < lengthof(p_PDN_AddressInfo); j := j + 1) { // @sic R5-213400 sic@ for (i := 0; i < lengthof(p_ConfigOptionsRX); i := i + 1) { v_ProtocolId := p_ConfigOptionsRX[i].protocolID; v_Contents := ''O; select (v_ProtocolId) { // See 24.008 Table 10.5.154 case ('0001'O) { // P-CSCF IPv6 address v_Contents := f_Convert_IPv6Addr2OctString(p_PDN_AddressInfo[j].PCSCF_IPAddressIPv6); // @sic R5s200643 sic@ } case ('0003'O) { // DNS Server IPv6 Address if (v_DNS_OverInternet){ v_Contents := f_Convert_IPv6Addr2OctString(p_PDN_AddressInfo[j].DNS_ServerAddressIPv6); } } case ('000C'O) { // P-CSCF IPv4 address v_Contents := f_Convert_IPv4Addr2OctString(p_PDN_AddressInfo[j].PCSCF_IPAddressIPv4); // @sic R5s200643 sic@ } case ('000D'O) { // DNS Server IPv4 Address if (v_DNS_OverInternet){ v_Contents := f_Convert_IPv4Addr2OctString(p_PDN_AddressInfo[j].DNS_ServerAddressIPv4); } } } if (lengthof(v_Contents) > 0) { v_ProtocolContainerList[v_PcoCnt] := cs_ProtocolContainer(v_ProtocolId, v_Contents); v_PcoCnt := v_PcoCnt + 1; } } } return v_ProtocolContainerList; } <<SKIPPED CODE>>

## Correction to function fl_AddrServerArray_GetAddrServer

| Function name | fl_AddrServerArray_GetAddrServer |
| --- | --- |
| Reason for change | As in NR internet is related to PDN_3 that may cause that array p_AddrServerArray is of length 2 but pos 1(PDN_2) does not contain any AddrInfo, that will cause a runtime error when trying to get the value. |
| Summary of change | Added additional if statement to check that p_ServerAddrInfo contains any value. |
| TTCN module | Common\IP_PTC\ IP_PTC_Main.ttcn |
| MCC160 Comment | Accepted |

Before Change:

<<SKIPPED CODE>> function fl_AddrServerArray_GetAddrServer(IP_AddrServerArray_Type p_AddrServerArray, IP_AddrInfo_Type p_ServerAddrInfo) return template (omit) AddrServerInfo_Type { /* @sic R5-145754: new generic function sic@ */ var integer i; for (i:=0; i < lengthof(p_AddrServerArray); i := i + 1) { if (f_IMS_IpAddrMatch(p_ServerAddrInfo, p_AddrServerArray[i].ServerAddr)) { return p_AddrServerArray[i]; } } FatalError(__FILE__, __LINE__, "No address server found matching given server address"); return omit; } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function fl_AddrServerArray_GetAddrServer(IP_AddrServerArray_Type p_AddrServerArray, IP_AddrInfo_Type p_ServerAddrInfo) return template (omit) AddrServerInfo_Type { /* @sic R5-145754: new generic function sic@ */ var integer i; for (i:=0; i < lengthof(p_AddrServerArray); i := i + 1) { if (isvalue(p_AddrServerArray[i].ServerAddr)){ if (f_IMS_IpAddrMatch(p_ServerAddrInfo, p_AddrServerArray[i].ServerAddr)) { return p_AddrServerArray[i]; } } } FatalError(__FILE__, __LINE__, "No address server found matching given server address"); return omit; }<<SKIPPED CODE>>