3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220853

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2653 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR test case 8.1.4.2.1.2 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2022-07-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add NR test case 8.1.4.2.1.2 to the IWD_22wk23 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 8.1.4.2.1.2 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.2.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	4

2.1	Function f_TC_8_1_4_2_1_2_NR5GC()	4

2.2	Function fl_TC_8_1_4_2_1_2_TestBody()	6

2.3	Module IRAT_Handover_NR5GC_NR	15

2.4	Function f_NR_GetSCGConfigDef()	15

2.5	Function f_NR_CellConfig_Def ()	18

2.6	Function f_TC_8_1_4_2_1_2_NR5GC_EUTRA ()	19

2.7	Function fl_TC_8_1_4_2_1_2_NR5GC_EUTRA_TestBody ()	20

2.8	New Function : f_NR_Config_Set	28

3	Branches executed	28

4	Execution Log Files	29

4.1	MediaTek MT6983 UE	29

5	References	29

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 8.1.4.2.1.2  which is part of the NR5GC_IRAT test suite in iwd-TTCN3-B2020-09_D22wk23.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	8.1.4.2.1.2

ATS Version:	iwd-TTCN3-B2020-09_D22wk23

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	MediaTek MT6983

Verification Status:	PASS

# Corrections required

## Function f_TC_8_1_4_2_1_2_NR5GC()

| Function name | f_TC_8_1_4_2_1_2_NR5GC() |
| --- | --- |
| Reason for change | - ServingCellIndex has to be set as ‘1’ for NR Cell 3 ( PScell ) in align to the other ENDC Cases ; Refer function implementation f_NR_ServCellIndex - Before releasing the Nr cells , need to make sure that EUTRA side Deregistration is done ; so coordination signal has to be introduced |
| Summary of change | - ServingCellIndex is set as ‘1’ for NR Cell 3 ( PScell ) - Wait for Postamble coordination signal from Eutra PTC is introduced |
| TTCN module | NR5GC_IRAT\8_1_4\IRAT_Handover_NR5GC_NR.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_8_1_4_2_1_2_NR5GC() runs on NR5GC_PTC { // Inter-RAT Handover / From NR to EN-DC / Success var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedure; //Use a specific init to initialise NR cells with ENDC band cobination and ENDC frequencies f_NR5GC_InitWithENDC(NR_6); fl_NR5GC_IRAT_InitSysinfo (NR_6); // Change cellGroupId of NR_Cell3 f_NR_CellInfo_Set_CellGroupId(nr_Cell3, tsc_NR_CellGroupId_SCG); f_NR_SetSysinfoCombinationToMIBOnly(nr_Cell3); //Create and configure cell // FFS Prose doesn't specify any power levels f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell3); //Configure nr_Cell3 as a normal cell including PCCH/SRBs that aren't used //Reconfigure RACH of nr_Cell3 for ENDC RACH v_NR_RachProcedure := cs_NR_RachProcedureConfig_CBRA_CrntiBased(f_NR_CellInfo_GetPhysicalParameters(nr_Cell3), -,-, tsc_NR_ENDC_Msg3Grant_Def); f_NR_SS_RachProcedureConfig(nr_Cell3, v_NR_RachProcedure); f_NR5GC_Preamble (nr_Cell1, STATE_CONNECTED_3A, PING_Or_TEST_LOOPModeB_ON); f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Preamble")); f_NR_TestBody_Set(true); fl_TC_8_1_4_2_1_2_TestBody(); f_NR_TestBody_Set(false); // UE now on EUTRA f_NR_ReleaseAllCells(); }

After Change:

function f_TC_8_1_4_2_1_2_NR5GC() runs on NR5GC_PTC { // Inter-RAT Handover / From NR to EN-DC / Success var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedure; //Use a specific init to initialise NR cells with ENDC band cobination and ENDC frequencies f_NR5GC_InitWithENDC(NR_6); fl_NR5GC_IRAT_InitSysinfo (NR_6); // Change cellGroupId of NR_Cell3 f_NR_CellInfo_Set_CellGroupId(nr_Cell3, tsc_NR_CellGroupId_SCG); f_NR_CellInfo_Set_ServingCellIndex(nr_Cell3,1); //WA#WI=942180 f_NR_SetSysinfoCombinationToMIBOnly(nr_Cell3); //Create and configure cell // FFS Prose doesn't specify any power levels f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell3); //Configure nr_Cell3 as a normal cell including PCCH/SRBs that aren't used //Reconfigure RACH of nr_Cell3 for ENDC RACH v_NR_RachProcedure := cs_NR_RachProcedureConfig_CBRA_CrntiBased(f_NR_CellInfo_GetPhysicalParameters(nr_Cell3), -,-, tsc_NR_ENDC_Msg3Grant_Def); f_NR_SS_RachProcedureConfig(nr_Cell3, v_NR_RachProcedure); f_NR5GC_Preamble (nr_Cell1, STATE_CONNECTED_3A, PING_Or_TEST_LOOPModeB_ON); f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Preamble")); f_NR_TestBody_Set(true); fl_TC_8_1_4_2_1_2_TestBody(); f_NR_TestBody_Set(false); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //WA#WI=942180 // UE now on EUTRA f_NR_ReleaseAllCells(); }

## Function fl_TC_8_1_4_2_1_2_TestBody()

| Function name | fl_TC_8_1_4_2_1_2_TestBody() |
| --- | --- |
| Reason for change | - Eps bearer Identity has to be filled in bit 5-8, refer 9.3.2 of 3GPP TS 24.301 [15]. Bits 1 to 4 are spare and shall be coded as zero - TFT has to be included as part of EPS dedicated Bearer Context Reference dedicated EPS bearer context #1 as specified in TS 36.508 [10], Table 6.6.2-1 ( 1,2 changes are required while sending Pdu Session Modification command in step 0A) - NR Cell3 has to be powered up before configuring Radio bearer ( SCG DRB ) - Ul grant configuration has to be provided on NR Cell 3 - In function f_NR_SendRRCReconfigurationContentsToEUTRA , parameter has to be set as noRACH_Check so that coordination signal will be in line with EUTRA PTC - f_NR_Config_Set has to be set as EN_DC after the reception of reconfiguration complete which gets used while validating UE loop back on NR Cell 3 in the function f_NR_CheckDataPathCommon in step 7 (new function is implemented , mentioned as part of section 2.8 ) - f_ENDC_GetKeysActivateSecurity has to be moved after reconfiguration complete reception so that , NR PTC can receive security Keys from EUTRA PTC after TAU procedure trigger ( as a result of f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys ) - At step 7 , f_NR5GC_CheckDataPath can not be used for Nr Cell3 as there is no SDAP involved in EN_DC ; it is proposed to use the derived implementation from f_NR_CheckDataPath ( as in ENDC Cases ) - f_NR_GetSCGConfigDef is passed with second parameter as true for the purpose of having DRB3 ( more clarification is given in section 2.4 ) - Coordination signal is introduced for Step 6 ( MCG DRB IP packet tx rx ) to get completed and further SCG DRB IP Packet tx rx to begin at Step 7 |
| Summary of change | - Eps bearer Identity is filled in bit 5-8 - TFT is included as part of EPS dedicated Bearer Context reference - NR Cell 3 is powered up before configuring SCG DRB - Ul grant configuration is provided on NR Cell 3 - f_NR_SendRRCReconfigurationContentsToEUTRA is called with noRACH_Check - NR Config is set to ENDC After receiving Reconfiguration Complete ( Mobility From NR Command procedure ) - f_ENDC_GetKeysActivateSecurity is moved after after reconfiguration complete reception , so that Keys can be received from EUTRA PTC after TAU procedure trigger - At Step 7 , f_NR_CheckDataPath implementation is used for NR Cell 3 ( in line with other ENDC cases ) - f_NR_GetSCGConfigDef is having second parameter passed as “True” - For Step 7 to begin ( SCG DRB IP Tx Rx ) coordination signal is introduced |
| TTCN module | NR5GC_IRAT\8_1_4\IRAT_Handover_NR5GC_NR.ttcn |
| MCC160 Comment | 1. Accepted in principle 2. Accepted subject to prose CR approval. Also corrected QoS to use dedicated QoS according to PDN type. 3. No power level is specified in the prose for either NR cell 3 or the EUTRA cell. This needs to be added to the prose CR. Accepted subject to prose CR approval 4, 5, 6, 8, 10. Accepted 7.Not needed 9. Accepted in principle, but passed in DRB id |

Before Change:

function fl_TC_8_1_4_2_1_2_TestBody() runs on NR5GC_PTC { var template (value) SDAP_Configuration_Type v_SDAP_Configuration; var NR_SRB_COMMON_IND v_ReceivedAsp; var octetstring v_EUTRA_HOCmd; var NasCountInfo_Type v_NASCountInfo; var EUTRA_NR_PduSessionInfo_Type v_CoordMsg; var template (omit) GSM_MobilityInfo_Type v_PDUSessionInfoTemplate := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Internet_DNN); var GSM_MobilityInfo_Type v_PDUSessionInfo; var template (omit) MappedEPSBearerContexts v_MappedEPSBearerContexts := omit; var template (value) MappedEPSContext v_MappedEPSContext; var bitstring v_EPSParam; var octetstring v_ParamOctets; var integer v_NewDRBId; var template (value) SDAP_Config v_SDAP_Config; var template (value) DRB_ToAddMod v_DRBToAddMod; var template (value) QoS_Flow v_QoSFlow; var template (value) NG_NAS_DL_Pdu_Type v_NG_NAS_DL_Pdu; var template(value) SpCellConfig v_SpCellConfig := f_NR_GetSpCellConfigDef(nr_Cell3, -, true); var template(value) CellGroupConfig v_SCGConfig := f_NR_GetSCGConfigDef(nr_Cell3); //Initialise CellGroupConfig as for ENDC var template (value) CellGroupConfig v_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_SS_Config; var boolean v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell3); var template (value) UL_AM_RLC v_ParametersTX := f_NR_UL_AM_RLC(v_IsFR1); var template (value) DL_AM_RLC v_ParametersRX := f_NR_DL_AM_RLC(v_IsFR1); var template(value) RadioBearerConfig v_RadioBearerConfig; var EUTRA_NR_Coordination_MSG v_ReceivedMsg; // get info for only one PDU session as ENDC test body, and prose, only expects 1 PDN if (isvalue(v_PDUSessionInfoTemplate)) { v_PDUSessionInfo := valueof(v_PDUSessionInfoTemplate); v_CoordMsg.PDU_SessionId := oct2int(v_PDUSessionInfo.SessionId); v_NewDRBId := f_NR_GetNextDRBId(v_PDUSessionInfo, false); v_CoordMsg.EPS_Bearer := v_PDUSessionInfo.EPS_Bearer; // This is the default bearer v_CoordMsg.QFI := 0; // Don't need the QFI of the default bearer v_CoordMsg.PdnIndex := v_PDUSessionInfo.PdnIndex; v_CoordMsg.IsIMS := false; } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "No internet PDU session found to modify"); } // Configure extra DRB on Cell 1, using PDU Session Modification // This will become Dedicated Bearer on EUTRA/SCG on Cell 3 v_QoSFlow := f_BuildDefaultQosFlow("5", true, int2oct((v_NewDRBId+4), 1)); v_QoSFlow.qfi := '000101'B; // QFI = 5 v_EPSParam := encvalue(f_Get_508_EPSQoS(DEF_1)); v_ParamOctets := bit2oct(v_EPSParam); // This is only correct if the mappedEPSContext contains EPSQoS v_MappedEPSContext := cs_OneMappedEPSContext(int2oct(v_NewDRBId+4, 1), lengthof(v_ParamOctets), cs_EPSParameter_QoS(v_ParamOctets)); // @sic R5s210302 sic@ v_MappedEPSBearerContexts := cs_OneMappedEPSBearerContextList (v_MappedEPSContext); v_NG_NAS_DL_Pdu := f_Get_NG_PDUSessionModificationCommand(v_PDUSessionInfo.SessionId, -, -, -, -, -, f_BuildDefaultQoSRules("5", '7A'O), cs_QoSFlowDescr({v_QoSFlow}), v_MappedEPSBearerContexts); // SDAP parameter for the new SCG DRB Id // The SDAP needs to include the QFI of the QoS Flow of the SCG bearer. This will never be a default bearer v_SDAP_Config := cs_SDAP_Config(oct2int(valueof(v_PDUSessionInfo.SessionId)), false, {5}); v_SDAP_Configuration:= f_ReturnNR_SDAP_Configuration_FromSDAP_Config(valueof(v_SDAP_Config)); v_DRBToAddMod := cs_38508_NRDRB_ToAddMod(v_SDAP_Config, v_NewDRBId, cs_38508_PDCP_Config()); v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-, {v_DRBToAddMod}); v_SS_Config := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, v_NewDRBId, -, -, -, v_SDAP_Configuration)}; v_CellGroupConfig := f_NR_GetCellgroupConfig_DRB(nr_Cell1, {v_NewDRBId}, false); v_CellGroupConfig.rlc_BearerToAddModList := {cs_38508_RLC_BearerConfig_DRB(v_NewDRBId, cs_38508_RLC_Config_AM_DRB(v_ParametersTX, v_ParametersRX), cs_38508_LogicalChannelConfig_DRB)}; //@siclog "Step 0A - 0B" siclog@ f_NR_RRCReconfigNewDRBWithSSConfig(nr_Cell1, v_RadioBearerConfig, v_SS_Config, v_CellGroupConfig, { cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_NG_NAS_DL_Pdu) }); //@siclog "Step 0C" siclog@ SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionInfo.SessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Modification Complete failed"); } //Prepare nr_Cell3 // Use same DRB Id on both Cell 1 and Cell 3 for the SCG //Modify CellGroupConfig with the correct SpCellConfig content v_SCGConfig.spCellConfig := v_SpCellConfig; v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-, {cs_NR_DRB_ToAddMod_ReestPDCP(v_NewDRBId, cs_38508_PDCP_Config)}, -, cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig(f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()), secondary)); //Configure the DRB in nr_Cell3 f_NR_SS_CommonRadioBearerConfig(nr_Cell3, { f_NR_GetRadioBearerConfig_FullAM(nr_Cell3, v_NewDRBId)}); // send HO command f_NR_SendRRCReconfigurationContentsToEUTRA(v_RadioBearerConfig, v_SCGConfig, omit, omit, false, reconfigurationWithSync_CheckRACH, nr_Cell3); f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_PduSessionInfoList({v_CoordMsg})); f_NR5GC_Send_GutiParametersToEUTRA (nr_Cell1); // wait for EUTRA to trigger that it's ok to get the NAS count value f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); v_NASCountInfo := f_NR5GC_Send_AuthParametersToEUTRA (); // get SgNB Keys for SCG f_ENDC_GetKeysActivateSecurity(nr_Cell3, EUTRA); //@siclog "Step 1" siclog@ v_EUTRA_HOCmd := f_EUTRA_NR_WaitForCoOrd_Data(EUTRA, "HO Cmd"); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_MobilityFromNRCommand (tsc_NR_RRC_TI_Def, v_EUTRA_HOCmd, substr(v_NASCountInfo.DL, 3, 1) ))); // @siclog "Step 2" siclog@ EUTRA.receive(cmr_EUTRA_NR_OctetData( "Complete")) -> value v_ReceivedMsg; f_NR_CheckReceived_RRCReconfigurationComplete(v_ReceivedMsg); // @siclog "Step 7 siclog@ f_NR5GC_CheckDataPath (nr_Cell3, v_PDUSessionInfo.PdnIndex, dedicatedBearer, v_NewDRBId, "Step 7"); }

After Change:

function fl_TC_8_1_4_2_1_2_TestBody() runs on NR5GC_PTC { var template (value) SDAP_Configuration_Type v_SDAP_Configuration; var NR_SRB_COMMON_IND v_ReceivedAsp; var octetstring v_EUTRA_HOCmd; var NasCountInfo_Type v_NASCountInfo; var EUTRA_NR_PduSessionInfo_Type v_CoordMsg; var template (omit) GSM_MobilityInfo_Type v_PDUSessionInfoTemplate := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Internet_DNN); var GSM_MobilityInfo_Type v_PDUSessionInfo; var template (omit) MappedEPSBearerContexts v_MappedEPSBearerContexts := omit; var template (value) MappedEPSContext v_MappedEPSContext; var bitstring v_EPSParam; var bitstring v_EPSParam_TFT; //WA#WI=942180 var octetstring v_ParamOctets; var octetstring v_ParamOctets_TFT; //WA#WI=942180 var integer v_NewDRBId; var template (value) SDAP_Config v_SDAP_Config; var template (value) DRB_ToAddMod v_DRBToAddMod; var template (value) QoS_Flow v_QoSFlow; var template (value) NG_NAS_DL_Pdu_Type v_NG_NAS_DL_Pdu; var template(value) SpCellConfig v_SpCellConfig := f_NR_GetSpCellConfigDef(nr_Cell3, -, true); var template(value) CellGroupConfig v_SCGConfig := f_NR_GetSCGConfigDef(nr_Cell3,true); //Initialise CellGroupConfig as for ENDC //WA#WI=942180 var template (value) CellGroupConfig v_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_SS_Config; var boolean v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell3); var template (value) UL_AM_RLC v_ParametersTX := f_NR_UL_AM_RLC(v_IsFR1); var template (value) DL_AM_RLC v_ParametersRX := f_NR_DL_AM_RLC(v_IsFR1); var template(value) RadioBearerConfig v_RadioBearerConfig; var EUTRA_NR_Coordination_MSG v_ReceivedMsg; var B8_Type EpsBearerId; //WA#WI=942180 var octetstring v_SduTX; //WA#WI=942180 var NR_PDCP_SDUList_Type v_PDCP_Data; //WA#WI=942180 var template (value) NR_DRB_COMMON_REQ v_DRB_DL; //WA#WI=942180 var template (present) NR_DRB_COMMON_IND v_DRB_UL; //WA#WI=942180 // get info for only one PDU session as ENDC test body, and prose, only expects 1 PDN if (isvalue(v_PDUSessionInfoTemplate)) { v_PDUSessionInfo := valueof(v_PDUSessionInfoTemplate); v_CoordMsg.PDU_SessionId := oct2int(v_PDUSessionInfo.SessionId); v_NewDRBId := f_NR_GetNextDRBId(v_PDUSessionInfo, false); v_CoordMsg.EPS_Bearer := v_PDUSessionInfo.EPS_Bearer; // This is the default bearer v_CoordMsg.QFI := 0; // Don't need the QFI of the default bearer v_CoordMsg.PdnIndex := v_PDUSessionInfo.PdnIndex; v_CoordMsg.IsIMS := false; } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "No internet PDU session found to modify"); } EpsBearerId := int2bit((v_NewDRBId+4),8) << 4; //WA#WI=942180 // Configure extra DRB on Cell 1, using PDU Session Modification // This will become Dedicated Bearer on EUTRA/SCG on Cell 3 v_QoSFlow := f_BuildDefaultQosFlow("5", true, bit2oct(EpsBearerId)); //WA#WI=942180 v_QoSFlow.qfi := '000101'B; // QFI = 5 v_EPSParam := encvalue(f_Get_508_EPSQoS(DEF_1)); v_ParamOctets := bit2oct(v_EPSParam); v_EPSParam_TFT := encvalue(f_EUTRA_NB_Get_508_TrafficFlowTemplate(6,tsc_EpsDedicatedBearerId2,tsc_PdnType_IPv4v6,v_PDUSessionInfo.PdnIndex)); //WA#WI=942180 v_ParamOctets_TFT := bit2oct(v_EPSParam_TFT); //WA#WI=942180 // This is only correct if the mappedEPSContext contains EPSQoS //v_MappedEPSContextVoice := cs_OneMappedEPSContextTwoParams(hex2oct(v_EPS_BearerIdVoice & '0'H), lengthof(v_QoSParamOctetsVoice) + lengthof(v_TFTParamOctetsVoice), cs_EPSParameter_QoS(v_QoSParamOctetsVoice), cs_EPSParameter_TFT(v_TFTParamOctetsVoice)); //WA#WI=942180 v_MappedEPSContext := cs_OneMappedEPSContextTwoParams(bit2oct(EpsBearerId), lengthof(v_ParamOctets) + lengthof(v_ParamOctets_TFT), cs_EPSParameter_QoS(v_ParamOctets),cs_EPSParameter_TFT(v_ParamOctets_TFT)); // @sic R5s210302 sic@ //WA#WI=942180 v_MappedEPSBearerContexts := cs_OneMappedEPSBearerContextList (v_MappedEPSContext); v_NG_NAS_DL_Pdu := f_Get_NG_PDUSessionModificationCommand(v_PDUSessionInfo.SessionId, -, -, -, -, -, f_BuildDefaultQoSRules("5", '7A'O), cs_QoSFlowDescr({v_QoSFlow}), v_MappedEPSBearerContexts); // SDAP parameter for the new SCG DRB Id // The SDAP needs to include the QFI of the QoS Flow of the SCG bearer. This will never be a default bearer v_SDAP_Config := cs_SDAP_Config(oct2int(valueof(v_PDUSessionInfo.SessionId)), false, {5}); v_SDAP_Configuration:= f_ReturnNR_SDAP_Configuration_FromSDAP_Config(valueof(v_SDAP_Config)); v_DRBToAddMod := cs_38508_NRDRB_ToAddMod(v_SDAP_Config, v_NewDRBId, cs_38508_PDCP_Config()); v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-, {v_DRBToAddMod}); v_SS_Config := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, v_NewDRBId, -, -, -, v_SDAP_Configuration)}; v_CellGroupConfig := f_NR_GetCellgroupConfig_DRB(nr_Cell1, {v_NewDRBId}, false); v_CellGroupConfig.rlc_BearerToAddModList := {cs_38508_RLC_BearerConfig_DRB(v_NewDRBId, cs_38508_RLC_Config_AM_DRB(v_ParametersTX, v_ParametersRX), cs_38508_LogicalChannelConfig_DRB)}; //@siclog "Step 0A - 0B" siclog@ f_NR_RRCReconfigNewDRBWithSSConfig(nr_Cell1, v_RadioBearerConfig, v_SS_Config, v_CellGroupConfig, { cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_NG_NAS_DL_Pdu) }); //@siclog "Step 0C" siclog@ SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered,tsc_SHT_IntegrityProtected)))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionModificationComplete (v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionInfo.SessionId)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Modification Complete failed"); } //Prepare nr_Cell3 f_NR_SetCellPower (nr_Cell3, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR2); //Power_Level_Changes //WA#WI=942180 //Prepare nr_Cell3 // Use same DRB Id on both Cell 1 and Cell 3 for the SCG //Modify CellGroupConfig with the correct SpCellConfig content v_SCGConfig.spCellConfig := v_SpCellConfig; v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(-, {cs_NR_DRB_ToAddMod_ReestPDCP(v_NewDRBId, cs_38508_PDCP_Config)}, -, cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig(f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()), secondary)); //Configure the DRB in nr_Cell3 f_NR_SS_CommonRadioBearerConfig(nr_Cell3, { f_NR_GetRadioBearerConfig_FullAM(nr_Cell3, v_NewDRBId)}); f_NR_ULGrantConfiguration_Start(nr_Cell3); //WA#WI=942180 // send HO command f_NR_SendRRCReconfigurationContentsToEUTRA(v_RadioBearerConfig, v_SCGConfig, omit, omit, false, noRACH_Check, nr_Cell3); //WA#WI=942180 f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_PduSessionInfoList({v_CoordMsg})); f_NR5GC_Send_GutiParametersToEUTRA (nr_Cell1); // wait for EUTRA to trigger that it's ok to get the NAS count value f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); v_NASCountInfo := f_NR5GC_Send_AuthParametersToEUTRA (); // get SgNB Keys for SCG //f_ENDC_GetKeysActivateSecurity(nr_Cell3, EUTRA); //WA#WI=942180 //@siclog "Step 1" siclog@ v_EUTRA_HOCmd := f_EUTRA_NR_WaitForCoOrd_Data(EUTRA, "HO Cmd"); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_MobilityFromNRCommand (tsc_NR_RRC_TI_Def, v_EUTRA_HOCmd, substr(v_NASCountInfo.DL, 3, 1) ))); // @siclog "Step 2" siclog@ EUTRA.receive(cmr_EUTRA_NR_OctetData( "Complete")) -> value v_ReceivedMsg; f_NR_CheckReceived_RRCReconfigurationComplete(v_ReceivedMsg); f_NR_Config_Set(EN_DC); //WA#WI=942180 f_ENDC_GetKeysActivateSecurity(nr_Cell3, EUTRA); //WA#WI=942180 f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "NR_DATA_TXRX_Start"); //WA#WI=942180 // @siclog "Step 7 siclog@ v_SduTX := f_LoopbackModeBorPing_GetDataTX(v_PDUSessionInfo.PdnIndex,dedicatedBearer,1); //WA#WI=942180 v_PDCP_Data := {v_SduTX}; //WA#WI=942180 v_DRB_DL := cas_NR_DRB_COMMON_REQ_PDCP_SDUList_DC(nr_Cell3, v_NewDRBId, cs_RlcBearerRouting_NR(nr_Cell3), -, v_PDCP_Data, omit); //WA#WI=942180 v_DRB_UL := car_NR_DRB_COMMON_IND_PDCP_SDUList_DC(nr_Cell3, v_NewDRBId, cs_RlcBearerRouting_NR(nr_Cell3), -, -, omit); //WA#WI=942180 f_NR_CheckDataPathCommon(v_SduTX, v_DRB_DL, v_DRB_UL, "Step 3"); //WA#WI=942180 /*f_NR_CheckDataPath (nr_Cell3, v_PDUSessionInfo.PdnIndex, dedicatedBearer, v_NewDRBId, "Step 7");*/ f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger("NR_DATA_TXRX")); //WA#WI=942180 }

## Module IRAT_Handover_NR5GC_NR

| Module name | IRAT_Handover_NR5GC_NR |
| --- | --- |
| Reason for change | For the changes listed down in section 2.2 , some of the modules have to be imported |
| Summary of change | Modules have to be imported to support the changes listed down in section 2.2 |
| TTCN module | NR5GC_IRAT\8_1_4\IRAT_Handover_NR5GC_NR.ttcn |
| MCC160 Comment | Accepted |

Before Change:

module IRAT_Handover_NR5GC_NR { import from NR_RRC_ASN1_Definitions language "ASN.1:2002" all with {encode "UNALIGNED_PER_OctetAligned"}; << SKIPPED CODE >> import from NR_CellInfoInit all;

After Change:

module IRAT_Handover_NR5GC_NR { import from NR_RRC_ASN1_Definitions language "ASN.1:2002" all with {encode "UNALIGNED_PER_OctetAligned"}; << SKIPPED CODE >> import from NR_CellInfoInit all; import from EPS_NAS_Constants all; //WA#WI=942180 import from NR_PDCP_TypeDefs all; //WA#WI=942180 import from NR_ASP_DrbDefs all; //WA#WI=942180 import from LoopbackIP all; //WA#WI=942180 import from NR_ASP_Templates_DRB_Data all; //WA#WI=942180 import from Common4G5G_Templates all; //WA#WI=942180

## Function f_NR_GetSCGConfigDef()

| Function name | f_NR_GetSCGConfigDef |
| --- | --- |
| Reason for change | tsc_NR_DRB2 is hardcoded as the SCG DRB to get used for EN-DC cases . But in this scenario , DRB3 is getting added as SCG DRB which is also communicated in Pdu session modification command to UE. In case of SAENDC HO , it is required to make sure SCG DRB is assigned as DRB 3 ( as DRB 2 is already used as NR Bearer in Nr Cell 1 ) |
| Summary of change | Second Parameter sa_to_endc is added in the function prototype which is set to True only in case of 8.1.4.2.1.2 so that other ENDC cases are unaffected |
| TTCN module | NR_CommonFunctions.ttcn |
| MCC160 Comment | Accepted in principle, but passed in value of DRB id |

Before Change:

function f_NR_GetSCGConfigDef(NR_CellId_Type p_NR_CellId) runs on NR_BASE_PTC return template (value) CellGroupConfig { var DRB_Identity v_DrbId := tsc_NR_DRB2; // for ENDC only var template (value) CellGroupConfig v_SCGConfig; var template (value) RLC_BearerConfig v_RLC_BearerConfig; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(p_NR_CellId); var template (value) PhysicalCellGroupConfig v_PhysicalCellGroupConfig := f_NR_CellInfo_GetPhysicalCellGroupConfig(p_NR_CellId); var boolean v_IsFR1 := f_NR_CellInfo_GetIsFR1(p_NR_CellId); var template (value) UL_AM_RLC v_ParametersTX := f_NR_UL_AM_RLC(v_IsFR1); var template (value) DL_AM_RLC v_ParametersRX := f_NR_DL_AM_RLC(v_IsFR1); var template(value) SpCellConfig v_SpCellConfig := f_NR_GetSpCellConfigDef(p_NR_CellId); var CellGroupId v_CellGroupId := f_NR_CellInfo_Get_CellGroupId(p_NR_CellId); //@sic R5s201387 sic@ v_RLC_BearerConfig := cs_38508_RLC_BearerConfig_DRB(v_DrbId, cs_38508_RLC_Config_AM_DRB(v_ParametersTX, v_ParametersRX), cs_38508_LogicalChannelConfig_DRB); v_SCGConfig := cs_38508_CellGroupConfig(v_CellGroupId, {v_RLC_BearerConfig}, omit, v_MAC_CellGroupConfig, v_PhysicalCellGroupConfig, v_SpCellConfig); return v_SCGConfig; }

After Change:

function f_NR_GetSCGConfigDef(NR_CellId_Type p_NR_CellId, boolean sa_to_endc := false) runs on NR_BASE_PTC return template (value) CellGroupConfig ////WA#WI=942180 { var DRB_Identity v_DrbId /*:= tsc_NR_DRB2*/; // for ENDC only //WA#WI=942180 var template (value) CellGroupConfig v_SCGConfig; var template (value) RLC_BearerConfig v_RLC_BearerConfig; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(p_NR_CellId); var template (value) PhysicalCellGroupConfig v_PhysicalCellGroupConfig := f_NR_CellInfo_GetPhysicalCellGroupConfig(p_NR_CellId); var boolean v_IsFR1 := f_NR_CellInfo_GetIsFR1(p_NR_CellId); var template (value) UL_AM_RLC v_ParametersTX := f_NR_UL_AM_RLC(v_IsFR1); var template (value) DL_AM_RLC v_ParametersRX := f_NR_DL_AM_RLC(v_IsFR1); var template(value) SpCellConfig v_SpCellConfig := f_NR_GetSpCellConfigDef(p_NR_CellId); var CellGroupId v_CellGroupId := f_NR_CellInfo_Get_CellGroupId(p_NR_CellId); //@sic R5s201387 sic@ if(sa_to_endc) //WA#WI=942180 { v_DrbId := tsc_NR_DRB3; } else{ v_DrbId := tsc_NR_DRB2; } v_RLC_BearerConfig := cs_38508_RLC_BearerConfig_DRB(v_DrbId, cs_38508_RLC_Config_AM_DRB(v_ParametersTX, v_ParametersRX), cs_38508_LogicalChannelConfig_DRB); v_SCGConfig := cs_38508_CellGroupConfig(v_CellGroupId, {v_RLC_BearerConfig}, omit, v_MAC_CellGroupConfig, v_PhysicalCellGroupConfig, v_SpCellConfig); return v_SCGConfig; }

## Function f_NR_CellConfig_Def ()

| Function name | f_NR_CellConfig_Def |
| --- | --- |
| Reason for change | In case of NR Cell 3 ( which is PScell ) – only SRB0 has to be configured along with SCG DRB to be inline with ENDC Cases For this to happen we need an extra check : if the serving cell index is not equal to one . In case of ENDC, PScell will have serving cell index as “one” Other reason is that we can not really rely on f_GetTestcaseAttrib_ENDC() condition as this particular case also has NR cell 1 which is SA cell. |
| Summary of change | Extra Check is added before configuring SRB to ensure Spcell has only SRB0 |
| TTCN module | NR_ConfigurationSteps.ttcn |
| MCC160 Comment | Rejected. Even though the other SRBs are not used in this test, they won’t have any impact if they are configured |

Before Change:

function f_NR_CellConfig_Def(NR_CellId_Type p_NR_CellId, template (omit) NR_CellCA_Capability_Type p_CA_CellCapability := None, boolean p_NRDC_SCG_Cell := false ) runs on NR_BASE_PTC { << SKIPPED CODE >> if (not f_GetTestcaseAttrib_ENDC(testcasename())) { //@sic R5s199033 Added CA configuration sic@ if (match(None, p_CA_CellCapability) or match(SpCell, p_CA_CellCapability)) { // no CA or CA but SPCell if (not p_NRDC_SCG_Cell) { //@sic R5s210891 sic@ f_NR_SS_ConfigureSRBs(p_NR_CellId); } else { // for NR-DC configure SRB0 f_NR_SS_CommonRadioBearerConfig(p_NR_CellId, {cs_NR_SS_SRB0_Config}); } } } }

After Change:

function f_NR_CellConfig_Def(NR_CellId_Type p_NR_CellId, template (omit) NR_CellCA_Capability_Type p_CA_CellCapability := None, boolean p_NRDC_SCG_Cell := false ) runs on NR_BASE_PTC { << SKIPPED CODE >> if (not f_GetTestcaseAttrib_ENDC(testcasename()) and (f_NR_CellInfo_Get_ServingCellIndex(p_NR_CellId) != 1)) { //@sic R5s199033 Added CA configuration sic@ //WA#WI=942180 if (match(None, p_CA_CellCapability) or match(SpCell, p_CA_CellCapability)) { // no CA or CA but SPCell if (not p_NRDC_SCG_Cell) { //@sic R5s210891 sic@ f_NR_SS_ConfigureSRBs(p_NR_CellId); } else { // for NR-DC configure SRB0 f_NR_SS_CommonRadioBearerConfig(p_NR_CellId, {cs_NR_SS_SRB0_Config}); } } } }

## Function f_TC_8_1_4_2_1_2_NR5GC_EUTRA ()

| Function name | f_TC_8_1_4_2_1_2_NR5GC_EUTRA |
| --- | --- |
| Reason for change | Coordination signal is added once the postamble is completed in EUTRA PTC . This indicates to NR PTC to release the NR cells . By doing so we avoid the T310 timer expiry at UE end ( If SCG release happens first , UE will send SCG Failure with T310 timer expiry ) |
| Summary of change | Coordination signal is added |
| TTCN module | IRAT_Handover_NR5GC_EUTRA |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_8_1_4_2_1_2_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { // Inter-RAT Handover / From NR to EN-DC / Success f_EUTRA38_IRAT_InitWithENDC(c31); //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Preamble"); // FFS Prose doesn't specify any power levels f_EUTRA38_SetCellPower(eutra_Cell1, tsc_ServingCellRS_EPRE, tsc_ServingCellRS_EPRE); f_EUTRA_TestBody_Set(true); fl_TC_8_1_4_2_1_2_NR5GC_EUTRA_TestBody (); f_EUTRA_TestBody_Set(false); f_EUTRA38_MultiPDN_Postamble(eutra_Cell1, E2_CONNECTED); }

After Change:

function f_TC_8_1_4_2_1_2_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { // Inter-RAT Handover / From NR to EN-DC / Success f_EUTRA38_IRAT_InitWithENDC(c31); //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Preamble"); // FFS Prose doesn't specify any power levels f_EUTRA38_SetCellPower(eutra_Cell1, tsc_ServingCellRS_EPRE, tsc_ServingCellRS_EPRE); f_EUTRA_TestBody_Set(true); fl_TC_8_1_4_2_1_2_NR5GC_EUTRA_TestBody (); f_EUTRA_TestBody_Set(false); f_EUTRA38_MultiPDN_Postamble(eutra_Cell1, E2_CONNECTED); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); //WA#WI=942180 }

## Function fl_TC_8_1_4_2_1_2_NR5GC_EUTRA_TestBody ()

| Function name | fl_TC_8_1_4_2_1_2_NR5GC_EUTRA_TestBody |
| --- | --- |
| Reason for change | - f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys is moved after the reception of TAU request , by that time , there will be availability of NAS_SecurityCap5G - pc_IP_Ping check is removed before doing Step 6 , MCG DRB IP Tx Rx - Coordination signal is sent after Step 6 “NR_DATA_TXRX_Start” to NR PTC , which makes sure Step 7 ( SCG DRB Tx Rx ) happens after Step 6 is over - Wait coordination signal is introduced in EUTRA PTC “NR_DATA_TXRX” , before proceeding to Postamble ( this makes sure step 7 is completed before UE is powered off in postamble ) |
| Summary of change | - f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys is moved after the reception of TAU request - pc_IP_Ping check is removed before doing Step 6 - Coordination signal is sent after Step 6 “NR_DATA_TXRX_Start” to NR PTC - Wait coordination signal is introduced in EUTRA PTC “NR_DATA_TXRX” , before proceeding to Postamble |
| TTCN module | IRAT_Handover_NR5GC_EUTRA |
| MCC160 Comment | - Alternative solution implemented: don’t include the NAS_SecurityCap5G details in the co-ordination message if not stored 2, 3, 4. Accepted |

Before Change:

function fl_TC_8_1_4_2_1_2_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var EUTRA_NR_CoOrd_GUTI_Type v_CoOrd_Guti; var EUTRA_NR_CoOrd_Security_Type v_SecurityRx; var template (value) MobileIdentity v_MappedGUTI; var SRB_COMMON_IND v_ReceivedMsg; var GutiParameters_Type v_GutiParams := f_EUTRA_CellInfo_GetGuti(eutra_Cell1); var NAS_PlmnId v_PLMN := f_Asn2Nas_PlmnId(v_GutiParams.PLMN_Identity); var TrackingAreaCode v_Tac := f_EUTRA_CellInfo_GetTAC(eutra_Cell1); var template (present) TrackingAreaId v_TAI := cr_TAI(v_PLMN, bit2oct(v_Tac)); var template AdditionalUpdateType v_AdditionalUpdateType := f_GetAdditionalUpdateType (NORMAL); var EUTRA_SecurityParams_Type v_Auth_Params := f_EUTRA_Security_Get(); var EUTRA38_SecurityParamsExtd_Type v_Security_Params; var template (value) EUTRA38_DRBInfoList_Type v_DRBIdList; var DRB_IdentityList_Type v_ListOfDRBs; var template (value) RadioBearerId_Type v_RbId_DRB; var template (omit) integer v_EPSBearerId; var integer i; var octetstring v_HOCmd; var octetstring v_NR_SCGConfig; var octetstring v_NR_RBConfig; var template (value) RRCConnectionReconfiguration_v1510_IEs v_RRCConnectionReconfiguration_v1510_IEs; v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig"); v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB"); v_RRCConnectionReconfiguration_v1510_IEs := cs_RRCConnectionReconfiguration_v1510_IEs(cs_RRCConnectionReconfiguration_v1510_IEs_NR_Config_Setup(v_NR_SCGConfig), 0, v_NR_RBConfig, omit, cs_RRCConnectionReconfiguration_v1530_IEs (cs_SecurityConfigHO_From5GToEPC())); v_HOCmd := f_Encode_RRCConnectionReconfiguration_HO5GToEUTRA(eutra_Cell1, f_EUTRA_CellInfo_GetFrequencyIEs(eutra_Cell1), f_EUTRA_CellInfo_GetDL_ChBandwidth (eutra_Cell1), f_EUTRA_CellInfo_GetUL_ChBandwidth (eutra_Cell1), f_EUTRA_CellInfo_GetPhyCellId(eutra_Cell1), f_EUTRA_CellInfo_GetRootSequenceIndex (eutra_Cell1), v_RRCConnectionReconfiguration_v1510_IEs); // This is the list of default bearers v_DRBIdList := f_EUTRA38_MobileInfo_GetDRBInfoList(); for (i := 0; i < lengthof(v_DRBIdList); i := i+1) { v_ListOfDRBs[i] := valueof(v_DRBIdList[i].DRB_Id); } v_CoOrd_Guti := f_EUTRA_NR_WaitForCoOrd_GutiParams(NR); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); v_SecurityRx := f_EUTRA_NR_WaitForCoOrd_Security (NR); v_MappedGUTI := f_EUTRA38_NR_Convert5G_CoOrdToEPS_GUTI (v_CoOrd_Guti, omit); // Configure C-RNTI based contention resolution in target cell f_EUTRA_SS_ConfigRachProcedure(eutra_Cell1, tsc_C_RNTI_Def2, cs_RachProcedureConfig_CRNTI_Dedicated(f_EUTRA_CellInfo_GetRAR_TA(eutra_Cell1), f_EUTRA_CellInfo_GetDL_ChBandwidth(eutra_Cell1))); // @sic R5-123712 sic@ f_EUTRA_CellInfo_SetC_RNTI ( eutra_Cell1, tsc_C_RNTI_Def2 ); // activate RRC security at SS target cell for SRB 1, 2 and 1 AM DRB v_Auth_Params := f_EUTRA38_MappedContextFromN1_InitNAS(eutra_Cell1, v_SecurityRx, true); v_Auth_Params.AS_Ciphering.ActTimeList := f_EUTRA_RRC_CipherActTime_GetForHO(eutra_Cell1, v_ListOfDRBs); f_EUTRA_SS_RRC_EnableIntProt_CiphULandDL(eutra_Cell1, v_Auth_Params.AS_Integrity, v_Auth_Params.AS_Ciphering, cs_TimingInfo_Now); f_EUTRA_Security_Set(v_Auth_Params); // Now initiate SgNB Keys for DRB2 v_Security_Params := f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys(NR); f_EUTRA38_Security_Set(v_Security_Params); //Configure UL grant configuration ("OnSR", periodic TA is NOT started) in target cell (cell 1) f_EUTRA_SS_CommonCellConfig(eutra_Cell1, cas_ULGrantAllocation_Def_REQ(eutra_Cell1, cs_TimingInfo_Now, cs_UplinkTimeAlignment_Synch_None)); // Send EUTRA HO command to NR f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_OctetData (v_HOCmd, "HO Cmd")); // @siclog "Step 2" siclog@ SRB.receive( car_SRB1_RrcPdu_IND( eutra_Cell1, cr_RRCConnectionReconfigurationComplete_r15)) -> value v_ReceivedMsg; f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_OctetData (v_ReceivedMsg.Signalling.Rrc.Dcch.message_.c1.rrcConnectionReconfigurationComplete.criticalExtensions.rrcConnectionReconfigurationComplete_r8.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.scg_ConfigResponseNR_r15, "Complete")); /* SS automatically checks that UE transmit an RRCConnectionReconfigurationComplete using the indicated security key and the indicated security algorithms */ f_EUTRA_PreliminaryPass( __FILE__, __LINE__, "Step 2" ); //Configure UL grant configuration f_EUTRA_SS_CommonCellConfig(eutra_Cell1, cas_PUCCH_Synch_Config_REQ(eutra_Cell1, cs_TimingInfo_Now, cs_UplinkTimeAlignment_Synch_Auto_Def)); // @siclog "Step 3" siclog@ SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1, cr_NAS_Indication(tsc_SHT_IntegrityProtected, cr_TAU_Request_N1ToS1((tsc_EpsUpdate_TaUpdate, tsc_EpsUpdate_Combined_IMSIAttach, tsc_EpsUpdate_Combined_TaLaUpdate), v_SecurityRx.KSI, cr_DRXparameter_Any ('5C'O) ifpresent, v_AdditionalUpdateType, v_MappedGUTI, v_TAI ifpresent, -, tsc_NasKsi_MappedSecurityContext, cdr_NAS_KeySetIdentifier_tlv (?, ?) ifpresent, -, -, cr_508_EPS_BearerContextStatusTAUReq ifpresent))))-> value v_ReceivedMsg; // @siclog "Step 4-5" siclog@ f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq (eutra_Cell1, v_ReceivedMsg.Signalling.Nas[0], -, noRrcConnectionRelease, -, true); // @siclog "Step 6" siclog@ v_EPSBearerId := f_Get_EPSBearerIdForAPNType(Internet_DNN); if (isvalue(v_EPSBearerId)) { v_RbId_DRB := cs_RadioBearerId_DRB((valueof(v_EPSBearerId) - 4)); if (pc_IP_Ping) { f_EUTRA_CheckDataPath(eutra_Cell1, PDCP, defaultBearer, "Step 6", v_RbId_DRB, -, -, f_GetMultiPdnIndex (Internet_DNN)); } } }

After Change:

function fl_TC_8_1_4_2_1_2_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var EUTRA_NR_CoOrd_GUTI_Type v_CoOrd_Guti; var EUTRA_NR_CoOrd_Security_Type v_SecurityRx; var template (value) MobileIdentity v_MappedGUTI; var SRB_COMMON_IND v_ReceivedMsg; var GutiParameters_Type v_GutiParams := f_EUTRA_CellInfo_GetGuti(eutra_Cell1); var NAS_PlmnId v_PLMN := f_Asn2Nas_PlmnId(v_GutiParams.PLMN_Identity); var TrackingAreaCode v_Tac := f_EUTRA_CellInfo_GetTAC(eutra_Cell1); var template (present) TrackingAreaId v_TAI := cr_TAI(v_PLMN, bit2oct(v_Tac)); var template AdditionalUpdateType v_AdditionalUpdateType := f_GetAdditionalUpdateType (NORMAL); var EUTRA_SecurityParams_Type v_Auth_Params := f_EUTRA_Security_Get(); var EUTRA38_SecurityParamsExtd_Type v_Security_Params; var template (value) EUTRA38_DRBInfoList_Type v_DRBIdList; var DRB_IdentityList_Type v_ListOfDRBs; var template (value) RadioBearerId_Type v_RbId_DRB; var template (omit) integer v_EPSBearerId; var integer i; var octetstring v_HOCmd; var octetstring v_NR_SCGConfig; var octetstring v_NR_RBConfig; var template (value) RRCConnectionReconfiguration_v1510_IEs v_RRCConnectionReconfiguration_v1510_IEs; var EUTRA_SecurityParams_Type v_AuthParams; //WA#WI=942180 v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig"); v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB"); v_RRCConnectionReconfiguration_v1510_IEs := cs_RRCConnectionReconfiguration_v1510_IEs(cs_RRCConnectionReconfiguration_v1510_IEs_NR_Config_Setup(v_NR_SCGConfig), 0, v_NR_RBConfig, omit, cs_RRCConnectionReconfiguration_v1530_IEs (cs_SecurityConfigHO_From5GToEPC())); v_HOCmd := f_Encode_RRCConnectionReconfiguration_HO5GToEUTRA(eutra_Cell1, f_EUTRA_CellInfo_GetFrequencyIEs(eutra_Cell1), f_EUTRA_CellInfo_GetDL_ChBandwidth (eutra_Cell1), f_EUTRA_CellInfo_GetUL_ChBandwidth (eutra_Cell1), f_EUTRA_CellInfo_GetPhyCellId(eutra_Cell1), f_EUTRA_CellInfo_GetRootSequenceIndex (eutra_Cell1), v_RRCConnectionReconfiguration_v1510_IEs); // This is the list of default bearers v_DRBIdList := f_EUTRA38_MobileInfo_GetDRBInfoList(); for (i := 0; i < lengthof(v_DRBIdList); i := i+1) { v_ListOfDRBs[i] := valueof(v_DRBIdList[i].DRB_Id); } v_CoOrd_Guti := f_EUTRA_NR_WaitForCoOrd_GutiParams(NR); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); v_SecurityRx := f_EUTRA_NR_WaitForCoOrd_Security (NR); v_MappedGUTI := f_EUTRA38_NR_Convert5G_CoOrdToEPS_GUTI (v_CoOrd_Guti, omit); // Configure C-RNTI based contention resolution in target cell f_EUTRA_SS_ConfigRachProcedure(eutra_Cell1, tsc_C_RNTI_Def2, cs_RachProcedureConfig_CRNTI_Dedicated(f_EUTRA_CellInfo_GetRAR_TA(eutra_Cell1), f_EUTRA_CellInfo_GetDL_ChBandwidth(eutra_Cell1))); // @sic R5-123712 sic@ f_EUTRA_CellInfo_SetC_RNTI ( eutra_Cell1, tsc_C_RNTI_Def2 ); // activate RRC security at SS target cell for SRB 1, 2 and 1 AM DRB v_Auth_Params := f_EUTRA38_MappedContextFromN1_InitNAS(eutra_Cell1, v_SecurityRx, true); v_Auth_Params.AS_Ciphering.ActTimeList := f_EUTRA_RRC_CipherActTime_GetForHO(eutra_Cell1, v_ListOfDRBs); f_EUTRA_SS_RRC_EnableIntProt_CiphULandDL(eutra_Cell1, v_Auth_Params.AS_Integrity, v_Auth_Params.AS_Ciphering, cs_TimingInfo_Now); f_EUTRA_Security_Set(v_Auth_Params); // Now initiate SgNB Keys for DRB2 //v_Security_Params := f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys(NR); //WA#WI=942180 //f_EUTRA38_Security_Set(v_Security_Params); //WA#WI=942180 //Configure UL grant configuration ("OnSR", periodic TA is NOT started) in target cell (cell 1) f_EUTRA_SS_CommonCellConfig(eutra_Cell1, cas_ULGrantAllocation_Def_REQ(eutra_Cell1, cs_TimingInfo_Now, cs_UplinkTimeAlignment_Synch_None)); // Send EUTRA HO command to NR f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_OctetData (v_HOCmd, "HO Cmd")); // @siclog "Step 2" siclog@ SRB.receive( car_SRB1_RrcPdu_IND( eutra_Cell1, cr_RRCConnectionReconfigurationComplete_r15)) -> value v_ReceivedMsg; f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_OctetData (v_ReceivedMsg.Signalling.Rrc.Dcch.message_.c1.rrcConnectionReconfigurationComplete.criticalExtensions.rrcConnectionReconfigurationComplete_r8.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.scg_ConfigResponseNR_r15, "Complete")); /* SS automatically checks that UE transmit an RRCConnectionReconfigurationComplete using the indicated security key and the indicated security algorithms */ f_EUTRA_PreliminaryPass( __FILE__, __LINE__, "Step 2" ); //Configure UL grant configuration f_EUTRA_SS_CommonCellConfig(eutra_Cell1, cas_PUCCH_Synch_Config_REQ(eutra_Cell1, cs_TimingInfo_Now, cs_UplinkTimeAlignment_Synch_Auto_Def)); // @siclog "Step 3" siclog@ SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1, cr_NAS_Indication(tsc_SHT_IntegrityProtected, cr_TAU_Request_N1ToS1((tsc_EpsUpdate_TaUpdate, tsc_EpsUpdate_Combined_IMSIAttach, tsc_EpsUpdate_Combined_TaLaUpdate), v_SecurityRx.KSI, cr_DRXparameter_Any ('5C'O) ifpresent, v_AdditionalUpdateType, v_MappedGUTI, v_TAI ifpresent, -, tsc_NasKsi_MappedSecurityContext, cdr_NAS_KeySetIdentifier_tlv (?, ?) ifpresent, -, -, cr_508_EPS_BearerContextStatusTAUReq ifpresent))))-> value v_ReceivedMsg; v_AuthParams := f_EUTRA_Security_Get(); //WA#WI=942180 if(ispresent(v_ReceivedMsg.Signalling.Nas[0].Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueAddSecurityCap)) { v_AuthParams.NAS_SecurityCap5G := v_ReceivedMsg.Signalling.Nas[0].Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueAddSecurityCap; f_EUTRA_Security_Set(v_AuthParams); v_Security_Params := f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys(NR); //NIRMAL_HACK f_EUTRA38_Security_Set(v_Security_Params); //NIRMALL_HACK } //WA#WI=942180 // @siclog "Step 4-5" siclog@ f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq (eutra_Cell1, v_ReceivedMsg.Signalling.Nas[0], -, noRrcConnectionRelease, -, true); // @siclog "Step 6" siclog@ v_EPSBearerId := f_Get_EPSBearerIdForAPNType(Internet_DNN); if (isvalue(v_EPSBearerId)) { v_RbId_DRB := cs_RadioBearerId_DRB((valueof(v_EPSBearerId) - 4)); // if (pc_IP_Ping) { //WA#WI=942180 f_EUTRA_CheckDataPath(eutra_Cell1, PDCP, defaultBearer, "Step 6", v_RbId_DRB, -, -, f_GetMultiPdnIndex (Internet_DNN)); // } //WA#WI=942180 } f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("NR_DATA_TXRX_Start")); //WA#WI=942180 f_EUTRA_NR_WaitForCoOrd_Trigger(NR,"NR_DATA_TXRX"); //WA#WI=942180 }

## New Function : f_NR_Config_Set

| Function name | f_NR_Config_Set |
| --- | --- |
| Reason for change | f_NR_Config_Set has to be set as EN_DC after the reception of reconfiguration complete ( Handover to ENDC ) which gets used while validating UE loop back on NR Cell 3 in the function f_NR_CheckDataPathCommon in step 7 |
| Summary of change | New function is implemented as below |
| TTCN module | NR_CellInfo |
| MCC160 Comment | Accepted |

Function Implementation :

/* * @desc Set NR_Configuration_Type * @return NR_Configuration_Type * @status APPROVED (ENDC, NR5GC, NR5GC_IRAT) */ function f_NR_Config_Set(NR_Configuration_Type p_NR_Configuration) runs on NR_BASE_PTC { vc_NR_Global.NR_Config := p_NR_Configuration; } //WA#WI=942180

# Branches executed

## This NR TC was executed in IRAT Band Combination n78-FDD3 with NEA1/NIA1

# Execution Log Files

## MediaTek MT6983 UE

The MediaTek MT6983 UE passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
TC_8_1_4_2_1_2_MediaTek.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s220854: Supporting information for addition of NR TC 8.1.4.2.1.2 |
| --- | --- |