3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220931

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2667 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 8.1.5.2.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-07-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - At step 2 of the test case, Prach configuration index and RaResponse window in the system information are updated to 94 and sl40 respectively - As per 38.331 : ra-ResponseWindow Msg2 (RAR) window length in number of slots. The network configures a value lower than or equal to 10 ms when Msg2 is transmitted in licensed spectrum and a value lower than or equal to 40 ms when Msg2 is transmitted with shared spectrum channel access (see TS 38.321 [3], clause 5.1.4). UE ignores the field if included in SCellConfig. If ra-ResponseWindow-v1610 or ra-ResponseWindow-v1700 is signalled, UE shall ignore the ra-ResponseWindow (without suffix). The field ra-ResponseWindow-v1700 is applicable to SCS 480 kHz and SCS 960 kHz. As per the above RRC specification snippet , RaResponse window has to be limited to sl20. - At Step 2 prach-ConfigurationIndex is set as 94 , for which Prach is expected to be received in the subframe = 4/9 ( every odd SFN ). If Prach is received in Subframe =9 and slot = 1 , no issue is observed . But If RACH is received in subframe = 4 & Slot = 1 , RAR Tx can be scheduled on sf = 0 slot = 0 (or) sf = 5 slot = 0 . Considering the interference due to SSB block transmission sf = 0 slot = 0 can not be used for RAR transmission . Sending Rar at sf = 5 slot = 0 will make the Rar window to expire (sl20 is the rar window configured) . To overcome this issue , it is proposed to make the prach-ConfigurationIndex as 91 for which Prach is expected to be received in the subframe = 8/9 ( every odd SFN ) and there is also no SSB interference during Rar transmission |  |  |  |  |  |  |  |  |
| Summary of change: |  | - At Step 2, ra-ResponseWindow is set as sl20 - At Step 2, prachConfigurationIndex is modified as 91 in case of TDD |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.2.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required for NR testcase 8.1.5.2.2	3

2.1	Correction to the function fl_TC_8_1_5_2_2_TestBody	3

- Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

- Corrections required for NR testcase 8.1.5.2.2

  - Correction to the function fl_TC_8_1_5_2_2_TestBody

| Function name | Function fl_TC_8_1_5_2_2_TestBody |
| --- | --- |
| Reason for change | - At step 2 of the test case, Prach configuration index and RaResponse window in the system information are updated to 94 and sl40 respectively - As per 38.331 : ra-ResponseWindow Msg2 (RAR) window length in number of slots. The network configures a value lower than or equal to 10 ms when Msg2 is transmitted in licensed spectrum and a value lower than or equal to 40 ms when Msg2 is transmitted with shared spectrum channel access (see TS 38.321 [3], clause 5.1.4). UE ignores the field if included in SCellConfig. If ra-ResponseWindow-v1610 or ra-ResponseWindow-v1700 is signalled, UE shall ignore the ra-ResponseWindow (without suffix). The field ra-ResponseWindow-v1700 is applicable to SCS 480 kHz and SCS 960 kHz. As per the above RRC specification snippet , RaResponse window has to be limited to sl20. - At Step 2 prach-ConfigurationIndex is set as 94 , for which Prach is expected to be received in the subframe = 4/9 ( every odd SFN ). If Prach is received in Subframe =9 and slot = 1 , no issue is observed . But If RACH is received in subframe = 4 & Slot = 1 , RAR Tx can be scheduled on sf = 0 slot = 0 (or) sf = 5 slot = 0 . Considering the interference due to SSB block transmission sf = 0 slot = 0 can not be used for RAR transmission . Sending Rar at sf = 5 slot = 0 will make the Rar window to expire (sl20 is the rar window configured) . To overcome this issue , it is proposed to make the prach-ConfigurationIndex as 91 for which Prach is expected to be received in the subframe = 8/9 ( every odd SFN ) and there is also no SSB interference during Rar transmission |
| Summary of change | - At Step 2, ra-ResponseWindow is set as sl20 - At Step 2, prachConfigurationIndex is modified as 91 in case of TDD |
| TTCN module | RRC_Others_NR5GC.ttcn |
| MCC160 Comment | Accepted conditional to prose CR agreement at RAN5#96e |

Before Change:

function fl_TC_8_1_5_2_2_TestBody() runs on NR5GC_PTC { var integer v_PrachConfigurationIndex; var NR_PhysicalParameters_Type v_PhysicalParameters; var template (present) TimingInfo_Type v_TimingInfo; var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var integer v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); var template (value) QosFlow_Identification_Type v_QosFlowId := f_NR_GetQosFlowIdentification_ForDRBId(v_DRBId); //Get the QosFlow ID information from the DRB ID var boolean v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1); var NR_DuplexMode_Type v_NR_DuplexMode := f_NR_CellInfo_GetDuplexMode(nr_Cell1); var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); //@sic R5s201136 sic@ var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedure := cs_NR_RachProcedureConfig_Msg3TBS(v_NR_PhysicalParameters, 640); //@sic R5s201136 R5-206337 sic@ //Set SIB1 content as Table 8.1.5.2.2.3.3-2: SIB1 if ( not v_IsFR1 ) { //FR2 v_PrachConfigurationIndex := 6; // 10 subframes with each 4 slots // slots (4,9,14,19,24,29,34,39) - As per 38.211 Table 6.3.3.2-4 4 // @sic R5s211650 sic@ // translate to 120kHz slots (8, 9, 18, 19, 28, 29, 38, 39, 48, 49, 58, 59, 68, 69, 78, 79) v_TimingInfo := ( cr_TimingInfo(?, (1), -, cs_SlotTimingInfo_Numerology3(0)), //Slot 8 cr_TimingInfo(?, (1), -, cs_SlotTimingInfo_Numerology3(1)), //Slot 9 cr_TimingInfo(?, (2), -, cs_SlotTimingInfo_Numerology3(2)), //Slot 18 cr_TimingInfo(?, (2), -, cs_SlotTimingInfo_Numerology3(3)), //Slot 19 cr_TimingInfo(?, (3), -, cs_SlotTimingInfo_Numerology3(4)), //Slot 28 cr_TimingInfo(?, (3), -, cs_SlotTimingInfo_Numerology3(5)), //Slot 29 cr_TimingInfo(?, (4), -, cs_SlotTimingInfo_Numerology3(6)), //Slot 38 cr_TimingInfo(?, (4), -, cs_SlotTimingInfo_Numerology3(7)), //Slot 39 cr_TimingInfo(?, (6), -, cs_SlotTimingInfo_Numerology3(0)), //Slot 48 cr_TimingInfo(?, (6), -, cs_SlotTimingInfo_Numerology3(1)), //Slot 49 cr_TimingInfo(?, (7), -, cs_SlotTimingInfo_Numerology3(2)), //Slot 58 cr_TimingInfo(?, (7), -, cs_SlotTimingInfo_Numerology3(3)), //Slot 59 cr_TimingInfo(?, (8), -, cs_SlotTimingInfo_Numerology3(4)), //Slot 68 cr_TimingInfo(?, (8), -, cs_SlotTimingInfo_Numerology3(5)), //Slot 69 cr_TimingInfo(?, (9), -, cs_SlotTimingInfo_Numerology3(6)), //Slot 78 cr_TimingInfo(?, (9), -, cs_SlotTimingInfo_Numerology3(7)) //Slot 79 ); } else if (v_IsFR1 and ischosen(v_NR_DuplexMode.TDD)) { //FR1 TDD v_PrachConfigurationIndex := 94; //FR1 TDD - As per 38.211 Table 6.3.3.2-3 v_TimingInfo := (cr_TimingInfo(?, (4)), cr_TimingInfo(?, (9))); } else { //FR1 FDD v_PrachConfigurationIndex := 119; //FR1 FDD - As per 38.211 Table 6.3.3.2-2 //nSFN mod 8 = 1 v_TimingInfo := (cr_TimingInfo(?, (2)), cr_TimingInfo(?, (6)), cr_TimingInfo(?, (9))); } //@sic R5s201136 sic@ f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedure); //@siclog "Step 1" siclog@ //The SS transmits one IP packet to the UE DRB.send(cas_NR_DRB_COMMON_REQ_SDAP_SDUList(nr_Cell1, v_QosFlowId, {v_IPData})); //@siclog "Steps 1a" siclog@ //The SS waits for 0.5 second. //@sic R5-206337 sic@ f_Delay(0.5); //Stop the UL Grant allocation //@sic R5s201136 R5-204334 sic@ f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); //@siclog "Steps 2/3" siclog@ //Reconfigure SS with wew PRACH configuration index //Set Prach configuration index in SIB1 and in Cell Array f_NR_CellInfo_SetPRACH_ConfigurationIndex(nr_Cell1, v_PrachConfigurationIndex); f_NR_CellInfo_SetRaResponseWindow(nr_Cell1, sl40);//@sic R5s220280 R5-221105sic@ <<SKIPPED CODE>> }

After Change:

function fl_TC_8_1_5_2_2_TestBody() runs on NR5GC_PTC { var integer v_PrachConfigurationIndex; var NR_PhysicalParameters_Type v_PhysicalParameters; var template (present) TimingInfo_Type v_TimingInfo; var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var integer v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); var template (value) QosFlow_Identification_Type v_QosFlowId := f_NR_GetQosFlowIdentification_ForDRBId(v_DRBId); //Get the QosFlow ID information from the DRB ID var boolean v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1); var NR_DuplexMode_Type v_NR_DuplexMode := f_NR_CellInfo_GetDuplexMode(nr_Cell1); var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); //@sic R5s201136 sic@ var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedure := cs_NR_RachProcedureConfig_Msg3TBS(v_NR_PhysicalParameters, 640); //@sic R5s201136 R5-206337 sic@ //Set SIB1 content as Table 8.1.5.2.2.3.3-2: SIB1 if ( not v_IsFR1 ) { //FR2 v_PrachConfigurationIndex := 6; // 10 subframes with each 4 slots // slots (4,9,14,19,24,29,34,39) - As per 38.211 Table 6.3.3.2-4 4 // @sic R5s211650 sic@ // translate to 120kHz slots (8, 9, 18, 19, 28, 29, 38, 39, 48, 49, 58, 59, 68, 69, 78, 79) v_TimingInfo := ( cr_TimingInfo(?, (1), -, cs_SlotTimingInfo_Numerology3(0)), //Slot 8 cr_TimingInfo(?, (1), -, cs_SlotTimingInfo_Numerology3(1)), //Slot 9 cr_TimingInfo(?, (2), -, cs_SlotTimingInfo_Numerology3(2)), //Slot 18 cr_TimingInfo(?, (2), -, cs_SlotTimingInfo_Numerology3(3)), //Slot 19 cr_TimingInfo(?, (3), -, cs_SlotTimingInfo_Numerology3(4)), //Slot 28 cr_TimingInfo(?, (3), -, cs_SlotTimingInfo_Numerology3(5)), //Slot 29 cr_TimingInfo(?, (4), -, cs_SlotTimingInfo_Numerology3(6)), //Slot 38 cr_TimingInfo(?, (4), -, cs_SlotTimingInfo_Numerology3(7)), //Slot 39 cr_TimingInfo(?, (6), -, cs_SlotTimingInfo_Numerology3(0)), //Slot 48 cr_TimingInfo(?, (6), -, cs_SlotTimingInfo_Numerology3(1)), //Slot 49 cr_TimingInfo(?, (7), -, cs_SlotTimingInfo_Numerology3(2)), //Slot 58 cr_TimingInfo(?, (7), -, cs_SlotTimingInfo_Numerology3(3)), //Slot 59 cr_TimingInfo(?, (8), -, cs_SlotTimingInfo_Numerology3(4)), //Slot 68 cr_TimingInfo(?, (8), -, cs_SlotTimingInfo_Numerology3(5)), //Slot 69 cr_TimingInfo(?, (9), -, cs_SlotTimingInfo_Numerology3(6)), //Slot 78 cr_TimingInfo(?, (9), -, cs_SlotTimingInfo_Numerology3(7)) //Slot 79 ); } else if (v_IsFR1 and ischosen(v_NR_DuplexMode.TDD)) { //FR1 TDD v_PrachConfigurationIndex := 91; //WA#WI=984104 //FR1 TDD - As per 38.211 Table 6.3.3.2-3 v_TimingInfo := (cr_TimingInfo(?, (8)), //WA#WI=984104 cr_TimingInfo(?, (9))); } else { //FR1 FDD v_PrachConfigurationIndex := 119; //FR1 FDD - As per 38.211 Table 6.3.3.2-2 //nSFN mod 8 = 1 v_TimingInfo := (cr_TimingInfo(?, (2)), cr_TimingInfo(?, (6)), cr_TimingInfo(?, (9))); } //@sic R5s201136 sic@ f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedure); //@siclog "Step 1" siclog@ //The SS transmits one IP packet to the UE DRB.send(cas_NR_DRB_COMMON_REQ_SDAP_SDUList(nr_Cell1, v_QosFlowId, {v_IPData})); //@siclog "Steps 1a" siclog@ //The SS waits for 0.5 second. //@sic R5-206337 sic@ f_Delay(0.5); //Stop the UL Grant allocation //@sic R5s201136 R5-204334 sic@ f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); //@siclog "Steps 2/3" siclog@ //Reconfigure SS with wew PRACH configuration index //Set Prach configuration index in SIB1 and in Cell Array f_NR_CellInfo_SetPRACH_ConfigurationIndex(nr_Cell1, v_PrachConfigurationIndex); //f_NR_CellInfo_SetRaResponseWindow(nr_Cell1, sl40);//@sic R5s220280 R5-221105sic@ //WA#WI=984104 << SKIPPED CODE >> }