3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220936

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2675 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC SOR test case 6.3.1.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | NR5GC PTC needs to receive a trigger from IMS PTC before initiating emergency call release (this is the trigger sent from IMS PTC in function f_IMS_EmergencyCallAndReleaseWithoutRegistration()) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add additional wait for trigger. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.7.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

- Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

- Corrections required

  - Change 1

| Function name | fl_TC_6_3_1_7_TestBody() |
| --- | --- |
| Reason for change | NR5GC PTC needs to receive a trigger from IMS PTC before initiating emergency call release (this is the trigger sent from IMS PTC in function f_IMS_EmergencyCallAndReleaseWithoutRegistration()) |
| Summary of change | Add additional wait for trigger. Note : In the code as delivered in iwd-TTCN3-B2020-09_D22wk23, an addition trigger would also be required to be sent to IMS PTC to indicate voice QoS bearer is established. This has already been added into the function f_NR5GC_EmergencyCallPDUSession() by the MCC comments for CR R5s220821. |
| TTCN module | SOR_NR5GC.ttcn |
| MCC160 Comment | Accepted. |

Before Change:

function fl_TC_6_3_1_7_TestBody() runs on NR5GC_PTC { var NAS_PlmnId v_PlmnID_2 := f_NR_Asn2Nas_PlmnId(tsc_NR_HPLMN_002_11); //@sic R5-204410 sic@ var O2_Type v_AccessTechId :='0800'O; var octetstring v_Data := v_PlmnID_2 & v_AccessTechId; var template (value) SOR_Header v_Header := crs_NG_SOR_Header('1'B,'1'B,'1'B,'0'B); //@siclog "Step 1" siclog@ //Power on the UE //@siclog "Step 1A1-1A13" siclog@ //Steps 1 to 13 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed //@sic R5-204410 sic@ f_NR5GC_Preamble_Steps1_13(nr_Cell11); //@siclog "Step 1B" siclog@ //SS transmits a REGISTRATION REJECT message with cause #22 (Congestion) and T3346 set to 3 minutes. //@sic R5-204410 sic@ SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell11, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause(omit, tsc_NR5GCCause_Congestion), cs_GPRS_Timer2_3_IEI('5F'O, '001'B, '00011'B))))); //@siclog "Step 1C" siclog@ //The SS releases the RRC connection //@sic R5-204410 sic@ f_NR_RRCRelease(nr_Cell11); //@siclog "Step 2" siclog@ //The UE is made to establish an emergency call. This can be done by an AT/MMI command f_UT_RequestIMSEmergencyCall (UT); //@siclog "Step 3" siclog@ // The UE transmits an RRCSetupRequest message on NR Cell 11 with establishmentCause set to 'emergency'. SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_emergency)))); //@sic R5s201466 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "RRCSetupRequest received"); //@sic R5s200305 sic@ //@siclog "Steps 4-5" siclog@ //Steps 3 to 4 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed //@siclog "Step 6" siclog@ //The UE transmit a REGISTRATION REQUEST message with IE 5GS registration type set to "Emergency registration" or "Initial registration"//@sic R5-210807 R5s201466 sic@ //@siclog "Steps 7-14" siclog@ //Steps 5 to 13 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed on NR Cell 13 //@siclog "Step 15" siclog@ //The SS transmits a DLInformationTransfer message and a REGISTRATION ACCEPT message containing //steering of roaming information, includes unmatched SOR-MAC-IAUSF that will result in unsuccessful security check while on UE reception f_NR5GC_SorRegistration(nr_Cell11,v_Data,v_Header,true,Initial_Secure, tsc_SHT_IntegrityProtected);//@sic R5s210236 R5s201455 R5-205361 sic@//using Initial_Secure to indicate the security context state. //@siclog "Step 16" siclog@ //The UE transmits an ULInformationTransfer message and REGISTRATION COMPLETE message without including an SOR transparent container f_NR5GC_Registration_Complete(nr_Cell11); //@siclog "Step 17" siclog@ //Check: Does the UE transmit an UL NAS TRANSPORT message with "Request type" set to "initial emergency request", and a PDU SESSION ESTABLISHMENT REQUEST message on NR Cell 13 //@siclog "Step 18-22" siclog@//@sic R5-216165 sic@ //Steps 9 to 13 of the procedure of "IMS Emergency call establishment in 5GC with IMS emergency registration" described in TS 38.508-1 [4] Table 4.9.11.2.2-1 are performed. f_NR5GC_EmergencyCallPDUSession(nr_Cell11,tsc_NR_RbId_SRB1);//@sic R5s211479 sic@ //@siclog "Step 23" siclog@//@sic R5-216165 sic@ //Generic procedure of "Test Procedure for IMS MO Emergency call release" described in TS 38.508-1 [4] Table 4.9.12A.2.2-1 are performed. f_NR5GC_EmergencyCallRelease(nr_Cell11);//@sic R5s211479 sic@ //@siclog "Step 24" siclog@//@sic R5-216165 sic@ //The SS releases the RRC connection. f_NR_RRCRelease(nr_Cell11); }//End of fl_TC_6_3_1_7_TestBody

After Change:

function fl_TC_6_3_1_7_TestBody() runs on NR5GC_PTC { var NAS_PlmnId v_PlmnID_2 := f_NR_Asn2Nas_PlmnId(tsc_NR_HPLMN_002_11); //@sic R5-204410 sic@ var O2_Type v_AccessTechId :='0800'O; var octetstring v_Data := v_PlmnID_2 & v_AccessTechId; var template (value) SOR_Header v_Header := crs_NG_SOR_Header('1'B,'1'B,'1'B,'0'B); //@siclog "Step 1" siclog@ //Power on the UE //@siclog "Step 1A1-1A13" siclog@ //Steps 1 to 13 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed //@sic R5-204410 sic@ f_NR5GC_Preamble_Steps1_13(nr_Cell11); //@siclog "Step 1B" siclog@ //SS transmits a REGISTRATION REJECT message with cause #22 (Congestion) and T3346 set to 3 minutes. //@sic R5-204410 sic@ SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell11, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause(omit, tsc_NR5GCCause_Congestion), cs_GPRS_Timer2_3_IEI('5F'O, '001'B, '00011'B))))); //@siclog "Step 1C" siclog@ //The SS releases the RRC connection //@sic R5-204410 sic@ f_NR_RRCRelease(nr_Cell11); //@siclog "Step 2" siclog@ //The UE is made to establish an emergency call. This can be done by an AT/MMI command f_UT_RequestIMSEmergencyCall (UT); //@siclog "Step 3" siclog@ // The UE transmits an RRCSetupRequest message on NR Cell 11 with establishmentCause set to 'emergency'. SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_emergency)))); //@sic R5s201466 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "RRCSetupRequest received"); //@sic R5s200305 sic@ //@siclog "Steps 4-5" siclog@ //Steps 3 to 4 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed //@siclog "Step 6" siclog@ //The UE transmit a REGISTRATION REQUEST message with IE 5GS registration type set to "Emergency registration" or "Initial registration"//@sic R5-210807 R5s201466 sic@ //@siclog "Steps 7-14" siclog@ //Steps 5 to 13 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed on NR Cell 13 //@siclog "Step 15" siclog@ //The SS transmits a DLInformationTransfer message and a REGISTRATION ACCEPT message containing //steering of roaming information, includes unmatched SOR-MAC-IAUSF that will result in unsuccessful security check while on UE reception f_NR5GC_SorRegistration(nr_Cell11,v_Data,v_Header,true,Initial_Secure, tsc_SHT_IntegrityProtected);//@sic R5s210236 R5s201455 R5-205361 sic@//using Initial_Secure to indicate the security context state. //@siclog "Step 16" siclog@ //The UE transmits an ULInformationTransfer message and REGISTRATION COMPLETE message without including an SOR transparent container f_NR5GC_Registration_Complete(nr_Cell11); //@siclog "Step 17" siclog@ //Check: Does the UE transmit an UL NAS TRANSPORT message with "Request type" set to "initial emergency request", and a PDU SESSION ESTABLISHMENT REQUEST message on NR Cell 13 //@siclog "Step 18-22" siclog@//@sic R5-216165 sic@ //Steps 9 to 13 of the procedure of "IMS Emergency call establishment in 5GC with IMS emergency registration" described in TS 38.508-1 [4] Table 4.9.11.2.2-1 are performed. f_NR5GC_EmergencyCallPDUSession(nr_Cell11,tsc_NR_RbId_SRB1);//@sic R5s211479 sic@ //@siclog "Step 23" siclog@//@sic R5-216165 sic@ f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); //Generic procedure of "Test Procedure for IMS MO Emergency call release" described in TS 38.508-1 [4] Table 4.9.12A.2.2-1 are performed. f_NR5GC_EmergencyCallRelease(nr_Cell11);//@sic R5s211479 sic@ //@siclog "Step 24" siclog@//@sic R5-216165 sic@ //The SS releases the RRC connection. f_NR_RRCRelease(nr_Cell11); }//End of fl_TC_6_3_1_7_TestBody