3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220945

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2679 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 7.1.1.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-07-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation - At steps 1-2 MAC PDU is sent with cs_NR_HarqProcessAssignment_Automatic. As a result, SS might pick harq id = 0 with ndi = x - At Steps 4-5 , PDCCH is sent in such a way that C-RNTI is different from the assigned to the UE ; this step is also done with cs_NR_HarqProcessAssignment_Automatic. As a result, SS might pick harq id = 0 but with ndi = !x ( toggled from step 1-2 ) But , UE will not decode this PDCCH and will not have knowledge about the harq id and NDI used in step 4-5 as C-RNTI is different from the assigned to the UE - At Step 7 , MAC PDU is sent with cs_NR_HarqProcessAssignment(0) which makes SS to use harq Id = 0 and ndi = x ( toggled from step 4-5 ) . as UE has not decoded the PDCCH sent on steps 4-5 , UE will consider the MAC PDU received at step 7 with harq id = 0 as Retransmission not a new data |  |  |  |  |  |  |  |  |
| Summary of change: |  | - At steps 4-5 , harq id = 1 is used , so that Steps 1-2 will use harq id = 0 ndi = x Steps 4-5 will use harq id = 1 ndi = x Step 7 will use harq id = 0 ndi = !x further will use harq id = 1 , ndi = !x |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.2.1.ENDC , 7.1.1.2.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required for NR testcase 7.1.1.2.1	3

2.1	Correction to the function f_TC_7_1_1_2_1_NR_TestBody1	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required for NR testcase 7.1.1.2.1

## Correction to the function f_TC_7_1_1_2_1_NR_TestBody1

| Function name | f_TC_7_1_1_2_1_NR_TestBody1 |
| --- | --- |
| Reason for change | In the current TTCN implementation - At steps 1-2 MAC PDU is sent with cs_NR_HarqProcessAssignment_Automatic. As a result, SS might pick harq id = 0 with ndi = x - At Steps 4-5 , PDCCH is sent in such a way that C-RNTI is different from the assigned to the UE ; this step is also done with cs_NR_HarqProcessAssignment_Automatic. As a result, SS might pick harq id = 0 but with ndi = !x ( toggled from step 1-2 ) But , UE will not decode this PDCCH and will not have knowledge about the harq id and NDI used in step 4-5 as C-RNTI is different from the assigned to the UE - At Step 7 , MAC PDU is sent with cs_NR_HarqProcessAssignment(0) which makes SS to use harq Id = 0 and ndi = x ( toggled from step 4-5 ) . as UE has not decoded the PDCCH sent on steps 4-5 , UE will consider the MAC PDU received at step 7 with harq id = 0 as Retransmission not a new data |
| Summary of change | At steps 4-5 , harq id = 1 is used , so that Steps 1-2 will use harq id = 0 ndi = x Steps 4-5 will use harq id = 1 ndi = x Step 7 will use harq id = 0 ndi = !x further will use harq id = 1 , ndi = !x |
| TTCN module | MAC_TC_Common_NR |
| MCC160 Comment | Accepted as improvement |

Before Change:

function f_TC_7_1_1_2_1_NR_TestBody1(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { var RNTI_Value_Type v_C_RNTI; var integer j; var integer v_Nack_Count; var NR_MAC_SDU_Type v_EncodedRlcPdu1; var NR_MAC_SDU_Type v_EncodedRlcPdu2; var NR_MAC_SDU_Type v_EncodedRlcPdu3; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var SubFrameTiming_Type v_TimingDL; //@sic R5s210725 sic@ var B6_Type v_LCID := int2bit(p_NR_DRB_Id+3, 6); //@sic R5s211241 sic@ timer t_Watchdog := 0.6; //configure SS for no Auto Time alignement //@sic R5s200009 sic@ f_NR_ULGrantConfiguration_Common(nr_Cell1, cs_TimingInfo_Now, -, cs_NR_UplinkTimeAlignment_Stop); // Configure for indication of UL HARQ ACK/NACK f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(enable)); // @siclog "Steps 1 - 2" siclog@ // PDCP SDU =36B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_36B))); t_Watchdog.start; v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu1) };//@sic R5s211241 sic@ v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListDL)); // @siclog "Step 3" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, ack)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); t_Watchdog.stop; // store cell info parameters for C-RNTI v_C_RNTI := f_NR_CellInfo_GetRNTI(nr_Cell1); // @siclog "Steps 4-5" siclog@ // Configure SS with a different RNTI than allocated to UE f_NR_SS_C_RNTI_Config(nr_Cell1, tsc_RNTI_Value_Diff); // PDCP SDU =36B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, crs_NR_PDCP_SDU_36B))); t_Watchdog.start; v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu1) };//@sic R5s211241 sic@ v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListDL)); // @siclog "Step 6" siclog@ alt { [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, (ack, nack))) { t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 6: UE is sending HARQ ACK/NACK when sent addressed to different CRNTI"); } [] t_Watchdog.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); } }

After Change:

function f_TC_7_1_1_2_1_NR_TestBody1(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { var RNTI_Value_Type v_C_RNTI; var integer j; var integer v_Nack_Count; var NR_MAC_SDU_Type v_EncodedRlcPdu1; var NR_MAC_SDU_Type v_EncodedRlcPdu2; var NR_MAC_SDU_Type v_EncodedRlcPdu3; var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; var SubFrameTiming_Type v_TimingDL; //@sic R5s210725 sic@ var B6_Type v_LCID := int2bit(p_NR_DRB_Id+3, 6); //@sic R5s211241 sic@ timer t_Watchdog := 0.6; //configure SS for no Auto Time alignement //@sic R5s200009 sic@ f_NR_ULGrantConfiguration_Common(nr_Cell1, cs_TimingInfo_Now, -, cs_NR_UplinkTimeAlignment_Stop); // Configure for indication of UL HARQ ACK/NACK f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(enable)); // @siclog "Steps 1 - 2" siclog@ // PDCP SDU =36B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_36B))); t_Watchdog.start; v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu1) };//@sic R5s211241 sic@ v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListDL)); // @siclog "Step 3" siclog@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, ack)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); t_Watchdog.stop; // store cell info parameters for C-RNTI v_C_RNTI := f_NR_CellInfo_GetRNTI(nr_Cell1); // @siclog "Steps 4-5" siclog@ // Configure SS with a different RNTI than allocated to UE f_NR_SS_C_RNTI_Config(nr_Cell1, tsc_RNTI_Value_Diff); // PDCP SDU =36B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header =44B=352 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(1, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(1, crs_NR_PDCP_SDU_36B))); t_Watchdog.start; v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(v_LCID, v_EncodedRlcPdu1) };//@sic R5s211241 sic@ v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_MAC_PDUListDL, cs_NR_HarqProcessAssignment(1))); //WA#WI=981964 // @siclog "Step 6" siclog@ alt { [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, (ack, nack))) { t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 6: UE is sending HARQ ACK/NACK when sent addressed to different CRNTI"); } [] t_Watchdog.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); } }